3GPP TSG-RAN WG2 NR AH#4			    R2-1810657

Montreal, Canada, 2nd – 6th July, 2018

Agenda item:	   10.4.1.3.4 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	Indication for the establishment or release of SDAP entity

Document for:	Discussion and Decision

1.	Introduction

According to TS 36.331, LTE UE indicates the establishment or release of DRBs to upper layers. Regarding this issue, RAN2 haven’t yet discussed on whether such indication is required in NR.

5.3.10.2	DRB release

The UE shall:

1> for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration (DRB release); or

1> for each drb-identity value that is to be released as the result of full configuration option according to 5.3.5.8:

……

2> if the DRB was configured with pdcp-config and new DRB is not added with same eps-BearerIdentity in drb-ToAddModList nor radioBearerConfig1 nor in radioBearerConfig2:

3>	if the procedure was triggered due to handover:

4> indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers after successful handover;

3>	else:

4> indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers immediately.

5.3.10.3	DRB addition/ modification

The UE shall:

1> for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2> if the concerned entry of drb-ToAddModList includes the drb-TypeLWA set to TRUE (i.e. add LWA DRB):

3> perform the LWA specific DRB addition or reconfiguration as specified in 5.3.10.3a2;

……

2> else if the entry of drb-ToAddModList includes pdcp-config (establishment of bearer with E-UTRA PDCP):

3> indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

2.	Discussion

When a new DRB is added, LTE UE indicates the establishment of the DRB to upper layers. Also, the LTE UE indicates the release of the DRB to upper layers when a DRB is released.

However, should NR UE indicate the establishment or release of the DRB(s) to upper layers?

Our answer is “NO” because NR QoS model is based on QoS flows, not bearers. In other words, all QoS flows belonging to a PDU session are associated with a single SDAP entity, and the SDAP entity maps each QoS flow to the DRB decided by QoS flow to DRB mapping rules. Moreover, if there is no QoS flow which is mapped to a new DRB, any packets are not sent through the DRB. Even, when a DRB is released, all QoS flows mapped to the DRB can be transmitted via default DRB. Therefore, the NR UE does NOT need to indicate the establishment or release of the DRB(s) to upper layers.

Proposal 1: NR UE does NOT need to indicate the establishment or release of the DRB(s) to upper layers.

However, we think that the NR UE needs to indicate the establishment or release of the SDAP entity to upper layers. As mentioned above, because all QoS flows within a PDU session are associated with a single SDAP entity, the upper layers should know if SDAP entity is established for which PDU session.

Proposal 2: NR UE needs to indicate the establishment or release of the SDAP entity(s) to upper layers.

3.	Proposal

In this document, we present our view on indication for the establishment or release of DRB and SDAP entity. We have following proposals:

Proposal 1: NR UE does NOT need to indicate the establishment or release of the DRB(s) to upper layers.

Proposal 2: NR UE needs to indicate the establishment or release of the SDAP entity(s) to upper layers.