3GPP TSG-RAN WG2 NR AH1807 Meeting	R2-1810742

# Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | TBD | rev |  | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RLC Failure Information. |
| --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document is to capture RAN2#102 agreements on RLC max retransmissions for CA duplication. In user plane session during RAN2#102, the contents of report of reaching RLC max retransmission for CA duplication were agreed as follows: - When RLC max retransmissions are met for a logical channel restricted to one or multiple SCell(s), the information with the report identifies the problematic RLC entity without ambiguity. - The report on RLC max retransmissions does not include measurement results. - In the email discussion a majority of companies seemed to want a new RRC message for this indication. Details TBD in common session. - If the failure is restricted to the SCG, the information need to be known by the SN, and for other cases the MN. UP session assumes SRB1 and SRB3 would be used accordingly. - UP session confirm current behaviour that when RLC max retransmissions are met for a logical channel restricted to PCell or PSCell, RLF or SCG failure is triggered. - LCH ID + MCG/SCG Indication is reported According to agreements, the report shall include LCH ID and MCG/SCG indcation. Since there is no existing message for this purpose, a new RRC message should be defined. FFSs on max RLC retransmission for CA duplication are resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RLC failure and procedure are added. FFS on RLC failure for CA duplication are removed. logicalChannelIdentity and CellGroupId are included in RLC failure information. - Note: This change is applicable to SA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When RLC max retransmissions are met for a logical channel restricted to one or multiple SCell(s), problematic RLC entity cannot be identified to gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.3, 5.7.x, 6.2.1, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification 1>

5.3.10.3	Detection of radio link failure

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while T311 is not running; or

1>	upon indication that the maximum number of retransmissions has been reached for an MCG RLC bearer that logical channel association is not configured or not restricted to only SCell(s) for CA duplication:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

Editor’s Note: FFS Whether indications related to beam failure recovery may affect the declaration of RLF.

Editor’s Note: FFS: RLF related measurement reports e.g. VarRLF-Report is supported in NR.

2>	if AS security has not been activated:

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7.

1>	upon indication that the maximum number of retransmissions has been reached for  an MCG RLC bearer that logical channel association is restricted to only SCell(s) for CA duplication:

2>	initiate RLC failure information procedure as specified in 5.7.x to report RLC failure;

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication that the maximum number of retransmissions has been reached for an SCG RLC bearer that logical channel association is not configured or not restricted to only SCell(s) for CA duplication;

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

1>	upon indication that the maximum number of retransmissions has been reached for an SCG RLC bearer that logical channel association is restricted to only SCell(s) for CA duplication;

2>	initiate RLC failure information procedure as specified in 5.7.x to report RLC failure;

<End of modification 1>

<Start of modification 2>

5.7.x	RLC failure information

5.7.x.1	General

Figure 5.7.x.1-1: RLC failure information

The purpose of this procedure is to inform NR MN or SN about an RLC failure the UE has experienced i.e. maximum number of RLC retransmissions for CA duplication.

5.7.x.2	Initiation

A UE initiates the procedure to report RLC failures when transmission is not suspended and when the following condition is met:

1>	upon detecting the maximum number of RLC retransmissions for the RLC entity not associated with SpCell of a radio bearer configured with CA duplication, in accordance with subclause 5.3.10.3;

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the RLCFailureInformation message as specified in 5.7.x.3;

5.7.x.3	Actions related to transmission of RLCFailureInformation message

The UE shall set the contents of the RLCFailureInformation message as follows:

1>	if the UE initiates transmission of the RLCFailureInformation message to provide RLC failure information for MCG:

2>	set the cellGroupId to MCG;

2>	set the logicalChannelIdentity to include LogicalChannelIdentity whre the maximum number of retransmissions are reached;

1>	else if the UE initiates transmission of the RLCFailureInformation message to provide RLC failure information for SCG:

2>	set the cellGroupId to SCG;

2>	set the logicalChannelIdentity to include LogicalChannelIdentity whre the maximum number of retransmissions are reached;

The UE shall:

1>	if the UE initiates transmission of the RLCFailureInformation message to provide RLC failure information for MCG:

2>	submit the RLCFailureInformation message via SRB1 to lower layers for transmission;

1>	else if the UE initiates transmission of the RLCFailureInformation message to provide RLC failure information for SCG:

2>	if SRB3 is configured;

3>	submit the RLCFailureInformation message via SRB3 to lower layers for transmission;

2>	else;

3>	submit the RLCFailureInformation message via SRB1 to lower layers for transmission;

<End of modification 2>

<Start of modification 3>

6.2.1	General message structure

–	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::= SEQUENCE {

message											UL-DCCH-MessageType

}

UL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

measurementReport							MeasurementReport,

rrcReconfigurationComplete					RRCReconfigurationComplete,

rrcSetupComplete							RRCSetupComplete,

rrcReestablishmentComplete					RRCReestablishmentComplete,

rrcResumeComplete							RRCResumeComplete,

securityModeComplete						SecurityModeComplete,

securityModeFailure						SecurityModeFailure,

ulInformationTransfer						ULInformationTransfer,

locationMeasurementIndication				LocationMeasurementIndication,

rlcFailureInformation 						RLCFailureInformation,

spare6 NULL,

spare5 NULL, spare4 NULL, spare3 NULL,

spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

<End of modification 3>

<Start of modification 4>

### 6.2.2	Message definitions

#### –	RLCFailureInformation

The RLCFailureInformation message is used to provide information regarding failures detected by the UE for RLC entity in CA duplication.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RLCFailureInformation message

-- ASN1START

-- TAG-RLCFAILUREINFORMATION-START

RLCFailureInformation ::=	SEQUENCE {

criticalExtensions							CHOICE {

rlcFailureInformation					RLCFailureInformation-IEs,

criticalExtensionsFuture				SEQUENCE {}

}

}

RLCFailureInformation-IEs ::=					SEQUENCE {

cellGroupId									CellGroupId,

logicalChannelIdentity						LogicalChannelIdentity,

spare										BIT STRING (SIZE (ffsValue))

}

-- TAG-RLCFAILUREINFORMATION-STOP

-- ASN1STOP

<End of modification 4>