3GPP TSG-RAN WG2 #102AH	R2-1810855

Montreal, Canada, 2nd – 6th July, 2018

Title:	Draft reply LS to RAN4 on SFTD measurements when an NR PSCell is configured

Response to:	R2-1809446

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson [to be changed to RAN2]

To:	RAN4

Contact Person:

#### Name:	Helka-Liina Määttänen

E-mail Address:	helka-liina.maattanen@ericsson.com

1. Overall Description

RAN2 would like to thank RAN4 for the reply LS on SFTD measurements [1]. RAN2 has discussed the SFTD measurements use cases, and would like to provide the following example use cases for SFTD for establishing a time offset relation between LTE and NR cells:

- One use case is when the serving eNB can select the UEs to perform SFTD measurements among UEs not configured with an NR PSCell, at least for NR neighbor cells at non-monitored carriers.

- Another use case is for a UE already configured with NR PSCell is performing SFTD for establishing time offsets between a served LTE cell and non-connected NR cells to use as input when selecting an appropriate NR PSCell for NR PSCell change. This use case is probably typical only for NR neighbour cells at a monitored carrier.

Therefore, one important question is:

What is the feasibility of and requirements for SFTD measurements of an NR neighbor cell at monitored carrier, within configured SMTC window – available via existing measurement config, in the case when a UE is configured with an LTE PCell and an NR PSCell.

2. Actions:

ACTION: 	RAN2 respectfully asks RAN4 to consider the above question.

3. References:

[1] R2-1809446 (R4-1808442), Reply LS on SFTD measurements

4. Date of Next RAN2 Meetings:

RAN2#103, Gothenburg, 20-24 Aug 2018

RAN2#103bis, Chengdu, 8-12 Oct 2018