3GPP TSG-RAN2 Meeting #NR AH1807 	R2-1810861

# Montreal, Canada, 2 - 6 July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [D001/002] Inter-node signalling for feature set coordination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (If agreed, this draft CR is supposed to be merged into the rapporteur's CR) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-07-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Due to introducing the concept of feature sets, inter-node signalling for Baseband Processing Combination (BPC) was removed. Nevertheless, the feature set also requires coordination between MN and SN and so the similar inter-node signalling as defined for BPC is needed for the feature set as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In CG-Config and CG-ConfigInfo, a list supported combinations of feature sets is added for a given band combination. Impact analysis: Impacted functionality: EN-DC Inter-operability: Although the proposed change is non-backward compatible, no inter-operability issue is forseen as it is related to inter-nod signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If multiple combinations of FeatureSets are supported for one band combination, SN cannot know which combination of FeatureSets can be configured in the SN side based on the chosen FeatureSets by MN. Likewise, in the same case, if SN renegotiate the configured band combination, MS cannot know which combination of FeatureSets can be configured in the MN based on the chosen FeatureSets by SN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

11.2	Inter-node RRC messages

11.2.1	General

This section specifies RRC messages that are sent either across the X2-, Xn- or the NG-interface, either to or from the gNB, i.e. a single 'logical channel' is used for all RRC messages transferred across network nodes. The information could originate from or be destined for another RAT.

-- ASN1START

-- TAG_NR-INTER-NODE-DEFINITIONS-START

NR-InterNodeDefinitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ARFCN-ValueNR,

CellIdentity,

CSI-RS-Index,

GapConfig,

maxBandComb,

maxFeatureSetsPerBand,

maxNrofSCells,

maxNrofServingCells-1,

maxNrofIndexesToReport,

MeasQuantityResults,

MeasResultList2NR,

MeasResultSCG-Failure,

MeasResultCellListSFTD,

P-Max,

PhysCellId,

RadioBearerConfig,

RRCReconfiguration,

ServCellIndex,

SetupRelease,

SSB-Index,

SSB-MTC,

ShortMAC-I,

UE-CapabilityRAT-ContainerList

FROM NR-RRC-Definitions;

-- TAG_NR-INTER-NODE-DEFINITIONS-STOP

-- ASN1STOP

11.2.2	Message definitions

<< skip unchanged part >>

–	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB.

Direction: Secondary gNB to master gNB or eNB.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

cg-Config					CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

CG-Config-IEs ::=			SEQUENCE {

scg-CellGroupConfig					OCTET STRING (CONTAINING RRCReconfiguration)	OPTIONAL,

scg-RB-Config						OCTET STRING (CONTAINING RadioBearerConfig)		OPTIONAL,

configRestrictModReq				ConfigRestrictModReqSCG							OPTIONAL,

drx-InfoSCG							DRX-Info										OPTIONAL,

candidateCellInfoListSN				OCTET STRING (CONTAINING MeasResultList2NR)	OPTIONAL,

measConfigSN						MeasConfigSN									OPTIONAL,

selectedBandCombinationNR			BandCombinationIndex							OPTIONAL,

fr-InfoListSCG						FR-InfoList										OPTIONAL,

nonCriticalExtension				SEQUENCE {}										OPTIONAL

}

MeasConfigSN ::= SEQUENCE {

measuredFrequenciesSN				SEQUENCE (SIZE (1.. maxMeasFreqsSN))	OF NR-FreqInfo	OPTIONAL,

...

}

NR-FreqInfo ::= SEQUENCE {

measuredFrequency 					ARFCN-ValueNR									OPTIONAL,

...

}

ConfigRestrictModReqSCG ::=			SEQUENCE {

requestedBC-MRDC					RequestedBandCombinationInfo								OPTIONAL,

requestedP-MaxFR1					P-Max												OPTIONAL,

...

}

RequestedBandCombinationInfo ::=	SEQUENCE {

bandCombinationIndex				BandCombinationIndex,

requestedFeatureSets				FeatureSetEntryIndex

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

FR-InfoList ::=	SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF FR-Info

FR-Info ::=	SEQUENCE {

servCellIndex		ServCellIndex,

fr-Type				ENUMERATED {fr1, fr2}

}

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-Config field descriptions

candidateCellInfoListSN Contains information regarding cells that the source secondary node suggests the target secondary gNB to consider configuring.

fr-InfoListSCG Contains information of FR information of serving cells.

measuredFrequenciesSN Used by SN to indicate a list of frequencies measured by the UE.

requestedP-MaxFR1 IRequested value for the maximum power for FR1 (see TS 38.104 [12]) the UE can use in NR SCG.

requestedBC-MRDC Used to request configuring an NR band combination which is forbidden to use by MN. This field comprises a band combination numbered according to supportedBandCombination in the UE-MRDC-Capability and the feature set applied for each band in the requested band combination. The FeatureSetEntryIndex pinpoints the entry position within FeatureSetsPerBand.

scg-CellGroupConfig Contains the RRCReconfiguration message, used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB

scg-RB-Config Contains the IE RadioBearerConfig, used to establish or reconfigure the SCG configuration, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB

selectedBandCombinationNR Indicates the band combination selected by SN for the EN-DC.

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can used to request configuring an NR band combination whose use MN has previously forbidden.

–	CG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB to set the SCG configuration.It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, modify or release an MCG or SCG.

Direction: Master eNB or gNB to secondary gNB, alternatively CU to DU.

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=				SEQUENCE {

criticalExtensions				CHOICE {

c1								CHOICE{

cg-ConfigInfo				CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture		SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=		SEQUENCE {

ue-CapabilityInfo			OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)		OPTIONAL,-- Cond SN-Addition

candidateCellInfoListMN			MeasResultList2NR									OPTIONAL,

candidateCellInfoListSN			OCTET STRING (CONTAINING MeasResultList2NR)			OPTIONAL,

measResultCellListSFTD			MeasResultCellListSFTD									OPTIONAL,

scgFailureInfo					SEQUENCE {

failureType						ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG					OCTET STRING (CONTAINING MeasResultSCG-Failure)

}																						OPTIONAL,

configRestrictInfo			ConfigRestrictInfoSCG										OPTIONAL,

drx-InfoMCG					DRX-Info													OPTIONAL,

measConfigMN				MeasConfigMN												OPTIONAL,

sourceConfigSCG				OCTET STRING (CONTAINING RRCReconfiguration)				OPTIONAL,

scg-RB-Config             	OCTET STRING (CONTAINING RadioBearerConfig)        			OPTIONAL,

mcg-RB-Config				OCTET STRING (CONTAINING RadioBearerConfig)					OPTIONAL,

nonCriticalExtension		SEQUENCE {}													OPTIONAL

}

ConfigRestrictInfoSCG ::=		SEQUENCE {

allowedBC-ListMRDC				BandCombinationInfoList									OPTIONAL,

powerCoordination-FR1				SEQUENCE {

p-maxNR							P-Max												OPTIONAL,

p-maxEUTRA						P-Max												OPTIONAL

}																						OPTIONAL,

servCellIndexRangeSCG			SEQUENCE {

lowBound						ServCellIndex,

upBound							ServCellIndex

}																						OPTIONAL,   -- Cond SN-Addition

maxMeasFreqsSCG-NR					INTEGER(1..maxMeasFreqsMN)							OPTIONAL,

maxMeasIdentitiesSCG-NR				INTEGER(1..maxMeasIdentitiesMN)						OPTIONAL,

...

}

BandCombinationInfoList ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationInfo

BandCombinationInfo ::=		SEQUENCE {

bandCombinationIndex		BandCombinationIndex,

allowedFeatureSetsList		SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSetEntryIndex

}

FeatureSetEntryIndex ::=	INTEGER (1.. maxFeatureSetsPerBand)

DRX-Info ::=					SEQUENCE {

drx-LongCycleStartOffset		CHOICE {

ms10							INTEGER(0..9),

ms20							INTEGER(0..19),

ms32							INTEGER(0..31),

ms40							INTEGER(0..39),

ms60							INTEGER(0..59),

ms64							INTEGER(0..63),

ms70							INTEGER(0..69),

ms80							INTEGER(0..79),

ms128							INTEGER(0..127),

ms160							INTEGER(0..159),

ms256							INTEGER(0..255),

ms320							INTEGER(0..319),

ms512							INTEGER(0..511),

ms640							INTEGER(0..639),

ms1024							INTEGER(0..1023),

ms1280							INTEGER(0..1279),

ms2048							INTEGER(0..2047),

ms2560							INTEGER(0..2559),

ms5120							INTEGER(0..5119),

ms10240							INTEGER(0..10239)

},

shortDRX							SEQUENCE {

drx-ShortCycle						ENUMERATED	{

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer					INTEGER (1..16)

}																					OPTIONAL

}

MeasConfigMN ::= SEQUENCE {

measuredFrequenciesMN				SEQUENCE (SIZE (1..maxMeasFreqsMN))	OF NR-FreqInfo	OPTIONAL,

measGapConfig					SetupRelease { GapConfig }							OPTIONAL,

gapPurpose							ENUMERATED {perUE, perFR1}							OPTIONAL,

...

}

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

allowedBandCombinationListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select an NR band combination. Each entry refers to a band combination numbered according to supportedBandCombination in the UE-MRDC-Capability. The list of feature sets supported for each band combination entry is also included. All MR-DC band combinations indicated by this field comprise the same LTE band combination. The FeatureSetEntryIndex pinpoints the entry position within FeatureSetsPerBand.

candidateCellInfoListMN, candidateCellInfoListSN Contains information regarding cells that the master node or the source node suggests the target gNB to consider configuring. Including CSI-RS measurement results in candidateCellInfoListMN is not supported in this version of the specification.

maxMeasFreqsSCG-NR Indicates the maximum number of NR inter-frequency carriers the SN is allowed to configure with PSCell for measurements.

maxMeasIdentitiesSCG-NR Indicates the maximum number of allowed measurement identities that the SCG is allowed to configure.

measuredFrequenciesMN Used by MN to indicate a list of frequencies measured by the UE.

measGapConfig Indicates the measurement gap configuration configured by MN.

mcg-RB-Config Contains the IE RadioBearerConfig of the MN, used to support delta configuration for bearer type change between MN terminated to SN terminated bearer and SN change.

p-maxEUTRA Indicates the maximum power for EUTRA (see TS 36.104 [XX]) the UE can use in LTE MCG.

p-maxNR Indicates the maximum power for NR (see TS 38.104 [12]) the UE can use in NR SCG.

powerCoordination-FR1 Indicates the maximum power that the UE can use in FR1.

scg-RB-Config Contains the IE RadioBearerConfig of the SN, used to support delta configuration e.g. during SN change. This field is absent when master eNB uses full configuration option.

sourceConfigSCG Includes the current dedicated SCG configuration in the same format as the RRCReconfiguration message, i.e. not only CellGroupConfig but also e.g. measConfig. This field is absent when master eNB uses full configuration option.

ConfigRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

servCellIndexRangeSCG Range of serving cell indices that SN is allowed to configure for SCG serving cells.

| Conditional Presence | Explanation |
| --- | --- |
| SN-Addition | The field is mandatory present upon SN addition. |