3GPP TSG-RAN WG2 Meeting NR Adhoc 1807				R2-1810917

Montreal, Canada, 2nd - 6th July 2018

Title:	[DRAFT] LS on security requirements for RRC connection release

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Intel Corporation [TSG RAN WG2]

To:	TSG SA WG3

Cc:

Contact Person:

#### Name:	Sudeep K Palat

E-mail Address:	sudeep.k.palat@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In NR, RRC connection release message can be sent to the UE before or after security activation.  During an RRC Connection Release, the UE can be provided with the following:

1) Re-direction request to move the UE to another carrier in NR or to LTE.  It could be used for example, to redirect the UE during a emergency VoIP call setup, to another NR carrier or LTE.

2) Dedicated frequency reselection priorities for other LTE and NR carriers.  UE uses this information to prioritise the carrier frequencies in LTE or NR for reselection.

3) Deprioritisation of NR RAT or the current NR frequency for a maximum period of 30min.  This will make NR or the current NR frequency the lowest priority for reselection and hence the UE will reselect the next highest priority carrier available in LTE or NR.

RAN2 discussed whether each the above can be provided without security to the UE.   If they are to be provided with security, there will be some additional signalling required and delay as the NG-RAN has to first establish the connection to the Core Network and use security mode procedure before providing the RRC connection release message.

RAN2 would like to have feedback from SA3 if AS security is needed for each of the above configurations.

2. Actions:

To SA3 group.

ACTION:

Question: RAN2 respectfully asks SA3 which of the above configurations can be provided to the UE only after AS security activation.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2 #103	2018.08.20 – 2018.08.24		Gothenburg

3GPP TSG RAN WG2 #103-Bis	2018.10.08 – 2018.10.12		Chengdu