3GPP TSG-RAN WG2 Meeting NR Adhoc 1807		R2-1810947

Montreal, Canada, 2nd - 6th July 2018

Title:	[DRAFT] LS on additional TDD configuration periodicities

Response to:	R1-1807873

Release:	Release 15

Work Item:	NR_newRAT

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG1

Cc:	TSG RAN WG4

Contact Person:

#### Name:	Tero Henttonen

E-mail Address:	tero.henttonen @nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed the TDD configuration periodicities and noticed that the current TDD DL-UL patterns are not directly compatible with LTE TDD DL-UL configurations 1 & 2 since the periodicities used for those are not currently supported in RRC signallling.

From RAN2 viewpoint, this can be done by either using frame shifting or by adding 4ms and 3ms periodicities to the signalling to match the TD-LTE config 1 and 2 respectively by concatenating two periodicities matching the TD-LTE pattern as shown below:

| TD-LTE config 1: | D | S | U | U | D |
| --- | --- | --- | --- | --- | --- |
| NR 4 ms + 1 ms periodicities concatenated | D | S | U | U | D |
| TD-LTE config 2: | D | S | U | D | D |
| NR 3 ms + 2 ms periodicities concatenated | D | S | U | D | D |

Since using frameshifting may require additional considerations in the future if frame-alignment is required for some features (like is done e.g. in LTE for some features), RAN2 has considered adding the 3ms and 4ms periodicities as a possible alternative.

Since this may affect RAN1 specifications, RAN2 would request RAN1 to indicate whether they have concerns with adding these periodicities for the common TDD configuration.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to indicate whether they have concerns to support the 3ms and 4ms TDD DL-UL pattern periodicities in common TDD configuration.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2 #103	20 – 24 Aug 2018		Gothenburg

3GPP TSG RAN WG2#103bis	8 - 12 Oct 2018   	 	Chengdu

3GPPRAN2#104 		12 - 16 Nov 2018    		Spokane