3GPP TSG RAN WG2 NR ad hoc #1807	R2-1810976

Montreal, Canada, 2 – 6 July, 2018

Title:	LS on ssb-PositionsInBurst mismatch in SIB1 and ServingcellConfigCommon

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN1

Cc

Contact Person:

Name:	Yue Ma

E-mail Address:	mayue@vivo.com

1. Overall Description

RAN2 discussed about the configuration of ssb-PositionsInBurst in ServingcellConfigCommon. In handover and serving cell addition cases, now the ssb-PositionsInBurst in ServingcellConfigCommon has only the 64-bit full bit map format for FR2, and if it does not match the ssb-PositionsInBurst in target cell SIB1, it will result in SSB-RO association problem, which RAN2 think should be avoided.

To respect RAN1 agreement to not change 8+8 bit format in SIB1, while keeping RAN1 intention that achieving configuration flexibility for the actually transmitted SSB, RAN2 think there are basically 2 options to handle this misalignment:

- Option1: Modify the field description of ssb-PositionsInBurst IE in ServingCellConfigCommon to align with target cell RMSI. Specifically, keep the full bitmap in ServingcellConfigCommon but put a restriction in field description that only the same scheme as in SIB1 can be configured during HO and serving cell addition

- Option2: Add two 8-bit bitmaps as the ssb-PositionsInBurst2 IE in ServingCellConfigCommon, i.e., introduce additional inOneGroup and groupPresence in ServingCellConfigCommon  and explain that it is used for deriving the association of RACH occasion and SSB in handover and serving cell addition case in field description. The full bitmap ssb-PositionsInBurst is kept in ServingcellConfigCommon for rate matching purpose.

From RAN2 point of view, above two options both can be implemented in backward compatible manner.

2. Actions:

ACTION:

- RAN2 respectfully asks RAN1 the specific intention/functionalities of 8+8 bit format in SIB1 and the 64 full bit map format in ServingCellConfigCommon.

- RAN2 respectfully asks RAN1 to consider the above options, and feedback the preferred solution according to RAN1 functionality.

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #103	20th – 24th August 2018		Gothenburg, Sweden

TSG-RAN2 Meeting #103bis      8th – 12th October 2018		Chengdu, China