3GPP TSG-RAN WG3 #100	Tdoc R3-183214

Busan, Korea, 21st – 25th May

Agenda Item:	23.3

Source:	Ericsson

Title:	Bearer Context Modification procedure

Document for:	pCR TS 38.463

# Introduction

At the last RAN3 meeting, it was agreed an initial structure for the Bearer Context Modification Request / Response messages. The structure of the messages is captured in TS 38.463 v0.2.0. In this contribution, we propose a resolution for the remaining issues.

# Discussion

We propose the following updates for the Bearer Context Modification Request message. The definitions of several IEs, such as the security information, DL UE AMBR, etc. and the reason for introducing / updating these parameters is provided in R3-183213 in the description of the Bearer Context Setup procedure.

- Security Information: The IE is introduced to allow the CU-CP to update security parameters, e.g., change the security algorithm for a UE.

- DL UE AMBR: The IE is introduced to allow the CU-CP to update the UE AMBR that is allocated to this CU-UP (which may be a portion of the overall UE AMBR in case that the UE is served by multiple CU-UPs at the same time)

- Bearer Context Status Change: The reason for this IE is provided in R3-183206.

- PDCP Count Request: The IE is introduced to allow the CU-CP to request the CU-UP to provide the current PDCP TX / RX Status. It is needed for example in case of handover, so that the source CU-CP can generate the SN Status Transfer message and send it to the target CU-UP.

- Flow Mapping Information is added to the list of DRBs to modify. This allows the CU-CP to modify the QoS-Flows to DRB mapping.

Proposal 1: 	RAN3 is kindly asked to agree with the updated structure for the Bearer Context Modification Request message.

In this contribution, we also propose a structure for the Bearer Context Modification Response message. The proposed structure includes several nested lists to report the resources that were successfully established/modified and the resources that failed to establish/modify:

- For E-UTRAN:

  - A list of DRBs successfully established;

  - A list of DRBs which failed to establish;

  - A list of DRBs successfully modified;

  - A list of DRBs which failed to modify;

- For NG-RAN:

  - A list of PDU Session Resource successfully established;

    - A list of DRBs successfully established;

      - A list of QoS-Flows successfully established;

      - A list of QoS-Flows which failed to establish;

    - A list of DRBs which failed to establish;

  - A list of PDU Session Resource which failed to establish;

  - A list of PDU Session Resource successfully modified;

    - A list of DRBs successfully established;

      - A list of QoS-Flows successfully established;

      - A list of QoS-Flows which failed to establish;

    - A list of DRBs which failed to establish;

    - A list of DRBs successfully modified;

      - A list of QoS-Flows successfully established;

      - A list of QoS-Flows which failed to establish;

    - A list of DRBs which failed to be modified;

  - A list of PDU Session Resource which failed to be modified;

The IEs in the Bearer Context Setup Response message are already defined and explained for the other messages. There are no new IEs defined as part of this messages.

Proposal 2: 	RAN3 is kindly asked to agree with the proposed structure for the Bearer Context Modification Response message.

# Conclusion

In this contribution, we discussed the E1 Bearer Context Modification procedure.

Proposal 1: 	RAN3 is kindly asked to agree with the updated structure for the Bearer Context Modification Request message.

Proposal 2: 	RAN3 is kindly asked to agree with the proposed structure for the Bearer Context Modification Response message.

# Annex I: TP for 38.463

<<<<<< START CHANGE >>>>>>

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT MODIFICATION RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	A list of DRBs which are successfully established shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed To Modify List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Modified List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed To Modify List IE;

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	A list of DRBs which are successfully established shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed To Modify List IE;

-	A list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	A list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Security Information IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Aggregate Maximum Bit Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider that the UE is changing RRC state and act as specified in TS 38.401 [2].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the E-UTRAN QoS IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the PDCP Count Request IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the PDCP UL Count IE and the PDCP DL Count IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP UL Count IE and the PDCP DL Count IE are contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the DL UP Parameters IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Cell Group To Add IE or the Cell Group To Modify IE or the Cell Group To Remove IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall add or modify or remove the corresponding cell group.

If the PDU Session Type IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the S-NSSAI IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Security Indication IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the PDU Session Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the SDAP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Flow Mapping Information IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

#### 8.3.2.3	Unsuccessful Operation

Figure 8.3.2.3-1: Bearer Context Modification procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot modify the requested bearer context, it should respond with a BEARER CONTEXT MODIFICATION FAILURE message and appropriate cause value.

#### 8.3.2.4	Abnormal Conditions

Not applicable.

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Security Information | O |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.xx5 |  | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates to Suspend or Resume the Bearer Context | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>PDCP Configuration | M |  |  | 9.3.1.xx23 | - | - |
| >>>>E-UTRAN QoS | M |  | 9.3.1.xx2 |  | - | - |
| >>>>S1 UL UP Trasnport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Request | O |  | 9.3.2.6 |  | - | - |
| >>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry | - | - |
| >>DRB To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>PDCP Configuration | O |  |  | 9.3.1..xx23 | - | - |
| >>>>E-UTRAN QoS | O |  | 9.3.1.xx2 |  | - | - |
| >>>>S1 UL UP Parameters | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Request | O |  | 9.3.2.6 |  | - | - |
| >>>>PDCP Count Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP Count in the response message. | - | - |
| >>>>PDCP UL Count | O |  | PDCP Count 9.3.1.xx20 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>PDCP DL Count | O |  | PDCP Count 9.3.1.xx20 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>DL UP Parameters | O |  | 9.3.1.13 |  | - | - |
| >>>>Cell Group To Add | O |  | 9.3.1.12 |  | - | - |
| >>>>Cell Group To Modify | O |  | 9.3.1.12 |  | - | - |
| >>>>Cell Group To Remove | O |  | 9.3.1.12 |  | - | - |
| >>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>DRB ID | M |  |  |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>PDU Session ID | M |  | 9.3.1.xx6 |  | - | - |
| >>>>PDU Session Type | M |  | 9.3.1.xx6b |  | - | - |
| >>>>S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >>>>Security Indication | M |  | 9.3.1.xx7 |  | - | - |
| >>>>PDU Session Resource Aggregate Maximum Bit Rate | M |  | 9.3.1.xx5 |  | - | - |
| >>>>NG UP UL Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 | Needed if there is at least one DRB that uses configuration with PDCP. | - | - |
| >>>>PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.6 |  | - | - |
| >>>>DRB To Setup List |  | 1 |  |  |  |  |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>>>SDAP Configuration | M |  |  | 9.3.1.xx24 | - | - |
| >>>>>>PDCP Configuration | M |  |  | 9.3.1.xx23 | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>>>>Flow Mapping Information | M |  | QoS Flow QoS Parameters List 9.3.1.xx9 |  | - | - |
| >>>>>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.6 |  | - | - |
| >>PDU Session Resource To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>PDU Session ID | M |  | 9.3.1.xx6 |  | - | - |
| >>>>PDU Session Type | O |  | 9.3.1.xx6b |  | - | - |
| >>>>S-NSSAI | O |  | 9.3.1.9 |  | - | - |
| >>>>Security Indication | O |  | 9.3.1.xx7 |  | - | - |
| >>>>PDU Session Resource Aggregate Maximum Bit Rate | O |  | 9.3.1.xx5 |  | - | - |
| >>>>NG UP UL Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 | Needed if there is at least one DRB that uses configuration with PDCP. | - | - |
| >>>>PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.6 |  | - | - |
| >>>>DRB To Setup List |  | 1 |  |  |  |  |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>>>>DRB ID | M |  | 9.3.xx1 |  | - | - |
| >>>>>>SDAP Configuration | M |  |  | 9.3.1.xx24 | - | - |
| >>>>>>PDCP Configuration | M |  |  | 9.3.1.xx23 | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>>>>Flow Mapping Information | M |  | 9.3.1.12 |  | - | - |
| >>>>>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.6 | Overrides any existing information. | - | - |
| >>>>DRB To Modify List |  | 0.. 1 |  |  |  |  |
| >>>>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>>>>>SDAP Configuration | O |  | OCTET STRING | SDAP-config as defined in TS 38.331 [10]. | - | - |
| >>>>>>PDCP Configuration | O |  | OCTET STRING | PDCP-config as defined in TS 38.331 [10]. | - | - |
| >>>>>>DRB Data forwarding information Request | O |  |  | Data Forwarding Information Request 9.3.2.6 | - | - |
| >>>>>>PDCP Count Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP Count in the response message. | - | - |
| >>>>>>PDCP UL Count | O |  | PDCP Count 9.3.1.xx20 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>>>PDCP DL Count | O |  | PDCP Count 9.3.1.xx20 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>>>DL UP Parameters | O |  | 9.3.1.13 |  | - | - |
| >>>>>>Cell Group To Add | O |  | 9.3.1.11 |  | - | - |
| >>>>>>Cell Group To Modify | O |  | 9.3.1.11 |  | - | - |
| >>>>>>Cell Group To Remove | O |  | 9.3.1.11 |  | - | - |
| >>>>>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.xx9 | Overrides previous mapping information. | - | - |
| >>>>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | EACH | reject |
| >>>>>>DRB ID | M |  |  |  | - | - |
| >>PDU Session Resource To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Remove Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | reject |
| >>>>PDU Session ID | M |  |  |  | - | - |

#### 9.2.2.5	BEARER CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | M |  |  |  | YES | ignore |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Setup List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>DRB ID | M |  | 9.3.1.1 |  | - | - |
| >>>>S1 DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Response | O |  | Data Forwarding Information Response 9.3.2.7 |  | - | - |
| >>>>UL UP Transport Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>Cause | M |  | 9.3.1.xx2 |  | - | - |
| >>DRB Modified List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>PDCP UL Count | O |  | PDCP Count 9.3.1.xx20 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>PDCP DL Count | O |  | PDCP Count 9.3.1.xx20 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>UL UP Transport Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters for newly added cell groups. | - | - |
| >>DRB Failed To Modify List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>Cause | M |  | 9.3.1.xx2 |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Setup List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | ignore |
| >>>>PDU Session ID | M |  | 9.3.1.xx6 |  | - | - |
| >>>>NG-U DL UP Tansport layer Information | O |  | UP Trasport Layer Information 9.3.2.1 | Needed if there is at least one DRB that uses configuration with PDCP. |  |  |
| >>>>PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information Respone 9.3.2.7 |  | - | - |
| >>>>DRB Setup List |  | 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3..1.xx1 |  | - | - |
| >>>>>>DRB Data forwarding information Response | O |  | Data Forwarding Information Respone 9.3.2.7 |  | - | - |
| >>>>>>UL UP Parameters | M |  | 9.3.1.13 |  | - | - |
| >>>>>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.xx2 |  | - | - |
| >>PDU Session Resource Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Failed Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | ignore |
| >>>>PDU Session ID | M |  | 9.3.1.xx6 |  | - | - |
| >>>>Cause | M |  | 9.3.1.xx2 |  | - | - |
| >>PDU Session Resource Modified List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Modified Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | ignore |
| >>>>PDU Session ID | M |  | 9.3.1..xx6 |  | - | - |
| >>>>DRB Setup List |  | 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>>>DRB Data forwarding information Response | O |  | Data Forwarding Information Respone 9.3.2.7 |  | - | - |
| >>>>>>UL UP Parameters | M |  | 9.3.1.13 |  | - | - |
| >>>>>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |  |  |
| >>>>>>Flow Failed List | M |  | QoS Flow List 9.3.1.12 |  |  |  |
| >>>>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.xx2 |  | - | - |
| >>>>DRB Modified List |  | 0.. 1 |  |  | YES | ignore |
| >>>>>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>>>PDCP UL Count | O |  | PDCP Count 9.3.1.xx20 | PDCP count for first un-acknowledged UL packet. | - | - |
| >>>>>>PDCP DL Count | O |  | PDCP Count 9.3.1.xx20 | PDCP count for next DL packet to be assigned. | - | - |
| >>>>>>UL UP Transport Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters for newly added cell groups. | - | - |
| >>>>>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>DRB Failed To Modify List |  | 0.. 1 |  |  | YES | ignore |
| >>>>>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  | EACH | ignore |
| >>>>>>DRB ID | M |  | 9.3.1.xx1 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.xx2 |  | - | - |
| >>PDU Session Resource Failed To Modify List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Failed To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | EACH | ignore |
| >>>>PDU Session ID | M |  | 9.3.1.xx6 |  | - | - |
| >>>>Cause | M |  | 9.3.1.xx2 |  | - | - |

#### 9.2.2.6	BEARER CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modifcation of the bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<< NEXT CHANGE >>>>>>

#### 9.3.1.xx20		PDCP Count

This IE include the PDCP Count information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| >PDCP SN | M |  | INTEGER (0 .. ..2PDCP_SN_Size-1) | The PDCP SN Size is provided in the PDCP Configuration IE. |
| >HFN | M |  | INTEGER (0 .. 232-PDCP_SN_Size-1) | The PDCP SN Size is provided in the PDCP Configuration IE. |