3GPP TSG-RAN WG3#101                                                                            R3-184763

Goteborg, Sweden, 20-24 August 2018

Agenda Item:	10.8.4.1

Source:	CATT

Title:	Introduction of Xn-U TEID in S-Node Modification Required message

Document for:	Discussion, Decision

1. Introduction

In EN-DC,it is agreed that SgNB may inform MeNB of the changed DL tunnel ID for X2-U.In this contribution,we propose to support this function for option 7.

2. Discussion

In EN-DC,to support SN initiated inter-SgNB-DU mobility, SgNB DL GTP TEID at SCG is introduced in SgNB Modification Required message which indicates the updated DL TEID for X2-U. So, similarly, for  option 7,the same information should be provided from SN to MN during SN initiated inter-SgNB-DU mobility procedure. Based on that, we have the following proposal:

Proposal 1: It is proposed to introduce Xn-U DL UP TNL Information IE in S-NODE MODIFICATION REQUIRED message.

3. Conclusion

The following proposal is made:

Proposal 1: It is proposed to introduce Xn-U DL UP TNL Information IE in S-NODE MODIFICATION REQUIRED message.

4. TP for 38.423

<<<<<<<<<<<<<<<<<<<<Start of the first change >>>>>>>>>>>>>>>>>>>>

9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| SCG Change Indication | O |  | ENUMRATED(true…) | This IE may need to be refined | YES | ignore |
| PDU Session Resources To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>NG-U DL UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| >> Xn-U DL UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the Xn transport bearer. For delivery of DL PDCP PDUs. |  |  |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>Cause | M |  | 9.2.3.2 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

<<<<<<<<<<<<<<<<<<<<End of the change >>>>>>>>>>>>>>>>>>>>