3GPP TSG RAN WG3 Meeting #102 	R3-186356

Spokane/USA, 12-16 November 2018

Title:	NTN multi connectivity

Source: 	Thales

Type:	pCR

Document for:	Agreement

Agenda Item:	20.2

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document aims at clarifying the various multi connectivity scenarios involving Non-Terrestrial networks.

It directly proposes some modifications to the clause 5.4.

# Discussion

Multi connectivity is a useful scheme to improve the service performance in terms of service rate, service availability and/or reliability.

This is of particular interest in low density populated areas where the 5G service deployment is a challenge.

A number of service scenarios as described in TS 22.261 (e.g. user in building, in vehicle, in high speed train or on board airplane), would benefit from the combination of several access technologies including satellite to meet the targeted service performances.

Given that several access technologies may be deployed in such areas including terrestrial based and satellite access, it is recommended to extend dual to multi connectivity to be able to combine as many access technologies as are available to the UE.

Proposal 1: Extend Dual connectivity to Multi connectivity as per [5] 3GPP TS 37.340; NR; Multi-connectivity; Overall description.

When considering regenerative and transparent satellite scenarios, some constraints may trigger the implementation choice (NG-RAN or 5GCN level) for the multi connectivity given specific requirements associated to Xn-AP protocols which may not be compatible with some satellite feeder links.

Proposal 2: Implementation of multi connectivity may differ when involving regenerative or transparent NTN-based NG-RAN

In rural areas, there could be available one or several NTN access based on NR and only one terrestrial access based on EUTRA which could be combined.

Proposal 3: Multi connectivity should be possible between NTN-based NG-RAN and terrestrial-based EUTRA.

Proposal 4: Multi connectivity between at least two NTN-based NG-RAN should be possible

# Conclusion

Proposal 1: Extend Dual connectivity to Multi connectivity as per [5] 3GPP TS 37.340; NR; Multi-connectivity; Overall description.

Proposal 2: Implementation of multi connectivity may differ when involving regenerative or transparent NTN-based NG-RAN

Proposal 3: Multi connectivity should be possible between NTN-based NG-RAN and terrestrial-based EUTRA.

Proposal 4: Multi connectivity between at least two NTN-based NG-RAN should be possible

Proposal 5: Reflect the above proposals in the TR 38.821 (See TP below)

# Text Proposal for TR 38.821 v0.2.0

START OF CHANGES

## 5.4	Multi connectivity involving NTN-based NG-RAN (FFS)

### 5.4.1	Overview

This clause discusses multi connectivity [5], either for transparent or regenerative NTN-based NG-RAN, and in combination or not with terrestrial-based NG-RAN or terrestrial-based EUTRA.

This may apply to transparent satellites as well as regenerative satellites with gNB or gNB-DU function on board.

A UE may be connected and served simultaneously by at least:

- One NTN-based NG-RAN and one terrestrial-based access (NR or EUTRA)

- One NTN-based NG-RAN and another NTN-based NG-RAN

As for terrestrial access, connectivity combining can occur for either the uplink or the downlink or both.

### 5.4.2	Regenerative NTN-based NG-RAN

In case of multi Connectivity involving regenerative NTN-based NG-RAN with on board gNB, setting up and maintaining Xn interfaces toward terrestrial gNBs over the feeder link would require all the corresponding traffic (CP and UP) to be transported over the SRI relevant to the satellite-hosted gNB. This may be a challenge.

It should be verified whether it is feasible to transport Xn over the SRI of the feeder link, taking also into consideration potential impacts of on board gNBs mobility.

Figure 5.4.2-1: Multi connectivity with regenerative NTN-based NG-RAN and terrestrial based NG-RAN (or EUTRA)

Figure 5.4.2-2: Multi connectivity between two regenerative NTN-based NG-RAN

Figure 5.4.2-3: Multi connectivity between two regenerative NTN-based NG-RAN and a terrestrial based access (NG-RAN or EUTRA)

In such cases, 5GCN and UE could support multi connectivity and manage the QoS flow mapping on the different access links. This is out of RAN scope (e.g. 3GPP TR 23.793, Study on Access Traffic Steering, Switching and Splitting support in the 5G system architecture, Release 16).

### 5.4.3	Transparent NTN-based NG-RAN

This case is illustrated in the figure below.

Figure 5.4.3-1: Multi connectivity with transparent NTN-based NG-RAN and terrestrial based NG-RAN (or EUTRA)

Figure 5.4.3-2: Multi connectivity between two transparent NTN-based NG-RAN

Figure 5.4.3-3: Multi connectivity between two transparent NTN-based NG-RAN and a terrestrial access (NG-RAN or EUTRA)

In this context, the multi connectivity procedure could be based on the one defined in [5].

The same gNB could serve NR cells via the terrestrial access network and via the satellite access network (e.g. with transparent payload on board the satellite).

### 5.4.4	NG-RAN impacts

Editor’s note: The TS 37.340 should cover Multi connectivity scenario combining several NG-RAN (NTN based or Terrestrial based). In particular the figures 4.2.2.1 and 4.2.2-2 of TS 37.340 should be adapted to illustrate multi connectivity with 2 NR bearers.

Editor’s note: in case of multi connectivity between Transparent NTN based NG-RAN and terrestrial based NG-RAN, the gNB serving UEs with a given set of NTN beams may be elected as Master node while the terrestrial based gNBs serving the same UEs may be elected as secondary nodes. This may reduce the complexity associated to mobility management.

END OF CHANGES