3GPP TSG-RAN WG3 #102                                              	R3-186441

Spokane, USA, 12 – 16 November 2018

# Agenda Item: 24.2

# Source: OMESH Networks

# Title: TP for Soft Topology Adaptation

# Document for: Approval

1.  Introduction

The following definitions and proposals have been made in R3-185402, and R3-185404 in the previous meeting.

Definition 1: A hard topology adaption sets up new links to recover a partially disconnected IAB network.

Definition 2: A soft topology adaption sets up new links to improve topological redundancy of a connected IAB network.

Definition 3: The cost of delivery between any two IAB nodes indicates a long-term average cost of sending a packet between the two IAB nodes under current IAB network topology.

Proposal 1: For maintaining IAB topology redundancy, every IAB nodes shall have at least two parents if possible, in order to reduce the risk of hard topology adaption.

Proposal 2: Current link status of next-hop relays can be a condition in the routing decision under multi-connectivity.

Proposal 3: Current buffer load of next-hop relays can be a condition in the routing decision under multi-connectivity.

Proposal 4: Soft topology adaption shall be performed to maintain topological redundancy, when 1) a new IAB node is joining the network; 2) an IAB node is leaving the network; 3) a BH RLF occurs over certain period of time and dual connectivity is affected at any IAB node.

Proposal 5: Both hard and soft topology adaptions can be performed centralized by the donor.

Proposal 6: Distributed routing shall be supported under multi-connectivity, where the next-hop relay can be selected with the least cost of delivery to the destination, under satisfied conditions of link quality and buffer load thresholds.

Proposal 7: Between any two IAB nodes, a cost of delivery shall be provided based on the current topology, which can be obtained from either a routing table or node ID calculation. The cost of delivery of IAB networks can be updated by the donor under topology adaptions.

Email discussion were sent in RAN3 reflector to collect feedbacks from companies.

This document provides a TP on soft topology adaptation for TR38.874 v0.5.0.

- TP for TR 38.874 v0.5.0

# [Start of PCR]

## 9.7	Topology adaptation

### 9.7.1	Goals and Principals of IAB topology adaptation

Topology adaptation has the goal to change the IAB-topology in order to ensure that the IAB node can continue to operate (incl. providing coverage and end user service continuity) even if the current active backhaul path is degraded or lost. Topology adaptation includes the following tasks:

- Information collection

  - Information includes, e.g., backhaul link quality, link- and node-load, neighbor-node signal strength.

  - Collection applies to sufficiently large area of the IAB topology.

- Topology determination

  - Deciding best topology based on the collected info and following a performance objective.

- Topology reconfiguration

  - Adjusting topology based on topology determination, through e.g. establishing new connections, releasing other connections, changing routes, etc.

The following discussion mainly focuses on topology reconfiguration. In this discussion, it is assumed that existing Rel-15 procedures for measurements, handover, dual-connectivity and F1-interface management are baseline for topology reconfiguration in architecture 1. Furthermore, Rel-16 related procedures should be considered when these procedures are available.

Two categories of topology adaptions can be defined depending on how the topology reconfiguration process is triggered:

- Definition 1: A hard topology adaptation sets up new links to recover a partially disconnected IAB network.

- Definition 2: A soft topology adaptation sets up new links to improve topological redundancy of a connected IAB network.

Both hard and soft topology adaptations may be performed centralized by the donor. It is also desirable to minimize service disruption and packet loss during topology adaptation. It is understood service disruption and packet loss may be inevitable in the process of hard topology adaptation; and no service interruption may be introduced in the process of soft topology adaptation. Therefore, hard topology adaptation should be avoided as much as possible in the IAB operation.

IAB Topology adaptation can be triggered by integration of a IAB node to the topology, detachment of an IAB node from the topology, detection of backhaul link over load, deterioration of backhaul link quality or link failure, or other events.

### 9.7.2	Goals and Principals of Topological Redundancy

Topological redundancy has the goal to enable robust operation, e.g., in case of backhaul link blockage, and to balance load across backhaul links. Establishment and management of topological redundancy is performed by soft topology adaptation.

Mechanisms for topology adaptation for IAB should be designed to ensure support of various multi-connectivity options of the IAB architecture design as described in section 9.2. Under a single IAB-donor-DU the following two cases for multi-connectivity in an IAB deployment should be considered. Note that multi-connectivity solutions could also be considered for the case where the redundant routes are to different IAB-donor-DUs.

- Case 1: UE is multi-connected to the IAB-donor via redundant routes (traditional DC scenario).

- Case 2: IAB-node is multi-connected to the IAB-donor node via redundant routes.

Multi-connectivity of IAB-node (Case 2 above) can be supported by:

- using a single MT function in the IAB node,

- using several independent MT functions in the IAB node, where each MT function makes an independent connection to the network (using normal MT setup).

Regardless of whether a single MT or multiple MT connections are used, the IAB-node will be configured with multiple backhaul RLC-channels associated with the different paths.

Multi-connectivity solutions designed for IAB should take into account the protocol stacks available at the IAB-node and IAB-donor depending on the architecture. This means that, for example, in some alternatives for architecture group 1a, a multi-path framework needs to be designed potentially based on the adaptation layer.

For maintaining IAB topology redundancy in Case 2, every IAB nodes shall have at least two parents if possible, in order to reduce the risk of introducing hard topology adaptation. When a backhaul-link-failure or a parent overloading occurs over certain period of time, a soft topology adaptation shall be triggered to recover such dual multi-connectivity, even though the IAB node can remain connected via its other parent.

Figure 9.7.2-1 Example of topological redundancy for backhaul-link-failure

Figure 9.7.2-1 shows a typical scenario where multiple IAB nodes are deployed, e.g., on street lights, every IAB node has at least two parents, so that a single backhaul link failure won’t introduce any hard topology adaptation. For example, if link A is blocked, DU9 can route through DU7 instead of DU8, via link B.

Figure 9.7.2-2: Example of more topological redundancy for backhaul-link-failure

Figure 9.7.2-2 further shows a scenario where more redundancy is provided, where DU9 has three parents. In this case, if any two links, e.g., A and B are both blocked, DU9 can route through DU6 instead of DU8 and DU7, via link C.

Figure 9.7.2-3: Example of topological redundancy for load balancing

In Figure 9.7.2-3, DU4 is overloaded, i.e., with an overflow of UL buffer, DU10 shall instantly route packets to DU3, and prevent sending any new packets to DU4 until the congestion is relieved. Similar mechanisms can be applied to DL as well.

All above scenarios requires DAG topology, and routing decision at any IAB node will be depending on the condition of current link status and potential overloading. As backhaul link failure and node overloading can occur frequently and swiftly in IAB, only the relays with satisfied link quality and buffer load will be used.

Figure 9.7.2-4: Scenario of an IAB node failure and soft topology adaption

Figure 9.7.2-4 shows a typical scenario where an IAB node DU4 is subject to permanent failure and leaves the network. Under the original topology, DU3 and DU10 only have one parent, which introduces a risk of hard topology adaptation if a further blockage of link A or link B occurs. In this case, a soft topology adaptation is performed by the CU to set up new links C and D which can recover the dual-connectivity of DU3 and DU10.

More details about how a multi-connectivity architecture design operating at the IAB node level would perform topology adaptation are still FFS. Such details will be addressed after details of the Adapt layer and RLC channel configuration have been defined.

…

### 9.7.10	Backhaul-link-failure recovery scenarios

Due to various reasons, different scenarios of backhaul-link failure may happen in IAB networks. In the following, some example scenarios are illustrated for backhaul-link failure. Each scenario is depicted with an illustrative figure (Figures 9.7.10-1 to 9.7.10-3) aiming at establishing a route between IAB-donor and IAB-node D after BH-link failure, where:

- Nodes A1 and A2 are IAB-donor nodes; nodes B to H are IAB nodes;

- The blue dashed line represents the established connection between two nodes;

- The red arrow represents the established route after BH-link failure, and the red dashed line represents the new established connection.

Figure 9.7.10-1: Example of backhaul-link failure scenario 1

Scenario 1

In this scenario (depicted in Figure 9.7.10-1), the backhaul-link failure occurs between on upstream IAB-node (e.g., IAB-node C) and one of its parent IAB-nodes (e.g. IAB-node B), where the upstream IAB-node (IAB-node C) has an additional link established to another parent node (IAB-node E). Because of the existing multi-connectivity on IAB-node C, no hard topology adaptation will be triggered, but soft topology adaptation may be applied to maintain topological redundancy.

Figure 9.7.10-2: Example backhaul-link failure scenario 2

Scenario 2

In this scenario (depicted in Figure 9.7.10-2), the backhaul-link failure occurs between an upstream IAB-node (e.g. IAB-node C) and all its parent IAB-nodes (e.g. IAB-nodes B and E). The upstream IAB-node (IAB-node C) has to reconnect to a new parent node (e.g. IAB-node F), and the connection between IAB-node F and IAB-node C is newly established). Since the part of the network is disconnected in the process (IAB-node C and D), this new link establishment requires hard topology adaptation. It may introduce service interruption and packet loss since any UE traffics on C and D will be blocked or dropped, and have to wait until a new route via node F is established.

Figure 9.7.10-3: Example backhaul-link failure scenario 3

Scenario 3

In this scenario (depicted in Figure 9.7.10-3), the backhaul-link failure occurs between IAB-node C and IAB-node D. IAB-node D has to reconnect to the new IAB-donor (e.g. IAB-donor A2) via a new route. Since the part of the network is disconnected in the process (IAB-node D), this new link establishment requires hard topology adaptation. It may introduce service interruption and packet loss, since any UE traffics on D will be blocked or dropped, and have to wait until the new route to A2 is established.

9.X Backhaul Routing with Topological Redundancy

Since backhaul-link-failure (BH RLF) and node congestions can occur very frequently and swiftly in IAB networks, distributed and cognitive routing shall be supported under multi-connectivity (topological redundancy) in order to have a fast response to the RLFs and overloading of IAB nodes. In essence, backhaul cognitive routing shall deal with short-term changes in IAB networks, whereas topology adaptation only deals with long-term changes.

Figure 9.X-1: A typical scenario of distributed routing decision

Figure 9.X-1 shows a typical scenario of distribute routing decision under topological redundancy. For example, DU9 has three potential next relays in UL - parents, DU8, DU6, and DU7. If all the three relays have satisfied link quality and buffer load, DU6 shall be selected as the next hop relay since it is having closest distance to the destination CU. However, if link C is temporarily blocked or DU7 is overloaded, DU8 shall be selected instead.

A metric of cost of delivery, between any two IAB nodes, shall be defined, which indicates the average (long-term) cost of sending a packet between the two IAB nodes under the current topology. The next-hop relay shall be selected at every IAB node with the least cost of delivery to the destination node, under the current (cognitive) conditions of satisfied link quality and buffer load. In Figure 9.x-1, the cost of delivery can either be the average number of hops between the two IAB nodes, or the actual distance between the two IAB nodes.

Destination ID based routing can be applied to such distributed routing under topological redundancy. The cost of delivery can be either calculated by the CU in a routing table, or it can be calculated at every IAB nodes by particular node ID assignment (e.g., at adaption layer) from the CU. If the cost of delivery is obtained by a routing table, the CU shall send updated routing table to every IAB nodes in topology adaptations; if it is obtained by node ID calculation, the CU shall update related IAB node IDs in topology adaptations.

Since consecutive UE packets may take different or opportunistic UL and DL routes in reaching the destination, i.e. CUs and UE, mechanisms for dealing with out-of-order packets shall be supported.

More details about distributed routing under topological redundancy at the IAB node level are still FFS. Such details will be addressed after details of the Adapt layer and RLC channel configuration have been defined.

[End of PCR]