3GPP TSG-RAN WG3 #102	  R3-186462

Spokane, USA, 12-16 November 2018

Source: Qualcomm Incorporated

Title: Dynamic Cell Group Configuration for Remote Interference Management

Document for: Approval

Agenda Item: 22.2

# Introduction

RAN3 #101bis meeting reached following agreements:

- gNBs can be grouped as Cluster.

- RAN3 study the design of IDs encoded into the RS and the mechanism to support the gNB grouping, which are common for all RIM solutions.

- The gNB cluster is configured by OAM.

And identified following issues:

- How to define the group ID?

- How the aggressor identifies the victim from the obtained group ID?

- How to support the inter-gNB coordination via the backhaul, especially considering that the two gNBs can be 300km far from each other?

- Co-ordination intra-cluster?

- Whether the member gNBs in the set can be changed or not?

- Whether the Set ID is static or dynamic?

This paper analyses how to configure the cell group (aka cluster).

# Cell group

There are three kinds of groups:

- Group for RI (remote interference) detection

- Group for RIM-RS (reference signal) receiving

- Group for RIM-RS transmission.

By default (without explicit indication), group means RIM-RS transmission group. Each group has a CGC (cell group coordinator). The CGC could be either a new logical entity or a function of existing network entity e.g. OAM, gNB, CU, MCE.

## Group for RI detection

The member in this group reports its RI detection result to CGC. The cells sharing same CGC address is one group. This group could be large, even as large as one PLMN. With neighbor assistance [1], each cell can detect RI with very high accuracy. The RI report is used for victim group configuration.

Proposal 1: An address should be configured in gNB for the gNB to send its RI report.

## Group for RIM-RS receiving

Based on RAN1 discussion, the group ID information in RIM-RS may be carried by following resources:

- RIM-RS sequence ID

- OCC (orthogonal cover code)

- Time: e.g. UL-DL periodicity offset

- Frequency: e.g. sub-band ID

- Space: beam configuration.

CGC should configure {group-ID, resource} set for group members to detect RIM-RS. When RIM-RS is detected, gNB reports detected group ID to CGC. This group could be same as RI detection group. The RIM-RS report is used for aggressor group configuration.

Proposal 2: {Group-ID, resource} set information should be configured in gNB for the gNB to detect RIM-RS.

Proposal 3: An address should be configured in gNB for the gNB to report the group ID detected from RIM-RS.

## Group for RIM-RS transmission

CGC dynamically configures this group based on gNBs reported RIM-RS and RI, e.g. only adjacent cells reported RI and have suitable beam/antenna configuration are grouped together. Member cells in the group transmit same RS in same frequency/time resource with same waveform, e.g. similar to MBSFN.

Proposal 4: Resource and Group-ID for RS-RIM transmission should be configured in gNB.

# Group ID

RI is caused by remote DL signal falls into UL slot, as shown in below figure.

So, the maximum range of remote interference is several hundreds of kilometres. Beyond this range, the RI would be either too weak or fall into another DL slot, which is not a problem. So, the group ID carried in RIM-RS can be reused, similar to PCI (physical cell ID) reuse. Therefore, we can name the group ID carried in RIM-RS as PCGI (physical cell group identifier).

Proposal 5: The Group ID carried in RIM-RS is PCGI (physical cell group ID), which may be reused in the network.

Based on RAN1 discussion, RIM-RS can carry up to 22 bits information. Due to PCGI reuse, it is not necessary to use such a long PCGI. For example, PCGI may use only 12 bits similar to PCI. The extra bit carried by RIM-RS can be used to deliver the coordination information, e.g. for framework 1.

In backhaul coordination, we need a global unique cell group ID for message routing.

Proposal 6: Define a global unique GCGI (global cell group identifier) from NR CGI name space for RIM message routing.

GCGI and TNL address are derived from PCGI, by querying DNS/OAM with PCGI and source gNB-ID as input, as shown in figure below.

# Dynamic Group Configuration

RAN3 #101bis meeting agreed: “The gNB cluster is configured by OAM”. The group configuration includes two aspects:

- group member identification;

- radio resource configuration for RIM-RS transmission and RIM-RS detection.

It is reasonable for OAM to dynamically configure the group members based on RI and RIM-RS report. However, radio resource configuration should be configured by a radio resource aware entity.

Based on above analysis, gNB should be configured with following information.

| Configuration Items | Configured by | Static/dynamic |
| --- | --- | --- |
| Address for RI reporting | OAM | Static |
| Address for RIM-RS reporting | OAM | Static |
| PCGI, GCGI, TNL address of CGC | OAM | Dynamic, based on RI and RIM-RS reporting |
| Address for target GCGI and TNL address resolution | OAM | static |
| {PCGI, Resource} set for RIM-RS detection | CGC | Semi-static |
| Resource for RIM-RS transmission | CGC | dynamic |

Proposal 7: OAM to dynamically configure the group members based on RI and RIM-RS report.

Proposal 8: The radio resource for RIM-RS transmission and detection are configured by a radio resource aware entity.

# Summary

Based on above analysis, we have following proposals.

Proposal 1: An address should be configured in gNB for the gNB to send its RI report.

Proposal 2: {Group-ID, resource} set information should be configured in gNB for the gNB to detect RIM-RS.

Proposal 3: An address should be configured in gNB for the gNB to report the group ID detected from RIM-RS.

Proposal 4: Resource and Group-ID for RS-RIM transmission should be configured in gNB.

Proposal 5: The Group ID carried in RIM-RS is PCGI (physical cell group ID), which may be reused in the network.

Proposal 6: Define a global unique GCGI (global cell group identifier) from NR CGI name space for RIM message routing.

Proposal 7: OAM to dynamically configure the group members based on RI and RIM-RS report.

Proposal 8: The radio resource for RIM-RS transmission and detection are configured by a radio resource aware entity.

# Reference

[1] R3-185514 Remote Interference Detection, Qualcomm Incorporated