3GPP TSG RAN WG3 Meeting #102	R3-186475

Spokane, WA, USA, 12th – 16th November 2018

Agenda Item:	22.3.2

Source:	Ericsson

Title:		TP to TR 38.866: The realization of RIM backhaul signalling

Document for:	pCR to TR 38.866

# Introduction

The RIM discussions at the RAN3#101bis meeting focused on reply LS to RAN1. Some of the remaining issues are as follows:

- Is there a need for intra-cluster coordination?

- Can the member gNBs in a set be changed? Is the Set ID static or dynamic?

- How does a gNB, based on the gNB set ID received in the RS, find out the destination ID for the backhaul messages it wants to send?

- How are the RIM messages forwarded over the backhaul?

This contribution discusses the issues 3-4 and presents a TP to TR 38.866. The issues 1-2 are discussed in R3-186477.

# Discussion

The realization of RIM backhaul communication requires at least the definition of:

- A mechanism for a gNB to acquire a destination ID for backhaul messages it intends to send. The destination ID may be acquired based on the gNB set ID, which is extracted from the received RS;

- A mechanism for passing the backhaul messages between the gNB sets.

The two concepts are jointly discussed below. The assumed communication framework is the RIM backhaul framework proposed in R3-186474. As agreed at the RAN3#101bis meeting, the OAM system handles the assignment and distribution of gNB set IDs and manages the gNB sets. Meanwhile, the AMF can transparently assist the RIM backhaul message exchange.

Proposal 1: The AMF can assist the RIM backhaul message exchange.

2.1 The realization of RIM backhaul communication

Having in mind that an RI scenario may involve hundreds or thousands of gNBs, the approach where each victim/aggressor gNB establishes a backhaul connection with each individual aggressor/victim gNB would simply not scale. A possible scalable solution is the one where a RIM backhaul message destined to a gNB set first arrives to the AMF affiliated to the gNB set, after which the AMF distributes the message to all the gNBs in the set. In fact, a reverse principle can be applied at the sender side – the AMF of the sending gNB set may collect the outgoing RIM messages from the set, merge them and forward the aggregated message to the AMF of the destination gNB set.

Proposal 2: In order to reduce the RIM backhaul traffic load, a Source gNB set AMF aggregates the outgoing RIM backhaul messages sent by the gNBs in the source gNB set. Furthermore, upon receiving a RIM backhaul message the AMF of the destination gNB set distributes the RIM message to the gNBs in the destination set.

The end-to-end RIM backhaul communication therefore consists of three segments:

- Sending gNBs -> Source AMF,

- Source AMF -> Destination gNB set AMF,

- Destination gNB set AMF -> all gNBs in the destination gNB set.

With respect to the above, the Destination ID in RIM backhaul messages from on the segment from individual gNBs to the Source AMF should be the gNB set ID that the sending gNB extracted from the received RS. In this this case, the AMF collects and aggregates all outgoing messages from a gNB set sent over a certain time window. The aggregated message should contain a list of all the source gNBs whose messages were merged into the aggregated RIM message. Furthermore, the Source AMF shall fill the Destination ID field of the aggregated message with the address of the AMF of the destination gNB set. This address can be a newly-defined on an existing AMF identifier.

Proposal 3: An end-to-end RIM backhaul communication path consists of three segments:

- Sending gNBs to Source AMF,

- Source AMF to destination gNB set AMF,

- Destination gNB set AMF to all gNBs in the destination gNB set.

Proposal 4: The Source gNB set AMF aggregates the outgoing messages from the source gNBs set. The aggregated RIM message contains the list of all the source gNBs whose messages were merged into the aggregated RIM message.

Proposal 5: The Destination gNB set AMF distributes an incoming RIM message to all gNBs in the destination set.

The mapping between the gNB set IDs and their corresponding AMF IDs can be e.g. in the form of a mapping table stored at the AMF. The mapping could be retrieved by the AMF from a database located in e.g. the core network (CN). Alternatively, to reduce the CN impact, the mapping between AMF IDs and gNB set IDs can be configured in the AMF nodes by the OAM.

Proposal 6: The mapping between the gNB set IDs and their corresponding AMF IDs can be e.g. in the form of a mapping table stored at the AMF. The mapping could be retrieved by the AMF from a database located in e.g. the core network (CN). Alternatively, to reduce the CN impact, the mapping between AMF IDs and gNB set IDs can be configured in the AMF nodes by the OAM.

The backhaul traffic overhead reduction achieved by the proposed approach can be illustrated by the following example. If two gNB sets involved in an RI scenario each comprise 1000 gNBs, setting the Destination ID to destination gNB set ID/AMF ID and having the destination AMF distributed the incoming message to the gNBs in the set may result in a 1000-fold reduction in the number of RIM messages that traverse the CN, compared to the approach where gNBs address each other individually. Moreover, even further reduction of RIM backhaul traffic load can be achieved if RIM Source gNB set AMF aggregates RIM backhaul messages received from the gNBs in the set and forwards the aggregated message to the AMF affiliated to the recipient gNB set. The assumption is that all gNBs in a set detect the incoming RS in a relatively short timespan.

The details of the RIM backhaul communication should be discussed in the Work Item phase. However, already today it may be assumed that the communication would be built on top of the existing RAN Information Management procedures.

Proposal 7: RAN3 adopts the existing RAN Information Management procedure as the baseline for the work on RIM backhaul communication in the Work Item (provided that RAN1 adopts a backhaul-based RIM framework).

# Conclusion

In this contribution, we discuss the realization aspects of RIM backhaul signalling. The following proposals are raised:

Proposal 1: The AMF can assist the RIM backhaul message exchange.

Proposal 2: In order to reduce the RIM backhaul traffic load, a Source gNB set AMF aggregates the outgoing RIM backhaul messages sent by the gNBs in the source gNB set. Furthermore, upon receiving a RIM backhaul message the AMF of the destination gNB set distributes the RIM message to the gNBs in the destination set.

Proposal 3: An end-to-end RIM backhaul communication path consists of three segments:

- Sending gNBs to Source AMF,

- Source AMF to destination gNB set AMF,

- Destination gNB set AMF to all gNBs in the destination gNB set.

Proposal 4: The Source gNB set AMF aggregates the outgoing messages from the source gNBs set. The aggregated RIM message contains the list of all the source gNBs whose messages were merged into the aggregated RIM message.

Proposal 5: The Destination gNB set AMF distributes an incoming RIM message to all gNBs in the destination set.

Proposal 6: The mapping between the gNB set IDs and their corresponding AMF IDs can be e.g. in the form of a mapping table stored at the AMF. The mapping could be retrieved by the AMF from a database located in e.g. the core network (CN). Alternatively, to reduce the CN impact, the mapping between AMF IDs and gNB set IDs can be configured in the AMF nodes by the OAM.

Proposal 7: RAN3 adopts the existing RAN Information Management procedure as the baseline for the work on RIM backhaul communication in the Work Item (provided that RAN1 adopts a backhaul-based RIM framework).

# ANNEX: TP to TR 38.866

-------------------------------------------Change 1-------------------------------------------

## 6.x.1	The realization of RIM backhaul communication

RAN3 concluded that the RIM backhaul communication is realized in the following manner.

An end-to-end RIM backhaul communication path consists of three segments:

- Source gNB set to Source gNB set AMF,

- Source gNB set AMF to destination gNB set AMF,

- Destination gNB set AMF to all gNBs in the destination gNB set.

The AMF affiliated to a gNB set assists the RIM backhaul message exchange. In order to reduce the RIM backhaul traffic load, an AMF collects the outgoing RIM backhaul messages from the gNBs in the Source gNB set, aggregates these messages into a single RIM backhaul message, and forwards the aggregated message towards the AMF of the Destination gNB set. The aggregated RIM backhaul message contains the list of all the source gNBs whose messages were merged into the aggregated RIM message. Upon receiving a RIM backhaul message destined to its affiliated gNB set, the Destination AMF distributes the incoming RIM backhaul message to the gNBs constituting the set.

The mapping between the gNB set IDs and their corresponding AMF IDs can be e.g. in the form of a mapping table stored at the AMF. The mapping could be retrieved by the AMF from a database located in e.g. the core network (CN). Alternatively, to reduce the CN impact, the mapping between AMF IDs and gNB set IDs can be configured in the AMF nodes by the OAM.

RAN3 decided that the existing RAN Information Management procedure is the baseline for the work on RIM backhaul communication in the Work Item phase (provided that RAN1 adopts a backhaul-based RIM framework).

-------------------------------------------End of changes-------------------------------------------