# 3GPP TSG-RAN WG3 #102	Tdoc R3-186854

# Spokane, USA, 12th – 16th November

Agenda item:	31.3.1.32

Source:	Ericsson

Title:	Multiple SCTPs in NG-RAN

Document for:	Discussion

# 1	Introduction

At the RAN101bis meeting in Chengdu, it was discussed how multiple SCTPs can be supported on different interfaces.

This paper reviews the status of the support of multiple SCTP on the NR interfaces, taking the functionality agreed forNG as the example.

# 2	Discussion

The table below summarizes the discussions and contributions thus far. Two methods have been used:

- Requesting TNLA addition / removal / modification, including TNLA role

- Sending CONFIG UPDATE from a new TNLA with node ID

Both methods require signalling specification (TS 38.4x3) and usage description (TS 38.4x2).

| Node / interface | Can multiple IP addresses be used? | Method | Status |
| --- | --- | --- | --- |
| AMF / NG | Yes | Adding / Removing / Updating TNLA | Fully supported |
| NGRN / NG | Yes | Sending CONFIG UPDATE with node ID | Agreed, signalling missing (TPs at the RAN3 #101bis meeting: [1], [2] and at RAN3 #102 meeting) |
|  |  | Dissent on whether the AMF should first ack the NG-RAN’s request for additional TNLAs |  |
| CU / F1 | Yes | Adding / Removing / Updating TNLA | Fully supported |
| DU / F1 | No | Sending CONFIG UPDATE with node ID | Part of “full package” CRs to be discussed in RAN3 #102 meeting: [3], [4]) |
| CP / E1 | Yes | Adding / Removing / Updating TNLA | Fully supported |
| UP / E1 | Yes | Sending CONFIG UPDATE with node ID | Agreed, description missing RAN3 #102 meeting:[5], [6]) |
| NGRN / Xn | No | Adding / Removing / Updating TNLA | Agreed, signalling and description missing |

Additional aspects to be discussed in the context of the Xn:

- Triangulation function of UE-associated messages (i.e. a response message in a class-1 procedure may be sent on a different TNLA than the one used for the initiating message), as supported on the NG.

- Disassociation of a TNLA.

- Stage-2, as present for F1 in TS 38.401.

### Disassociation contributions for this meeting

The following list provides a reference to the contribution on dissociating TNLA from interface instance on the various interfaces

- F1AP dissociating TNLA from interface instance, [7]

- E1AP dissociating TNLA from interface instance, [8]

# 3	Conclusion

Way forward:

- To complete the work on the functionalities marked as “Agreed”, where some part is missing (considering the additional functions from NG);

- To discuss further on the parts marked as “full package”.

# 4	References

- R3-185670, TP for NR BL CR for TS 38.413: Support of RAN initiated multiple SCTP, RAN3 #101bis, Chengdu

- R3-185930, “Associating an additional TNLA with the NG-C interface instance by means of the RAN Configuration Update procedure”, RAN3 #101bis, Chengdu

- R3-186779, “CR for F1 Multiple TNLA transport”, RAN3 #102, Spokane

- R3-186780, “CR for F1 Multiple TNLA initialization”, RAN3 #102, Spokane

- R3-186781, “CR for E1 Multiple TNLA transport”, RAN3 #102, Spokane

- R3-186855, “CR for E1 Multiple TNLA initialization”, RAN3 #102, Spokane

- R3-186857, “TP F1AP dissociating TNLA from interface instance”, RAN3 #102, Spokane

- R3-186856, “TP E1AP dissociating TNLA from interface instance”, RAN3 #102, Spokane