3GPP TSG-RAN WG3 Meeting #102	R3-187175

# Spokane, U.S.A. 12th- 16th November 2018

Agenda Item:	31.3.4.12

Source:	Ericsson

Title: 	[TP BL CR TS 38.413] NGAP Correction related to Intra system Handover

Document for:	Discussion and Approval

# 1	Discussion

There are two issues identified in the Intra system handover:

- The procedural text and the tabular have mismatch for the Intra system handover part in the handover preparation procedure.

- The presence of DRBs to QoS Flows Mapping List is missing in the Source NG-RAN Node to Target NG-RAN Node Transparent Container. It is correct to Optional, as it is Optional in ASN.1.

We propose to align the procedural text to tabular, so there is no impact on ASN.1.

Proposal 1: RAN 3 to discuss and agree the text proposal.

# 2	Text Proposal on TS 38.413 v15.1.0

--------------------------------

Chang starts.

---------------------------------

## 8.4	UE Mobility Management Procedures

### 8.4.1	Handover Preparation

#### 8.4.1.1	General

The purpose of the Handover Preparation procedure is to request the preparation of resources at the target side via the 5GC. There is only one Handover Preparation procedure ongoing at the same time for a certain UE.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Handover preparation: successful operation

The source NG-RAN node initiates the handover preparation by sending the HANDOVER REQUIRED message to the serving AMF. When the source NG-RAN node sends the HANDOVER REQUIRED message, it shall start the timer TNGRELOCprep. The source NG-RAN node shall indicate the appropriate cause value for the handover in the Cause IE.

In case of intra-system handover, the information in the Source to Target Transparent Container IE shall be encoded according to the definition of the Source NG-RAN node to Target NG-RAN node Transparent Container IE.

If the DL Forwarding IE is included for a given QoS flow in the PDU Session Resource Information Item IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message and it is set to "DL forwarding proposed", it indicates that the source NG-RAN node proposes forwarding of downlink data for that QoS flow.

If the DRBs to QoS Flows Mapping  List IE is included in the PDU Session Resource Information Item IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message, it implicitly indicates that the source NG-RAN node proposes forwarding of downlink data for those DRBs. If the HANDOVER COMMAND message contains the DL Forwarding UP TNL Information IE for a given DRB within the Handover Command Transfer IE, the source NG-RAN node shall consider that the forwarding of downlink data for this DRB is accepted by the target NG-RAN node. If the HANDOVER COMMAND message contains the UL Forwarding UP TNL Information IE for a given DRB in the Data Forwarding Response DRB List IE within the Handover Command Transfer IE, it means the target NG-RAN node has requested the forwarding of uplink data for this DRB.

In case of inter-system handover to LTE, the information in the Source to Target Transparent Container IE shall be encoded according to the Source eNB to Target eNB Transparent Container IE definition as specified in TS 36.413 [16].

If the Direct Forwarding Path Availability IE is included in the HANDOVER REQUIRED message the AMF shall handle it as specified in TS 23.502 [10].

If the Direct Forwarding Path Availability IE is included within the Handover Required Transfer IE of the HANDOVER REQUIRED message the SMF shall handle it as specified in TS 23.502 [10].

When the preparation, including the reservation of resources at the target side is ready, the AMF responds with the HANDOVER COMMAND message to the source NG-RAN node.

Upon reception of the HANDOVER COMMAND message the source NG-RAN node shall stop the timer TNGRELOCprep and start the timer TNGRELOCoverall.

If there are any PDU Sessions that could not be admitted in the target, they shall be indicated in the PDU Session Resources to Release List IE.

If the HANDOVER COMMAND message contains the QoS Flow to be Forwarded List IE within the Handover Command Transfer IE for a given PDU session, then the source NG-RAN node should initiate data forwarding for the listed QoS flows as specified in TS 38.300 [8].

If the Target to Source Transparent Container IE has been received by the AMF from the handover target then the transparent container shall be included in the HANDOVER COMMAND message.

In case of inter-system handover to LTE, the information in the Target to Source Transparent Container IE shall be encoded according to the definition of the Target eNB to Source eNB Transparent Container IE as specified in TS 36.413 [16].

If the Index to RAT/Frequency Selection Priority IE is contained in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE, the target NG-RAN node shall store the content of the received Index to RAT/Frequency Selection Priority IE in the UE context and use it as defined in TS 23.501 [9].

Interactions with other NGAP procedures:

NOTE:	Description of the interaction of the Handover Preparation procedure with other NGAP procedures may need to be refined.

------------------------

Skip to next change

------------------------

#### 9.3.1.29	Source NG-RAN Node to Target NG-RAN Node Transparent Container

This IE is produced by the source NG-RAN node and is transmitted to the target NG-RAN node. For inter-system handovers to 5G, the IE is transmitted from the external handover source to the target NG-RAN node.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in TS 38.331 [18] if the target is a gNB. Includes the RRC HandoverPreparationInformation message as defined in TS 36.331 [21] if the target is an ng-eNB. |
| PDU Session Resource Information List |  | 0..1 |  | For intra-system handovers in NG-RAN. |
| >PDU Session Resource Information Item |  | 1..<maxnoofPDUSessions> |  |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  |
| >>QoS Flow Information List |  | 1 |  |  |
| >>>QoS Flow Information Item |  | 1..<maxnoofQoSFlows> |  |  |
| >>>>QoS Flow Indicator | M |  | 9.3.1.51 |  |
| >>>>DL Forwarding | O |  | 9.3.1.33 |  |
| >>DRBs to QoS Flows Mapping List | O |  | 9.3.1.34 |  |
| E-RAB Information List |  | 0..1 |  | For inter-system handovers to 5G. |
| >E-RAB Information Item |  | 1..<maxnoofE-RABs> |  |  |
| >>E-RAB ID | M |  | 9.3.2.3 |  |
| >>DL Forwarding | O |  | 9.3.1.33 |  |
| Target Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  |
| UE History Information | M |  | 9.3.1.95 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofE-RABs | Maximum no. of E-RABs allowed towards one UE. Value is 256. |