3GPP TSG-RAN WG3 Meeting #103	R3-190352

Athens, Greece, 25th February - 1st  March, 2019

Agenda Item:	13.2.1

Source:	Ericsson

Title:	IP Address Assignment for IAB Nodes

Document for:	Agreement

# Introduction

During the IAB Study Item the topic of IP address assignment was briefly discussed. The focus was mainly on the feasibility of assigning IP addresses to the IAB node. At least two methods of IP address assignment were mentioned in the TR 38.874, where one was based on IPv6 address assignment currently supported by UEs connected to 5GC, and the other method was based on using the RRC protocol.

In our view, it is beneficial if existing methods for managing the IP addresses (and other related parameters) used in today’s operator networks could also be applied for IAB nodes avoiding the need to define 3GPP-specific methods, such as using the RRC protocol. One such common method used for IP address management is DHCP. This paper shows how DHCP can be supported for IAB nodes.

# Discussion

Figure 1: Example call flow for IAB node integration

Figure 1 shows an example of a call flow for integrating an IAB node (for more detail, please refer to [1]). The IAB node will first perform initial registration/attach in a similar way to normal UEs. During this registration, the IAB node can indicate that it is an IAB node by requesting access to an IAB node-specific network slice (or CN instance in LTE/EPC). The CN functions serving IAB node can authenticate the IAB node to the network and generate security keys to be used for NAS/AS security (for more detail, please refer to [2]).

When the RAN node (e.g. CU-CP) receives the IAB node context and sees that the IAB node has been authenticated as an IAB node, the RAN node establishes the backhaul bearers (or RLC channels) and configures the IAB Adaptation Layer to support forwarding of data packets to the IAB node [1].

At this point, the IAB node needs to obtain an IP address which is reachable from the operator’s internal network. Today, many networks use DHCP as a method to autoconfigure the IP addresses as well as other related network parameters (such as domain, gateway etc.). In our view, this should also be supported over IAB network.

In practice, this means that IAB node can send a DHCP discovery message over any of the backhaul bearers it has been assigned. When the Donor-DU receives this message, it can either assign an IP address directly to the IAB node (i.e. act as a DHCP server) or use a DHCP relay function to relay the message to an external DHCP server. DHCP protocol uses UDP/IP as lower layer (initially it uses broadcast/multicast addresses) and can easily be delivered over the backhaul bearers. DHCP is also supported for both for IPv4 and IPv6 networks.

Once the IAB node has obtained one or more IP addresses, it can communicate with its OAM server, set up NDS/IP, F1 etc.

DHCP is a feasible option for IP address assignment to IAB nodes for both IPv4 and IPv6 networks.

The DHCP messages can be transmitted on the backhaul bearers (RLC channels) as any other UDP/IP messages.

The Donor DU could either provide DHCP server function or DHCP relay function.

DHCP should be supported in IAB nodes and Donor DU to enable assignment of IP addresses to IAB nodes.

# Conclusions

In earlier sections we made the following observations:

Observation 1	DHCP is a feasible option for IP address assignment to IAB nodes for both IPv4 and IPv6 networks.

Observation 2	The DHCP messages can be transmitted on the backhaul bearers (RLC channels) as any other UDP/IP messages.

Observation 3	The Donor DU could either provide DHCP server function or DHCP relay function.

Based on the discussion in earlier sections we propose the following:

Proposal 1	DHCP should be supported in IAB nodes and Donor DU to enable assignment of IP addresses to IAB nodes.

# References

[1] R3-190354; IAB Node Integration Procedure, Ericsson, RAN3#103, February 2019.

[2] R3-190359; IAB Node Authentication and Authorization, Ericsson, RAN3#103, February 2019.