3GPP TSG-RAN3 Meeting #104	R3-192495

Reno, NV, USA, 13-17 May 2019

3GPP TSG-RAN WG2 #105-Bis			R2-1905475

Xi’an, PRC, 08 – 12 Apr 2019

Title:	LS to SA2 and SA5 on IAB impact to CN

Response to:

Release:	Rel-16

Work Item:	NR_IAB-Core

Source:	RAN2

To:	SA2, SA5

CC:	CT1, SA3, RAN3

Contact Person:

#### Name:	Milos TESANOVIC

E-mail Address:	m [dot] tesanovic [at] samsung [dot] com

1. Overall Description:

Following the successful completion of the Rel-16 Integrated Access and Backhaul (IAB) SI – results of which are captured in TR 38.874 (v16.0.0) – an IAB WI was approved for Rel-16. The following features need to be supported in the Rel-16 IAB system design (please see RP-182882 for further detail of the WID):

- Multi-hop backhauling for flexible range extension for both FR1 and FR2.

- Topology adaptation including redundant connectivity to optimize backhauling performance and to respond to backhaul (BH) link failure.

- Mapping of UE bearers to backhaul RLC channels and QoS enforcement over backhaul RLC channels to meet E2E QoS requirements.

- Scalability to a large number of UEs.

- Flexible deployment allowing IAB-node operation in EN-DC mode with EPC or in SA-mode with 5GC.

- Support for NR-NR DC from the UE and IAB-node perspective.

- Efficient operation for both inband and out-of-band relaying.

- OTA synchronization across IAB topology.

- Support of Rel-15 UEs.

The DU part of an IAB node should not visible to the CN for the same reason that the “regular” wired DU is not exposed. A relay (IAB) node will however also have a UE function, in order to connect to its parent (upstream – towards the Donor) node: (from the IAB TR) “MT is referred to as a function residing on an IAB-node that terminates the radio interface layers of the backhaul Uu interface toward the IAB-donor or other IAB-nodes.”

As indicated in the WID, the full impact on the CN is currently unknown – therefore CN specs have been listed in the IAB WID as being potentially impacted. For instance, based on the IAB TR, some CN impact is possibly envisaged through the support for authentication/authorization of the IAB-node, and key generation in the IAB-node.

In the timeframe until Rel-16 completion, RAN2 expects to make agreements for IAB that need to be captured in stage-2 descriptions of TS 23.501 and/or TS 23.502. This expectation is based on analogue descriptions on Relay Nodes in TS 23.401.

Further, at the RAN3#103 meeting in Athens, the following agreements were made:

1. The donor needs to know that the IAB-node MT is not a normal UE

2. SA for IAB node:

AMF includes “IAB Authorized” IE in the INITIAL CONTEXT SETUP REQUEST/CONTEXT MODIFICATION REQUEST messages

3. NSA for IAB node:

MME includes “IAB Authorized” IE in the INITIAL CONTEXT SETUP REQUEST/CONTEXT MODIFICATION REQUEST messages

The eNB should include “IAB Authorized” IE in SgNB ADDITION REQUEST/MODIFICATION REQUEST messages

4. Routing/forwarding for F1-C and for F1-U should be the same

Based on these recent RAN3 agreements and more specifically agreements 2 and 3 above, it is clear that a network node needs to be authorized by the CN in order to function as an IAB node. It therefore follows that some impact on CN procedures is expected.

To sum up, RAN2 as the lead WG for IAB WI would like to inform SA2 and SA5 about impending efforts needed for IAB across RAN and CN groups. RAN2 would further like to inform SA2 and SA5 of the recently made RAN3 agreements introducing an “IAB Authorized” IE in the INITIAL CONTEXT SETUP REQUEST/CONTEXT MODIFICATION REQUEST messages.

2. Actions:

RAN2 would like to kindly ask SA2 and SA5 to take these developments into account when planning their Rel-16 work, by making allowances in their schedule for any IAB impact on CN procedures until completion of Rel-16 scheduled for March 2020 – this may include considering initiating a new SA-groups based IAB-related WI or SI. Please note that an IAB-related SI has already been approved in SA3 (FS_NR_IAB_Sec) to study the potential security solutions for securing the wireless backhaul link.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #106	13 - 17 May 2019	Reno, USA

TSG-RAN WG2 Meeting #107	26 - 30 Aug 2019	Prague, CZ