3GPP TSG-RAN3 Meeting #104	R3-194055

Ljubljana, Slovenia, 26-30 August, 2019                                        Revision of R3-192946

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 088 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dynamic CN Component of PDB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IIOT |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Last SA2#131 meeting agreed to introduce the E2E PDB division in S2-1902833. Specifcally, the dynamic CN component of the PDB may be derived by the NG-RAN node. One way is that: - Configured in the SMF, per delay critical GBR 5QI, for different combinations of PSA-to-NG-RAN-node delays and signalled to NG-RAN during the PDU Session Establishment, PDU Session Modification, Xn/N2 handover and the Service Request procedures. Hence it requires to introduce the dynamic CN component of the PDB so that the CU-UP can determine the delay budget applicable to the radio interface. At SA2#132 meeting, it was also agreed to shorten the terminology “dynamic CN component of the PDB” to “CN PDB” in S2-1904695. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the CN Packet Delay Budget to GBR QoS Flow Information. Rev1: Update based on the latest specification. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN specification is not aligned with TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0147 TS 38.423 CR 0142 TS 38.473 CR 0368 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R3-192946 |  |  |  |  |  |  |  |  |

9.3.1.30	GBR QoS Flow Information

This IE indicates QoS parameters for a GBR QoS flow for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.20 | Maximum Bit Rate in DL. Details in TS 23.501 [20]. |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.20 | Maximum Bit Rate in UL. Details in TS 23.501 [20]. |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.20 | Guaranteed Bit Rate (provided there is data to deliver) in DL. Details in TS 23.501 [20]. |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.20 | Guaranteed Bit Rate (provided there is data to deliver). Details in TS 23.501 [20]. |
| Maximum Packet Loss Rate Downlink | O |  | Packet Loass Rate 9.3.1.46 | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Details in TS 23.501 [20]. |
| Maximum Packet Loss Rate Uplink | O |  | Packet Loss Rate 9.3.1.46 | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Details in TS 23.501 [20]. |
| CN Packet Delay Budget | O |  | Packet Delay Budget 9.3.1.47 | Delay between the UPF and the NG-RAN node as specified in TS 23.501 [20]. This IE may be present in case of Delay Critical GBR QoS flows, and shall be ignored otherwise. |