3GPP TSG-RAN3 Meeting #105	R3-194321

Ljubljana, Slovenia, 26th August – 30th August 2019

Agenda Item:	20.2.3

Source: 	Ericsson

Title:  	Resource coordination between NG-RAN nodes for NR V2X sidelink communication

Document for:	Discussion and decision

# Introduction

During the RAN3 meetings of NR V2X Study Item (SI) phase, companies have discussed the issues relative to resource pool coordination between NG-RAN nodes (e.g., ng-eNB and gNB). At the end of the SI, it was agreed that resource pool coordination between NG-RAN nodes is beneficial [1] and that the corresponding solution(s) will be discussed during the Work Item (WI) phase [2]. Below is the agreement from TR 38.885:

10.5	Resource coordination

Resource pool coordination between NG-RAN nodes (e.g., ng-eNB and gNB) is considered beneficial.

In this paper, we propose to enable NR V2X sidelink (SL) resource coordination between NG-RAN nodes in MR-DC, by leveraging UE-associated signaling between Master Node (MN) and Secondary Node (SN). The signaling includes messages from MN to SN, and vice versa, carrying the intended SL resource allocation for the V2X UE as well as the V2X Authorization.

# Discussion

In Rel-14 and Rel-15 LTE V2X, it was assumed that eNB could obtain the inter-cell SL resource pool configuration directly from OAM. However, OAM generally manages the RAN nodes of one PLMN, and may not be aware of the SL resource configuration of the neighboring nodes of another PLMN. Besides, in MR-DC scenario, it is possible to have two RAN nodes managed by different OAM systems; as well as in CU/DU split scenario, where a gNB-DU and a gNB-CU of the same gNB can be managed by different OAM systems, requiring on top of that a cross-OAM coordination for configuring the gNB-CU and gNB-DU OAMs, etc.

Therefore, we can remark that the OAM-based solution is lacking of flexibility and cannot be considered suitable for supporting the NR V2X SL operation.

Observation 1: The OAM configuration-based scheme is not suitable for cross-RAT sidelink resource scheduling.

One important thing that is needed for the NR V2X operation is the support UE-specific SL resource coordination in MR-DC scenarios. For instance, if we look at the MR-DC scenarios in figures 1,2,3 below (TR 38.885):

Figure 1: NE-DC 	                Figure 2: NGEN-DC                    	     Figure 3: EN-DC

sidelink resource coordination between MN and SN may be necessary if the SL UE and its MN/SN are using different RATs:

  - Scenario #1: LTE SL UE connected to NR MN and LTE SN (NE-DC)

    - In this case LTE SN may propose to NR MN some resource allocation for LTE SL.

  - Scenario #2: NR SL UE connected to LTE MN and NR SN (NGEN-DC)

    - In this case NR SN may propose to LTE MN some SLRB configuration and resource allocation for NR SL.

  - Scenario #3: NR SL UE connected to LTE MN and LTE SN (EN-DC)

    - In the case the UE change its LTE SN to an NR SN, then the new SN needs to propose to the LTE MN some resource coordination for the NR SL.

If, for all these scenarios, there is no support for V2X dedicated scheduling in another RAT, then MR-DC solutions are needed to support SL coordination between the MN and SN, where each node can perform scheduling within its own RAT for a specific UE SL resources, and coordinate this scheduling with the other node using UE-associated signaling.

Observation 2: SL resource coordination between NG-RAN nodes is needed, especially if the SL UE and its MN/SN are using different RATs.

As an example, the MN and SN can provide V2X scheduling sidelink information to each other, related to UE sidelink resources, via proper inter-node signaling. The signaling may include messages from MN to SN and vice versa. The messages can contain the intended resource allocation for the V2X UE and the V2X Service Authorization.

Proposal 1: MR-DC support of V2X sidelink resource coordination should be enabled by proper inter-node signalling between MN and SN, and vice-versa.

Proposal 2: The X2/Xn signalling should contain the intended resource allocation for the V2X UE and the V2X authorization.

From our view, the coordination of UE V2X sidelink resources for MR-DC can be ensured by simply leveraging some existing mechanisms for UE-associated signalling between MN and SN. Specifically, the E-UTRA-NR UE-level resource coordination present in X2 specification TS 36.423, which is used to avoid that a UE in an EN-DC scenario is configured with an E-UTRA-NR carrier frequency combination that would lead to a 1TX and harmonic interference problems.

In the case of NR V2X resource coordination in EN-DC, the MeNB can send to the en-gNB over X2 its scheduled UE sidelink resources in the SGNB ADDITION REQUEST message, which contains:

- the MeNB resource Coordination information IE (as defined in clause 9.2.116).

- And the V2X Services Authorization IE [3].

The en-gNB can send back the SgNB resource Coordination information IE (as defined in clause 9.2.117) in the SGNB ADDITION REQUEST ACKNOWLEDGE message. It can later forward this information to its lower layers and use it for sidelink resource coordination with the MeNB. In fact, the en-gNB, based on the received information, can realize which resources the UE uses for sidelink and refrains from scheduling other users or this UE for Uu and/or SL operations in those resources at those time instances.

Figure 3: example of V2X SL resource coordination in EN-DC

Proposal 3: Coordination of V2X sidelink resources for MR-DC can be enabled by leveraging some existing mechanisms for UE-associated signalling between MN and SN introduced in TS 36.423.

Note that this proposed solution can be applied to any other type of dual connectivity, e.g. NGEN-DC, NE-DC etc., with the respective changes to Xn.  We provide in [4] a CR with the proposed changes in X2 for covering EN-DC scenario. Regarding NR MR-DC cases (EN-DC and NGEN-DC), once the NR V2X Authorization is agreed and available in the Xn specification, we will provide the changes in future CRs.

Proposal 4: RAN3 to agree on the proposed solution for MR-DC V2X sidelink coordination and capture the specification changes proposed in [4].

Proposal 5: Send an LS to RAN2 to inform about the RAN3 solution.

# Conclusion

In this paper, we discussed on the issue of MR-DC sidelink coordination in NR V2X. Based on the discussion in this paper, we observed the following:

Observation 1: The OAM configuration-based scheme is not suitable for cross-RAT sidelink resource scheduling.

Observation 2: SL resource coordination between NG-RAN nodes is needed, especially if the SL UE and its MN/SN are using different RATs.

We also provide the following proposals:

Proposal 1: MR-DC support of V2X sidelink resource coordination should be enabled by proper inter-node signalling between MN and SN, and vice-versa.

Proposal 2: The X2/Xn signalling should contain the intended resource allocation for the V2X UE and the V2X authorization.

Proposal 3: Coordination of V2X sidelink resources for MR-DC can be enabled by leveraging some existing mechanisms for UE-associated signalling between MN and SN introduced in TS 36.423.

Proposal 4: RAN3 to agree on the proposed solution for MR-DC V2X sidelink coordination and capture the specification changes proposed in [4]

Proposal 5: Send an LS to RAN2 to inform about the RAN3 solution

# Reference

- 3GPP TR 38.885: Study on Vehicle-to-Everything; NR; Release 16

- R3-191060, “Summary of Offline Discussion: (WF) Potential agreements in RAN3#103”, LG Electronics Inc., RAN3#103

- 3GPP TS 36.423: Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)

- R3-194322, CR for MR-DC V2X SL coordination over X2, Ericsson, RAN3#105