3GPP TSG-RAN3 Meeting #105	R3-194470

Ljubljana, Slovenia, 26-30 August 2019

Title:	TP to TR 37.985 on LTE V2X

Source:	Huawei, LG Electronics

Agenda item:	20.1

Document for:	Approval

Introduction

This document is a Text Proposal to Techincal Report 37.985 “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR” providing contents to “Network aspects” section for LTE V2X.

2	Text proposal

----------------------------------------------- Start of TP -----------------------------------------------

5.4	Sidelink higher-layer protocols

----------------------------------------------- Next change -----------------------------------------------

5.4.9	Multi-PLMN operation

The same sidelink resource pool may be configured to the eNBs within a PLMN.

----------------------------------------------- Next change -----------------------------------------------

5.6	Network aspects

5.6.1	V2X service authorization

Considering the different UE types based on subscription information in the HSS, the eNB receives the authorization status of the UE provided by a MME or a neighbour eNB, to know whether the UE is authorized as Vehicle UE and/or Pedestrian UE.

Only the authorized UEs can perform V2X sidelink communication. And the sidelink radio resouces will be provided to the UEs in different ways due to the different services supported.

5.6.2	Sidelink AMBR

UE Sidelink Aggregate Maximum Bit Rate is provided to the eNB by a MME or a neighbour eNB, and used for the concerned UE’s sidelink communication in network scheduled mode for V2X services. The sidelink AMBR is based on subscription information from the HSS and the operator’s policy, which is used to cap the PC5 transmission from a particular UE.

Figure 5.6-1 Network Infrastructure for LTE V2X

----------------------------------------------- End of TP -----------------------------------------------