3GPP TSG-RAN WG3 Meeting #105	R3-194711

# Ljubljana, Slovenien, 26th – 30th August 2019

Agenda Item:	19.5

Source:	Ericsson

Title: 	Summary of offline discussion on F1AP measurement support

Document for:	Discussion and Approval

# 1	Summary of offline discussion

CB: # 65_MeasSuppF1AP

- general purpose measurement F1AP procedures / dedicated UL/DL F1AP procedures?

- information structure in message?

- all details are still FFS pending RAN1/RAN2

- handling of UL RTOA measurements

- further details?

(E///)

Summary of offline disc R3-194711

Structure of NRPPa / F1AP IE’s in messages

R3-193585 proposed structure NRPPa / F1AP IE’s in messages to allow separation of CU and TRP level information. The idea was that IE’s containing TRP level information should be easy to move from NRPPa messages to F1 messages. For this to work in the long run, the TRP related information would need to be captured in an extensible sequence that would allow future extensions to keep the separation for future additions of TRP related information.

This should be taken into account when we start to populate F1 messages with TRP related information.

F1AP Procedure structure

R3-194173 proposed the following set of messages (procedures):

Introduce 4 UE-associated F1AP procedures with the following messages: POSITIONING MEASUREMENT REQUEST/RESPONSE/FAILURE (Class 1), POSITIONING MEASUREMENT REPORT (Class 2), POSITIONING MEASUREMENT TERMINATION COMMAND (Class 2) and POSITIONING MEASUREMENT FAILURE INDICATION (Class 2)

R3-194270 proposed the following set of messages (procedures):

Class 1 procedures: DL positioning information exchange procedure; UL positioning information exchange procedure; positioning measurement procedure.

Class 2 procedures: UL position information update procedure; Positioning measurement update procedure; positioning measurement terminate procedure; Positioning measurement report procedure. Where the assumption is that the messages will support multiple positioning techniques.

It is proposed to focus discussion on the message/procedure structure at RAN3#105bis.

In summary:

- It is proposed to focus discussion on the message/procedure structure at RAN3#105bis.

- An attempt should be made to group TRP related information in an extensible sequence.

# 2	Conclusion

For RAN3#105bis it is recommended resolve the different proposals on the F1AP procedure and message structures.

And when we start to populate F1 (NRPPa) with details related to TRP’s, the target is to group such information in an extensible SEQUENCE.