3GPP TSG-RAN WG2 Meeting #106	                                                     R2-196795

Reno, USA, 18th – 22nd November 2019

Agenda item:	15.2.3

Source:	Qualcomm Incorporated

Title:	Per DRB DAPS HO

WID/SID:	NR_Mob_enh-Core

Document for:	Approval

# Introduction

RAN2 has agreed to support DAPS HO per DRB. This paper analyses the RAN3 impact.

RAN #107bis agreements:

Working assumption

19	DAPS configuration per DRB is agreed as working assumption as long as the specification impact is small.

# Discussion

RAN2/RAN3 has agreed to support DAPS HO (Dual active protocol stack handover). DSPA refers to the AS stack design where the UE maintains separate PHY/MAC/RLC entities for source connection DRBs and target connection DRBs with a common PDCP entity handling both DRBs, as discussed in [2] and described below.

The common PDCP entity can receive DL data from both the source connection and target connection during DAPS HO. This could happen if the source cell retransmits the PDUs not Acked by RLC or duplicates the PDCP PDUs sent to the target gNB to reduce delays. Also, these packets could be ciphered with different set of security keys in case of an Inter-CU HO, where the security keys used by source CU would be different from the security keys used by the target CU. The source CU and target CU compress the PDCP SDUs with different ROHC contexts. Thus, the common PDCP receiving entity maintains a common PDCP reordering and duplicate discarding entity, two separate DL ROHC contexts and two different security key information (source cell keys and target cell keys).

RAN2 has agreed that the UE does not need to transmit uplink data to both source and target and there will be a point in time where the UL data tx switch from source to target happens. To switch the UL data tx from source to target connection without any reconfiguration interruption, UE stores the configuration and security key information for the target connection on receiving the handover command while maintaining the source connection configuration. Thus, the common PDCP transmitting entity has to maintain two different security key information (source cell keys and target cell keys) as well.

Maintaining the dual security key information, dual ROHC context for DL, and, dual radio resource configuration during DAPS HO for every UL/DL DRB results in additional memory usage and complexity on the UE. Also, maintaining two separate PHY/MAC/RLC entities results in setup of separate logical channels for source and target connection during DAPS HO, and, could result in limiting the number of DRBs with DAPS HO support to not exceed the max. logical channel limit per UE.

Observation 1: DAPS HO is actually supported in DRB granularity anyway, because:

- the PDCP/RLC is per DRB;

- the admission control is per DRB;

- the data forwarding is per DRB level typically.

UE can signal in the DAPS HO capability for the max. number of supported MBB bearers or even request which specific bearers to consider for DAPS HO. Source gNB and target gNB consider UE capability and choose which DRBs are to be configured for DAPS HO and not all DRBs corresponding to the UE. The remaining DRBs not configured with DAPS HO will be handled as traditional HO DRBs.  In some cases, where all the DRBs for a UE need reduced user data interruption during HO, it is efficient to configure DAPS HO per UE. Thus, it is beneficial to keep the support for per UE DAPS HO configuration and use per DRB DAPS HO optionally as needed.

Proposal 1: Support per DRB DAPS HO in addition to per UE DAPS HO, i.e. configure DAPS HO only for DRBs that requires reduced user data interruption.

## Per DRB DAPS HO configuration

In order to support the per DRB DAPS HO configuration, network includes a flag in each DRB configuration to indicate whether DAPS HO is supported or not. gNB decides whether to configure DAPS HO for a DRB based on:

- Service need (e.g. info from QoS profile and potential UE assistance information)

- UE capability

- Low layer resource status

- PDCP configuration.

This information is known by RRC and it is better for RRC to configure the per DRB eMBB in RRC container. In XnAP/X2AP, the DAPS HO indicator can be supported in UE level.

Proposal 2: Support UE level DAPS HO indicator in XnAP/X2AP and DRB level DAPS HO indicator in RRC container.

# 3. Conclusion

Based on the above discussions, we recommend RAN2 discusses the following observations and proposals:

Observation 1: DAPS HO is actually supported in DRB granularity anyway, because:

- the PDCP/RLC is per DRB;

- the admission control is per DRB;

- the data forwarding is per DRB level typically.

Proposal 1: Support per DRB DAPS HO in addition to per UE DAPS HO, i.e. configure DAPS HO only for DRBs that requires reduced user data interruption.

Proposal 2: Support UE level DAPS HO indicator in XnAP/X2AP and DRB level DAPS HO indicator in RRC container.

# 4. References

[1] RP-192277 New WID: NR mobility enhancements

[2] R2-1908912 User Plane changes to minimize the mobility interruption during Make-Before-Break (DAPS) HO

[3] R2-1908935 RLM and RLF handling during DAPS HO