3GPP TSG-RAN WG3 #107bis-e	R3-201999

E-meeting, 20th – 30th April 2020

Source:	CATT

Title:	(TP on SON BLCR for 36.423) TP on Load Metrics

Agenda Item:	10.2.2.1

Document for:	Discussion and decision

# Introduction

This is a TP based on the discussion paper R3-201996.

# Text proposal for TS 36.423

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

8.7.x	EN-DC Resource Status Reporting Initiation

8.7.x.1	General

This procedure is used by the MeNB to request the reporting of load measurements to the en-gNB .

The procedure uses non UE-associated signalling.

8.7.x.2	Successful Operation

Figure 8.7.x.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from eNB to en-gNB to start a measurment, stop a measurements, add cells to report for a measurement.

If the Report Characteristics IE is included in the RESPOURCE STATUS REQUEST message and this indicates cell specific measurements, the Cell To Report IE shall be included.

Upon receipt of the RESOURCE STATUS REQUEST message, the en-gNB:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report IE list to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report IE, this information shall be ignored.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects en-gNB shall perform measurements on. For each cell, the en-gNB shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall include the requested SSB Area Radio Resource Status List IE.

-	the S1 TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB, and if the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE, providing the SSB area capacity with respect to the Cell Capacity Class Value.

-	the RRC Connections IE, if the sixth bit, "RRC Connections" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

8.7.x.3	Unsuccessful Operation

Figure 8.2.x.3-1: Resource Status Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, en-gNB shall send a RESOURCE STATUS FAILURE message.

8.7.x.4	Abnormal Conditions

FFS

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.1.2.x4	EN-DC RESOURCE STATUS UPDATE

This message is sent by en-gNB to neighbouring MeNB to report the results of the requested measurements.

Direction: en-gNB  eNB.

[Reporting per cell is FFS for TNL Load Indicator, Hardware Load Indicator]

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| MeNB Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by MeNB | YES | reject |
| en-gNB Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by en-gNB | YES | reject |
| Hardware Load Indicator [FFS] | O |  | FFS |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellinengNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.2.111 |  |  |  |
| >>Radio Resource Status [FFS] | O |  | NR Radio Resource Status 9.2.yy |  |  |  |
| >>S1 TNL Capacity Indicator [FFS Whether it is per cell] | O |  | S1 TNL Capacity Indicator 9.2.xx |  |  |  |
| >>Composite Available Capacity Group[FFS] | O |  | NR Composite Available Capacity Group 9.2.zz |  |  |  |
| >>Number of Active UEs [FFS], | O |  | INTEGER (0..65535,...) | Value 1 means 0.1 active UEs, value 2 means 0.2 active UEs, value n means n/10 Active UEs. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinengNB | Maximum no. cells that can be served by an en-gNB. Value is 16384. |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.2.yy	NR Radio Resource Status

[The information is FFS]

The NR Radio Resource Status IE indicates the usage of the PRBs per cell and per SSB area for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SSB Area Radio Resource Status List |  | 1 |  |  |
| >SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | 9.2.o |  |
| >>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage [FFS] |
| >>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage [FFS] |
| >>SSB Area SUL GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL GBR PRB usage [FFS] |
| >>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage [FFS] |
| >>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage [FFS] |
| >>SSB Area SUL non-GBR PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL non-GBR PRB usage [FFS] |
| >>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage [FFS] |
| >>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage [FFS] |
| >>SSB Area SUL Total PRB usage | O |  | INTEGER (0..100) | Per SSB area SUL Total PRB usage [FFS] |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| >>SUL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

9.3.4	PDU Definitions

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

CellMeasurementResult-ENDC-List ::= SEQUENCE (SIZE (1..maxCellinengNB)) OF ProtocolIE-Single-Container { {CellMeasurementResult-ENDC-ItemIEs} }

CellMeasurementResult-ENDC-ItemIEs X2AP-PROTOCOL-IES ::= {

{ ID id-CellMeasurementResult-ENDC-Item	CRITICALITY ignore	TYPE CellMeasurementResult-ENDC-Item	PRESENCE mandatory}

}

CellMeasurementResult-ENDC-Item ::= SEQUENCE {

cell-ID								NRCGI,

radioResourceStatus					NRRadioResourceStatus								OPTIONAL,

s1TNLCapacityIndicator				S1TNLCapacityIndicator								OPTIONAL,

compositeAvailableCapacityGroup		NRCompositeAvailableCapacityGroup					OPTIONAL,

numberofActiveUEs					INTEGER (0..65535, ...)								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {CellMeasurementResult-Item-ExtIEs} }	OPTIONAL,

...

}

CellMeasurementResult-Item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

9.3.5	Information Element definitions

/////////////////////////////////////skip unchanged/////////////////////////////////////

#### -- A

/////////////////////////////////////skip unchanged/////////////////////////////////////

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- N

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

/////////////////////////////////////skip unchanged/////////////////////////////////////

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

#### -- S

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////

SSBAreaRadioResourceStatus-List	::=	SEQUENCE (SIZE (1.. maxnoofSSBAreas)) OF SSBAreaRadioResourceStatus-Item

SSBAreaRadioResourceStatus-Item	::= SEQUENCE {

ssbIndex							SSBIndex

ssbAreaDLGBRPRBUsage				INTEGER (0..100),

ssbAreaULGBRPRBUsage				INTEGER (0..100),

ssbAreaSULGBRPRBUsage				INTEGER (0..100)									OPTIONAL,

ssbAreaDLNonGBRPRBUsage				INTEGER (0..100),

ssbAreaULNonGBRPRBUsage				INTEGER (0..100),

ssbAreaSULNonGBRPRBUsage			INTEGER (0..100)									OPTIONAL,

ssbAreaDLTotalPRBUsage				INTEGER (0..100),

ssbAreaULTotalPRBUsage				INTEGER (0..100),

ssbAreaSULTotalPRBUsage				INTEGER (0..100)									OPTIONAL,

ssbAreaDLSchedulingPDCCHCCEUsage	INTEGER (0..100)									OPTIONAL,

ssbAreaULSchedulingPDCCHCCEUsage	INTEGER (0..100)									OPTIONAL,

ssbAreaSULSchedulingPDCCHCCEUsage	INTEGER (0..100)									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {SSBAreaRadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

SSBAreaRadioResourceStatus-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////