3GPP TSG-RAN WG3 #107bis-e																	R3-202872

20-30 April 2020

Online

Agenda item:		19.2.1

Source:	Intel Corporation

Title:	(TP to BL CR for TS 38.455) Introduction of TRP support in NRPPa

Document for:		Discussion

# Introduction

This is the TP to capture agreements related to TRP support in NRPPa for the following email approval post RAN3#107bis-e:

Add an optional TRP list to the TRP INFORMATION REQUEST NRPPa message, to allow LMF to request information about all and specific TRPs from a gNB

LMF can request using TRP INFORMATION REQUEST and the gNB can report using TRP INFORMATION REPORT the following information: PRS ID, NR PCI, NR CGI, NR ARFCN, Timing Information, DL PRS Configuration, SSB Information/Configuration, Spatial Direction Information, Geographical Coordinates/Access Point Position, SFN Initialization Time; all of the above should be added with FFS

TRP INFORMATION UPDATE NRPPa message is not supported in this release

Remove TRP id from NRPPa MEASURMENT FAILURE and ABORT messages

Define a list of TRP ids in the MEASUREMENT REQUEST and RESPONSE messages; all the details, i.e.: weather the TRP ids are mandatory or optional, whether the message is UE-associated or non-UE associated, and whether it supports E-CID are FFS.

Include “Additional Path List” in RTOA measurement results and gNB RxTx measurement results

Email# 6 NRPPa TP to capture RAN3 #107bis-e Agreements on TRPs

Deadline 2020-05-08 1000 UTC

(Intel)

(TP to BL CR for TS 38.455) Introduction of TRP support in NRPPa R3-202872

# TP for TS 38.455

START OF CHANGES

### 8.2.Z	TRP Information Exchange

#### 8.2.Z.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node.

#### 8.2.Z.2	Successful Operation

Figure 8.2.Z.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information. If the LMF includes the TRP List IE in the TRP INFORMATION REQUEST message, the NG-RAN node includes in the TRP INFORMATION RESPONSE message, information for all TRPs included for the TRP List IE. If the LMF includes no TRPs for the TRP List IE in the TRP INFORMATION REQUEST message, the NG-RAN node includes information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message

#### 8.2.Z.3	Unsuccessful Operation

Figure 8.2.Z.3-1: TRP Information Exchange procedure, unsuccessful operation

If the NG-RAN node cannot provide any of the requested information, the NG-RAN node shall respond with a TRP INFORMATION FAILURE message.

NEXT CHANGE

#### 9.1.1.e	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 .. <maxnoTRPs> |  |  | EACH | ignore |
| >TRP ID | M |  | 9.2.aa |  |  |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >TRP Information Type Item | M |  | ENUMERATED (prs id, nr pci, nr cgi, nr arfcn, timing info, prs config, ssb config, snf init time, spatial direction info, geo-coordinates, FFS, …) [FFS] |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 16384. |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is FFS. |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.1.f	TRP INFORMATION RESPONSE

This message is sent by an NG-RAN node to convey TRP information to an LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Information List | M | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >TRP Information | M |  | 9.2.bb |  |  |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 16384. |

NEXT CHANGE

### 9.1.x	Messages for Measurement Information Transfer Procedures

#### 9.1.x.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| TRP List |  |  |  |  |  |  |
| >TRP Item |  | 1..<maxnoofTRPs> |  |  |  |  |
| >>TRP ID | FFS |  | 9.2.aa |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) [FFS] |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA, FFS,…) |  | - |  |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is FFS. |
| maxnoofTRPs | Maxmum no. of TRPs that can be included within one message. Value is 16 |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.x.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| TRP List |  |  |  |  |  |  |
| >TRP Item |  | 1..<maxnoofTRPs> |  |  |  |  |
| >>TRP ID | FFS |  | 9.2.aa |  | YES | reject |
| >>Measurement Result | O |  | 9.2.z1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maxmum no. of TRPs that can be included within one message. Value is 16 |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.x.3	MEASUREMENT FAILURE

This message is sent by the NG-RAN node to report measurement failure.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.11 |  | YES | ignore |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.x.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| TRP ID | FFS |  | 9.2.aa |  | YES | reject |
| Measurement Result | O |  | 9.2.z1 |  | YES | reject |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.x.5	MEASUREMENT UPDATE

This message is sent by the LMF to update a previously configured measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| TRP ID | FFS |  | 9.2.aa |  | YES | reject |
| SRS Configuration | O |  | 9.2.y |  | YES | ignore |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.x.6	MEASUREMENT ABORT

This message is sent by the LMF to request the NG-RAN node to abort a measurement.

Direction: LMF   NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

#### 9.1.x.7	MEASUREMENT FAILURE INDICATION

This message is sent by the NG-RAN node to indicate that the previously requested measurements can no longer be reported.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…) |  | YES | reject |
| TRP ID (FFS) | FFS |  | 9.2.aa |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

NEXT CHANGE

### 9.2.bb	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.aa |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  |
| >CHOICE TRP Information Item | M |  |  |  |
| >>PRS ID | M |  | INTEGER (0..255) | DL PRS ID |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >>NR CGI | M |  | 9.2.z5 |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  |
| >>Timing Information | M |  | 9.2.z6 |  |
| >>PRS Configurations | M |  | 9.2.z7 |  |
| >>SSB Configurations | M |  | 9.2.z8 |  |
| >>SFN Initialisation Time | M |  | BIT STRING (64) |  |
| >>Spatial Direction Information | M |  | 9.2.z9 |  |
| >>Geographical Coordinates | M |  | 9.2.10 |  |

[Editor’s Note: further details on the IEs are FFS]

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is FFS. |

NEXT CHANGE

### 9.2.z	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path List | O |  | 9.2.z12 |  |

### 9.2.z1	Measurement Result

This information element contains the measurement result.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.z2 |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..127) |  |
| >>UL RTOA | M |  | 9.2.z |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.z11 |  |
| >Time Stamp | M |  | 9.2.z3 |  |
| >Measurement Quality | M |  | 9.2.z4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is FFS. |

NEXT CHANGE

### 9.2.z5	NR CGI

This information element is used to globally identify an NR cell (see TS 38.300 [3]).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.8 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  |

### 9.2.z6	Timing Information

This information element contains timing information for the TRP.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |

### 9.2.z7	PRS Configuration

This information element contains the DL PRS configuration for the TRP.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |

### 9.2.z8	SSB Configuration

This information element contains the SSB information (e.g. time/frequency occupancy of SSBs) for the TRP.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |

### 9.2.z9	Spatial Direction Information

This information element contains the spatial direction information of the DL PRS resources for the TRP.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |

### 9.2.z10	Geographical Coordinates

This information element contains the geographical coordinates for the TRP.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |

### 9.2.z11	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Rx-Tx Time Difference | M |  | INTEGER(0..FFS) |  |
| Additional Path List | O |  | 9.2.z12 |  |

### 9.2.z12	Additional Path List

This information element contains the additional path results of time measurement.

[Editor’s Note: further details on the IEs are FFS / pending RAN2]

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnopath> |  |  |
| Relative Time of Path | M |  | FFS |  |
| Path Quality | O |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnopath | Maximum no. of additional path measurement. Value is 2. |