3GPP TSG RAN WG3 Meeting #108-e	R3-20xxxx

Online, 1st – 11th June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0033 | rev | 21 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BL CR to 38.401 Support for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Nokia Shanghai Bell. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  |  | Date: |  | 2020-06-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the support for IAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture agreements from RAN3#103: - Adding definitions and abbreviations for IAB specific terms. - Introduce the overall architecture for IAB. #5 Adding the description to state the applicability of gNB-CU/DU functionalities in IAB network. #6: capture agreed TPs in RAN3#103bis - Adding example for backhaul RLC channel establishment. - Adding contents about IAB node release. #7: capture agreed TPs in RAN3#104 - Add general procedure for IAB node integration - Move the IAB-specific changes from clause 6.1.2 to the newly-defined IAB-specific clause 6.1.x. - Introduce a statement on the reuse of UE context management procedures for IAB-MT. - Use term “BAP” instead of “Adaptation layer” to align with RAN2. #8: capture agreed TPs in RAN3#105 - Add IAB OAM connectivity content in clause 8.9.D and revise 8.z accordingly. - Modify IAB integration procedure to capture the OTA based IAB node parent node selection. - Modify IAB integration procedure to capture the IAB node indication in RRCSetupComplete message. #9: Merge the TPs (R3-195527, R3-196171, R3-196199, R3-196200, R3-196278, R3-196285) agreed in RAN3 #105bis meeting. - Fix the message name, step number issues, and make some editorial change about BH RLC channel establishment procedure. - Add intra-CU topopogy update procedure - Add more details about IAB integration procedure - routing configuration and BH RLC channel establishment. - IP address allocation for IAB node - IAB topology discovery for IAB-donor-CU #10: Include the TPs (R3-197784, R3-197662, R3-196997, R3-197640) agreed in RAN3 #106 meeting. - Introduce the NSA IAB integration procedure. - Add description for UL bearer mapping configuration in BH RLC channel management procedure. - Capture the protocol stacks for IAB network - Add the Intra-donor IAB topological redundancy procedure. #11:Merge the following TPs agreed in RAN3 #107-e meeting. - R3-201356: Revise the protocol stack figure for IAB, and cleanup for IAB integration procedure. - R3-201363: Introduce the BH RLF recovery procedure for IAB. - R3-201419: Cleanup the FFS part for intra-CU IAB migration procedure. Use hyphenation terms IAB-donor, IAB-donor-DU, IAB-donor-CU, IAB-DU, IAB-MT across the whole BL CR in a unified way, and add new definitions in clause 3.1 accordingly. #12:Merge the following TPs agreed in RAN3 #107bis-e meeting. - R3-202836: Miscellaneous change for stage 2 cleanup. - R3-202751: Update IAB migration procedure to include UL mapping configuration via RRC when IAB performs intra-CU migaration scenario. - R3-202842: Include IP address allocation for IAB. - R3-202860: Include intra-CU migration procedure for EN-DC case. Made some additional editorial changes for clause 8.2.x and clause 8.2.y, i.e. use “these bearers can be used” instead of “these bearers are used”, since the DRB for IAB-MT is not mandatory to be established. #13:Merge the following TPs agreed in RAN3 #108-e meeting: R3-203778; R3-204078; R3-204244; R3-204246; R3-204250; R3-204251; R3-204322; R3-204381; R3-204382. Add TS38.425 as new reference documents in clause 2. Made some editorial changes on the number of title in some new added clauses of section 8.9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 3.1; 3.2; 6.1.x (new); 6.1.y (new); 8.2.x (new); 8.2.x.1(new); 8.2.x.2(new); 8.2.x.3(new); 8.2.y(new); 8.2.z(new);8.9.A(new); 8.9.B(new); 8.9.B.1(new); 8.9.B.2(new); 8.9.C(new); 8.9.C.1(new); 8.9.C.2(new); 8.9.D(new); 8.9.E(new); 8.9.F(new); 8.z (new); 8.z.1 (new); 8.z.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | rev -: R3-190551 Add IAB related abbrevations and definitions, and the new clause 6.1.x to introduce the overall architecture of IAB. rev 1: R3-191001 - Update the title of the BL CR, - Add description about the IAB node’ in the overall architecture part shown in clause 6.1.2. - Update the introduction description and figure about the IAB architecture in clause 6.1.x rev 2: R3-191139 - Remove the bullet “An IAB-node only supports RRC_CONNECTED state in this release.” in summary of change part. - Update the clause affected part accordingly. rev3: R3-191143 Replace the “Donor” in “IAB-Donor” with “donor”, and update the according term shown in figure 6.1.x-1 rev 4:R3-191174 Update the meeting title and the spec version. rev 5: R3-192162 Add the description to state the applicability of gNB-CU/DU functionalities in IAB network. rev 6: R3-192187 Capture agreed TPs (R3-192165,R3-192175) in RAN3#103bis - Add example for backhaul RLC channel establishment. - Add contents about IAB node release. Rev 7:R3-192621 Use latest CR template, change the revision history and revision number. Rev 8:R3-193288 Include the two TPs(R3-193175, R3-193176) agreed in RAN3 #104 meeting - Add general procedure for IAB node integration - Move the IAB-specific changes from clause 6.1.2 to the newly-defined IAB-specific clause 6.1.x. - Introduce a statement on the reuse of UE context management procedures for IAB-MT. - Substitute the term “adaptation layer” with “BAP layer” to align with RAN2 determined protocol layer name. Rev 9: R3-193351 Update according to latest spec version v15.6.0, submit to RAN3 #105 meeting. Rev 10: R3-194800 Include the two agreed TPs(R3-194689, R3-194691) in RAN3 105 meeting. - Add IAB OAM connectivity content in clause 8.9.D and revise 8.z accordingly. - Modify IAB integration procedure to capture the OTA based IAB node parent node selection. - Modify IAB integration procedure to capture the IAB node indication in RRCSetupComplete message. Rev 11: R3-194967 Update according to latest spec version v15.6.0, submit to RAN3 #105bis meeting. Rev 12: R3-196306 Merge the TPs (R3-195527, R3-196171, R3-196199, R3-196200, R3-196278, R3-196285) agreed in RAN3 #105bis meeting. - Fix the message name, step number issues, and make some editorial change about BH RLC channel establishment procedure. - Add intra-CU topopogy update procedure - Add more details about IAB integration procedure - routing configuration and BH RLC channel establishment. - IP address allocation for IAB node - IAB topology discovery for IAB-donor-CU Rev 13: R3-196504 submit to RAN3 #106 meeting. Rev 14: R3-197808 Include the following TPs agreed in RAN3 #106 meeting. - R3-197784: Introduce the NSA IAB integration procedure. - R3-197662: Add description for UL bearer mapping configuration in BH RLC channel management procedure. - R3-196997: Capture the protocol stacks for IAB network - R3-197640: Add the Intra-donor IAB topological redundancy procedure. Rev 15: R3-200022 Update according to latest spec version v16.0.0, submit to RAN3 #107-e meeting. Rev 16: R3-201287 Two editorial changes: - Use “BAP sublayer” instead of “BAP layer” - rephrase the sentence of last paragraph in subclause 8.9.A to avoid using term “bearer mapping”. Rev 17: R3-201461 Merge the following TPs agreed in RAN3 #107-e meeting. - R3-201356: Revise the protocol stack figure for IAB, and cleanup for IAB integration procedure. - R3-201363: Introduce the BH RLF recovery procedure for IAB. - R3-201419: Cleanup the FFS part for intra-CU IAB migration procedure. Use hyphenation terms IAB-donor, IAB-donor-DU, IAB-donor-CU, IAB-DU, IAB-MT across the whole BL CR in a unified way, and add new definitions in clause 3.1 accordingly. Rev 18: R3-201597 Update according to latest spec version v16.1.0, submit to RAN3 #107bis-e meeting. Rev 19: R3-202896 Merge the following TPs agreed in RAN3 #107bis-e meeting. - R3-202836: Miscellaneous change for stage 2 cleanup. - R3-202751: Update IAB migration procedure to include UL mapping configuration via RRC when IAB performs intra-CU migaration scenario. - R3-202842: Include IP address allocation for IAB. - R3-202860: Include intra-CU migration procedure for EN-DC case. Made some additional editorial changes for clause 8.2.x and clause 8.2.y, i.e. use “these bearers can be used” instead of “these bearers are used”, to avoid the DRB for IAB-MT is not mandatory to be established. Rev 20: R3-203067 Add TS38.340 and TS38.331 as two new reference documents in clause 2. Check the whole BL CR to use “IAB-node” instead of “IAB node”. Submit to RAN3 #108-e meeting. Rev 21: R3-20xxxx Merge the following TPs agreed in RAN3 #108-e meeting. - R3-203778: In the step 12 of the intra-CU topological redundancy procedure, remove the condition of new TNL address allocation. - R3-204078: Change 8.9.F to modify the description about when the IAB node send IP address to CU to “before the IAB node use the IP address for UL and/or DL traffic”. - R3-204244: Add 8.9.M, 8.9.M.1 and 8.9.M.2, about the traffic mapping for intermediate IAB node and IAB-donor-DU. - R3-204246: Add sentence about IP address allocation for F1AP over LTE in section 8.9.F, and configuration of LTE and NR leg for NSA case in 8.z.2. - R3-204250: Change about the descendent nodes defualt configuration in 8.2.x and 8.2.z, add detailed description of order of IAB release in section 8.9.C.1. - R3-204251: Miscellaneous change for IAB topology update. - R3-204322: Add 8.9.E about IAB-MT in Inactive. - R3-204381: Add “SRBs and optionally DRBs” to clarify the IAB node ‘s radio bearer in phase 1 of integration procedure. - R3-204382: revise the description in the 8.9.F about the conflict mapping for IP prefix and full IP address. Add TS38.425 as new reference documents in clause 2. Made some editorial changes on the number of title in some new added clauses of section 8.9. |
| --- | --- |

Start of Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System".

[4]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[5]	3GPP TS 38.414: "NG-RAN; NG data transport".

[6]	3GPP TS 38.424: "NG-RAN; Xn data transport".

[7]	3GPP TS 38.474: "NG-RAN; F1 data transport".

[8]	ITU-T Recommendation G.823 (2000-03): "The control of jitter and wander within digital networks which are based on the 2048 kbit/s hierarchy".

[9]	ITU-T Recommendation G.824 (2000-03): "The control of jitter and wander within digital networks which are based on the 1544 kbit/s hierarchy".

[10]	ITU-T Recommendation G.825 (2001-08): "The control of jitter and wander within digital networks which are based on the synchronous digital hierarchy (SDH)".

[11]	ITU-T Recommendation G.8261/Y.1361 (2008-04): "Timing and Synchronization aspects in Packet networks".

[12]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[13]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[14]	3GPP TS 38.410: "NG-RAN; NG general aspect and principles".

[15]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles"

[16]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[17]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[18]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP Network Layer Security".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA), Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[20]	3GPP TS 32.422: "Trace control and configuration management".

[21]	3GPP TS 37.470: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and NG-RAN; W1 general aspects and principles; Stage-2".

[xx]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[yy]	3GPP TS 38.331: " NR; Radio Resource Control (RRC) protocol specification".

[zz]	3GPP TS 38.425: " NG-RAN; NR user plane Protocol".

# 3	Definitions and abbreviations

- 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

en-gNB: as defined in TS 37.340 [12].

gNB: as defined in TS 38.300 [2].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols of the gNB or RRC and PDCP protocols of the en-gNB that controls the operation of one or more gNB-DUs. The gNB-CU terminates the F1 interface connected with the gNB-DU.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates the F1 interface connected with the gNB-CU.

gNB-CU-Control Plane (gNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the gNB-CU for an en-gNB or a gNB. The gNB-CU-CP terminates the E1 interface connected with the gNB-CU-UP and the F1-C interface connected with the gNB-DU.

gNB-CU-User Plane (gNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol of the gNB-CU for an en-gNB, and the user plane part of the PDCP protocol and the SDAP protocol of the gNB-CU for a gNB. The gNB-CU-UP terminates the E1 interface connected with the gNB-CU-CP and the F1-U interface connected with the gNB-DU.

IAB-node: as defined in TS 38.300 [2].

IAB-donor: as defined in TS 38.300 [2].

IAB-donor-CU: the gNB-CU of an IAB-donor, terminating the F1 interface towards IAB-nodes and IAB-donor-DU.

IAB-donor-DU: the gNB-DU of an IAB-donor, hosting the IAB BAP sublayer (as defined in TS 38.340 [xx]), providing wireless backhaul to IAB-nodes.

IAB-DU: as defined in TS 38.300 [2].

IAB-MT: as defined in TS 38.300 [2].

ng-eNB: as defined in TS 38.300 [2].

ng-eNB Central Unit (ng-eNB-CU): as defined in TS 37.470 [21].

ng-eNB Distributed Unit (ng-eNB-DU): as defined in TS 37.470 [21].

NG-RAN node: as defined in TS 38.300 [2].

PDU Session Resource: This term is used for specification of NG, Xn, and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support a PDU Session.

Next Change

- 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

BH	Backhaul

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

NAS	Non-Access Stratum

O&M	Operation and Maintenance

PWS	Public Warning System

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SAP	Service Access Point

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SM	Session Management

SMF	Session Management Function

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

Next Change

# 6	NG-RAN Architecture

## 6.1	Overview

>>>> unchanged parts are skipped<<<<

### 6.1.x	Overall Architecture of IAB

Figure 6.1.x-1: Overall architecture of IAB

The NG-RAN supports IAB by the IAB-node wirelessly connecting to the gNB capable of serving the IAB-nodes, named IAB-donor.

The IAB-donor consists of an IAB-donor-CU and one or more IAB-donor-DU(s). In case of separation of gNB-CU-CP and gNB-CU-UP, the IAB-donor may consist of an IAB-donor-CU-CP, multiple IAB-donor-CU-UPs and multiple IAB-donor-DUs.

The IAB-node connects to an upstream IAB-node or an IAB-donor-DU via a subset of the UE functionalities of the NR Uu interface (named IAB-MT function of IAB-node). The IAB-node provides wireless backhaul to the downstream IAB-nodes and UEs via the network functionalities of the NR Uu interface (named IAB-DU function of IAB-node).

The F1-C traffic between an IAB-node and IAB-donor-CU is backhauled via the IAB-donor-DU and the optional intermediate hop IAB-node(s).

The F1-U traffic between an IAB-node and IAB-donor-CU is backhauled via the IAB-donor-DU and the optional intermediate hop IAB-node(s).

All functions specified for a gNB-DU are equally applicable for an IAB-DU and IAB-donor-DU, unless otherwise stated, and all functions specified for a gNB-CU are equally applicable for an IAB-donor-CU, unless otherwise stated. All functions specified for the UE context are equally applicable for managing the context of IAB-MT, unless otherwise stated.

Next Change

6.1.y	Protocol stacks of IAB

Figure 6.1.y-1 shows the protocol stack for F1-U between IAB-DU and the IAB-donor-CU-UP, and Figure 6.1.y-2 shows the protocol stack for F1-C between IAB-DU and the IAB-donor-CU-CP. In these example figures, F1-U and F1-C are carried over two backhaul hops.

NOTE:    F1 needs to be security-protected as described in TS 33.501, the security layer is not shown in the Figures 6.1.y-1/2.

Figure 6.1.y-1: Protocol stack for F1-U of IAB

Figure. 6.1.y-2: Protocol stack for F1-C of IAB

Figure 6.1.y-3 shows the protocol stack for F1-C between IAB-DU and the IAB-donor-CU-CP, when the F1-C traffic is transmitted via the MeNB.

Fig. 6.1.y-3: Protocol stack for IAB F1-C traffic delivered via the MeNB

Next Change

8	Overall procedures in gNB-CU/gNB-DU Architecture

>>>> unchanged parts are skipped<<<<

### 8.2.x	Intra-CU topology adaptation procedure

- 8.2.x.1	Intra-CU topology adaptation procedure in SA

During the intra-CU topology adaptation in SA, both the source and the target parent node are served by the same IAB-donor-CU. The target parent node may use a different IAB-donor-DU than the source parent node. The source path may have common nodes with the target path. Figure 8.2.x.1-1 shows an example of the topology adaptation procedure, where the target parent node uses a different IAB-donor-DU than the one used by the source parent node.

Figure 8.2.x.1-1: IAB intra-CU topology adaptation procedure

1.	The migrating IAB-MT sends a MeasurementReport message to the source parent node IAB-DU. This report is based on a Measurement Configuration the migrating IAB-MT received from the IAB-donor-CU before.

2.	The source parent node IAB-DU sends an UL RRC MESSAGE TRANSFER message to the IAB-donor-CU to convey the received MeasurementReport.

3.	The IAB-donor-CU sends a UE CONTEXT SETUP REQUEST message to the target parent node IAB-DU to create the UE context for the migrating IAB-MT and setup one or more bearers. These bearers can be used by the migrating IAB-MT for its own signalling, and, optionally, data traffic.

4.	The target parent node IAB-DU responds to the IAB-donor-CU with a UE CONTEXT SETUP RESPONSE message.

5.	The IAB-donor-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source parent node IAB-DU, which includes a generated RRCReconfiguration message. The RRCReconfiguration message includes a default BH RLC channel and a default BAP Routing ID configuration for UL F1-C traffic mapping on the target path. It may include additional BH RLC channels. This step may also include allocation of TNL address(es) that is (are) routable via the target IAB-donor-DU. The new TNL address(es) may be included in the RRCReconfiguration message as a replacement for the TNL address(es) that is (are) routable via the source IAB-donor-DU. In case IPsec tunnel mode is used to protect the F1 and non-F1 traffic, the allocated TNL address is outer IP address. The TNL address replacement is not necessary if the source and target paths use the same IAB-donor-DU. The Transmission Action Indicator in the UE CONTEXT MODIFICATION REQUEST message indicates to stop the data transmission to the migrating IAB-node.

6.	The source parent node IAB-DU forwards the received RRCReconfiguration message to the migrating IAB-MT.

7.	The source parent node IAB-DU responds to the IAB-donor-CU with the UE CONTEXT MODIFICATION RESPONSE message.

8.	A Random Access procedure is performed at the target parent node IAB-DU.

9.	The migrating IAB-MT responds to the target parent node IAB-DU with an RRCReconfigurationComplete message.

10.	The target parent node IAB-DU sends an UL RRC MESSAGE TRANSFER message to the IAB-donor-CU to convey the received RRCReconfigurationComplete message. Also, uplink packets can be sent from the migrating IAB-MT, which are forwarded to the IAB-donor-CU through the target parent node IAB-DU. These UL packets belong to the IAB-MT’s own signalling and, optionally, data traffic.

11.	The IAB-donor-CU configures BH RLC channels and BAP-sublayer routing entries on the target path between the target parent IAB-node and target IAB-donor-DU as well as DL mappings on the target IAB-donor-DU for the migrating IAB-node’s target path. These configurations may be performed at an earlier stage, e.g. immediately after step 3. The IAB-donor-CU may establish additional BH RLC channels to the migrating IAB-MT via RRC message.

12.	The F1-C connections are switched to use the migrating IAB-node’s new TNL address(es), IAB-donor-CU updates the UL BH information associated to each GTP-tunnel to migrating IAB-node. This step may also update UL FTEID and DL FTEID associated to each GTP-tunnel. All F1-U tunnels are switched to use the migrating IAB-node’s new TNL address(es). This step may use non-UE associated signaling in E1 and/or F1 interface to provide updated UP configuration for F1-U tunnels of multiple connected UEs or child IAB-MTs. The IAB-donor-CU may also update the UL BH information associated with Non-UP traffic

13.	The IAB-donor-CU sends a UE CONTEXT RELEASE COMMAND message to the source parent node IAB-DU.

14.	The source parent node IAB-DU releases the migrating IAB-MT’s context and responds to the IAB-donor-CU with a UE CONTEXT RELEASE COMPLETE message.

15.	The IAB-donor-CU releases BH RLC channels and BAP-sublayer routing entries on the source path between source parent IAB-node and source IAB-donor-DU.

NOTE:     In case that the source path and target path have common nodes, the BH RLC channels and BAP-sublayer routing entries of those nodes may not need to be released in Step 15.

Steps 11, 12 and 15 should also be performed for the migrating IAB-node’s descendant nodes, as follows:

- The IAB-donor-CU may allocate new TNL address(es) that is (are) routable via the target IAB-donor-DU to the descendent nodes via RRCReconfiguration message.

- If needed, the IAB-donor-CU may also provide a new default UL mapping which includes a default BH RLC channel and a default BAP Routing ID for UL F1-C/non-F1 traffic on the target path, to the descendant nodes via RRCReconfiguration message.

- If needed, the IAB-donor-CU configures BH RLC channels, BAP-sublayer routing entries on the target path for the descendant nodes and the BH RLC channel mappings on the descendant nodes in the same manner as described for the migrating IAB-node in step 11.

- The descendant nodes switch their F1-C connections and F1-U tunnels to new TNL addresses that are anchored at the new IAB-donor-DU, in the same manner as described for the migrating IAB-node in step 12.

- Based on implementation, these steps can be performed after or in parallel with the handover of the migrating IAB-node.

NOTE:     In upstream direction, in-flight packets between the source parent node and the IAB-donor-CU can be delivered even after the target path is established.

NOTE:	In-flight downlink data in the source path may be discarded, up to implementation via the NR user plane protocol (TS 38.425 [zz]).

NOTE:     The IAB-donor-CU can determine the unsuccessfully transmitted downlink data over the backhaul link by implementation.

Next Change

- 8.2.x.2	Intra-CU topology adaptation procedure in NSA using MCG SRB

This procedure is used when the migrating IAB-MT moves from source parent node to target parent node within the same IAB-donor-CU while only MCG SRB is available for IAB-node during EN-DC operation. The target parent node may use a different IAB-donor-DU than the one used by the source parent node. The source path may have common nodes with the target path. Figure 8.2.x.2-1 shows the topology adaptation procedure using MCG SRB of IAB-MT in EN-DC, where the target parent node uses a different IAB-donor-DU than the source parent node.

Figure 8.2.x.2-1: IAB intra-CU topology adaptation procedure using MCG SRB in EN-DC

1.	The migrating IAB-MT sends an ULInformationTransferMRDC message to the MeNB

2.	The MeNB sends RRC TRANSFER message to the IAB-donor-CU.

3.	The IAB-donor-CU may send UE CONTEXT MODIFICATION REQUEST message to the source parent node IAB-DU, to query the latest SCG configuration.

4.	The source parent node IAB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message that includes full configuration information.

5.	The IAB-donor-CU sends a UE CONTEXT SETUP REQUEST message to the target parent node IAB-DU, to create a UE context for migrating IAB-MT and set up one or more bearers. These bearers can be used by the migrating IAB-MT for its own signalling, and, optionally, data traffic. The UE CONTEXT SETUP REQUEST message includes CG-ConfigInfo.

6.	The target parent node IAB-DU responds to the IAB-donor-CU with a UE CONTEXT SETUP RESPONSE message.

7.	The IAB-donor-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source parent node IAB-DU, this message includes the Transmission Action Indicator IE, which instructs the source parent node IAB-DU to stop the data transmission to the migrating IAB-node. The source parent node IAB-DU also sends a Downlink Data Delivery Status frame to inform the IAB-donor-CU about the unsuccessfully transmitted downlink data to the migrating IAB-node.

8.	The source parent node IAB-DU responds to the IAB-donor-CU with a UE CONTEXT MODIFICATION RESPONSE message.

9. 	The IAB-donor-CU sends an SGNB MODIFICATION REQUIRED message to the MeNB.

10/11.	 The MeNB initiated SgNB Modification procedure may be triggered by the SgNB initiated SgNB Modification procedure (e.g. to provide information such as data forwarding addresses, new SN security key, measurement gap, etc...).

12.	The MeNB and the migrating IAB-MT perform RRC Connection Reconfiguration procedure. The RRCConnectionReconfiguration message includes information as described for the intra-CU topology adaptation procedure in SA in section 8.2.x.1.

13.	The MeNB sends an SGNB MODIFICATION CONFIRM message to the IAB-donor-CU.

14.	The migrating IAB-MT performs Random Access procedure at the target parent node IAB-DU.

15-19. The remaining steps of the procedure follow the steps 11-15 of the intra-CU topology adaptation procedure in SA scenario, as defined in clause 8.2.x.1. The main difference is that the RRC message for the migrating IAB-node, if involved, will be transmitted using the MCG SRB.

- 8.2.x.3	Intra-CU topology adaptation procedure in NSA using SCG SRB (SRB3)

This procedure is used when the migrating IAB-MT moves from source parent node to target parent node within the same IAB-donor-CU, when SCG SRB (SRB3) is available for IAB-node during EN-DC operation. The target parent node may use a different IAB-donor-DU than the source parent node. The source path may have common nodes with the target path. The procedure is the same as intra-CU topology adaptation procedure in SA scenario as defined in clause 8.2.x.1 but the UE CONTEXT SETUP REQUEST message includes CG-ConfigInfo in step 3.

Next Change

8.2.y	Intra-CU topological redundancy procedure

The intra-CU topological redundancy procedure enables the establishment and release of redundant paths in the IAB-topology underneath the same IAB-donor-CU. The redundant paths may use different IAB-donor-DUs. They may also have common intermediate nodes. Since topological redundancy uses NR-DC for the IAB-MT, it is only supported for IAB-nodes operating in SA mode.

Figure 8.2.y-1 shows an example for an IAB topology, where one IAB-node, referred to as the dual-connecting IAB-node, has two paths towards the IAB-donor via different IAB-donor-DUs.

Figure 8.2.y-1: Example for IAB topology with two redundant paths

Figure 8.2.y-2: Procedure for establishment of redundant path in IAB topology

Figure 8.2.y-2 shows the procedure for the establishment of the second path. This procedure has the following steps:

1. The dual-connecting IAB-MT sends a Measurement Report message to the first parent node IAB-DU. This report is based on a Measurement Configuration the dual-connecting IAB-MT received from the IAB-donor-CU before.

2.	The first parent node IAB-DU sends an UL RRC MESSAGE TRANSFER message to the IAB-donor-CU to convey the received Measurement Report.

3.	The IAB-donor-CU sends the UE CONTEXT SETUP REQUEST message to the second parent node IAB-DU, to create the UE context for the dual-connecting IAB-MT and to set up one or more bearers. These bearers can be used by the dual-connecting IAB-MT for its own signalling, and, optionally, data traffic.

4.	The second parent node IAB-DU responds to the IAB-donor-CU with a UE CONTEXT SETUP RESPONSE message.

5.	The IAB-donor-CU sends a DL RRC MESSAGE TRANSFER message to the first parent node IAB-DU, which includes a generated RRCReconfiguration message. The RRCReconfiguration message may contain one or more TNL address(es) for the dual-connecting IAB-DU, which are anchored at the second-path IAB-donor-DU. The IAB-donor-CU can proactively obtain these TNL addresses from the second-path IAB-donor-DU. In case IPsec tunnel mode is used to protect the F1 and non-F1 traffic, the allocated TNL address is the outer IP address. The TNL address allocation is not necessary if the first and second paths use the same IAB-donor-DU.

6.	The first parent node IAB-DU forwards the received RRCReconfiguration message to the dual-connecting IAB-MT.

7.	The dual-connecting IAB-MT responds to the first parent node IAB-DU with an RRCReconfigurationComplete message.

8.	The first parent node IAB-DU sends an UL RRC MESSAGE TRANSFER message to the IAB-donor-CU, to convey the received RRCReconfigurationComplete message.

9.	A Random Access procedure is performed at the second parent node IAB-DU.

10.	The IAB-donor-CU configures BH RLC channels and BAP-layer route entries on the second path between dual-connecting IAB-node and second-path IAB-donor-DU. These configurations may be performed at an earlier stage, e.g. immediately after step 3.

11.	The new TNL addresses allocated in step 5 (if any) are added to the dual-connecting IAB-DU’s F1-C association(s) with the IAB-donor-CU. The IAB-donor-CU may configure a new UL BH information on the second path for F1AP messages.

12. The IAB-donor-CU may migrate the F1-U tunnels it has with the dual-connecting IAB-DU from the first path to the second path via the UE CONTEXT MODIFICATION REQUEST message.

13.	The dual-connectivity IAB-DU replies with a UE CONTEXT MODIFICATION RESPONSE message.

Steps 12 and 13 can be performed for a subset of UE bearers, e.g., to balance the load between the first and the second path.

In case the second path is used for the dual-connecting IAB-node’s descendant node(s), Steps 10 and 11 are also performed for the descendant node(s), as follows:

- When the second path uses a different IAB-donor-DU, the IAB-donor-CU shall configure the descendent IAB-DU(s) with one or more new TNL addresses, which are anchored on the IAB-donor-DU of the second path.

- If needed, the IAB-donor-CU configures BH RLC channels, BAP-layer route entries on the target path for the descendant nodes and the BH RLC channel mappings on the descendant nodes in the same manner as described for the dual-connecting IAB-node in step 10.

- The descendant nodes’ new TNL addresses are added to the dual-connecting IAB-DU’s F1-C association(s) with the IAB-donor-CU. The IAB-donor-CU may further configure UL BH information for the second path to carry F1AP messages.

- The IAB-donor-CU may migrate the F1-U tunnels it has with the dual-connecting IAB-DU from the first path to the second path as described for Step 12.

- Based on implementation, these steps can be performed after or in parallel with the redundancy path addition of the dual connecting IAB-node.

The IAB-donor-CU may initiate the release of the redundant path by releasing the BAP routing entries and updating/modifying the BH RLC channels on that path.

Next Change

### 8.2.z	Intra-CU Backhaul RLF recovery for IAB-nodes in SA mode

The intra-CU backhaul RLF recovery procedure for IAB-nodes in SA mode enables migration of an IAB-node to another parent node underneath the same IAB-donor-CU, when the IAB-MT declares a backhaul RLF. The declaration of backhaul RLF is described in TS 38.331 [yy].

NOTE:     Determination of whether the recovery occurs at the same or at a different IAB-donor-CU is up to implementation.

Figure 8.2.z-1 shows an example of the BH RLF recovery procedure for an IAB-node in SA mode. In this example, the IAB-node changes from its initial parent node to a new parent node, where the new parent node is served by an IAB-donor-DU different than the one serving its initial parent node.

Figure 8.2.z-1: IAB intra-CU backhaul RLF recovery procedure for an IAB-node in SA mode

- The IAB-MT declares BH RLF for the MCG as described in TS 38.331 [yy], clause 5.3.10.3.

- The IAB-MT undergoing recovery from RLF conducts the RRC re-establishment procedure at the new parent node, as defined in clause 8.7. In this procedure, the IAB-donor-CU may provide new TNL address(es), which is(are) anchored at the new IAB-donor-DU, to the IAB-MT via RRC signalling. Besides, the IAB-donor-CU may also provide a new default UL mapping which includes a default BH RLC channel and a default BAP Routing ID for UL F1-C/non-F1 traffic on the target path, to the IAB-node undergoing recovery from RLF via RRCReconfiguration message in this procedure.

3.	The remaining part of the procedure follows the steps 11-15 of the intra-CU topology adaptation procedure defined in clause 8.2.x.

Descendant node(s) of the IAB-node undergoing recovery from RLF may also need to switch to new TNL address(es) anchored in the target-path IAB-donor-DU following the same mechanism as described for IAB intra-CU topology adaptation procedure in clause 8.2.x. The descendant node(s) may also be provided with new default UL mapping via RRC, after the IAB-node undergoing recovery from RLF connect the IAB-donor-CU via the recovery path.

Next Change

## 8.9	Overall procedures involving E1 and F1

The following clauses describe the overall procedures involving E1 and F1.

>>>> unchanged parts are skipped<<<<

### 8.9.A		BH RLC channel establishment procedure

The BH RLC channel is an RLC channel used for backhauling between IAB-node and IAB-donor-DU, or between different IAB-nodes. The BH RLC channel establishment may be triggered by a UE accessing the network and establishing a DRB.

Figure 8.9.A-1: Signalling flow for IAB BH RLC channel establishment procedure

1. The IAB-donor-CU sends to the IAB-donor-DU a UE CONTEXT MODIFICATION REQUEST message for setting up the parent-node IAB-DU side of the BH link between IAB-donor-DU and IAB-node 1. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

2. The IAB-donor-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the IAB-donor-CU.

3. The IAB-donor-CU sends to the IAB-donor-DU a DL RRC MESSAGE TRANSFER message encapsulating the RRCReconfiguration message for configuring the IAB-MT of IAB-node 1. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

4. The IAB-donor-DU decapsulates and forwards the RRCReconfiguration message to the IAB-MT of IAB-node 1. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

5. The IAB-MT of IAB-node 1 sends to the IAB-donor-DU an RRCReconfigurationComplete message destined to the IAB-donor-CU. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

6. The IAB-donor-DU sends the UL RRC MESSAGE TRANSFER message encapsulating the RRCReconfigurationComplete message to the IAB-donor-CU. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-donor-DU and IAB-node 1.

7. The IAB-donor-CU sends to the IAB-DU of IAB-node 1 a UE CONTEXT MODIFICATION REQUEST message for setting up the parent node IAB-DU side of the BH link between IAB-node 1 and IAB-node 2. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

8. The IAB-node 1 sends the UE CONTEXT MODIFICATION RESPONSE message to the IAB-donor-CU.

9. The IAB-donor-CU sends to the IAB-DU of IAB-node 1 a DL RRC MESSAGE TRANSFER message encapsulating the RRCReconfiguration message for configuring the IAB-MT of IAB-node 2. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

10. The IAB-DU of IAB-node 1 decapsulates and forwards the RRCReconfiguration message to the IAB-MT of IAB-node 2. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

11. The IAB-MT of IAB-node 2 sends to IAB-node 1 an RRCReconfigurationComplete message destined to the IAB-donor-CU. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

12. IAB-node 1 sends the UL RRC MESSAGE TRANSFER message encapsulating the RRCReconfigurationComplete message to the IAB-donor-CU. This step is optional and is required only when a new BH RLC channel needs to be established on the BH link between IAB-node 1 and IAB-node 2.

The IAB-donor-CU uses the existing CU-DU split principles and the F1-AP UE Context Setup procedure or UE Context Modification procedure to configure the parent IAB-DU side of the BH RLC channel. The IAB-donor-CU uses RRC signaling (which is encapsulated in the F1-AP DL RRC MESSAGE TRANSFER message terminating at the parent node IAB-DU side of the BH RLC channel) to configure the child node IAB-MT side of the BH RLC channel.

The IAB-donor-CU configures the IAB-DU with a mapping to the BH RLC channel to be used for a specific UL F1-U during the F1-AP UE Context Setup procedure or the F1-AP UE Context Modification procedure for the UE.

Next Change

### 8.9.B	Traffic Mapping

#### 8.9.B.1	Traffic Mapping from IP-layer to Layer-2

When forwarding IP packets, the IAB-donor-DU performs the traffic mapping from IP-layer to layer-2 as defined in TS 38.340 [xx]. The traffic mapping information is configured by the IAB-donor-CU, which contains the IP header information, and the BH information including the BAP routing ID and a list of egress link and BH RLC channel pairs.

Multiple traffic mappings can contain the same BAP routing ID and/or list of egress link and BH RLC channel pairs.

The traffic mappings can be configured as part of the F1AP UE Context Setup or F1AP UE Context Modification procedures. They may also be configured via the non-UE-associated BAP Mapping Configuration procedure.

NOTE:     Implementation must ensure the avoidance of potential race conditions, i.e. no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

#### 8.9.B.2		BH RLC Channel Mapping on BAP Layer

When traffic is forwarded on BAP layer as described in TS 38.300 [2], the IAB-node performs the BH RLC channel mapping as defined in TS 38.340 [xx]. The BH RLC channel mapping information is configured by the IAB-donor-CU.

The BH RLC channel mappings can be configured as part of the F1AP UE Context Setup or F1AP UE Context Modification procedures. They may also be configured via the non-UE-associated BAP Configuration procedure.

NOTE:     Implementation must ensure the avoidance of potential race conditions, i.e. no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

Next Change

8.9.C 	IAB-node release

An IAB-node may depart the network either in an orderly fashion, which implies that both the network and the IAB-node are aware in advance, or in a disorderly fashion (e.g. due to an RLF with failed recovery).

#### 8.9.C.1 	IAB-node orderly release

For orderly release, the IAB-donor-CU can remove F1 interface to the IAB-DU without releasing IAB-MT. If the IAB-MT needs to be released, IAB-MT will perform deregistration procedure. If both F1 interface and IAB-MT need to be released, the IAB-donor-CU should remove the F1 interface to the IAB-DU before it releases the collocated IAB-MT. The deregistration procedure is the same as the UE deregistration procedure. The IAB-donor-CU hands over the UEs or child IAB-nodes currently connected to the IAB-node’s cell(s) to another cell(s), or releases the UEs and may stop accepting incoming handovers or connections to the IAB-node that is about to be released. The IAB-donor-CU may also update/release the BH RLC channels in the intermediate hops. At this point, the F1 interface will be released and the corresponding SCTP associations will be removed.

#### 8.9.C.2 	IAB-node disorderly release

For the disorderly release case, how to remove the IAB-node context is up to network implementation.

Next Change

### 8.9.D	IAB-node OAM

The IAB-node receives commands, configuration data and software downloads (e.g. for equipment software upgrades) from its OAM system. The IAB-node can also send alarms and traffic counter information to its OAM system. The transport connection between the IAB-node and its OAM, using IP, is provided by the IAB-MT’s PDU session via 5G network, or the IAB-MT’s PDN connection via LTE network when IAB-MT uses EN-DC.

NOTE:     The transport connection between the IAB-node and its OAM may also be provided using the Backhaul IP layer by implementation.

Alarms in the IAB generate bursts of high-priority traffic, to be transported in real time. Traffic counters generate bursts of traffic, but their transport need not be real-time. Configuration messages from OAM to the IAB will also generate small bursts of traffic, possibly with lower priority than alarms but still delay-sensitive: when a configuration is committed on the OAM, the time interval between the commitment and the effect on the equipment shall be small.  Alarm messages and commands should be transported on a high-priority bearer, while counters may be transported on a lower priority bearer.

OAM software download to the IAB may generate larger amounts of data, but both the required data rate and the priority of this kind of traffic are much lower than in the case of alarms, commands and counters.

For different types of OAM traffic, it is necessary to use different DRBs between the IAB-MT and the serving DU, and different BH RLC channels for intermediate hops, with different QoS parameters. Aggregation of F1-U traffic for OAM with other F1-U traffic on the same BH RLC channels is not precluded. The QoS parameters are provided to the IAB-donor during the IAB-MT’s PDU session establishment, or the IAB-MT’s PDN connection establishment when IAB-MT uses EN-DC.

NOTE:     When the transport connection between the IAB-node and its OAM is provided by the Backhaul IP layer, the OAM traffic may be aggregated with other traffic types on the same BH RLC channel. The QoS for OAM is ensured by implementation.

Next Change

- 8.9.E		Handling of IAB-MTs in INACTIVE State

The IAB-MT optionally supports the RRC INACTIVE state. Upon the IAB-MT entering the RRC INACTIVE state, it is up to implementation to keep or release the F1 connection of the collocated IAB-DU.

Next Change

### 8.9.F	IP Address Allocation for IAB-nodes

An IAB-node may obtain IP address(es) either from the IAB-donor or from the OAM system. The IP address(es) is(are) used by the IAB-node for F1 and non-F1 traffic exchange via the backhaul. In case IPsec tunnel mode is used to protect this F1 and non-F1 traffic, the IP address(es) refer to the outer tunnel addresses. The allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

In case of IAB-donor-based IP address allocation, the IP address(es) is(are) allocated by the IAB-donor-CU or IAB-donor-DU. In both cases, the IAB-node requests the IP address(es) via RRC from the IAB-donor-CU. It includes a separate IP address request for each usage, where the usages defined are F1-U, F1-C and non-F1. The IAB-donor-CU may initiate the IAB TNL Address Allocation procedure to obtain IP addresses from the IAB-donor-DU. The IAB-donor-CU sends the IP addresses allocated for each usage to the IAB-node via RRC.

The IAB-node may be allocated one or multiple IPv6 addresses or one 64-bit IPv6 prefix for each usage and/or one or multiple IPv4 addresses for each usage. Each allocated IP address/IPv6 prefix is unique within the IAB network and routable from the wireline network.

In case of OAM-based IP address allocation, the IAB-node informs the IAB-donor-CU via an UL RRC message about the IP address(es) it received for each purpose. This occurs before the IAB node uses the IP address(es) for UL and/or DL traffic. The IAB-donor-CU configures the IAB-donor-DU with mappings between IP header fields and L2 parameters (BAP Routing ID, BH RLC channels) used for DL traffic.

Each mapping configuration may hold an IPv4 address, IPv6 address or a 64-bit IPv6 prefix. In case of two mapping entries matching the same IP header where one holds an IPv6 prefix and the other holds a full IPv6 address, the one with full IPv6 address takes precedence at the IAB-donor-DU.

In case of IAB-donor-allocated IP addresses, the IAB-node’s IP address(es) can be updated using DL RRC signalling.

For F1-C traffic transfer for NSA IAB, the LTE leg and NR leg should use separate IP address pairs {IAB-DU’s IP address, IAB-donor-CU’s IP address}. How the IAB-DU gets the remote IP end point(s) and its own IP address for LTE leg is not specified in this release.

Next Change

## 8.z	IAB-node Integration Procedure

### 8.z.1	Standalone IAB integration

A high-level flow chart for SA-based IAB integration is shown in the Figure 8.z-1:

Figure 8.z.1-1: The integration procedure for IAB-node in SA

Phase 1: IAB-MT setup. In this phase, the IAB-MT of the new IAB-node (e.g. IAB-node 2 in Figure 8.z.1-1) connects to the network in the same way as a UE, by performing RRC connection setup procedure with IAB-donor-CU, authentication with the core network, IAB-node 2-related context management, IAB-node 2’s access traffic-related radio bearer configuration at the RAN side (SRBs and optionally DRBs), and, optionally, OAM connectivity establishment by using the IAB-MT’s PDU session. The IAB-node can select the parent node for access based on an over-the-air indication from potential parent node IAB-DU (transmitted in SIB1). To indicate its IAB capability, the IAB-MT includes the IAB-node indication in RRCSetupComplete message, to assist the IAB-donor to select an AMF supporting IAB.

NOTE: The signalling flow for UE initial access procedure as shown in Figure 8.1-1/Figure 8.9.1-1 is used for the setup of the IAB-MT.

Phase 2-1: BH RLC channel establishment. During the bootstrapping procedure, one default BH RLC channel for non-UP traffic e.g. carrying F1-C traffic/non-F1 traffic to and from the IAB-node 2 in the integration phase, is established. This may require the setup of a new BH RLC channel or modification of an existing BH RLC channel between IAB-node 1 and IAB-donor-DU. The IAB-donor-CU may establish additional (non-default) BH RLC channels. This phase also includes configuring the BAP Address of the IAB-node 2 and default BAP Routing ID for the upstream direction.

NOTE: If the OAM connectivity is supported via backhaul IP layer by implementation, one or more BH RLC channels used for OAM traffic can also be established.

Phase 2-2: Routing update. In this phase, the BAP sublayer is updated to support routing between the new IAB-node 2 and the IAB-donor-DU. For the downstream direction, the IAB-donor-CU initiates F1AP procedure to configure the IAB-donor-DU with the mapping from IP header field(s) to the BAP Routing ID related to IAB-node 2. The routing tables are updated on all ancestor IAB-nodes (e.g. IAB-node 1 in Figure 8.z.1-1) and on the IAB-donor-DU, with routing entries for the new BAP Routing ID(s). This phase may also include the IP address allocation procedure for IAB-node 2. IAB-node 2 may request one or more IP addresses from the IAB-donor-CU via RRC. The IAB-donor-CU may send the IP address(es) to the IAB-node 2 via RRC. The IAB-donor-CU may obtain the IP address(es) from the IAB-donor-DU via F1-AP or by other means (e.g. OAM, DHCP). IP address allocation procedure may occur at any time after RRC connection has been established.

Phase 3: IAB-DU part setup. In this phase, the IAB-DU of IAB-node 2 is configured via OAM. The IAB-DU of IAB-node 2 initiates the TNL establishment, and F1 setup (as defined in clause 8.5) with the IAB-donor-CU using the allocated IP address(es). The IAB-donor-CU discovers collocation of IAB-MT and IAB-DU from the IAB-node’s BAP Address included in the F1 SETUP REQUEST message. After the F1 is set up, the IAB-node 2 can start serving the UEs.

NOTE: The IAB-DU can discover the IAB-donor-CU’s IP address in the same manner as a non-IAB gNB-DU.

### 8.z.2	NSA IAB Integration procedure

The IAB integration procedure for NSA is shown in Figure 8.z.2-1.

Figure 8.z.2-1: Signalling flow for IAB integration procedure in NSA

Phase 1-1. IAB-MT part setup with E-UTRAN. In this phase, the IAB-MT part connects to the LTE network as a UE, by performing RRC connection setup procedure with an eNB, authentication with the EPC, IAB-node’s access traffic-related radio bearer configuration at the E-UTRAN side, and optionally, OAM connectivity establishment by using the IAB-MT’s PDN connection. The IAB-node can select the IAB-supporting eNB based on an over-the-air indication from eNB (transmitted in SIB1). To indicate its IAB capability, the IAB-MT includes the IAB-node indication in RRCConnectionSetupComplete message, to assist the eNB to select an MME supporting IAB. The eNB then configures the IAB-MT part with an NR measurement configuration in order to perform discovery, measurement and measurement reporting of candidate gNBs. To enable the eNB choose an SgNB which supports IAB function, the IAB capability of neighbour gNBs can be pre-configured in the eNB (e.g. by OAM).

NOTE: Other ways to enable the eNB know the IAB capability of neighbour gNBs are not precluded.

Phase 1-2. SgNB addition. In this phase, the IAB-MT part connects to the parent node IAB-DU and IAB-donor-CU via the EN-DC SgNB Addition procedure. The procedure defined in section 8.4.1 is reused. The eNB includes “IAB Node Indication” in SGNB ADDITION REQUEST message to inform the IAB-donor-CU that the request is for an IAB-node. In addition, SRB3 can be set up for the IAB-MT, to transmit RRC message between the IAB-MT and the IAB-donor-CU via the NR links directly.

Phase 2-1: BH RLC channel establishment. This phase is the same as Phase 2-1 in the standalone IAB integration procedure (refer to the Phase 2-1 in clause 8.z.1). This step may occur in Phase 1-2.

Phase 2-2: Routing update. This phase is the same as Phase 2-2 in the standalone IAB integration procedure (refer to the Phase 2-2 in clause 8.z.1), except that the IP traffic on the F1-C interface may be transmitted via the MeNB.

Phase 3. IAB-DU part setup. This phase is the same as Phase 3 in the standalone IAB integration procedure (refer to the Phase 3 in clause 8.z.1), except that the IP traffic on the F1-C interface may be transmitted via the MeNB.

The IAB-donor-CU decides to only configure LTE leg, or only configure NR leg, or configure both LTE leg and NR leg, to be used for F1-C traffic transfer. The configuration may be performed before IAB-DU part setup. IAB-donor-CU may also change the configuration after IAB-DU part setup. In case the configuration is not performed before IAB-DU part setup, the IAB node uses the NR leg as the default one. When both LTE leg and NR leg are configured, it is up to the implementation to select the leg for F1-C traffic transfer.

End of Change