3GPP TSG-RAN WG3 #108-e	R3-20xxxx

1-11 June 2020

Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0103 | rev | 7 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BL CR to 38.425: Support for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  |  | Date: |  | 2020-06-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The support of DL end-to-end flow control for IAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification that the existing NR user plane protocol can be reused for end-to-end flow control in IAB networks. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of end-to-end flow control in IAB networks is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1; 3.2; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev#7 - Update WI Code to “NR_IAB-Core” - Add Nokia, Nokia Shanghai Bell as co-sign companies - Change the information for Rev#1: change the agree tdoc to R3-197786; delete “Add one sentence “The NR user plane protocol in this specification is also applicable to IAB networks, where it is also used for the end-to-end flow control and congestion prevention/mitigation over wireless backhaul link(s).” ” Rev#6 - For endorsement in RAN3#108-e Rev#5 (During RAN3#107bis-e) - Change “IAB-node DU” to “IAB-DU” in “In case of IAB, this node refers to the IAB-node DU functionality serving the UE for the corresponding data radio bearer” Rev#4 (RAN3#107bis-e) - Rebase TS38.425 v16.0.0 (2020-03) Rev#3 - For endorsed as BL in RAN3#107 - Update “Title”, “Reason for change”, “Summary of change” and “Consequences if not approved” Rev#2 - Endorsed as BL in RAN3#106 Rev#1(R3-197786) - Add clarification to corresponding node; Add IAB abbreviation |
| --- | --- |

-------------------------------------------Change 1-------------------------------------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Corresponding node: a node interacting with a node hosting NR PDCP for flow control. In an IAB network, this is the IAB-DU serving the UE for the corresponding data radio bearer.

Master node: as defined in TS 37.340 [3].

Secondary node: as defined in TS 37.340 [3].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

MR-DC	Multi-RAT Dual Connectivity

# 4	General

## 4.1	General aspects

The NR user plane protocol is located in the User Plane of the Radio Network layer over either the Xn or the X2 or the F1 interface.

The NR user plane protocol is used to convey control information related to the user data flow management of data radio bearers.

Each NR user plane protocol instance is associated to one data radio bearer only. There is one NR user plane instance per GTP tunnel. When a GTP tunnel is set up, a new NR user plane instance is set up.

If configured, NR user plane protocol instances exist at the Master node and the Secondary node in the context of DC or at nodes hosting F1-U protocol terminations. The NR user plane protocol supports direct communication between NR user plane protocol entities, regardless of whether they terminate the same or different user plane interfaces.

NOTE:	User data radio bearers may be setup for data forwarding purposes during Xn HO or during DC related mobility without requiring the execution of any additional data radio bearer related user plane protocol functions related to an NR user plane protocol instance.

On each data radio bearer, the NR user plane protocol operates with RLC AM or RLC UM.

In this version of the present document, NR user plane protocol data is conveyed by GTP-U protocol means, more specifically, by means of the "NR RAN Container" GTP-U extension header as defined in TS 29.281 [2].