3GPP TSG-RAN WG3 Meeting #108-e	R3-20oooo

Electronic Meeting, June 1st – 11th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0110 | rev | 6 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Baseline CR for introducing Rel-16 NR mobility enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation, CATT, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2020-06-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture agreements for Rel-16 NR mobility enhancement into 38.401 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3-106 (Reno) - Adding definition and abbreviation of DAPS HO - DAPS HO support for Inter-gNB-DU Mobility (based on R3-196898) - DAPS HO support for Inter-gNB handover involving gNB-CU-UP change (based on R3-196723) RAN3-107-e (Electronic Meeting) - R3-201423 (Google, ZTE, Intel Corporation) : Introducing conditional mobility - R3-201342 (Intel Corporation, Google) : Early Forwarding support for DAPS/CHO RAN3-107bis-e (Electronic Meeting) - Editorial change to mark FFS for still open issue - R3-202871 (Intel Corporation) : Correction and Resolving FFS on inter-gNB-DU conditional mobility scenarios - R3-202718 (Intel Corporation) : Early Forwarding Transfer needs CU-UP level distinction RAN3-108-e (Electronic Meeting) - R3-203763 (Intel Corporation) : Clean-up from the correction R3-202823 - R3-204176 (Google) : CPC Complete message correction - R3-204227 (Intel Corporation) : Correction for DAPS HO on inter-gNB-DU mobility scenarios - R3-204229 (Intel Corporation, ZTE, NEC) : F1 needs a new class-2 message like HO SUCCESS + miscellaneous clean-ups - R3-204285 (Intel Corporation) : Stage-2 for E1 - Rapporteur’s clean-up, message name correction/change (e.g. EARLY FORWARDING TRANSFER  EARLY STATUS TRANSFER), etc. Impact analysis, same release: the CR has functional and protocol impacts limited to Rel-16 NR mobility enhancement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-16 NR mobility enhancement is missing in 38.401 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 8.2.1.1, 8.2.1.X, 8.2.2.X, 8.9.2, 8.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 0481, TS 38.463 CR 0490 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: submitted to RAN3-107-e. Rev2: TPs agreed at RAN3-107-e incorporated. Rev3: submitted to RAN3-107bis-e Rev4: TPs agreed at RAN3-107bis-e incorporated. Rev5: submitted to RAN3-108-e Rev6: TPs agreed at RAN3-108-e incorporated. |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

- 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Conditional Handover: as defined in TS 38.300 [2].

Conditional PSCell Change: as defined in TS 37.340 [12].

en-gNB: as defined in TS 37.340 [12].

DAPS Handover: as defined in TS 38.300 [2].

Early Data Forwarding: as defined in TS 38.300 [2].

gNB: as defined in TS 38.300 [2].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols of the gNB or RRC and PDCP protocols of the en-gNB that controls the operation of one or more gNB-DUs. The gNB-CU terminates the F1 interface connected with the gNB-DU.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates the F1 interface connected with the gNB-CU.

gNB-CU-Control Plane (gNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the gNB-CU for an en-gNB or a gNB. The gNB-CU-CP terminates the E1 interface connected with the gNB-CU-UP and the F1-C interface connected with the gNB-DU.

gNB-CU-User Plane (gNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol of the gNB-CU for an en-gNB, and the user plane part of the PDCP protocol and the SDAP protocol of the gNB-CU for a gNB. The gNB-CU-UP terminates the E1 interface connected with the gNB-CU-CP and the F1-U interface connected with the gNB-DU.

ng-eNB: as defined in TS 38.300 [2].

ng-eNB Central Unit (ng-eNB-CU): as defined in TS 37.470 [21].

ng-eNB Distributed Unit (ng-eNB-DU): as defined in TS 37.470 [21].

NG-RAN node: as defined in TS 38.300 [2].

PDU Session Resource: This term is used for specification of NG, Xn, and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support a PDU Session.

- 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

AP	Application Protocol

AS	Access Stratum

CHO	Conditional Handover

CLI	Cross-Link Interference

CM	Connection Management

CMAS	Commercial Mobile Alert Service

DAPS	Dual Active Protocol Stack

ETWS	Earthquake and Tsunami Warning System

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

FDD	Frequency Division Duplex

GTP-U	GPRS Tunnelling Protocol

IP	Internet Protocol

NAS	Non-Access Stratum

O&M	Operation and Maintenance

PWS	Public Warning System

QoS	Quality of Service

RET	Remote Electrical Tilting

RIM	Remote Interference Management

RIM-RS Remote Interference Management Reference Signal

RNL	Radio Network Layer

RRC	Radio Resource Control

SAP	Service Access Point

SCTP	Stream Control Transmission Protocol

SFN	System Frame Number

SM	Session Management

SMF	Session Management Function

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TMA	Tower Mounted Amplifier

TNL	Transport Network Layer

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

- 8.2.1.1	Inter-gNB-DU Mobility

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation. Figure 8.2.1.1-1 shows the inter-gNB-DU mobility procedure for intra-NR.

Figure 8.2.1.1-1: Inter-gNB-DU Mobility for intra-NR

1.	The UE sends a MeasurementReport message to the source gNB-DU.

2.	The source gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received MeasurementReport message.

3.	The gNB-CU sends an UE CONTEXT SETUP REQUEST message to the target gNB-DU to create an UE context and setup one or more data bearers. The UE CONTEXT SETUP REQUEST message includes a HandoverPreparationInformation.

4.	The target gNB-DU responds to the gNB-CU with an UE CONTEXT SETUP RESPONSE message.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU, which includes a generated RRCReconfiguration message, and, indicates to stop the data transmission for the UE. The source gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE.

NOTE:	In case of DAPS Handover, the UE CONTEXT MODIFICATION REQUEST message in step 5 does not indicate to stop the data transmission for the UE. Instead, the DL RRC Message Transfer procedure can be used which carries the handover command to the UE. The UE CONTEXT MODIFICATION REQUEST message that indicates to stop the data transmission for the UE is sent to the source gNB-DU once the gNB-CU knows that the UE has successfully accessed the target gNB-DU, for which the source gNB-DU sends a DDDS frame about the unsuccessfully transmitted downlink data to the gNB-CU.

6.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

7.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

8.	A Random Access procedure is performed at the target gNB-DU. The target gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE:	It is up to gNB-CU implementation whether to start sending DL User Data to gNB-DU before or after reception of the Downlink Data Delivery Status.

9.	The UE responds to the target gNB-DU with an RRCReconfigurationComplete message.

10.	The target gNB-DU sends an UL RRC MESSAGE TRANSFER message to the gNB-CU to convey the received RRCReconfigurationComplete message. Downlink packets are sent to the UE. Also, uplink packets are sent from the UE, which are forwarded to the gNB-CU through the target gNB-DU.

11.	The gNB-CU sends an UE CONTEXT RELEASE COMMAND message to the source gNB-DU.

12.	The source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

- 8.2.1.X	Inter-gNB-DU Conditional Handover or Conditional PSCell Change

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for conditional handover or conditional PSCell change. Figure 8.2.1.X-1 shows the inter-gNB-DU conditional mobility procedure for intra-NR.

Figure 8.2.1.X-1: Inter-gNB-DU Conditional Handover or Conditional PSCell Change for intra-NR

1-2.	The steps 1-2 are as defined in clause 8.2.1.1.

3.	The gNB-CU sends an UE CONTEXT SETUP REQUEST message to the candidate gNB-DU to create an UE context and setup one or more data bearers. The UE CONTEXT SETUP REQUEST message is sent for each candidate cell and includes a HandoverPreparationInformation (conditional handover) or a CG-ConfigInfo (conditional PSCell change).

4.	The candidate gNB-DU responds to the gNB-CU with an UE CONTEXT SETUP RESPONSE message including the target cell ID that was requested from the gNB-CU. The response message is sent for each requested candidate cell.

5.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes a generated RRCReconfiguration message.

6.	The step 6 is as defined in clause 8.2.1.1.

7-8.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message, for which the source gNB-DU forwards to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

9.	 An execution condition to trigger initiation of conditional handover or conditional PSCell change is fulfilled.

10. A Random Access procedure is performed at the candidate gNB-DU, which becomes the target gNB-DU if successful. The target gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU. The target gNB-DU also sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed.

11-12.	The steps 11-12 are as defined in steps 9-10 in clause 8.2.1.1.

13.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU and indicates to stop the data transmission for the UE. The source gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE:	The step 13 may happen before step 12, as soon as the gNB-CU knows which cell the UE has successfully accessed.

NOTE:	The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate target gNB-DUs, if any, to cancel conditional handover or conditional PSCell change for the UE.

14.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

15 -16. 	The steps 15-16 are as defined in steps 11-12 in clause 8.2.1.1.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.2.X	Inter-gNB-DU Conditional PSCell Change using MCG SRB without MN negotiation

This procedure is used for the case where the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU when only MCG SRB is available and the MN’s configuration is not changed during EN-DC operation for conditional PSCell change. Figure 8.2.2.X-1 shows the inter-gNB-DU conditional PSCell change procedure using MCG SRB in EN-DC.

Figure 8.2.2.X-1: Inter-gNB-DU Conditional PSCell Change using MCG SRB without MN negotiation in EN-DC

1-4.	The steps 1-4 are as defined in clause 8.2.2.1.

5.	The gNB-CU sends an UE CONTEXT SETUP REQUEST message to the candidate gNB-DU to create an UE context and setup one or more data bearers. The UE CONTEXT SETUP REQUEST message is sent for each candidate cell and includes a CG-ConfigInfo.

6.	The candidate gNB-DU responds the gNB-CU with an UE CONTEXT SETUP RESPONSE message including the target cell ID that was requested from the gNB-CU. The response message is sent for each requested candidate cell.

7. 	The gNB-CU sends an SGNB MODIFICATION REQUIRED message to the MeNB, which includes a generated RRCReconfiguration message.

8.	The MeNB and the UE perform RRC Connection Reconfiguration/Complete procedure.

9.	The MeNB sends an SGNB MODIFICATION CONFIRM message to the gNB-CU, to convey the received RRCReconfigurationComplete message at step 8.

10. An execution condition to trigger initiation of conditional PSCell change is fulfilled.

11.	Random Access procedure is performed at the candidate gNB-DU, which becomes the target gNB-DU if successful. The target gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU. The target gNB-DU also sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed.

12-13. The UE responds with an RRCReconfigurationComplete message (embedded in an ULInformationTransferMRDC message), which the MeNB forwards to the gNB-CU via an SGNB RECONFIGURATION COMPLETE message.

14. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU and indicates to stop the data transmission for the UE. The source gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU. Downlink packets are sent to the UE. Also, uplink packets are sent from the UE, which are forwarded to the gNB-CU through the target gNB-DU.

NOTE:	The step 14 may happen before step 13, as soon as the gNB-CU knows which cell the UE has successfully accessed.

NOTE:	The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate target gNB-DUs, if any, to cancel conditional PSCell change for the UE.

15.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

16-17. The steps 16-17 are as defined in steps 11-12 in clause 8.2.1.1.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.9.2	Bearer context setup over F1-U

Figure 8.9.2-1 shows the procedure used to setup the bearer context in the gNB-CU-UP.

Figure 8.9.2-1: Bearer context setup over F1-U

0.	Bearer context setup (e.g., following an SGNB ADDITION REQUEST message from the MeNB) is triggered in gNB-CU-CP.

1.	The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for S1-U or NG-U, and if required, DL TNL address information for X2-U or Xn-U to setup the bearer context in the gNB-CU-UP. For NG-RAN, the gNB-CU-CP decides flow-to-DRB mapping and sends the generated SDAP and PDCP configuration to the gNB-CU-UP.

NOTE:	In case of Conditional Handover, the BEARER CONTEXT SETUP REQUEST message indicates to ignore the included security context and not to initiate sending downlink packets until the UE successfully accesses. Up to implementation, the gNB-CU-CP may request to establish bearer context as if a regular HO was requested.

2.	The gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, and DL TNL address information for S1-U or NG-U, and if required, UL TNL address information for X2-U or Xn-U.

NOTE:	The indirect data transmission for split bearer through the gNB-CU-UP is not precluded.

3.	F1 UE context setup procedure is performed to setup one or more bearers in the gNB-DU.

4.	The gNB-CU-CP sends a BEARER CONTEXT MODIFICATION REQUEST message containing the DL TNL address information for F1-U and PDCP status.

5.	The gNB-CU-UP responds with a BEARER CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

- 8.9.4	Inter-gNB handover involving gNB-CU-UP change

Figure 8.9.4-1 shows the procedure used for inter-gNB handover involving gNB-CU-UP change. Overall inter-gNB handover procedure is specified in TS 37.340 [12].

Figure 8.9.4-1: Inter-gNB handover involving gNB-CU-UP change

1.	The source gNB-CU-CP sends HANDOVER REQUEST message to the target gNB-CU-CP. In case of Conditional Handover, the target gNB is regarded as a candidate gNB which is only accessed by the UE when the CHO condition(s) are fulfilled.

2-4.	Bearer context setup procedure is performed as described in Section 8.9.2.

5.	The target gNB-CU-CP responds the source gNB-CU-CP with an HANDOVER REQUEST ACKNOWLEDGE message.

NOTE:	In case of Conditional Handover, it is up to target gNB-CU-CP implementation to make sure that the EARLY STATUS TRANSFER information is forwarded to the right gNB-CU-UP (e.g. separate UE-associated signalling connection over Xn interface for each gNB-CU-UP).

6.	The F1 UE Context Modification procedure is performed to stop UL data transmission at the gNB-DU and to send the handover command to the UE.

NOTE:	In case of DAPS Handover or Conditional Handover, the F1 UE Context Modification procedure in step 6 does not indicate to stop the data transmission for the UE. Instead, the F1 DL RRC Message Transfer procedure can be used which carries the handover command to the UE.

7-8.	Bearer context modification procedure (gNB-CU-CP initiated) is performed to enable the gNB-CU-CP to retrieve the PDCP UL/DL status and to exchange data forwarding information for the bearer.

9.	The source gNB-CU-CP sends an SN STATUS TRANSFER message to the target gNB-CU-CP.

NOTE:	In case of DAPS Handover, the EARLY STATUS TRANSFER message is sent for DRBs configured with DAPS instead of the SN STATUS TRANSFER message.

NOTE:	In case of Conditional Handover, the EARLY STATUS TRANSFER message is sent only if the early data forwarding is applied.

NOTE:	The COUNT related info for the EARLY STATUS TRANSFER message is retrieved from the source gNB-CU-UP via the steps 7/8.

10-11.	Bearer context modification procedure is performed as described in Section 8.9.2. The COUNT related info carried by the EARLY STATUS TRANSFER message is also provided to the target gNB-CU-UP.

12.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

NOTE:	In case of Conditional Handover, the UE performs RACH when the CHO condition(s) are fulfilled. Once successfully accessed, the target gNB-DU sends an ACCESS SUCCESS message to inform the target gNB-CU-CP of which cell the UE has accessed through. The target gNB-CU-CP may forward to the target gNB-CU-UP, necessary information for sending downlink packets (i.e. DL TNL address information for F1-U and UP security information corresponding to the accessed cell), via a Bearer context modification procedure. The target gNB-CU-CP may initiate Bearer context release procedure toward the other signalling connections or other candidate target gNB-CU-UPs, if any, to release the prepared conditional handover resources for the UE.

12a.	In case of DAPS Handover or Conditional Handover, the target gNB-CU-CP sends the HANDOVER SUCCESS message to the source gNB-CU-CP to inform that the UE has successfully accessed the target cell.

12b	In case of DAPS Handover or Conditional Handover, the F1 UE Context Modification procedure is performed to indicate to stop the data transmission for the UE.

12c-12d.	In case of DAPS Handover or Conditional Handover, the Bearer context modification procedure (gNB-CU-CP initiated) is performed to indicate the source gNB-CU-UP to stop packet delivery and also to retrieve the PDCP UL/DL status.

12e.	In case of DAPS Handover or Conditional Handover, the source gNB-CU-CP sends the SN STATUS TRANSFER message to the target gNB-CU-CP.

12f-12g.	In case of DAPS Handover or Conditional Handover, the Bearer context modification procedure is performed to provide the PDCP UL/DL status to the target gNB-CU-UP.

NOTE:	In case of Conditional Handover, inactivity monitoring is performed after step 12g.

13-15.	Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

16.	The target gNB-CU-CP sends an UE CONTEXT RELEASE message to the source gNB-CU-CP.

17.	and 19. Bearer context release procedure is performed.

18.	F1 UE context release procedure is performed to release the UE context in the source gNB-DU.