3GPP TSG-RAN WG3 #108-e 	R3-20xxxx

E-meeting, 1st - 11th June, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1468 | rev | 4 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | X2AP support for Radio Capability Signaling Optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, CATT, Ericsson, Huawei, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-RAN |  |  |  |  |  |  | Date: |  | 2020-06-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | X2 signaling shall be enhanced to support the Radio Capability Signaling Optimization (RACS). |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3-107-e (R3-201299) The followins are included to support the RACS feature: - Include the UE Radio Capability ID IE in the following X2/NG messages: . HANDOVER REQUEST . RETRIEVE UE CONTEXT RESPONSE - Support RACS feature in EN-DC and MR-DC: . Include the UE Radio Capability ID IE in the X2/Xn SN Addition Request messages. RAN3-107bis-e (R3-202731) Remove the editor’s note about the Assigned Crriticality of the UE Radio Capability ID IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RACS is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.3.13.2, 8.7.4.2, 9.1.1.1, 9.1.2.29, 9.1.4.1, 9.2.x (new), 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 36.413 CR 1746 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------------------- Start of 1st change ---------------------------

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an eNB for an incoming handover.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source eNB initiates the procedure by sending the HANDOVER REQUEST message to the target eNB. When the source eNB sends the HANDOVER REQUEST message, it shall start the timer TRELOCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

The source eNB may include in the GUMMEI IE any GUMMEI corresponding to the source MME node.

If at least one of the requested non-GBR E-RABs is admitted to the cell indicated by the Target Cell ID IE, the target eNB shall reserve necessary resources, and send the HANDOVER REQUEST ACKNOWLEDGE message back to the source eNB. The target eNB shall include the E-RABs for which resources have been prepared at the target cell in the E-RABs Admitted List IE. The target eNB shall include the E-RABs that have not been admitted in the E-RABs Not Admitted List IE with an appropriate cause value.

At reception of the HANDOVER REQUEST message the target eNB shall:

-	prepare the configuration of the AS security relation between the UE and the target eNB by using the information in the UE Security Capabilities IE and the AS Security Information IE in the UE Context Information IE.

For each E-RAB for which the source eNB proposes to do forwarding of downlink data, the source eNB shall include the DL Forwarding IE within the E-RABs To be Setup Item IE of the HANDOVER REQUEST message. For each E-RAB that it has decided to admit, the target eNB may include the DL GTP Tunnel Endpoint IE within the E-RABs Admitted Item IE of the HANDOVER REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer. This GTP tunnel endpoint may be different from the corresponding GTP tunnel endpoint, i.e. the information contained in the Transport Layer address IE and GTP TEID IE in the E-RAB To Be Switched in Downlink List IE of the PATH SWITCH REQUEST message (see TS 36.413 [4]) depending on implementation choice.

For each bearer in the E-RABs Admitted List IE, the target eNB may include the UL GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

Upon reception of the HANDOVER REQUEST ACKNOWLEDGE message the source eNB shall stop the timer TRELOCprep, start the timer TX2RELOCoverall and terminate the Handover Preparation procedure. The source eNB is then defined to have a Prepared Handover for that X2 UE-associated signalling.

If the Trace Activation IE is included in the HANDOVER REQUEST message then the target eNB shall, if supported, initiate the requested trace function as described in TS 32.422 [6]. In particular, the target eNB shall, if supported:

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [6];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to "Immediate MDT and Trace" initiate the requested trace session and MDT session as described in TS 32.422 [6];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to "Immediate MDT Only" initiate the requested MDT session as described in TS 32.422 [6] and the target eNB shall ignore Interfaces To Trace IE, and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the eNB may use it to propagate the MDT Configuration as described in TS 37.320 [31];

-	if the Trace Activation IE includes the UE Application layer measurement configuration IE, initiate the requested trace session and QoE Measurement Collection function as described in TS 36.300 [15].

-	if the Trace Activation IE includes the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [31].

-	if the Trace Activation IE includes the WLAN Measurement Configuration IE, within the MDT Configuration IE, take it into account for MDT Configuration as described in TS 37.320 [31].

If the Management Based MDT Allowed IE only or the Management Based MDT Allowed IE and the Management Based MDT PLMN List IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [6].

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

The source eNB shall, if supported and available in the UE context, include the Management Based MDT Allowed IE and the Management Based MDT PLMN List IE in the HANDOVER REQUEST message, except if the source eNB selects a serving PLMN in the target eNB which is not included in the Management Based MDT PLMN List. If the Management Based MDT PLMN List IE is not present, the source eNB shall, if supported, include the Management Based MDT Allowed IE, if this information is available in the UE context, in the HANDOVER REQUEST message, except if the source eNB selects a serving PLMN in the target eNB different from the serving PLMN in the source eNB.

If the Handover Restriction List IE is

-	contained in the HANDOVER REQUEST message, the target eNB shall

-	store the information received in the Handover Restriction List IE in the UE context;

-	use this information to determine a target for the UE during subsequent mobility action for which the eNB provides information about the target of the mobility action towards the UE, except when one of the E-RABs has a particular ARP value (TS 23.401 [12]) in which case the information shall not apply;

-	use this information to select a proper SCG during dual connectivity operation.

-	not contained in the HANDOVER REQUEST message, the target eNB shall consider that no roaming and no access restriction apply to the UE.

If the Location Reporting Information IE is included in the HANDOVER REQUEST message then the target eNB should initiate the requested location reporting functionality as defined in TS 36.413 [4].

If the SRVCC Operation Possible IE is included in the HANDOVER REQUEST message, the target eNB shall store the content of such IE in the UE context and use it as defined in TS 23.216 [20].

If the UE Security Capabilities IE included in the HANDOVER REQUEST message only contains the EIA0 algorithm as defined in TS 33.401 [18] and if this EIA0 algorithm is defined in the configured list of allowed integrity protection algorithms in the eNB (TS 33.401 [18]), the eNB shall take it into use and ignore the keys received in the AS Security Information IE.

The HANDOVER REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE, if available.

If the Subscriber Profile ID for RAT/Frequency priority IE is contained in the HANDOVER REQUEST message, the target eNB shall store this information and the target eNB should use the information as defined in TS 36.300 [15].

If the Additional RRM Policy Index IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information and the target eNB should use the information as defined in TS 36.300 [15].

Upon reception of UE History Information IE in the HANDOVER REQUEST message, the target eNB shall collect the information defined as mandatory in the UE History Information IE and shall, if supported, collect the information defined as optional in the UE History Information IE, for as long as the UE stays in one of its cells, and store the collected information to be used for future handover preparations.

Upon reception of the UE History Information from the UE IE in the HANDOVER REQUEST message, the target eNB shall, if supported, store the collected information to be used for future handover preparations.

If the Mobility Information IE is provided in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information and use it as defined in TS 36.300 [15]. The target eNB shall, if supported, store the C-RNTI of the source cell received in the HANDOVER REQUEST message.

If the Expected UE Behaviour IE is provided in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information and may use it to determine the RRC connection time.

If the ProSe Authorized IE is contained in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the eNB shall, if supported, consider that the UE is authorized for the relevant ProSe service(s).

If the V2X Services Authorized IE is contained in the HANDOVER REQUEST message and it contains one or more IEs set to "authorized", the eNB shall, if supported, consider that the UE is authorized for the relevant service(s).

If the UE Context Reference at the SeNB IE is contained in the HANDOVER REQUEST message the target eNB may use it as specified in TS 36.300 [15]. In this case, the source eNB may expect the target eNB to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 36.300 [15]. If the UE Context Reference at the WT IE is contained in the HANDOVER REQUEST message, the target eNB may use it as specified in TS 36.300 [15]. In this case, the source eNB may expect the target eNB to include the WT UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message; the source eNB shall use this information as specified in TS 36.300 [15].

If the UE Context Reference at the SgNB IE is contained in the HANDOVER REQUEST message the target eNB may use it as specified in TS 37.340 [32]. In this case, the source eNB may expect the target eNB to include the UE Context Kept Indicator IE set to "True" in the HANDOVER REQUEST ACKNOWLEDGE message, which shall use this information as specified in TS 37.340 [32].

If the Bearer Type IE is included in the HANDOVER REQUEST message and is set to "non IP", then the target eNB shall not perform header compression for the concerned E-RAB.If the UE Sidelink Aggregate Maximum Bit Rate IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for V2X services.

If the NR UE Security Capabilities IE is included in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and send it to the respective peer node during subsequent handover preparations and/or EN-DC operations for the UE as defined in TS 33.401 [15].

If the Aerial UE subscription information IE is included in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and use it as defined in TS 36.300 [15].

If the Subscription Based UE Differentiation Information IE is included in the HANDOVER REQUEST message, the eNB shall, if supported, store this information in the UE context for further use according to TS 23.401 [12].

If the EPC Handover Restriction List Container IE is included in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and shall use it as specified in TS 36.300 [15].

If the UE Radio Capability ID IE is contained in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and use it as specified in TS 23.401 [12].

Interaction with SN Status Transfer procedure:

If the UE Context Kept Indicator IE set to "True" and the E-RABs transferred to MeNB IE are included in the HANDOVER REQUEST ACKNOWLEDGE message, then the source eNB shall, if supported, include the uplink/downlink PDCP SN and HFN status received from the SgNB in the SN Status Transfer procedure towards the target eNB, as specified in TS 37.340 [32].

---------------------------- End of 1st change ---------------------------

---------------------------- Start of 2nd change ---------------------------

### 8.3.13	Retrieve UE Context

#### 8.3.13.1	General

The purpose of the Retrieve UE Context procedure is to retrieve the UE context from the eNB where the RRC connection has been suspended (old eNB) and transfer it to the eNB where the RRC Connection has been requested to be resumed (new eNB) or to retrieve the UE context for a UE which attempts to re-establish its RRC connection in an eNB (the new eNB) different from the eNB (the old eNB) where the RRC connection failed, e.g. due to RLF.

The procedure uses UE-associated signalling.

#### 8.3.13.2	Successful Operation

Figure 8.3.13.2-1: Retrieve UE Context, successful operation

The new eNB initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old eNB.

If the old eNB is able to identify the UE context and to successfully verify the UE by means of the Resume ID, the ShortMAC-I, optionally the C-RNTI, the failure cell PCI and the E-UTRAN Cell Identifier of the new cell contained in the RETRIEVE UE CONTEXT REQUEST message, it shall respond with the RETRIEVE UE CONTEXT RESPONSE message. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

If the C-RNTI IE is present in the RETRIEVE UE CONTEXT REQUEST, the old eNB shall ignore the Resume ID IE.

The old eNB may include in the GUMMEI IE any GUMMEI corresponding to the source MME node.

If the PLMN of the new cell is not the Serving PLMN stored in the UE Context the old eNB shall replace the Serving PLMN with the PLMN of the new cell and move the Serving PLMN to the equivalent PLMN list, before propagating the roaming and access restriction information to the new eNB.The new eNB shall act upon reception of the

-	UE Security Capabilities IE,

-	AS Security Information IE,

-	Subscriber Profile ID for RAT/Frequency priority IE,

-	Additional RRM Policy Index IE,

-	Handover Restriction List IE,

-	Location Reporting Information IE,

-	Management Based MDT Allowed IE

-	Management Based MDT PLMN List IE

-	Trace Activation IE,

-	SRVCC Operation Possible IE,

-	Masked IMEISV IE

-	Expected UE Behaviour IE,

-	ProSe Authorized IE,

-	V2X Services Authorized IE,

-	Aerial UE subscription information IE,

-	Subscription Based UE Differentiation Information IE,

-	EPC Handover Restriction List Container IE,

within the RETRIEVE UE CONTEXT RESPONSE message as specified for the target eNB upon reception of the HANDOVER REQUEST message for the Handover Preparation procedure.

If the UE Sidelink Aggregate Maximum Bit Rate IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new eNB shall, if supported, use it for the concerned UE’s sidelink communication in network scheduled mode for V2X services.

If the Aerial UE subscription information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the target eNB shall, if supported, store this information in the UE context and use it as defined in TS 36.300 [15].

For each E-RAB for which the old eNB proposes to do forwarding of downlink data, the old eNB shall include the DL Forwarding IE within the E-RABs To Be Setup Item IE of the RETRIEVE UE CONTEXT RESPONSE message.

If the UE Radio Capability ID IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the target eNB shall, if supported, store this information in the UE context and use it as specified in TS 23.401 [12].

---------------------------- End of 2nd change ---------------------------

---------------------------- Start of 3rd change ---------------------------

### 8.7.4	SgNB Addition Preparation

#### 8.7.4.1	General

The purpose of the SgNB Addition Preparation procedure is to request the en-gNB to allocate resources for EN-DC connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.7.4.2	Successful Operation

Figure 8.7.4.2-1: SgNB Addition Preparation, successful operation

The MeNB initiates the procedure by sending the SGNB ADDITION REQUEST message to the en-gNB. When the MeNB sends the SGNB ADDITION REQUEST message, it shall start the timer TDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the Full E-RAB Level QoS Parameters IE or in the Requested MCG E-RAB Level QoS Parameters IE or in the Requested SCG E-RAB Level QoS Parameters IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [4].

If the SGNB ADDITION REQUEST message contains the Serving PLMN IE, the en-gNB may use it for RRM purposes.

If the SGNB ADDITION REQUEST message contains the Expected UE Behaviour IE, the en-gNB shall, if supported, store this information and may use it to optimize resource allocation.

If the SGNB ADDITION REQUEST message contains the Handover Restriction List IE, the en-gNB node, if supported, shall store this information and use it to select an appropriate NR cell.

If the SGNB ADDITION REQUEST message contains the MeNB Resource Coordination Information IE, the en-gNB should forward it to lower layers and it may use it for the purpose of resource coordination with the MeNB. The en-gNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The en-gNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the MeNB Coordination Assistance Information IE is contained in the MeNB Resource Coordination Information IE, the en-gNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

The en-gNB shall choose the ciphering algorithm based on the information in the NR UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the SgNB Security Key IE as specified in the TS 33.401 [18].

If the SGNB ADDITION REQUEST message contains the Subscriber Profile ID for RAT/Frequency Priority IE, the en-gNB may use it for RRM purposes.

If the SGNB ADDITION REQUEST message contains the Additional RRM Policy Index IE, the en-gNB may use it for RRM purposes.

The en-gNB shall search for the target NR cell among the NR neighbour cells of the E-UTRAN cell indicated in MeNB Cell ID IE, as specified in the TS 37.340 [32].

If the Masked IMEISV IE is contained in the SGNB ADDITION REQUEST message the en-gNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

The en-gNB shall report to the MeNB, in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the result for all the requested E-RABs in the following way:

-	a list of E-RABs which are successfully established shall be included in the E-RABs Admitted To Be Added List IE;

-	a list of E-RABs which failed to be established shall be included in the E-RABs Not Admitted List IE.

NOTE:	The MeNB may trigger the SgNB Addition Preparation procedure in the course of the Inter-MeNB handover without SgNB change procedure as described in TS 37.340 [32]. The deleted E-RABs are not included in the E-RABs To Be Added List IE in the SGNB ADDITION REQUEST message, from MeNB point of view. If the en-gNB reports a certain E-RAB to be successfully established, respective SCG resources, from an en-gNB point of view, may be actually successfully established or modified or kept; if a certain E-RAB is reported to be failed to be established, respective SCG resources, from an en-gNB point of view, may be actually failed to be established or modified or kept.

For each E-RAB successfully established in the en-gNB, the en-gNB shall report to the MeNB, in the SGNB ADDITION REQUEST ACKNOWLEDGE message, the same value in the EN-DC Resource Configuration IE as received in the SGNB ADDITION REQUEST message.

For each E-RAB for which allocation of the PDCP entity is requested at the en-gNB:

-	the MeNB may propose to apply forwarding of downlink data by including the DL Forwarding IE within the E-RABs To be Added Item IE of the SGNB ADDITION REQUEST message. For each E-RAB that it has decided to admit, the en-gNB may include the DL Forwarding GTP Tunnel Endpoint IE within the E-RABs Admitted To Be Added Item IE of the SGNB ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer. This GTP tunnel endpoint may be different from the corresponding GTP tunnel endpoint, i.e the information contained in the Transport Layer Address IE and the DL GTP TEID IE in the E-RAB To Be Modified List IE of the E-RAB MODIFICATION INDICATION message (see TS 36.413 [4]) depending on implementation choice;

-	the en-gNB may include for each bearer in the E-RABs Admitted To Be Added List IE the UL Forwarding GTP Tunnel Endpoint IE to indicate that it requests data forwarding of uplink packets to be performed for that bearer.

-	the en-gNB shall use the S1 UL GTP Tunnel Endpoint IE of the SGNB ADDITION REQUEST message as the UL S1-U address.

-	the MeNB shall use the SgNB UL GTP Tunnel Endpoint at PDCP IE of the SGNB ADDITION REQUEST ACKNOWLEDGE message as the UL X2-U address.

-	if the SGNB ADDITION REQUEST message contains for an E-RAB to be added which is requested to be configured with MCG resources the MeNB DL GTP Tunnel Endpoint at MCG IE the en-gNB shall use it as the DL X2-U address for delivery of DL PDCP PDUs.

-	the en-gNB shall include in the SGNB ADDITION REQUEST ACKNOWLEDGE message the S1 DL GTP Tunnel Endpoint at the SgNB IE.

-	the en-gNB shall include in the SGNB ADDITION REQUEST ACKNOWLEDGE message the RLC Mode IE.

-	the en-gNB may include for each bearer in the E-RABs Admitted To Be Added List IE in the SGNB ADDITION REQUEST ACKNOWLEDGE the PDCP SN Length IE to indicate the PDCP SN length for that bearer.

-	If the RLC Mode IE is included for an E-RAB within the E-RABs To be Added List IE in the SGNB ADDITION REQUEST message, it indicates the mode that the MeNB used for the E-RAB when it was hosted at the MeNB.

-	If the Bearer Type IE for the concerned E-RAB is received by the en-gNB and is set to "non IP", the en-gNB shall, if supported,  not perform header compression for the concerned E-RAB.

Upon reception of the SGNB ADDITION REQUEST ACKNOWLEDGE message the MeNB shall stop the timer TDCprep.

If the SGNB ADDITION ACKNOWLEDGE message contains the SgNB Resource Coordination Information IE, the MeNB may use it for the purpose of resource coordination with the en-gNB. The MeNB shall consider the received UL Coordination Information IE value valid until reception of a new update of the IE for the same UE. The MeNB shall consider the received DL Coordination Information IE value valid until reception of a new update of the IE for the same UE. If the SgNB Coordination Assistance Information IE is contained in the SgNB Resource Coordination Information IE, the MeNB shall, if supported, use the information to determine further coordination of resource utilisation between the en-gNB and the MeNB.

If the SgNB UE X2AP ID IE is contained in the SGNB ADDITION REQUEST message, the en-gNB shall, if supported, store this information and use it as defined in TS 37.340 [32].

If the SGNB ADDITION REQUEST message contains the SGNB Addition Trigger Indication, the en-gNB shall include the RRC config indication IE in the SGNB ADDITION REQUEST ACKNOWLEDGE message to inform the MeNB if the en-gNB applied full or delta configuration, as specified in TS 37.340 [32].

If the en-gNB receives for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE and the Duplication Activation IE in the SGNB ADDITION REQUEST message, it may provide the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE and the LCID IE to the MeNB in the SGNB ADDITION REQUEST ACKNOWLEDGE message if PDCP duplication is configured at the en-gNB.

If the SGNB ADDITION REQUEST message contains the UL PDCP SN Length IE and the DL PDCP SN Length IE, the en-gNB shall, if supported, store this information and use it for lower layer configuration of the concerned MN terminated bearer.

The SgNB may include the Location Information at SgNB IE in the SGNB ADDITION REQUEST ACKNOWLEDGE message, if respective information is available at the SgNB.

If the Location Information at SgNB Reporting IE set to "pscell" is included in the SGNB ADDITION REQUEST, the SgNB shall start providing information about the current location of the UE. If the Location Information at SgNB IE is included in the SGNB ADDITION REQUEST ACKNOWLEDGE, the MeNB shall store the included information so that it may be transferred towards the MME.

If Trace Activation IE has previously been received for this UE, it shall be included in the SGNB ADDITION REQUEST message. If the Trace Activation IE is included in the SGNB ADDITION REQUEST message, the en-gNB shall, if supported, initiate the requested trace function as described in TS 32.422 [6].

If the UE Context Reference at Source NG-RAN IE is contained in the SGNB ADDITION REQUEST message, the en-gNB shall, if supported, store this information and use it for UE context retrieval and allocate data forwarding resources as specified in TS 37.340 [32].

The MeNB may request the en-gNB to configure the SRB3 by including the Requested Fast MCG recovery via SRB3 IE in the SGNB ADDITION REQUEST message. The en-gNB may include the Available fast MCG recovery via SRB3 IE in the SGNB ADDITION REQUEST ACKNOWLEDGE message to indicate that the SRB3 is enabled.

If the UE Radio Capability ID IE is contained in the SGNB ADDITION REQUEST message, the en-gNB shall, if supported, store this information and use it as specified in TS 23.401 [12].

- Interactions with the MeNB initiated SgNB Modification procedure:

If the en-gNB provides for an E-RAB for which the PDCP entiy is allocated at the MeNB the Secondary SgNB DL GTP Tunnel Endpoint at SCG IE to the MeNB in the SGNB ADDITION REQUEST ACKNOWLEDGE message and the MeNB has not provided the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE in the SGNB ADDITION REQUEST message, the MeNB shall trigger the MeNB initiated SgNB Modification procedure to provide the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE to the SgNB.

- Interactions with the SgNB Reconfiguration Completion procedure:

If the en-gNB admits at least one E-RAB, the en-gNB shall start the timer TDCoverall when sending the SGNB ADDITION REQUEST ACKNOWLEDGE message to the MeNB. The reception of the SGNB RECONFIGURATION COMPLETE message shall stop the timer TDCoverall.

Interaction with the Activity Notification procedure

Upon receiving an SGNB ADDITION REQUEST message containing the Desired Activity Notification Level IE, the en-gNB shall, if supported, use this information to decide whether to trigger subsequent SgNB Activitity Notification procedures according to the requested notification level.

---------------------------- End of 3rd change ---------------------------

---------------------------- Start of 4th change ---------------------------

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source eNB to the target eNB to request the preparation of resources for a handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – |  |
| >UE Security Capabilities | M |  | 9.2.29 |  | – |  |
| >AS Security Information | M |  | 9.2.30 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – |  |
| >E-RABs To Be Setup List |  | 1 |  |  | – |  |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >RRC Context | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9], or the RRC HandoverPreparationInformation-NB message as defined in 10.6.2 of TS 36.331 [9]. | – |  |
| >Handover Restriction List | O |  | 9.2.3 |  | – |  |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – |  |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | Ignore |
| >Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| >EPC Handover Restriction List Container | O |  | 9.2.153 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.x |  | YES | reject |
| UE History Information | M |  | 9.2.38 | Same definition as in TS 36.413 [4] | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source eNB provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| UE Context Reference at the SeNB | O |  |  |  | YES | ignore |
| >Global SeNB ID | M |  | Global eNB ID 9.2.22 |  |  |  |
| >SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB |  |  |
| >SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB |  |  |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| UE Context Reference at the WT | O |  |  |  | YES | ignore |
| >WT ID | M |  | 9.2.95 |  |  |  |
| >WT UE XwAP ID | M |  | 9.2.96 |  |  |  |
| UE Context Reference at the SgNB | O |  |  |  | YES | ignore |
| >Global en-gNB ID | M |  | 9.2.112 |  |  |  |
| >SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the SgNB. | – |  |
| NR UE Security Capabilities | O |  | 9.2.107 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.129 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.136 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

---------------------------- End of 4th change ---------------------------

---------------------------- Start of 5th change ---------------------------

#### 9.1.2.29	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old eNB to transfer the UE context to the new eNB.

Direction: old eNB  new eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| New eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the new eNB | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the new eNB | YES | ignore |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the old eNB | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the old eNB | YES | ignore |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – |  |
| >UE Security Capabilities | M |  | 9.2.29 |  | – |  |
| >AS Security Information | M |  | 9.2.30 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – |  |
| >E-RABs To Be Setup List |  | 1 |  |  | – |  |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>Bearer Type | O |  | 9.2.92 |  | – |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | YES | reject |
| >>>DL Forwarding | O |  | 9.2.5 |  | YES | ignore |
| >RRC Context | M |  | OCTET STRING | Includes either the RRC Handover Preparation Information message as defined in subclause 10.2.2 of TS 36.331 [9], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [9]. | – |  |
| >Handover Restriction List | O |  | 9.2.3 |  | – |  |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – |  |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | – |  |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | – |  |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| >Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| >EPC Handover Restriction List Container | O |  | 9.2.153 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.x |  | YES | reject |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.129 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.136 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

---------------------------- End of 5th change ---------------------------

---------------------------- Start of 6th change ---------------------------

#### 9.1.4.1	SGNB ADDITION REQUEST

This message is sent by the MeNB to the en-gNB to request the preparation of resources for EN-DC operation for a specific UE

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| NR UE Security Capabilities | M |  | 9.2.107 |  | YES | reject |
| SgNB Security Key | M |  | 9.2.101 | The S-KgNB which is provided by the MeNB, see TS 33.401 [18]. | YES | reject |
| SgNB UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.12 | The UE Aggregate Maximum Bit Rate is split into MeNB UE Aggregate Maximum Bit Rate and SgNB UE Aggregate Maximum Bit Rate which are enforced by MeNB and en-gNB respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.4 | The selected PLMN of the SCG in the en-gNB. | YES | ignore |
| Handover Restriction List | O |  | 9.2.3 |  | YES | ignore |
| E-RABs To Be Added List |  | 1 |  |  | YES | reject |
| >E-RABs To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>DRB ID | M |  | 9.2.122 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>Full E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes the E-RAB level QoS parameters as received on S1-MME. | – |  |
| >>>>Maximum MCG admittable E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBR |  | GBR QoS Information 9.2.10 | Includes the GBR QoS Information admittable by the MCG. | – |  |
| >>>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>>MeNB DL GTP Tunnel Endpoint at MCG | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at MCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>S1 UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1-U transport bearer. For delivery of UL PDUs from the en-gNB. | – |  |
| >>>>RLC Mode | O |  | RLC Mode 9.2.119 | Indicates the RLC mode at the MeNB for PDCP transfer to en-gNB. | YES | ignore |
| >>>>Bearer Type | O |  | 9.2.92 |  | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>Requested SCG E-RAB Level QoS Parameters | M |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the SCG. | – |  |
| >>>>MeNB UL GTP Tunnel Endpoint at PDCP | M |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs. | – |  |
| >>>>Secondary MeNB UL GTP Tunnel Endpoint at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer. For delivery of UL PDCP PDUs in case of PDCP duplication. | – |  |
| >>>>RLC Mode | M |  | RLC Mode 9.2.119 | Indicates the RLC mode to be used in the assisting node. | – |  |
| >>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the en-gNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>>Duplication activation | O |  | 9.2.137 | Indicated the initial staus of PDCP duplication. | YES | ignore |
| MeNB to SgNB Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in TS 38.331 [31]. | YES | reject |
| SgNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Requested split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRB are requested. | YES | reject |
| MeNB Resource Coordination Information | O |  | 9.2.116 | Information used to coordinate resources utilisation between MeNB and en-gNB. | YES | ignore |
| SGNB Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-eNB HO, intra-eNB HO, ...) | This IE indicates the trigger for SGNB Addition procedure. | YES | reject |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | YES | ignore |
| MeNB Cell ID | M |  | ECGI 9.2.14 | Indicates the cell ID for PCell in MeNB. | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.141 |  | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| Location Information at SgNB reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s location information is to be provided. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Context Reference at Source NG-RAN | O |  | RAN UE NGAP ID 9.2.152 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.x |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifMCGandSCGpresent | This IE shall be present if, for the E-RAB requested to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present". |
| ifMCGpresent | This IE shall be present if, for the E-RAB requested to be added, the MCG resources IE in the EN-DC Resource Configuration IE is set to the value "present". |
| C-ifMCGandSCGpresent_GBR | This IE shall be present if, for the E-RAB requested to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present", and GBR QoS Information IE is present in Full E-RAB Level QoS Parameters IE. |

---------------------------- End of 6th change ---------------------------

---------------------------- Start of 7th change ---------------------------

### 9.2.x	UE Radio Capability ID

This IE contains the UE Capability ID as defined in TS 23.003[29].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Radio Capability ID | M |  | OCTET STRING | NOTE: IE type is defined in 23.003 [29]. |

---------------------------- End of 7th change ---------------------------

---------------------------- Start of 8th change ---------------------------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

- -- PDU definitions for X2AP.

--

-- **************************************************************

X2AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

- -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ABSInformation,

ABS-Status,

AS-SecurityInformation,

BearerType,

Cause,

CompositeAvailableCapacityGroup,

Correlation-ID,

COUNTvalue,

CellReportingIndicator,

AerialUEsubscriptionInformation,

CriticalityDiagnostics,

CRNTI,

CSGMembershipStatus,

CSG-Id,

DeactivationIndication,

DL-Forwarding,

DynamicDLTransmissionInformation,

ECGI,

E-RAB-ID,

E-RAB-Level-QoS-Parameters,

E-RAB-List,

EUTRANTraceID,

GlobalENB-ID,

GTPtunnelEndpoint,

GUGroupIDList,

GUMMEI,

HandoverReportType,

HandoverRestrictionList,

Masked-IMEISV,

InvokeIndication,

LocationReportingInformation,

LowerLayerPresenceStatusChange,

MDT-Configuration,

ManagementBasedMDTallowed,

MDTPLMNList,

Neighbour-Information,

PCI,

PDCP-SN,

PLMN-Identity,

ReceiveStatusofULPDCPSDUs,

Registration-Request,

RelativeNarrowbandTxPower,

RadioResourceStatus,

RLC-Status,

RRCConnReestabIndicator,

RRCConnSetupIndicator,

UE-RLF-Report-Container,

UEAppLayerMeasConfig,

RRC-Context,

ServedCell-Information,

ServedCells,

ShortMAC-I,

SRVCCOperationPossible,

SubscriberProfileIDforRFP,

TargetCellInUTRAN,

TargeteNBtoSource-eNBTransparentContainer,

TimeToWait,

TraceActivation,

TraceDepth,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-HistoryInformation,

UE-HistoryInformationFromTheUE,

UE-S1AP-ID,

UESecurityCapabilities,

UEsToBeResetList,

UE-X2AP-ID,

UL-HighInterferenceIndicationInfo,

UL-InterferenceOverloadIndication,

HWLoadIndicator,

S1TNLLoadIndicator,

Measurement-ID,

ReportCharacteristics,

MobilityParametersInformation,

MobilityParametersModificationRange,

ReceiveStatusOfULPDCPSDUsExtended,

COUNTValueExtended,

SubframeAssignment,

ExtendedULInterferenceOverloadInfo,

ExpectedUEBehaviour,

SeNBSecurityKey,

MeNBtoSeNBContainer,

SeNBtoMeNBContainer,

SCGChangeIndication,

CoMPInformation,

ReportingPeriodicityRSRPMR,

RSRPMRList,

UE-RLF-Report-Container-for-extended-bands,

ProSeAuthorized,

CoverageModificationList,

ReportingPeriodicityCSIR,

CSIReportList,

ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

COUNTvaluePDCP-SNlength18,

LHN-ID,

UE-ContextKeptIndicator,

UE-X2AP-ID-Extension,

SIPTOBearerDeactivationIndication,

TunnelInformation,

V2XServicesAuthorized,

X2BenefitValue,

ResumeID,

EUTRANCellIdentifier,

MakeBeforeBreakIndicator,

WTID,

WT-UE-XwAP-ID,

UESidelinkAggregateMaximumBitRate,

SgNBSecurityKey,

MeNBtoSgNBContainer,

SgNBtoMeNBContainer,

SplitSRBs,

RRCContainer,

SRBType,

GlobalGNB-ID,

GNB-ID,

SCGConfigurationQuery,

SplitSRB,

NRUeReport,

EN-DC-ResourceConfiguration,

TAC,

NRFreqInfo,

NRCGI,

NRPCI,

NRUESecurityCapabilities,

PDCPChangeIndication,

ULConfiguration,

SgNB-UE-X2AP-ID,

SecondaryRATUsageReportList,

ActivationID,

MeNBResourceCoordinationInformation,

SgNBResourceCoordinationInformation,

NR-TxBW,

BroadcastPLMNs-Item,

AdditionalPLMNs-Item,

RLCMode,

GBR-QosInformation,

DRB-ID,

FiveGS-TAC,

SULInformation,

Packet-LossRate,

ResourceType,

DataTrafficResourceIndication,

SpectrumSharingGroupID,

RRC-Config-Ind,

SGNB-Addition-Trigger-Ind,

UserPlaneTrafficActivityReport,

ERABActivityNotifyItemList,

PDCPSnLength,

Subscription-Based-UE-DifferentiationInfo,

LCID,

DuplicationActivation,

GNBOverloadInformation,

NewDRBIDrequest,

DesiredActNotificationLevel,

LocationInformationSgNB,

LocationInformationSgNBReporting,

EndcSONConfigurationTransfer,

NRNeighbour-Information,

InterfaceInstanceIndication,

BPLMN-ID-Info-NR,

EPCHandoverRestrictionListContainer,

AdditionalRRMPriorityIndex,

RequestedFastMCGRecoveryViaSRB3,

AvailableFastMCGRecoveryViaSRB3,

RequestedFastMCGRecoveryViaSRB3Release,

ReleaseFastMCGRecoveryViaSRB3,

FastMCGRecovery,

PartialListIndicator,

MaximumCellListSize,

MessageOversizeNotification,

TNLConfigurationInfo,

TNLA-To-Add-List,

TNLA-To-Update-List,

TNLA-To-Remove-List,

TNLA-Setup-List,

TNLA-Failed-To-Setup-List,

RAN-UE-NGAP-ID,

UERadioCapabilityID

FROM X2AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

X2AP-PRIVATE-IES,

X2AP-PROTOCOL-EXTENSION,

X2AP-PROTOCOL-IES,

X2AP-PROTOCOL-IES-PAIR

FROM X2AP-Containers

id-ABSInformation,

id-ActivatedCellList,

id-BearerType,

id-Cause,

id-CellInformation,

id-CellInformation-Item,

id-CellMeasurementResult,

id-CellMeasurementResult-Item,

id-CellToReport,

id-CellToReport-Item,

id-CompositeAvailableCapacityGroup,

id-AerialUEsubscriptionInformation,

id-CriticalityDiagnostics,

id-DeactivationIndication,

id-DynamicDLTransmissionInformation,

id-E-RABs-Admitted-Item,

id-E-RABs-Admitted-List,

id-E-RABs-NotAdmitted-List,

id-E-RABs-SubjectToStatusTransfer-List,

id-E-RABs-SubjectToStatusTransfer-Item,

id-E-RABs-ToBeSetup-Item,

id-GlobalENB-ID,

id-GUGroupIDList,

id-GUGroupIDToAddList,

id-GUGroupIDToDeleteList,

id-GUMMEI-ID,

id-Masked-IMEISV,

id-InvokeIndication,

id-New-eNB-UE-X2AP-ID,

id-Old-eNB-UE-X2AP-ID,

id-Registration-Request,

id-ReportingPeriodicity,

id-RLC-Status,

id-ServedCells,

id-ServedCellsToActivate,

id-ServedCellsToAdd,

id-ServedCellsToModify,

id-ServedCellsToDelete,

id-SRVCCOperationPossible,

id-TargetCell-ID,

id-TargeteNBtoSource-eNBTransparentContainer,

id-TimeToWait,

id-TraceActivation,

id-UE-ContextInformation,

id-UE-HistoryInformation,

id-UE-X2AP-ID,

id-Measurement-ID,

id-ReportCharacteristics,

id-ENB1-Measurement-ID,

id-ENB2-Measurement-ID,

id-ENB1-Cell-ID,

id-ENB2-Cell-ID,

id-ENB2-Proposed-Mobility-Parameters,

id-ENB1-Mobility-Parameters,

id-ENB2-Mobility-Parameters-Modification-Range,

id-FailureCellPCI,

id-Re-establishmentCellECGI,

id-FailureCellCRNTI,

id-ShortMAC-I,

id-SourceCellECGI,

id-FailureCellECGI,

id-HandoverReportType,

id-UE-RLF-Report-Container,

id-PartialSuccessIndicator,

id-MeasurementInitiationResult-List,

id-MeasurementInitiationResult-Item,

id-MeasurementFailureCause-Item,

id-CompleteFailureCauseInformation-List,

id-CompleteFailureCauseInformation-Item,

id-CSGMembershipStatus,

id-CSG-Id,

id-MDTConfiguration,

id-ManagementBasedMDTallowed,

id-ABS-Status,

id-RRCConnSetupIndicator,

id-RRCConnReestabIndicator,

id-TargetCellInUTRAN,

id-MobilityInformation,

id-SourceCellCRNTI,

id-ManagementBasedMDTPLMNList,

id-ReceiveStatusOfULPDCPSDUsExtended,

id-ULCOUNTValueExtended,

id-DLCOUNTValueExtended,

id-IntendedULDLConfiguration,

id-ExtendedULInterferenceOverloadInfo,

id-RNL-Header,

id-x2APMessage,

id-UE-HistoryInformationFromTheUE,

id-ExpectedUEBehaviour,

id-MeNB-UE-X2AP-ID,

id-SeNB-UE-X2AP-ID,

id-UE-SecurityCapabilities,

id-SeNBSecurityKey,

id-SeNBUEAggregateMaximumBitRate,

id-ServingPLMN,

id-E-RABs-ToBeAdded-List,

id-E-RABs-ToBeAdded-Item,

id-MeNBtoSeNBContainer,

id-E-RABs-Admitted-ToBeAdded-List,

id-E-RABs-Admitted-ToBeAdded-Item,

id-SeNBtoMeNBContainer,

id-ResponseInformationSeNBReconfComp,

id-UE-ContextInformationSeNBModReq,

id-E-RABs-ToBeAdded-ModReqItem,

id-E-RABs-ToBeModified-ModReqItem,

id-E-RABs-ToBeReleased-ModReqItem,

id-E-RABs-Admitted-ToBeAdded-ModAckList,

id-E-RABs-Admitted-ToBeModified-ModAckList,

id-E-RABs-Admitted-ToBeReleased-ModAckList,

id-E-RABs-Admitted-ToBeAdded-ModAckItem,

id-E-RABs-Admitted-ToBeModified-ModAckItem,

id-E-RABs-Admitted-ToBeReleased-ModAckItem,

id-SCGChangeIndication,

id-E-RABs-ToBeReleased-ModReqd,

id-E-RABs-ToBeReleased-ModReqdItem,

id-E-RABs-ToBeReleased-List-RelReq,

id-E-RABs-ToBeReleased-RelReqItem,

id-E-RABs-ToBeReleased-List-RelConf,

id-E-RABs-ToBeReleased-RelConfItem,

id-E-RABs-SubjectToCounterCheck-List,

id-E-RABs-SubjectToCounterCheckItem,

id-CoMPInformation,

id-ReportingPeriodicityRSRPMR,

id-RSRPMRList,

id-UE-RLF-Report-Container-for-extended-bands,

id-ProSeAuthorized,

id-CoverageModificationList,

id-ReportingPeriodicityCSIR,

id-CSIReportList,

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18,

id-ULCOUNTValuePDCP-SNlength18,

id-DLCOUNTValuePDCP-SNlength18,

id-LHN-ID,

id-Correlation-ID,

id-SIPTO-Correlation-ID,

id-UE-ContextReferenceAtSeNB,

id-UE-ContextReferenceAtWT,

id-UE-ContextKeptIndicator,

id-UEs-ToBeReset,

id-UEs-Admitted-ToBeReset,

id-WT-UE-ContextKeptIndicator,

id-New-eNB-UE-X2AP-ID-Extension,

id-Old-eNB-UE-X2AP-ID-Extension,

id-MeNB-UE-X2AP-ID-Extension,

id-SeNB-UE-X2AP-ID-Extension,

id-SIPTO-BearerDeactivationIndication,

id-Tunnel-Information-for-BBF,

id-SIPTO-L-GW-TransportLayerAddress,

id-GW-TransportLayerAddress,

id-X2RemovalThreshold,

id-CellReportingIndicator,

id-V2XServicesAuthorized,

id-resumeID,

id-UE-ContextInformationRetrieve,

id-E-RABs-ToBeSetupRetrieve-Item,

id-NewEUTRANCellIdentifier,

id-MakeBeforeBreakIndicator,

id-UESidelinkAggregateMaximumBitRate,

id-uL-GTPtunnelEndpoint,

id-SgNBSecurityKey,

id-SgNBUEAggregateMaximumBitRate,

id-E-RABs-ToBeAdded-SgNBAddReqList,

id-MeNBtoSgNBContainer,

id-SgNB-UE-X2AP-ID,

id-RequestedSplitSRBs,

id-E-RABs-ToBeAdded-SgNBAddReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList,

id-SgNBtoMeNBContainer,

id-AdmittedSplitSRBs,

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item,

id-ResponseInformationSgNBReconfComp,

id-UE-ContextInformation-SgNBModReq,

id-E-RABs-ToBeAdded-SgNBModReq-Item,

id-E-RABs-ToBeModified-SgNBModReq-Item,

id-E-RABs-ToBeReleased-SgNBModReq-Item,

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList,

id-E-RABs-Admitted-ToBeModified-SgNBModAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList,

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item,

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList,

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item,

id-E-RABs-ToBeReleased-SgNBModReqdList,

id-E-RABs-ToBeModified-SgNBModReqdList,

id-E-RABs-ToBeReleased-SgNBModReqd-Item,

id-E-RABs-ToBeModified-SgNBModReqd-Item,

id-E-RABs-ToBeReleased-SgNBChaConfList,

id-E-RABs-ToBeReleased-SgNBChaConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqList,

id-E-RABs-ToBeReleased-SgNBRelReq-Item,

id-E-RABs-ToBeReleased-SgNBRelConfList,

id-E-RABs-ToBeReleased-SgNBRelConf-Item,

id-E-RABs-ToBeReleased-SgNBRelReqdList,

id-E-RABs-ToBeReleased-SgNBRelReqd-Item,

id-E-RABs-SubjectToSgNBCounterCheck-List,

id-E-RABs-SubjectToSgNBCounterCheck-Item,

id-Target-SgNB-ID,

id-RRCContainer,

id-SRBType,

id-HandoverRestrictionList,

id-SCGConfigurationQuery,

id-SplitSRB,

id-NRUeReport,

id-InitiatingNodeType-EndcX2Setup,

id-InitiatingNodeType-EndcConfigUpdate,

id-RespondingNodeType-EndcX2Setup,

id-RespondingNodeType-EndcConfigUpdate,

id-NRUESecurityCapabilities,

id-PDCPChangeIndication,

id-ServedEUTRAcellsENDCX2ManagementList,

id-ServedEUTRAcellsToModifyListENDCConfUpd,

id-ServedEUTRAcellsToDeleteListENDCConfUpd,

id-ServedNRcellsToModifyListENDCConfUpd,

id-ServedNRcellsToDeleteListENDCConfUpd,

id-CellAssistanceInformation,

id-Globalen-gNB-ID,

id-ServedNRcellsENDCX2ManagementList,

id-Old-SgNB-UE-X2AP-ID,

id-UE-ContextReferenceAtSgNB,

id-SecondaryRATUsageReportList,

id-ActivationID,

id-ServedNRCellsToActivate,

id-ActivatedNRCellList,

id-MeNBResourceCoordinationInformation,

id-SgNBResourceCoordinationInformation,

id-UEAppLayerMeasConfig,

id-SelectedPLMN,

id-SubscriberProfileIDforRFP,

id-InitiatingNodeType-EutranrCellResourceCoordination,

id-RespondingNodeType-EutranrCellResourceCoordination,

id-DataTrafficResourceIndication,

id-SpectrumSharingGroupID,

id-ListofEUTRACellsinEUTRACoordinationReq,

id-ListofEUTRACellsinEUTRACoordinationResp,

id-ListofEUTRACellsinNRCoordinationReq,

id-ListofNRCellsinNRCoordinationReq,

id-ListofNRCellsinNRCoordinationResp,

id-RRCConfigIndication,

id-SGNB-Addition-Trigger-Ind,

id-RequestedSplitSRBsrelease,

id-AdmittedSplitSRBsrelease,

id-E-RABs-AdmittedToBeModified-SgNBModConfList,

id-E-RABs-AdmittedToBeModified-SgNBModConf-Item,

id-UEContextLevelUserPlaneActivity,

id-ERABActivityNotifyItemList,

id-MeNBCell-ID,

id-InitiatingNodeType-EndcX2Removal,

id-RespondingNodeType-EndcX2Removal,

id-uLpDCPSnLength,

id-dL-Forwarding,

id-E-RABs-DataForwardingAddress-List,

id-E-RABs-DataForwardingAddress-Item,

id-Subscription-Based-UE-DifferentiationInfo,

id-RLCMode-transferred,

id-dLPDCPSnLength,

id-secondarysgNBDLGTPTEIDatPDCP,

id-secondarymeNBULGTPTEIDatPDCP,

id-lCID,

id-duplicationActivation,

id-GNBOverloadInformation,

id-new-drb-ID-req,

id-NRNeighbourInfoToModify,

id-DesiredActNotificationLevel,

id-LocationInformationSgNB,

id-LocationInformationSgNBReporting,

id-endcSONConfigurationTransfer,

id-EUTRANTraceID,

id-additionalPLMNs-Item,

id-InterfaceInstanceIndication,

id-BPLMN-ID-Info-NR,

id-EPCHandoverRestrictionListContainer,

id-ERABs-transferred-to-MeNB,

id-AdditionalRRMPriorityIndex,

id-LowerLayerPresenceStatusChange,

id-FastMCGRecovery-SN-to-MN,

id-FastMCGRecovery-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-AvailableFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-ReleaseFastMCGRecoveryViaSRB3,

id-PartialListIndicator,

id-MaximumCellListSize,

id-MessageOversizeNotification,

id-CellandCapacityAssistInfo,

id-TNLConfigurationInfo,

id-TNLA-To-Add-List,

id-TNLA-To-Update-List,

id-TNLA-To-Remove-List,

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-UEContextReferenceatSourceNGRAN,

id-UERadioCapabilityID,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands

FROM X2AP-Constants;

-- **************************************************************

--

- -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-Old-eNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory}|

{ ID id-TargetCell-ID								CRITICALITY reject	TYPE ECGI													PRESENCE mandatory}|

{ ID id-GUMMEI-ID									CRITICALITY reject	TYPE GUMMEI													PRESENCE mandatory}|

{ ID id-UE-ContextInformation						CRITICALITY reject	TYPE UE-ContextInformation								PRESENCE mandatory}|

{ ID id-UE-HistoryInformation						CRITICALITY ignore	TYPE UE-HistoryInformation								PRESENCE mandatory}|

{ ID id-TraceActivation								CRITICALITY ignore	TYPE TraceActivation										PRESENCE optional}|

{ ID id-SRVCCOperationPossible						CRITICALITY ignore	TYPE SRVCCOperationPossible								PRESENCE optional}|

{ ID id-CSGMembershipStatus							CRITICALITY reject	TYPE CSGMembershipStatus									PRESENCE optional}|

{ ID id-MobilityInformation							CRITICALITY ignore	TYPE MobilityInformation									PRESENCE optional}|

{ ID id-Masked-IMEISV								CRITICALITY ignore	TYPE Masked-IMEISV											PRESENCE optional}|

{ ID id-UE-HistoryInformationFromTheUE				CRITICALITY ignore	TYPE UE-HistoryInformationFromTheUE					PRESENCE optional}|

{ ID id-ExpectedUEBehaviour							CRITICALITY ignore	TYPE ExpectedUEBehaviour									PRESENCE optional}|

{ ID id-ProSeAuthorized								CRITICALITY ignore	TYPE ProSeAuthorized										PRESENCE optional}|

{ ID id-UE-ContextReferenceAtSeNB					CRITICALITY ignore	TYPE UE-ContextReferenceAtSeNB							PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-V2XServicesAuthorized						CRITICALITY ignore	TYPE V2XServicesAuthorized								PRESENCE optional}|

{ ID id-UE-ContextReferenceAtWT						CRITICALITY ignore	TYPE UE-ContextReferenceAtWT							PRESENCE optional}|

{ ID id-NRUESecurityCapabilities						CRITICALITY ignore	TYPE NRUESecurityCapabilities							PRESENCE optional}|

{ ID id-UE-ContextReferenceAtSgNB					CRITICALITY ignore	TYPE UE-ContextReferenceAtSgNB							PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation				CRITICALITY ignore	TYPE AerialUEsubscriptionInformation					PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo	CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo	PRESENCE optional},

...

}

UE-ContextInformation ::= SEQUENCE {

mME-UE-S1AP-ID						UE-S1AP-ID,

uESecurityCapabilities				UESecurityCapabilities,

aS-SecurityInformation				AS-SecurityInformation,

uEaggregateMaximumBitRate			UEAggregateMaximumBitRate,

subscriberProfileIDforRFP			SubscriberProfileIDforRFP		OPTIONAL,

e-RABs-ToBeSetup-List				E-RABs-ToBeSetup-List,

rRC-Context							RRC-Context,

handoverRestrictionList				HandoverRestrictionList		OPTIONAL,

locationReportingInformation		LocationReportingInformation	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {UE-ContextInformation-ExtIEs} } OPTIONAL,

...

}

UE-ContextInformation-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-ManagementBasedMDTallowed			CRITICALITY ignore	EXTENSION ManagementBasedMDTallowed					PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore	EXTENSION MDTPLMNList									PRESENCE optional }|

{ ID id-UESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION UESidelinkAggregateMaximumBitRate		PRESENCE optional }|

{ ID id-EPCHandoverRestrictionListContainer CRITICALITY ignore	EXTENSION EPCHandoverRestrictionListContainer		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex					PRESENCE optional }|

{ ID id-UERadioCapabilityID					CRITICALITY reject	EXTENSION UERadioCapabilityID							PRESENCE optional },

...

}

E-RABs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeSetup-ItemIEs} }

---------------------------- End of 8th change ---------------------------

---------------------------- Start of 9th change ---------------------------

-- **************************************************************

--

- -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-New-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-New-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-Old-eNB-UE-X2AP-ID							CRITICALITY ignore	TYPE UE-X2AP-ID												PRESENCE mandatory}|

{ ID id-Old-eNB-UE-X2AP-ID-Extension				CRITICALITY ignore	TYPE UE-X2AP-ID-Extension								PRESENCE optional}|

{ ID id-GUMMEI-ID									CRITICALITY reject	TYPE GUMMEI													PRESENCE mandatory}|

{ ID id-UE-ContextInformationRetrieve				CRITICALITY reject	TYPE UE-ContextInformationRetrieve					PRESENCE mandatory}|

{ ID id-TraceActivation								CRITICALITY ignore	TYPE TraceActivation										PRESENCE optional}|

{ ID id-SRVCCOperationPossible						CRITICALITY ignore	TYPE SRVCCOperationPossible								PRESENCE optional}|

{ ID id-Masked-IMEISV								CRITICALITY ignore	TYPE Masked-IMEISV											PRESENCE optional}|

{ ID id-ExpectedUEBehaviour							CRITICALITY ignore	TYPE ExpectedUEBehaviour									PRESENCE optional}|

{ ID id-ProSeAuthorized								CRITICALITY ignore	TYPE ProSeAuthorized										PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional}|

{ ID id-V2XServicesAuthorized						CRITICALITY ignore	TYPE V2XServicesAuthorized								PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation				CRITICALITY ignore	TYPE AerialUEsubscriptionInformation					PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo	CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo	PRESENCE optional},

...

}

UE-ContextInformationRetrieve ::= SEQUENCE {

mME-UE-S1AP-ID						UE-S1AP-ID,

uESecurityCapabilities				UESecurityCapabilities,

aS-SecurityInformation				AS-SecurityInformation,

uEaggregateMaximumBitRate			UEAggregateMaximumBitRate,

subscriberProfileIDforRFP			SubscriberProfileIDforRFP		OPTIONAL,

e-RABs-ToBeSetup-ListRetrieve		E-RABs-ToBeSetup-ListRetrieve,

rRC-Context							RRC-Context,

handoverRestrictionList				HandoverRestrictionList			OPTIONAL,

locationReportingInformation		LocationReportingInformation	OPTIONAL,

managBasedMDTallowed				ManagementBasedMDTallowed		OPTIONAL,

managBasedMDTPLMNList				MDTPLMNList						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {UE-ContextInformationRetrieve-ExtIEs} } OPTIONAL,

...

}

UE-ContextInformationRetrieve-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

{ ID id-UESidelinkAggregateMaximumBitRate	CRITICALITY ignore	EXTENSION UESidelinkAggregateMaximumBitRate		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex					PRESENCE optional }|

{ ID id-EPCHandoverRestrictionListContainer CRITICALITY ignore	EXTENSION EPCHandoverRestrictionListContainer		PRESENCE optional }|

{ ID id-UERadioCapabilityID					CRITICALITY reject	EXTENSION UERadioCapabilityID							PRESENCE optional },

...

}

E-RABs-ToBeSetup-ListRetrieve ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeSetupRetrieve-ItemIEs} }

---------------------------- End of 9th change ---------------------------

---------------------------- Start of 10th change ---------------------------

-- **************************************************************

--

- -- SGNB ADDITION REQUEST

--

-- **************************************************************

SgNBAdditionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container {{SgNBAdditionRequest-IEs}},

...

}

SgNBAdditionRequest-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY reject	TYPE UE-X2AP-ID										PRESENCE mandatory}|

{ ID id-NRUESecurityCapabilities					CRITICALITY reject	TYPE NRUESecurityCapabilities					PRESENCE mandatory}|

{ ID id-SgNBSecurityKey							CRITICALITY reject	TYPE SgNBSecurityKey									PRESENCE mandatory}|

{ ID id-SgNBUEAggregateMaximumBitRate			CRITICALITY reject	TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-SelectedPLMN								CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional}|

{ ID id-HandoverRestrictionList					CRITICALITY ignore	TYPE HandoverRestrictionList						PRESENCE optional}|

{ ID id-E-RABs-ToBeAdded-SgNBAddReqList			CRITICALITY reject	TYPE E-RABs-ToBeAdded-SgNBAddReqList			PRESENCE mandatory}|

{ ID id-MeNBtoSgNBContainer						CRITICALITY reject	TYPE MeNBtoSgNBContainer								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY reject	TYPE SgNB-UE-X2AP-ID									PRESENCE optional}|

{ ID id-ExpectedUEBehaviour						CRITICALITY ignore	TYPE ExpectedUEBehaviour								PRESENCE optional}|

{ ID id-MeNB-UE-X2AP-ID-Extension				CRITICALITY reject	TYPE UE-X2AP-ID-Extension							PRESENCE optional}|

{ ID id-RequestedSplitSRBs						CRITICALITY reject	TYPE SplitSRBs										PRESENCE optional}|

{ ID id-MeNBResourceCoordinationInformation		CRITICALITY ignore	TYPE MeNBResourceCoordinationInformation		PRESENCE optional}|

{ ID id-SGNB-Addition-Trigger-Ind				CRITICALITY reject	TYPE SGNB-Addition-Trigger-Ind					PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP				CRITICALITY ignore	TYPE SubscriberProfileIDforRFP					PRESENCE optional}|

{ ID id-MeNBCell-ID								CRITICALITY reject	TYPE ECGI											PRESENCE mandatory}|

{ ID id-DesiredActNotificationLevel				CRITICALITY ignore	TYPE DesiredActNotificationLevel				PRESENCE optional}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional}|

{ ID id-LocationInformationSgNBReporting			CRITICALITY ignore	TYPE LocationInformationSgNBReporting			PRESENCE optional}|

{ ID id-Masked-IMEISV							CRITICALITY ignore	TYPE Masked-IMEISV									PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex					PRESENCE optional}|

{ ID id-RequestedFastMCGRecoveryViaSRB3			CRITICALITY ignore	TYPE	 RequestedFastMCGRecoveryViaSRB3			PRESENCE optional}|

{ ID id-UEContextReferenceatSourceNGRAN			CRITICALITY ignore	TYPE RAN-UE-NGAP-ID									PRESENCE optional}|

{ ID id-UERadioCapabilityID						CRITICALITY reject	TYPE UERadioCapabilityID							PRESENCE optional},

...

}

E-RABs-ToBeAdded-SgNBAddReqList ::= SEQUENCE (SIZE(1..maxnoofBearers)) OF ProtocolIE-Single-Container { {E-RABs-ToBeAdded-SgNBAddReq-ItemIEs} }

---------------------------- End of 10th change ---------------------------

---------------------------- Start of 11th change ---------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

X2AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

*** skipped unchanged part

UEID ::= BIT STRING (SIZE (16))

UE-HistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCells)) OF LastVisitedCell-Item

UE-HistoryInformationFromTheUE ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the VisitedCellInfoList field contained in the UEInformationResponse message as defined in TS 36.331 [9]

UE-S1AP-ID ::= INTEGER (0.. 4294967295)

UE-X2AP-ID ::= INTEGER (0..4095)

UE-X2AP-ID-Extension ::= INTEGER (0..4095, ...)

UERadioCapabilityID ::= OCTET STRING

UE-RLF-Report-Container::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-r9 field contained in the UEInformationResponse message as defined in TS 36.331 [9]

UE-RLF-Report-Container-for-extended-bands ::= OCTET STRING

-- This IE is a transparent container and shall be encoded as the RLF-Report-v9e0 field contained in the UEInformationResponse message as defined in TS 36.331 [9]

---------------------------- End of 11th change ---------------------------

---------------------------- Start of 12th change ---------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

- -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

*** skipped unchanged part

-- **************************************************************

--

- -- IEs

--

-- **************************************************************

id-E-RABs-Admitted-Item														ProtocolIE-ID ::= 0

id-E-RABs-Admitted-List														ProtocolIE-ID ::= 1

id-E-RAB-Item																ProtocolIE-ID ::= 2

id-E-RABs-NotAdmitted-List													ProtocolIE-ID ::= 3

id-E-RABs-ToBeSetup-Item													ProtocolIE-ID ::= 4

id-Cause																	ProtocolIE-ID ::= 5

id-CellInformation															ProtocolIE-ID ::= 6

id-CellInformation-Item														ProtocolIE-ID ::= 7

id-New-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 9

id-Old-eNB-UE-X2AP-ID														ProtocolIE-ID ::= 10

id-TargetCell-ID															ProtocolIE-ID ::= 11

id-TargeteNBtoSource-eNBTransparentContainer								ProtocolIE-ID ::= 12

id-TraceActivation															ProtocolIE-ID ::= 13

id-UE-ContextInformation													ProtocolIE-ID ::= 14

id-UE-HistoryInformation													ProtocolIE-ID ::= 15

id-UE-X2AP-ID																ProtocolIE-ID ::= 16

id-CriticalityDiagnostics													ProtocolIE-ID ::= 17

id-E-RABs-SubjectToStatusTransfer-List										ProtocolIE-ID ::= 18

id-E-RABs-SubjectToStatusTransfer-Item										ProtocolIE-ID ::= 19

id-ServedCells																ProtocolIE-ID ::= 20

id-GlobalENB-ID																ProtocolIE-ID ::= 21

id-TimeToWait																ProtocolIE-ID ::= 22

id-GUMMEI-ID																ProtocolIE-ID ::= 23

id-GUGroupIDList															ProtocolIE-ID ::= 24

id-ServedCellsToAdd															ProtocolIE-ID ::= 25

id-ServedCellsToModify														ProtocolIE-ID ::= 26

id-ServedCellsToDelete														ProtocolIE-ID ::= 27

id-Registration-Request														ProtocolIE-ID ::= 28

id-CellToReport																ProtocolIE-ID ::= 29

id-ReportingPeriodicity														ProtocolIE-ID ::= 30

id-CellToReport-Item														ProtocolIE-ID ::= 31

id-CellMeasurementResult													ProtocolIE-ID ::= 32

id-CellMeasurementResult-Item												ProtocolIE-ID ::= 33

id-GUGroupIDToAddList														ProtocolIE-ID ::= 34

id-GUGroupIDToDeleteList													ProtocolIE-ID ::= 35

id-SRVCCOperationPossible													ProtocolIE-ID ::= 36

id-Measurement-ID															ProtocolIE-ID ::= 37

id-ReportCharacteristics													ProtocolIE-ID ::= 38

id-ENB1-Measurement-ID														ProtocolIE-ID ::= 39

id-ENB2-Measurement-ID														ProtocolIE-ID ::= 40

id-Number-of-Antennaports													ProtocolIE-ID ::= 41

id-CompositeAvailableCapacityGroup											ProtocolIE-ID ::= 42

id-ENB1-Cell-ID																ProtocolIE-ID ::= 43

id-ENB2-Cell-ID																ProtocolIE-ID ::= 44

id-ENB2-Proposed-Mobility-Parameters										ProtocolIE-ID ::= 45

id-ENB1-Mobility-Parameters													ProtocolIE-ID ::= 46

id-ENB2-Mobility-Parameters-Modification-Range								ProtocolIE-ID ::= 47

id-FailureCellPCI															ProtocolIE-ID ::= 48

id-Re-establishmentCellECGI													ProtocolIE-ID ::= 49

id-FailureCellCRNTI															ProtocolIE-ID ::= 50

id-ShortMAC-I																ProtocolIE-ID ::= 51

id-SourceCellECGI															ProtocolIE-ID ::= 52

id-FailureCellECGI															ProtocolIE-ID ::= 53

id-HandoverReportType														ProtocolIE-ID ::= 54

id-PRACH-Configuration														ProtocolIE-ID ::= 55

id-MBSFN-Subframe-Info														ProtocolIE-ID ::= 56

id-ServedCellsToActivate													ProtocolIE-ID ::= 57

id-ActivatedCellList														ProtocolIE-ID ::= 58

id-DeactivationIndication													ProtocolIE-ID ::= 59

id-UE-RLF-Report-Container													ProtocolIE-ID ::= 60

id-ABSInformation															ProtocolIE-ID ::= 61

id-InvokeIndication															ProtocolIE-ID ::= 62

id-ABS-Status																ProtocolIE-ID ::= 63

id-PartialSuccessIndicator													ProtocolIE-ID ::= 64

id-MeasurementInitiationResult-List											ProtocolIE-ID ::= 65

id-MeasurementInitiationResult-Item											ProtocolIE-ID ::= 66

id-MeasurementFailureCause-Item												ProtocolIE-ID ::= 67

id-CompleteFailureCauseInformation-List										ProtocolIE-ID ::= 68

id-CompleteFailureCauseInformation-Item										ProtocolIE-ID ::= 69

id-CSG-Id																	ProtocolIE-ID ::= 70

id-CSGMembershipStatus														ProtocolIE-ID ::= 71

id-MDTConfiguration															ProtocolIE-ID ::= 72

id-ManagementBasedMDTallowed												ProtocolIE-ID ::= 74

id-RRCConnSetupIndicator													ProtocolIE-ID ::= 75

id-NeighbourTAC																ProtocolIE-ID ::= 76

id-Time-UE-StayedInCell-EnhancedGranularity									ProtocolIE-ID ::= 77

id-RRCConnReestabIndicator													ProtocolIE-ID ::= 78

id-MBMS-Service-Area-List													ProtocolIE-ID ::= 79

id-HO-cause																	ProtocolIE-ID ::= 80

id-TargetCellInUTRAN														ProtocolIE-ID ::= 81

id-MobilityInformation														ProtocolIE-ID ::= 82

id-SourceCellCRNTI															ProtocolIE-ID ::= 83

id-MultibandInfoList														ProtocolIE-ID ::= 84

id-M3Configuration															ProtocolIE-ID ::= 85

id-M4Configuration															ProtocolIE-ID ::= 86

id-M5Configuration															ProtocolIE-ID ::= 87

id-MDT-Location-Info														ProtocolIE-ID ::= 88

id-ManagementBasedMDTPLMNList												ProtocolIE-ID ::= 89

id-SignallingBasedMDTPLMNList												ProtocolIE-ID ::= 90

id-ReceiveStatusOfULPDCPSDUsExtended										ProtocolIE-ID ::= 91

id-ULCOUNTValueExtended														ProtocolIE-ID ::= 92

id-DLCOUNTValueExtended														ProtocolIE-ID ::= 93

id-eARFCNExtension															ProtocolIE-ID ::= 94

id-UL-EARFCNExtension														ProtocolIE-ID ::= 95

id-DL-EARFCNExtension														ProtocolIE-ID ::= 96

id-AdditionalSpecialSubframe-Info											ProtocolIE-ID ::= 97

id-Masked-IMEISV															ProtocolIE-ID ::= 98

id-IntendedULDLConfiguration												ProtocolIE-ID ::= 99

id-ExtendedULInterferenceOverloadInfo										ProtocolIE-ID ::= 100

id-RNL-Header																ProtocolIE-ID ::= 101

id-x2APMessage																ProtocolIE-ID ::= 102

id-ProSeAuthorized															ProtocolIE-ID ::= 103

id-ExpectedUEBehaviour														ProtocolIE-ID ::= 104

id-UE-HistoryInformationFromTheUE											ProtocolIE-ID ::= 105

id-DynamicDLTransmissionInformation											ProtocolIE-ID ::= 106

id-UE-RLF-Report-Container-for-extended-bands								ProtocolIE-ID ::= 107

id-CoMPInformation															ProtocolIE-ID ::= 108

id-ReportingPeriodicityRSRPMR												ProtocolIE-ID ::= 109

id-RSRPMRList																ProtocolIE-ID ::= 110

id-MeNB-UE-X2AP-ID															ProtocolIE-ID ::= 111

id-SeNB-UE-X2AP-ID															ProtocolIE-ID ::= 112

id-UE-SecurityCapabilities													ProtocolIE-ID ::= 113

id-SeNBSecurityKey															ProtocolIE-ID ::= 114

id-SeNBUEAggregateMaximumBitRate											ProtocolIE-ID ::= 115

id-ServingPLMN																ProtocolIE-ID ::= 116

id-E-RABs-ToBeAdded-List													ProtocolIE-ID ::= 117

id-E-RABs-ToBeAdded-Item													ProtocolIE-ID ::= 118

id-MeNBtoSeNBContainer														ProtocolIE-ID ::= 119

id-E-RABs-Admitted-ToBeAdded-List											ProtocolIE-ID ::= 120

id-E-RABs-Admitted-ToBeAdded-Item											ProtocolIE-ID ::= 121

id-SeNBtoMeNBContainer														ProtocolIE-ID ::= 122

id-ResponseInformationSeNBReconfComp										ProtocolIE-ID ::= 123

id-UE-ContextInformationSeNBModReq											ProtocolIE-ID ::= 124

id-E-RABs-ToBeAdded-ModReqItem												ProtocolIE-ID ::= 125

id-E-RABs-ToBeModified-ModReqItem											ProtocolIE-ID ::= 126

id-E-RABs-ToBeReleased-ModReqItem											ProtocolIE-ID ::= 127

id-E-RABs-Admitted-ToBeAdded-ModAckList										ProtocolIE-ID ::= 128

id-E-RABs-Admitted-ToBeModified-ModAckList									ProtocolIE-ID ::= 129

id-E-RABs-Admitted-ToBeReleased-ModAckList									ProtocolIE-ID ::= 130

id-E-RABs-Admitted-ToBeAdded-ModAckItem										ProtocolIE-ID ::= 131

id-E-RABs-Admitted-ToBeModified-ModAckItem									ProtocolIE-ID ::= 132

id-E-RABs-Admitted-ToBeReleased-ModAckItem									ProtocolIE-ID ::= 133

id-E-RABs-ToBeReleased-ModReqd												ProtocolIE-ID ::= 134

id-E-RABs-ToBeReleased-ModReqdItem											ProtocolIE-ID ::= 135

id-SCGChangeIndication														ProtocolIE-ID ::= 136

id-E-RABs-ToBeReleased-List-RelReq											ProtocolIE-ID ::= 137

id-E-RABs-ToBeReleased-RelReqItem											ProtocolIE-ID ::= 138

id-E-RABs-ToBeReleased-List-RelConf											ProtocolIE-ID ::= 139

id-E-RABs-ToBeReleased-RelConfItem											ProtocolIE-ID ::= 140

id-E-RABs-SubjectToCounterCheck-List										ProtocolIE-ID ::= 141

id-E-RABs-SubjectToCounterCheckItem											ProtocolIE-ID ::= 142

id-CoverageModificationList													ProtocolIE-ID ::= 143

id-ReportingPeriodicityCSIR													ProtocolIE-ID ::= 145

id-CSIReportList															ProtocolIE-ID ::= 146

id-UEID																		ProtocolIE-ID ::= 147

id-enhancedRNTP																ProtocolIE-ID ::= 148

id-ProSeUEtoNetworkRelaying													ProtocolIE-ID ::= 149

id-ReceiveStatusOfULPDCPSDUsPDCP-SNlength18									ProtocolIE-ID ::= 150

id-ULCOUNTValuePDCP-SNlength18												ProtocolIE-ID ::= 151

id-DLCOUNTValuePDCP-SNlength18												ProtocolIE-ID ::= 152

id-UE-ContextReferenceAtSeNB												ProtocolIE-ID ::= 153

id-UE-ContextKeptIndicator													ProtocolIE-ID ::= 154

id-New-eNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 155

id-Old-eNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 156

id-MeNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 157

id-SeNB-UE-X2AP-ID-Extension												ProtocolIE-ID ::= 158

id-LHN-ID																	ProtocolIE-ID ::= 159

id-FreqBandIndicatorPriority												ProtocolIE-ID ::= 160

id-M6Configuration															ProtocolIE-ID ::= 161

id-M7Configuration															ProtocolIE-ID ::= 162

id-Tunnel-Information-for-BBF												ProtocolIE-ID ::= 163

id-SIPTO-BearerDeactivationIndication										ProtocolIE-ID ::= 164

id-GW-TransportLayerAddress													ProtocolIE-ID ::= 165

id-Correlation-ID															ProtocolIE-ID ::= 166

id-SIPTO-Correlation-ID														ProtocolIE-ID ::= 167

id-SIPTO-L-GW-TransportLayerAddress											ProtocolIE-ID ::= 168

id-X2RemovalThreshold														ProtocolIE-ID ::= 169

id-CellReportingIndicator													ProtocolIE-ID ::= 170

id-BearerType																ProtocolIE-ID ::= 171

id-resumeID																	ProtocolIE-ID ::= 172

id-UE-ContextInformationRetrieve											ProtocolIE-ID ::= 173

id-E-RABs-ToBeSetupRetrieve-Item											ProtocolIE-ID ::= 174

id-NewEUTRANCellIdentifier													ProtocolIE-ID ::= 175

id-V2XServicesAuthorized													ProtocolIE-ID ::= 176

id-OffsetOfNbiotChannelNumberToDL-EARFCN									ProtocolIE-ID ::= 177

id-OffsetOfNbiotChannelNumberToUL-EARFCN									ProtocolIE-ID ::= 178

id-AdditionalSpecialSubframeExtension-Info									ProtocolIE-ID ::= 179

id-BandwidthReducedSI														ProtocolIE-ID ::= 180

id-MakeBeforeBreakIndicator													ProtocolIE-ID ::= 181

id-UE-ContextReferenceAtWT													ProtocolIE-ID ::= 182

id-WT-UE-ContextKeptIndicator												ProtocolIE-ID ::= 183

id-UESidelinkAggregateMaximumBitRate										ProtocolIE-ID ::= 184

id-uL-GTPtunnelEndpoint														ProtocolIE-ID ::= 185

id-DL-scheduling-PDCCH-CCE-usage											ProtocolIE-ID ::= 193

id-UL-scheduling-PDCCH-CCE-usage											ProtocolIE-ID ::= 194

id-UEAppLayerMeasConfig														ProtocolIE-ID ::= 195

id-extended-e-RAB-MaximumBitrateDL											ProtocolIE-ID ::= 196

id-extended-e-RAB-MaximumBitrateUL											ProtocolIE-ID ::= 197

id-extended-e-RAB-GuaranteedBitrateDL										ProtocolIE-ID ::= 198

id-extended-e-RAB-GuaranteedBitrateUL										ProtocolIE-ID ::= 199

id-extended-uEaggregateMaximumBitRateDownlink								ProtocolIE-ID ::= 200

id-extended-uEaggregateMaximumBitRateUplink									ProtocolIE-ID ::= 201

id-NRrestrictioninEPSasSecondaryRAT											ProtocolIE-ID ::= 202

id-SgNBSecurityKey															ProtocolIE-ID ::= 203

id-SgNBUEAggregateMaximumBitRate												ProtocolIE-ID ::= 204

id-E-RABs-ToBeAdded-SgNBAddReqList											ProtocolIE-ID ::= 205

id-MeNBtoSgNBContainer														ProtocolIE-ID ::= 206

id-SgNB-UE-X2AP-ID															ProtocolIE-ID ::= 207

id-RequestedSplitSRBs														ProtocolIE-ID ::= 208

id-E-RABs-ToBeAdded-SgNBAddReq-Item											ProtocolIE-ID ::= 209

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAckList								ProtocolIE-ID ::= 210

id-SgNBtoMeNBContainer														ProtocolIE-ID ::= 211

id-AdmittedSplitSRBs															ProtocolIE-ID ::= 212

id-E-RABs-Admitted-ToBeAdded-SgNBAddReqAck-Item								ProtocolIE-ID ::= 213

id-ResponseInformationSgNBReconfComp											ProtocolIE-ID ::= 214

id-UE-ContextInformation-SgNBModReq											ProtocolIE-ID ::= 215

id-E-RABs-ToBeAdded-SgNBModReq-Item											ProtocolIE-ID ::= 216

id-E-RABs-ToBeModified-SgNBModReq-Item										ProtocolIE-ID ::= 217

id-E-RABs-ToBeReleased-SgNBModReq-Item										ProtocolIE-ID ::= 218

id-E-RABs-Admitted-ToBeAdded-SgNBModAckList									ProtocolIE-ID ::= 219

id-E-RABs-Admitted-ToBeModified-SgNBModAckList								ProtocolIE-ID ::= 220

id-E-RABs-Admitted-ToBeReleased-SgNBModAckList								ProtocolIE-ID ::= 221

id-E-RABs-Admitted-ToBeAdded-SgNBModAck-Item									ProtocolIE-ID ::= 222

id-E-RABs-Admitted-ToBeModified-SgNBModAck-Item								ProtocolIE-ID ::= 223

id-E-RABs-Admitted-ToBeReleased-SgNBModAck-Item								ProtocolIE-ID ::= 224

id-E-RABs-ToBeReleased-SgNBModReqdList										ProtocolIE-ID ::= 225

id-E-RABs-ToBeModified-SgNBModReqdList										ProtocolIE-ID ::= 226

id-E-RABs-ToBeReleased-SgNBModReqd-Item										ProtocolIE-ID ::= 227

id-E-RABs-ToBeModified-SgNBModReqd-Item										ProtocolIE-ID ::= 228

id-E-RABs-ToBeReleased-SgNBChaConfList										ProtocolIE-ID ::= 229

id-E-RABs-ToBeReleased-SgNBChaConf-Item										ProtocolIE-ID ::= 230

id-E-RABs-ToBeReleased-SgNBRelReqList										ProtocolIE-ID ::= 231

id-E-RABs-ToBeReleased-SgNBRelReq-Item										ProtocolIE-ID ::= 232

id-E-RABs-ToBeReleased-SgNBRelConfList										ProtocolIE-ID ::= 233

id-E-RABs-ToBeReleased-SgNBRelConf-Item										ProtocolIE-ID ::= 234

id-E-RABs-SubjectToSgNBCounterCheck-List										ProtocolIE-ID ::= 235

id-E-RABs-SubjectToSgNBCounterCheck-Item										ProtocolIE-ID ::= 236

id-RRCContainer																ProtocolIE-ID ::= 237

id-SRBType																	ProtocolIE-ID ::= 238

id-Target-SgNB-ID															ProtocolIE-ID ::= 239

id-HandoverRestrictionList													ProtocolIE-ID ::= 240

id-SCGConfigurationQuery														ProtocolIE-ID ::= 241

id-SplitSRB																	ProtocolIE-ID ::= 242

id-NRUeReport																ProtocolIE-ID ::= 243

id-InitiatingNodeType-EndcX2Setup											ProtocolIE-ID ::= 244

id-InitiatingNodeType-EndcConfigUpdate										ProtocolIE-ID ::= 245

id-RespondingNodeType-EndcX2Setup											ProtocolIE-ID ::= 246

id-RespondingNodeType-EndcConfigUpdate										ProtocolIE-ID ::= 247

id-NRUESecurityCapabilities													ProtocolIE-ID ::= 248

id-PDCPChangeIndication														ProtocolIE-ID ::= 249

id-ServedEUTRAcellsENDCX2ManagementList										ProtocolIE-ID ::= 250

id-CellAssistanceInformation													ProtocolIE-ID ::= 251

id-Globalen-gNB-ID															ProtocolIE-ID ::= 252

id-ServedNRcellsENDCX2ManagementList											ProtocolIE-ID ::= 253

id-UE-ContextReferenceAtSgNB													ProtocolIE-ID ::= 254

id-SecondaryRATUsageReport													ProtocolIE-ID ::= 255

id-ActivationID																ProtocolIE-ID ::= 256

id-MeNBResourceCoordinationInformation										ProtocolIE-ID ::= 257

id-SgNBResourceCoordinationInformation										ProtocolIE-ID ::= 258

id-ServedEUTRAcellsToModifyListENDCConfUpd									ProtocolIE-ID ::= 259

id-ServedEUTRAcellsToDeleteListENDCConfUpd									ProtocolIE-ID ::= 260

id-ServedNRcellsToModifyListENDCConfUpd										ProtocolIE-ID ::= 261

id-ServedNRcellsToDeleteListENDCConfUpd										ProtocolIE-ID ::= 262

id-E-RABUsageReport-Item														ProtocolIE-ID ::= 263

id-Old-SgNB-UE-X2AP-ID														ProtocolIE-ID ::= 264

id-SecondaryRATUsageReportList												ProtocolIE-ID ::= 265

id-SecondaryRATUsageReport-Item												ProtocolIE-ID ::= 266

id-ServedNRCellsToActivate													ProtocolIE-ID ::= 267

id-ActivatedNRCellList														ProtocolIE-ID ::= 268

id-SelectedPLMN																ProtocolIE-ID ::= 269

id-UEs-ToBeReset															ProtocolIE-ID ::= 270

id-UEs-Admitted-ToBeReset													ProtocolIE-ID ::= 271

id-RRCConfigIndication														ProtocolIE-ID ::= 272

id-DownlinkPacketLossRate													ProtocolIE-ID ::= 273

id-UplinkPacketLossRate														ProtocolIE-ID ::= 274

id-SubscriberProfileIDforRFP												ProtocolIE-ID ::= 275

id-serviceType																ProtocolIE-ID ::= 276

id-AerialUEsubscriptionInformation											ProtocolIE-ID ::= 277

id-SGNB-Addition-Trigger-Ind												ProtocolIE-ID ::= 278

id-MeNBCell-ID																ProtocolIE-ID ::= 279

id-RequestedSplitSRBsrelease												ProtocolIE-ID ::= 280

id-AdmittedSplitSRBsrelease													ProtocolIE-ID ::= 281

id-NRS-NSSS-PowerOffset														ProtocolIE-ID ::= 282

id-NSSS-NumOccasionDifferentPrecoder										ProtocolIE-ID ::= 283

id-ProtectedEUTRAResourceIndication											ProtocolIE-ID ::= 284

id-InitiatingNodeType-EutranrCellResourceCoordination						ProtocolIE-ID ::= 285

id-RespondingNodeType-EutranrCellResourceCoordination						ProtocolIE-ID ::= 286

id-DataTrafficResourceIndication											ProtocolIE-ID ::= 287

id-SpectrumSharingGroupID													ProtocolIE-ID ::= 288

id-ListofEUTRACellsinEUTRACoordinationReq									ProtocolIE-ID ::= 289

id-ListofEUTRACellsinEUTRACoordinationResp									ProtocolIE-ID ::= 290

id-ListofEUTRACellsinNRCoordinationReq										ProtocolIE-ID ::= 291

id-ListofNRCellsinNRCoordinationReq											ProtocolIE-ID ::= 292

id-ListofNRCellsinNRCoordinationResp										ProtocolIE-ID ::= 293

id-E-RABs-AdmittedToBeModified-SgNBModConfList								ProtocolIE-ID ::= 294

id-E-RABs-AdmittedToBeModified-SgNBModConf-Item								ProtocolIE-ID ::= 295

id-UEContextLevelUserPlaneActivity											ProtocolIE-ID ::= 296

id-ERABActivityNotifyItemList												ProtocolIE-ID ::= 297

id-InitiatingNodeType-EndcX2Removal											ProtocolIE-ID ::= 298

id-RespondingNodeType-EndcX2Removal											ProtocolIE-ID ::= 299

id-RLC-Status																ProtocolIE-ID ::= 300

id-CNTypeRestrictions														ProtocolIE-ID ::= 301

id-uLpDCPSnLength															ProtocolIE-ID ::= 302

id-BluetoothMeasurementConfiguration										ProtocolIE-ID ::= 303

id-WLANMeasurementConfiguration												ProtocolIE-ID ::= 304

id-NRrestrictionin5GS														ProtocolIE-ID ::= 305

id-dL-Forwarding															ProtocolIE-ID ::= 306

id-E-RABs-DataForwardingAddress-List										ProtocolIE-ID ::= 307

id-E-RABs-DataForwardingAddress-Item										ProtocolIE-ID ::= 308

id-Subscription-Based-UE-DifferentiationInfo								ProtocolIE-ID ::= 309

id-GNBOverloadInformation													ProtocolIE-ID ::= 310

id-dLPDCPSnLength															ProtocolIE-ID ::= 311

id-secondarysgNBDLGTPTEIDatPDCP												ProtocolIE-ID ::= 312

id-secondarymeNBULGTPTEIDatPDCP												ProtocolIE-ID ::= 313

id-lCID																		ProtocolIE-ID ::= 314

id-duplicationActivation													ProtocolIE-ID ::= 315

id-ECGI																		ProtocolIE-ID ::= 316

id-RLCMode-transferred														ProtocolIE-ID ::= 317

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAckList							ProtocolIE-ID ::= 318

id-E-RABs-Admitted-ToBeReleased-SgNBRelReqAck-Item							ProtocolIE-ID ::= 319

id-E-RABs-ToBeReleased-SgNBRelReqdList										ProtocolIE-ID ::= 320

id-E-RABs-ToBeReleased-SgNBRelReqd-Item										ProtocolIE-ID ::= 321

id-NRCGI																	ProtocolIE-ID ::= 322

id-MeNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 323

id-SgNBCoordinationAssistanceInformation									ProtocolIE-ID ::= 324

id-new-drb-ID-req															ProtocolIE-ID ::= 325

id-endcSONConfigurationTransfer												ProtocolIE-ID ::= 326

id-NRNeighbourInfoToAdd														ProtocolIE-ID ::= 327

id-NRNeighbourInfoToModify													ProtocolIE-ID ::= 328

id-DesiredActNotificationLevel												ProtocolIE-ID ::= 329

id-LocationInformationSgNBReporting											ProtocolIE-ID ::= 330

id-LocationInformationSgNB													ProtocolIE-ID ::= 331

id-LastNG-RANPLMNIdentity													ProtocolIE-ID ::= 332

id-EUTRANTraceID															ProtocolIE-ID ::= 333

id-additionalPLMNs-Item														ProtocolIE-ID ::= 334

id-InterfaceInstanceIndication												ProtocolIE-ID ::= 335

id-BPLMN-ID-Info-EUTRA														ProtocolIE-ID ::= 336

id-BPLMN-ID-Info-NR															ProtocolIE-ID ::= 337

id-NBIoT-UL-DL-AlignmentOffset												ProtocolIE-ID ::= 338

id-ERABs-transferred-to-MeNB												ProtocolIE-ID ::= 339

id-AdditionalRRMPriorityIndex												ProtocolIE-ID ::= 340

id-LowerLayerPresenceStatusChange											ProtocolIE-ID ::= 341

id-FastMCGRecovery-SN-to-MN													ProtocolIE-ID ::= 342

id-RequestedFastMCGRecoveryViaSRB3											ProtocolIE-ID ::= 343

id-AvailableFastMCGRecoveryViaSRB3											ProtocolIE-ID ::= 344

id-RequestedFastMCGRecoveryViaSRB3Release									ProtocolIE-ID ::= 345

id-ReleaseFastMCGRecoveryViaSRB3									ProtocolIE-ID ::= 346

id-FastMCGRecovery-MN-to-SN													ProtocolIE-ID ::= 347

id-PartialListIndicator														ProtocolIE-ID ::= 348

id-MaximumCellListSize														ProtocolIE-ID ::= 349

id-MessageOversizeNotification												ProtocolIE-ID ::= 350

id-CellandCapacityAssistInfo												ProtocolIE-ID ::= 351

id-TNLConfigurationInfo														ProtocolIE-ID ::= 352

id-TNLA-To-Add-List															ProtocolIE-ID ::= 353

id-TNLA-To-Update-List														ProtocolIE-ID ::= 354

id-TNLA-To-Remove-List														ProtocolIE-ID ::= 355

id-TNLA-Setup-List															ProtocolIE-ID ::= 356

id-TNLA-Failed-To-Setup-List													ProtocolIE-ID ::= 357

id-UnlicensedSpectrumRestriction												ProtocolIE-ID ::= 358

id-UEContextReferenceatSourceNGRAN											ProtocolIE-ID ::= 359

id-EPCHandoverRestrictionListContainer										ProtocolIE-ID ::= 360

id-UERadioCapabilityID														ProtocolIE-ID ::= xxx -- to be assigned

END

-- ASN1STOP

---------------------------- End of 12th change ---------------------------

---------------------------- End of Chnages ---------------------------