3GPP TSG-RAN3 WG Meeting #109-e                                              	　　　　　　　　    　        R3-204698

Online, August 17th – August 28th 2020

Source: 	KDDI Corporation

Title:	Considerations on differentiation inter-donor/intra-donor migration

Tdoc Type:	Discussion

Agenda Item:	13.2.1 Inter-Donor IAB Node Migration

Document for:	Discussion and decision

Release:	Rel-17

Introduction

In this contribution we discuss inter-donor IAB-node migration, the objective is to be specified in Rel-17 [1].

Topology adaptation enhancements [RAN3-led, RAN2]: - Specification of procedures for inter-donor IAB-node migration to enhance robustness and load-balancing, including enhancements to reduce signalling load. - Specification of enhancements to reduce service interruption due to IAB-node migration and BH RLF recovery. - Specification of enhancements to topological redundancy, including support of CP/UP separation.

Discussion

In-Rel16, RAN3 agreed the stage2 statements for intra-CU topology adaptation procedure [2]. In this intra-CU topology adaptation, established F1 association between the IAB node and the donor-CU is maintained after the migration. On the other hand, the inter-CU topology adaptation is to be specified in Rel-17, it requires a migrating IAB node to initiate F1 setup procedure (defined in TS38.473) with a new donor-CU, so that the migrating IAB node can obtain the necessary information from the new IAB donor. To initiate the F1 setup procedure for this case, a migrating IAB node needs to recognize whether the migration is intra-CU or inter-CU.

Observation:

For inter-CU migration case, a migrating IAB node needs to initiate new F1 setup procedure with a new donor-CU. To initiate F1 setup procedure, a migrating IAB node needs to recognize whether a migration is intra-CU or inter-CU.

Proposal:

RAN3 requests RAN2 to develop an RRC indication which is used at IAB-DU to figure out which migration is triggered, intra-CU or inter-CU.

Conclusions

In this contribution, the following observation and proposal is made:

Observation:

For inter-CU migration case, a migrating IAB node needs to initiate new F1 setup procedure with a new donor-CU. To initiate F1 setup procedure, a migrating IAB node needs to recognize whether a migration is intra-CU or inter-CU.

Proposal:

RAN3 requests RAN2 to develop an RRC indication which is used at IAB-DU to figure out which migration is triggered, intra-CU or inter-CU.

References

[1] RP-201293   	WID Enhancements to Integrated Access and Backhaul for NR		Qualcomm

[2] R3-197808		BL CR to 38.401 Support for IAB (38.401CR0033rev14)		Huawei