3GPP TSG-RAN3 Meeting #109-e	R3-5G0654

E-meeting, 17 - 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | <CR#> | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PWS cancel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-07-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Error during implementation: - One set of CRs to correct duplicated section numbering 8.5.1(->2).3 + 8.5.1(->2).4 in CR0614 (Rel15) and CR0615 (Rel16 mirror) - One set of CRs to add an abnormal corrections to 8.5.1.4 – but this should should be in the renumbered 8.5.2.4 in CR0617 (Rel15) and CR0618 (Rel16 mirror) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the abnormal text from 8.5.1.4 to 8.5.2.4 Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it should only impact PWS cancel. This CR has an impact under functional point of view. The impact can be considered isolated because the change affects PWS cancel and write replace warning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Previously agreed solution not captured correctly in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.5	Warning Message Transmission Procedures

### 8.5.1	Write-Replace Warning

#### 8.5.1.1	General

The purpose of Write-Replace Warning procedure is to start or overwrite the broadcasting of warning messages. The procedure uses non UE-associated signalling.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Write-Replace Warning procedure: successful operation

The gNB-CU initiates the procedure by sending a WRITE-REPLACE WARNING REQUEST message to the gNB-DU.

Upon receipt of the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall prioritise its resources to process the warning message.

The gNB-DU acknowledges the WRITE-REPLACE WARNING REQUEST message by sending a WRITE-REPLACE WARNING RESPONSE message to the gNB-CU.

Upon receipt of the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the WRITE-REPLACE WARNING RESPONSE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the WRITE-REPLACE WARNING RESPONSE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the Notification Information IE is included in the PWS System Information IE in the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall use this information to avoid that duplications trigger new broadcast or replace existing broadcast.

If the gNB-DU receives a WRITE-REPLACE WARNING REQUEST message with the Notification Information IE in the PWS System Information IE which are different from those of ongoing broadcast warning messages, and if the SIB Type IE is set to “8”, the gNB-DU shall broadcast the received warning message concurrently with other ongoing messages.

If the gNB-DU receives a WRITE-REPLACE WARNING REQUEST message with the Notification Information IE in the PWS System Information IE which are different from those of ongoing broadcast warning messages, and if the SIB Type IE is set to the value other than ‘8’ , the gNB-DU shall use the newly received one to replace the ongoing broadcast warning message with the same value of SIB Type IE.

If the SIB Type IE in the PWS System Information IE in the WRITE-REPLACE WARNING REQUEST message is set to “8” and if a value "0" is received in the Number of Broadcast Requested IE and if the Repetition Period IE is different from "0", the gNB-DU shall broadcast the received warning message indefinitely.

If Additional SIB Message List IE is included in PWS System Information IE, the gNB-DU shall store all SIB message(s) in PWS System Information IE, and consider that the first segment of public warning message is included in SIB message IE, and the remaining segments are listed in Additional SIB Message List IE in segmentation sequence order.

#### 8.5.1.3	Unsuccessful Operation

Not applicable.

#### 8.5.1.4	Abnormal Conditions

Not applicable.

8.5.2	PWS Cancel

#### 8.5.2.1	General

The purpose of the PWS Cancel procedure is to cancel an already ongoing broadcast of a warning message. The procedure uses non UE-associated signalling.

#### 8.5.2.2	Successful Operation

Figure 8.5.2.2-1: PWS Cancel procedure: successful operation

The gNB-CU initiates the procedure by sending a PWS CANCEL REQUEST message to the gNB-DU.

The gNB-DU shall acknowledge the PWS CANCEL REQUEST message by sending the PWS CANCEL RESPONSE message.

If the Cancel-All Warning Messages Indicator IE is present in the PWS CANCEL REQUEST message, then the gNB-DU shall stop broadcasting and discard all warning messages for the area as indicated in the Cell Broadcast To Be Cancelled List IE or in all the cells of the gNB-DU if the Cell Broadcast To Be Cancelled List IE is not included. The gNB-DU shall acknowledge the PWS CANCEL REQUEST message by sending the PWS CANCEL RESPONSE message, and shall, if there is area to report where an ongoing broadcast was stopped successfully, include the Cell Broadcast Cancelled List IE with the Number of Broadcasts IE set to 0.

If the Cell Broadcast To Be Cancelled List IE is not included in the PWS CANCEL REQUEST message, the gNB-DU shall stop broadcasting and discard the warning message identified by the Message Identifier IE and the Serial Number IE in the Notification Information IE in all of the cells in the gNB-DU.

If the Notification Information IE is included in the PWS CANCEL REQUEST, the gNB-DU shall cancel broadcast of the public warning message identified by the Notification Information IE.

If an area included in the Cell Broadcast To Be Cancelled List IE in the PWS CANCEL REQUEST message does not appear in the Cell Broadcast Cancelled List IE in the PWS CANCEL RESPONSE, the gNB-CU shall consider that the gNB-DU had no ongoing broadcast to stop for the public warning message identified, if present, by the Notification Information IE in that area.

If the Cell Broadcast Cancelled List IE is not included in the PWS CANCEL RESPONSE message, the gNB-CU shall consider that the gNB-DU had no ongoing broadcast to stop for the public warning message identified, if present, by the Notification Information IE.

#### 8.5.2.3	Unsuccessful Operation

If the gNB-DU receives a PWS CANCEL REQUEST message which contains neither the Cancel-all Warning Messages Indicator IE nor the Notification Information IE, the gNB-DU shall consider it as a logical error.

- 8.5.2.4	Abnormal Conditions

Not applicable.