3GPP TSG-RAN WG3#113-e	R3-214509

- E-meeting, 16 – 26 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0xxx | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per RP-193248, NR MBS (Multicast Broadcast Service for NR) is supported in release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR MBS is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR MBS is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 16.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 7 (4509): R3-213179 + agreed TP R3-214416 Rev 6 (3179): R3-212982 + new venue RAN3#113 (and new version 16.6.0) Rev 5 (2982): R3-211496 + agreed TP R3-212908 Rev 4 (1496): R3-211351 + new venue RAN3#112 Rev 3 (1351): baseline 0015 + TPs agreed from RAN3#111: R3-211147. Rev 2 (0015): baseline R3-207249 + new cover page + new venue RAN3#111 Rev 1: baseline R3-207049 + integrated agreed TPs from RAN3#110: R3-206384, R3-207055, R3-207057. |  |  |  |  |  |  |  |  |

- 3	Abbreviations and Definitions

- 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

MBS	Multicast Broadcast Service

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MT	Mobile Termination

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PTP	Point To Point

PTM	Point To Multipoint

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

- 3.2	Definitions

## Next change

## 16.x	NR MBS

### 16.x.1	General

Editor’s Note: General aspects to be covered here.

### 16.x.2	Architecture

Editor’s Note: Architecture aspects to be covered here.

The overall NG-RAN architecture specified in section 4 applies for NR MBS.

### 16.x.3	Session Management

Editor’s Note: Session Management aspects to be covered here.

- 16.x.3.1	QoS Model

The following QoS model applies to both multicast and broadcast:

- An MBS Session Resource may be associated with one or more MBS QoS flows.

- Each MB QoS flow is associated with a QoS profile.

Editor’s Note: whether 5GC sends MBS Session AMBR to NG-RAN and how NG-RAN node would handle it is FFS.

Editor’s Note: specification of applicability of QoS flow QoS parameters and PDU Session parameters to an MBS Session Resources is expected to be specified. How to reference to TS 23.501 (by SA2) is FFS.

### 16.x.4	Transmission

Editor’s Note: covers layer 1 and layer 2 description of NR MBS transmission.

#### 16.x.4.y	PTP-PTM Switching

Editor’s Note: covers both PTP to PTM switching and PTM to PTP switching.

The PTP-PTM Switching function is only applicable for a Multicast session and resides in NG-RAN node. It enables the NG-RAN node to decide for which UEs to use PTP or PTM (PTP, PTM to be defined with RAN2) for the MBS session. The NG-RAN node takes its decision based on information such as MBS Session QoS requirements, number of joined UEs, UE individual feedback on reception quality, and other criteria. The same QoS requirements apply regardless of the decision.

PTP and PTM modes can be used simultaneously in the same cell. For Broadcast session only PTM is applicable. For Multicast session both PTP and PTM are applicable.

### 16.x.5	Mobility

Editor’s Note: Mobility aspects to be covered here.

#### 16.x.5.1	General

Mobility principles builds on existing functionality including functions described in section 9.2.

#### 16.x.5.2	Multicast Mobility from MBS supporting cell to MBS supporting cell

During handover preparation phase, the source NG-RAN node transfers to the target NG-RAN node in the UE context information about the MBS sessions the UE has joined. For each Multicast session with ongoing user data transmission for which no MBS Session Resources exist at the target NG-RAN node, the target NG-RAN node triggers the setup of MBS user plane resources towards the 5GC. Which procedures to use is FFS.

During handover execution, the MBS configuration decided at target NG-RAN node is sent to the UE via the source NG-RAN node within an RRC container (FFS) as specified in TS 38.331 [12].

#### 16.x.5.3	Multicast Mobility from/to MBS supporting cell to/from MBS non-supporting cell

During an active multicast MBS session, at mobility from an MBS-supporting NG-RAN node to a non-MBS supporting NG-RAN node, the target NG-RAN node sets up PDU Session resources associated to the multicast MBS Session. The SMF infers from the absence of an ‘MBS-support“ indication in the Path Switch Request message (Xn handover) or Handover Request Acknowledge message (NG handover) that the 5GC has to switch to 5GC individual MBS traffic delivery for that UE as specified in TS 23.247 [x].

If data forwarding is applied the source NG-RAN node infers from the handover preparation response message that the target NG-RAN node does not support MBS and changes the QFI(s) in the forwarded packets to the associated unicast QFI(s) if respective mapping information is available.

Editor’s Note: whether the source NG-RAN node may also learn before the handover preparation response message that the target NG-RAN node is non-MBS supporting is FFS.

Editor’s Note: whether other options for mobility from supporting nodes to non-supporting nodes are specified to minimize data loss is FFS.

For mobility from non-MBS supporting NG-RAN node to MBS-supporting NG-RAN node, the existing Xn/NG handover procedures apply. The 5GC infers from the presence of the ‘MBS-supporting“ indicator in the Path Switch Request message (Xn handover) or Handover Request Acknowledge message (NG handover) that PDU sessions resources associated with active multicast MBS session(s) can be switched from 5GC MBS individual traffic delivery to 5GC shared traffic delivery [details FFS].

#### 16.x.5.4	Broadcast Mobility (FFS)

Editor’s Note: intends to cover information related to broadcast exchanged during mobility. FFS.