3GPP TSG-RAN WG3 #113-e							          								     R3-214511

16-26 Aug 2021

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.410 | CR | 0030 | rev | 6 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS BL CR for TS 38.410 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2021-08-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per RP-193248, NR MBS is supported in release 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR MBS is introduced. Editorial change Capture agreed TP：R3-207054, R3-214381 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR MBS cannot be supported in Rel 17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.4, 5, 6, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-1: R3-206526 rev in R3-207051 with editorial changes. Rev-2: R3-207250 = baseline R3-207051 + integrated agreed TP from RAN3#110: R3-207054 Rev-3: R3-207250 rev in R3-210016 for resubmit. Rev-4: R3-211478 for resubmit. Rev-5: R3-213144 +agreed TP R3-214381 |  |  |  |  |  |  |  |  |

--------------------------------Start of the First Change-----------------------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

gNB: as defined in TS 38.300 [7].

NB-IoT: as defined in TS36.300 [11].

ng-eNB: as defined in TS 38.300 [7].

NG-RAN node: as defined in TS 38.300 [7].

UPF: as defined in TS 23.501 [8].

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

CIoT	Cellular IoT

MBS	Multicast/Broadcast Service

NB-IoT	Narrow Band Internet of Things

NG-U	NG User plane interface

PTP	Point to Point

PTM	Point to Multipoint

RIM	Remote Interference Management

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

# 4	General aspects

//skip unchanged part

## 4.4	NG interface capabilities

The NG interface supports:

-	procedures to establish, maintain and release NG-RAN part of PDU sessions

-	procedures to perform intra-RAT handover and inter-RAT handover;

-	the separation of each UE on the protocol level for user specific signalling management;

-	the transfer of NAS signalling messages between UE and AMF;

-	mechanisms for resource reservation for packet data streams.

-	procedures to establish, maintain and release NG-RAN part of MBS sessions

Editor’s note: The definition of MBS session can be refined.

# 5	Functions of the NG interface

//skip unchanged part

- 5.xx	NR MBS Session Management function

The MBS Session Management function is responsible for establishing, modifying and releasing the involved NR MBS sessions NG-RAN resources for user data transport once an MBS context is available in the NG-RAN node.

Editor’s Note: this only applies to broadcast.

- 5.yy	Multicast Group Paging Function

The Multicast group paging function supports the sending of multicast group paging requests to the NG-RAN nodes in order to group-page UEs that have joined the Multicast MBS Session

# 6	Signalling procedures of the NG interface

//skip unchanged part

- 6.xx	NR MBS Session Management Procedures

The following list of MBS Session management procedures are used to establish, release, or modify NG-RAN resources for a NR MBS session:

-	Broadcast Session Resource Setup;

-	Broadcast Session Resource Release;

Editor’s Note: this only applies to broadcast.

- 6.yy	Multicast Group Paging Procedures

The following Multicast Group Paging procedure is used to send multicast group paging requests to the NG-RAN nodes:

-	Multicast Group Paging.

# 7	NG interface protocol structure

## 7.2	NG User Plane

The NG user plane (NG-U) interface is defined between a NG-RAN node and a UPF. The NG-U interface provides non guaranteed delivery of PDU Session /MBS session user plane PDUs between the NG-RAN node and the UPF.

Editor’s note: Changes are expected for MBS session.

The protocol stack for NG-U is shown in Figure 7.2-1.

Figure 7.2-1: NG-U protocol structure for PDU Session

< TBD >

Figure 7.2-2: NG-U protocol structure for MBS Session

Editor’s note: If NG-U protocol structure is the same for both PDU Session and MBS Session, the Figure 7.2-2 shall be removed.

--------------------------------End of the Changes-----------------------------