3GPP TSG-RAN WG3 Meeting #114-e	R3-216165

E-meeting, 1-11 Nov 2021

Title:	Reply LS on inter-MN handover without SN change

Response to:	LS R2-2106682 on inter-MN handover without SN change from RAN2

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG3

To:	TSG RAN WG2

Cc:

Contact person:

#### Name:	Yan Wang

E-mail Address:	wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN3 thanks RAN2 for the LS on inter-MN handover without SN change, RAN3 discussed the questions and would like to provide the answers as follows:

Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN?

RAN3 answer:

- If the target MN decides to keep the SN, the target MN sends SN Addition Request to the SN including the SN UE X2/XnAP ID as a reference to the UE context in the SN that was established by the source MN.

- No consensus on if the absence of SN UE X2/XnAP ID is applicable in case the source SN and target SN are the same SN.

Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration?

RAN3 answer:

- In case the SN UE X2/XnAP ID is provided alone, the SN is able to retrieve the stored UE context, there is no description in RAN3 specifications on whether the SN may perform delta configuration or not.

2	Actions

To RAN2 group

ACTION: 	RAN3 kindly ask RAN2 to take the answers above into account.

3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#114bis-e	2022-01-17 – 2022-01-26

RAN3#115-e 	2022-02-21 - 2022-03-03

RAN3#115bis-e	2022-04-04 – 2022-04-08