3GPP TSG-RAN WG3 #117-e	R3-225123

Electronic Meeting, Aug 15th – 25th, 2022

Agenda Item:	12.2.2

Source:	ZTE (moderator)

Title:	[draft] Summary of CB: # AIRAN2_Stage3

Document for:	Approval

Introduction

CB: # AIRAN2_Stage3

- Analyze and coverage on supporting input, output and feedback through existing procedures or new defined procedures

- Standard impact analysis on network energy saving, mobility optimization and load balancing

- Discuss the remaining issues, e.g., validity time

- Potential impacts over interfaces

- Capture agreements and open issues

(ZTE - moderator)

Summary of offline disc R3-225014

Two phases of this email discussion:

- Phase 1 Deadline: 23:59UTC, Thursday, 19th Aug (before online session starts).

- Phase 2 Deadline: 11:00UTC, Tuesday, 23th Aug. , we will try to come up with agreements in the 2nd phase discussion before online session.

For the Chairman’s Notes

Propose to capture the following for the chairman’s notes (2nd round discussion):

Proposal 1: Current Energy Efficiency metric can be exchanged between RAN nodes for the energy saving use case.

FFS what the exact node behaviour at reception of the Energy Efficiency metric will be.

Proposal 2: Energy Efficiency constitutes a metric that reflects the energy consumption of a cell or a node. It is FFS what the granularity and exact coding of this metric is.

FFS on how is energy efficiency represented in the Xn message.

Proposal 3: Predicted cell-granularity UE trajectory can be exchanged over Xn for AI/ML based mobility optimization.

Other granularity of UE trajectory, and how UE trajectory computation and representation over Xn needs to be further discussed.

Details of the solution how to transfer predicted UE trajectory over Xn interface can be discussed further, e.g., whether to reuse the existing message and whether to transfer in requested way or not.

Proposal 4: The new procedure over Xn used for AI/ML related information should be non-UE associated as a start point.

FFS on whether UE associated procedure is needed.

Propose to capture the following for the chairman’s notes (1st round discussion):

Proposal 1-1: Define the new dedicated new procedure for AI/ML predicted information (e.g., predicted resource status, etc).

FFS whether to use the existing procedure or dedicated new procedure for other input , output and feedback information.

Proposal 1-2: The new procedure for predicted information should be based on subscription-based solution.

FFS whether the procedure should be non-UE associated procedure or UE associated procedure.

Proposal 2: RAN3 focus on the cell-level energy saving strategy as a start point, to avoid overlapped discussion for network energy saving in RAN1 SI.

Proposal 3: Regarding AI/ML based Energy Saving, the following information should be specified as a start point:

- Predcited resource status information from neighbour NG-RAN nodes over Xn

- Current/Predicted Energy Efficiency from neighbour NG-RAN nodes over Xn

- UE performance (UL/DL throughput, packet delay, packet loss)

Proposal 4: Regarding AI/ML based Load Balancing, the following information should be specified as a start point:

- Predicted resource status information of neighbouring NG-RAN node(s)

- UE performance (UL/DL throughput, packet delay, packet loss)

Proposal 5: Predicted resource status information of neighbouring NG-RAN node(s) as output is internally used, and no standard impacts.

Proposal 6: Regarding AI/ML based mobility optimization, the following information should be specified as a start point:

- UE performance (e.g., UL/DL throughput, packet delay, packet loss)

- Predicted resource status information of neighbouring NG-RAN node(s)

- UE trajectory prediction over Xn (e.g., predicted serving cell, …)

Proposal 7: Validity time is a local node model output with no standards impact unless an output is identified to be sent to another node in assistance.

Proposal 8: Potential MDT enhancement related issues as follows, need more time to discuss the details and potential standard impacts, coordination with RAN2/SA5 if needed:

- Enhance the MDT procedure to solve the issue how to support the consecutive AI/ML data collection for the certain time-series AI/ML model.

- How the source NG-RAN node obtains logged UE trajectory information when UE enters RRC Connected state and reports to the new NG-RAN node.

- How to enable a more granular selection of UEs based on enhanced MDT Configuration information in management based MDT

- How to map AI/ML Feedback information to AI/ML Actions and report them over MDT

Open issues discussed in the 2nd round discussion:

FFS whether the procedure should be non-UE associated procedure or UE associated procedure.

How to support the below information transmission over Xn:

-      Predicted resource status information of neighbouring NG-RAN node(s)

-      UE trajectory prediction over Xn (e.g., predicted serving cell, …)

-      UE performance (e.g., UL/DL throughput, packet delay, packet loss)

Discussion -  2nd

During the online discussion, there are several open issues as follows to be discussed in the 2nd round.

Energy Efficiency

Regarding AI/ML based Network Energy Saving, companies want to discuss the usage and benefits of the current/predicted energy efficiency as AI/ML related information, and the calculation of energy efficiency.

Q1: Companies are invited to provide their views on whether the current or predicted energy efficiency is needed for AI/ML based Network Energy Saving? If yes, please clarify the benefits, usage, and elaborate the details of its potential standard impacts.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | We think EE is a straight forward metric to indicate the performance of ES, it is also the direct performance feedback after inference is executed. For example, it help the NG-RAN nodes to decide which cell(s) to switch on/off. The potential standard impact is Current/Predicted Energy Efficiency from neighbour NG-RAN nodes over Xn |
| Samsung | Yes | Energy efficiency can serve as input and feedback information of the model for energy saving strategy generation. As the input, the energy saving decision should consider the neighbour cell status to generate an optimized decision. When the EE of neighbour cells is low, the node may not switch-off the cell to improve the energy efficiency of such area. As the feedback, the energy efficiency can reflect whether the energy saving strategy is good or not. If the EE of neighbour cells is decreased dramatically after cell switch-off, the energy saving decision is not good due to “local” optimization instead of “global” optimization. Focusing on non-split architecture first, the potential standard impact is transferring the current/predicted energy efficiency via Xn interface. |
| China Telecom | Yes | Energy efficiency is the most straightforward metric for a NG-RAN node to make ES decisions and evaluate the effects of the ES decisions that been made, thus it’s the essential component for input and feedback information. On Xn interface impact, we share the same view as Huawei and Samsung. |
| Ericsson | Yes | The use and usefulness of the current and predicted Energy Efficiency have already been studied and agreed in TR37.817. The TR in fact quotes: “ML algorithms may predict the energy efficiency and load state of the next period, which can be used to make better decisions on cell activation/deactivation for energy saving. Based on the predicted load, the system may dynamically configure the energy-saving strategy (e.g., the switch-off timing and granularity, offloading actions) to keep a balance between system performance and energy efficiency and to reduce the energy consumption. ” The current energy efficiency is needed to determine the status of nodes in terms of energy consumption. The predicted energy efficiency is needed to take better decisions that maximise energy efficiency. The current energy efficiency can also be used as feedback information. The impact on the standard is to have this information exchanged between neighbour over Xn. However, the exact encoding of this information remains to be discussed and agreed, so we propose to agree that there will be a Current/Predicted energy efficiency information signalled over Xn, but that the encoding of this information is FFS. |
| CATT | Yes for current energy efficiency, No for predicted energy efficiency unless clear on how to use it | We agree that the predicted load information maybe useful for the energy saving decision. For example, if the predicted load information of neighbor node is high, then UEs should be not handover to this neighbor node and thereby may impact the energy saving decision. We also agree that the current energy efficiency is helpful for energy saving case as input and feedback, as the intention of energy saving is to improve the energy efficiency of one area. However, for the predicted energy efficiency, it is still not clear to us how it help the energy saving decision. The intention of energy saving is to improve the energy efficiency of one area. |
| ZTE | No strong view | From our perspective, energy saving is a system-level use case, which is a global optimization. Other parameters in the real environment will also impact the energy efficiency. |
| AT&T | Yes | We agree that predicted and current information is useful to be exchanged across neighbor nodes as highlighted during the SI phase. As mentioned by Samsung, the Xn signaling can be the starting point for a non-split architecture with FFS additional enhancements for the split architecture case. |
| Nokia | Depends on the understanding of the energy efficiency | In TR 37.817 we have agreed that input data for AI/ML Energy Saving includes current and predicted Energy Efficiency allowing the possible outputs of AI/ML Energy Saving to be energy saving strategy, handover strategy, predicted energy efficiency, predicted energy state, etc. In addition, we identified two solutions, one where AI/ML Model Training can be located in OAM and one where it can be located in a gNB. During the study, we didn’t define what we mean by Energy Efficiency. In TS28.554, an EE KPI is defined to equal the overall Data Volume (DV) divided by the Energy Consumption (EC) of the different network elements over a given time. This calculates EE of the different components of the RAN and is reported to OAM. Note that such EE KPI can be difficult to interpret by a neighboring gNB. First, DV highly depends on the load level, where the larger the load the larger the DV. This, in turn, depends on the time of the day in which the KPI is observed since the load largely fluctuates during the day. In addition, the EC also varies based on the DV level. Furthermore, such EE KPI may be also misleading sometimes. When looking at a single cell’s EE KPI, this will be low(er) at times of low(er) traffic demands (lower DV), and vice-versa, it will be high(er) at times of high(er) traffic demands (higher DV). For example, after cell deactivation at the time of low load, a neighboring cell’s EE KPI can appear low(er), and this could be wrongly interpreted as a wrong decision. So, when it comes to Energy Efficiency sent to a neighbouring gNB this may be misleading and not be easily interpretable by it, unless additional parameters are reconfigured to a gNB. Besides the parameters required to calculate the Energy Efficiency, e.g. Data Volume (DV) and Energy Consumption (EC), other parameters may be needed to describe the network characteristics with regards to user density, geographical conditions and climate zones. Given the above, the two possibilities that we see meaningful are the following: - When AI/ML Model Training is in OAM, OAM may receive EE from the RAN, may train an AI/ML Energy Saving Model which may then be deployed to RAN. Standards impacts in this case could be (possibly) identified on the output of AI/ML Energy Efficiency (inference) exchanged between neighbours - When AI/ML Model Training is in the gNB, then in our view we need to find meaningful metrics of EE that could be exchanged between neighbours to enable Training an AI/ML Model and which do not have the problems listed above. For example, energy efficiency could be indicated across neighbors using a metric that would account for the effects of delta traffic on the energy consumption of the cell, which can be beneficial when making energy saving decisions. |
| Lenovo |  | There is no doubt measured current EE from local RAN node or neighbor RAN node can be used as feedback. On the other hand, CATT’s doubt about the usefulness of predicted EE from neighbor RAN node is also valid. How does neighbor RAN node’s future EE impact the ES decision at the local RAN node? It would be good if the proponent could give reasonable example, which is so far missing in the comments above. |
| China Unicom | Yes | Maximizing the energy efficiency of the system is also one of the purposes of our pursuit for energy saving and EE should be considered as a performance metric. The potential standard impact would be transferring current/predicted Energy Efficiency from neighbour NG-RAN nodes over Xn. |
| Qualcomm | See Comments | We think transferring Current or Predicted Energy Efficiency to neighbors via XN will not be a useful information until there is a standardized way of calculating and interpreting Energy Efficiency among the Neighboring nodes. Also, we agree with CATT’s concern on how will Current or Predicted Energy Efficiency be used at the receiver. Should the source decide to Handover a UE to a neighbor which has higher value of Energy Efficiency? The use case is unclear. |
| Intel | Yes | We also agree that both current and predicted energy efficiency is beneficial to be collected for AI/ML based energy saving use case. As for predicted energy efficiency, one of the objectives of AI/ML based network energy saving is to avoid optimize system performance and energy efficiency conflicting between source and target NG-RAN node, as captured in TR37.817 Section 5.1.1 use case description. The predicted energy efficiency is exactly beneficial for the above objective, which can avoid energy efficiency down-grade at the neighbouring NG-RAN node. |

Moderator’s summary:

7 companies acknowledge the use and benefits of the current/predicted energy efficiency for AI/ML based network energy saving.

2 companies agree that current energy efficiency can be used as input and feedback information, while they think how the predicted energy efficiency could help the energy saving decision, needs clear clarification. And 1 company also needs the clarification, how current or predicted Energy Efficiency be used at the receiver. 1 company consider energy saving is a system-level use case, which is a global optimization.

In addition, 1 company provides two possibilities based on the deployment of AI/ML model training, and think when it comes to Energy Efficiency sent to a neighbour gNB this may be misleading and not be easily interpretable by it, unless additional parameters are reconfigured to a gNB.

Moderator here tries to conclude that, the key issue raised by companies, is how the NG-RAN nodes generates the energy saving decision based on the predicted energy efficiency from neighbour NG-RAN nodes.

Proposal 1: How the NG-RAN node generates the energy saving decision based on the predicted energy efficiency from the neighbour NG-RAN nodes needs to be clarified.

In addition to the current/predicted energy efficiency, the calculation of energy efficiency is asked to discuss during 2nd discussion.

In the current specification TS 28.554, following is the definition of the energy efficiency:

- 6.7.1 NG-RAN data Energy Efficiency (EE) - 6.7.1.1 Definition a) EEMN,DV. b) A KPI that shows mobile network data energy efficiency in operational NG-RAN. Data Volume (DV) divided by Energy Consumption (EC) of the considered network elements. The unit of this KPI is bit/J. c) EEMN,DV =SamplesDRB. PdcpSduVolumnUl+DRB.PdcpSduVolumnDlSamplesPEE.Energy=SamplesDRB. PdcpSduVolumnUl+DRB.PdcpSduVolumnDlSamplesPEE.Energy - for non-split gNBs; =SamplesF1uPdcpSduVolumeUl+XnuPdcpSduVolumeUl+X2uPdcpSduVolumeUl+(F1uPdcpSduVolumeDl+XnuPdcpSduVolumeDl+X2uPdcpSduVolumeDl)SamplesPEE.Energy=SamplesF1uPdcpSduVolumeUl+XnuPdcpSduVolumeUl+X2uPdcpSduVolumeUl+(F1uPdcpSduVolumeDl+XnuPdcpSduVolumeDl+X2uPdcpSduVolumeDl)SamplesPEE.Energy - for split-gNBs;

Moderator thinks this definition can be reused for AI/ML based network energy saving, and can be considered as a baseline.

Proposal: The definition of energy efficiency in TS28.554 can be regarded as a baseline for energy efficiency in AI/ML based network energy saving.

Q2: Companies are invited to provide their views on whether the proposal above can be agreed?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | We agree with moderator’s analysis, we think the general target of ES is to improve the EE of the whole network. |
| Samsung | Yes | The ratio of data volume and energy consumption defined in 28.554 is the general understanding of energy efficiency. We can take it as the baseline. |
| China Telecom | Yes | We agree to take the EE definition of TS28.5543 as baseline. |
| Ericsson | No | It is virtually impossible to measure the energy consumption associated to a cell, especially if we consider cloud based RAN, where a common platform is used to serve many cells. Also, the EE defined above is a very sensitive piece of information that is not appropriate so signal over the Xn. We propose to discuss more on the encoding of the EE IE and to adopt an indexed value, which reveals how close to minimum and maximum EE levels the node is, as well as that reveals the delat changes of the EE metric. |
| CATT | Mostly Yes | Section 6.1.1 of TS 28.310 captures the same, although the formula is shown in a more general manner (the meaning is the same as described in the text beneath the formula): EEMN,DV=DVMNECMN And this is exactly why we observed that DV and EC for each network component should be delivered separately: The total EE of an area (which can be used as a reward of AI/ML) can only be got by dividing the sum of DVs by the sum of ECs—you can never get the total EE=∑DV/∑EC by operating many individual DV/EC quotients. In addition, for split case DV is measured at the gNB-CU-UP (different from the EC) according to Section 5.1.3.6.2.3/4 of TS 28.552, e.g.: c) This measurement is obtained by counting the number of DL PDCP SDU bits sent to GNB-DU (F1-U interface), sent to external gNB-CU-UP (Xn-U interface) and sent to external eNB (X2-U interface). The measurement is performed in GNB-CU-UP per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI and per PLMN ID, and reported per interface (F1-U, Xn-U, X2-U). But there might be some technical obstacle: the granularity (e.g. per-XXX) of this measurement may be unnecessarily small, or even impractical as the gNB-CU-UP is never explicitly indicated whether a tunnel used to deliver PDCP PDUs is an F1-U one or an Xn-U/X2-U one (we do indicate MCG or SCG explicitly, but that doesn’t help). Therefore, maybe the details on F1uPdcpSduVolumeUl etc have to be FFS. |
| ZTE | Yes | Take this definition as baseline. Any other metrics for energy efficiency can be discussed later. |
| AT&T | Maybe | Support having an explicit KPI as defined in TS28.554, but exactly how to signal it can be further discussed to address the issues raised by Ericsson and CATT. |
| Nokia | No | As mentioned above, we support defining an alternative, meaningful EE metric, that can be well-understood and exchanged between neighbours, e.g., capturing the effects of additional or decreased traffic on the energy consumption of the cell. |
| Lenovo | Yes as baseline | Agree with companies above, such definition can be a baseline. RAN3 can further discuss more meaningful metric based on it. |
| China Unicom | Yes | We are fine with reusing the EE definition above. |
| Qualcomm | See comments | We can take this as a baseline to progress further. But RAN3 needs to discuss further on how Energy consumption is calculated, and it is possible to have a standard way of calculating it across the neighbor nodes. Also, as per our comments in Q3.1, we also need to understand how this information can be used in neighbor. If it is just for local consumption, then the existing SA5 metric can be followed as it is. |
| Intel | Yes | We also think reusing TS28.554 definition can be reused. |

Moderator’s summary:

Majority companies agree the proposal that the definition of energy efficiency in TS28.554 can be regarded as a baseline for energy efficiency in AI/ML based network energy saving. And two companies think the other metrics to represent energy efficiency needs to be discussed.

Proposal 2: The definition of energy efficiency in TS28.554 can be regarded as a baseline for energy efficiency in AI/ML based network energy saving. And the other metrics for energy efficiency are not precluded.

UE trajectory prediction

During the online discussion, companies want to clarify whether there is necessary to transfer the predicted UE trajectory over Xn. Supporting companies believe that predicted UE trajectory can benefit the subsequent network optimization in the target NG-RAN node.

Q3: Companies are invited to provide their views on whether the predicted UE trajectory is needed for AI/ML based mobility optimization? If yes, please clarify the benefits, usage and elaborate the details of its potential standard impacts.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | We think the predicted trajectory info, together with other info, could help the AI/Model better to infer a more proper HO target cell. The potential standard impacts could be: UE trajectory prediction over Xn, we also think the detailed info should be further discussed, the granularity of cell should be enough. |
| Samsung | Yes | The predicted trajectory helps target node for further mobility optimization. For high-mobility UEs, the collected data amount of a small-coverage cell (i.e. mmW cell) is small, which is not sufficient for AI/ML model to generate accurate predicted UE trajectory information. And the node capability to support AI/ML model is diverse, so there may exist some nodes with no ability for AI/ML model inference or the function for UE trajectory prediction. Thus, exchanging predicted UE information is beneficial for node to set the proper further mobility optimization strategy to improve the handover robustness and efficiency. The potential standard impacts is that source node can transfer the predicted trajectory information to target node. |
| China Telecom | Yes | UE trajectory prediction as an important output for mobility optimization use case could assist the target NG-RAN node to make further predictions of UE trajectory and UE handover decisions. Besides, in our point view, since the predicted UE trajectory may comprise of locations or camps cell and the corresponding time interval, it can be taken as feedback information to be compared to the actual UE trajectory, for the purpose of performance evaluation of one AI/ML model. |
| Ericsson | Needs further discussions | The feasibility of transferring the UE trajectory depends on how this information is encoded. If this information is encoded as a prediction in terms of cells the UE will pass through, then this may be ok. But if the information is supposed to provide predicted geolocation of the UE in time, this becomes first very complex, secondly very sensitive and thirdly it imposes a requirement on the RAN to be able to geolocate the UE. Hence, we would first like to converge on the definition of UE trajectory before agreeing whether the information shall be exchanged. It is not possible to agree to the inclusion of a piece of information without at least knowing what that information is. |
| CATT | Yes | Delivering the predicted trajectory during handover can be useful, e.g. beam-level prediction can be used for configuring target beams. |
| InterDigital | Yes but | It should definitely be supported, but we agree with Ericsson we have to have discussions on exactly what is feasible and not too sensitive, for example cell ids predicted to be visited. |
| ZTE | Yes | Predicted UE trajectory could be transferred via Xn interface to benefit the target NG-RAN node to perform the subsequent network optimization. And for the definition of the predicted UE trajectory, it could involve UE serving cells which will be resided in, or the predicted UE geographic location. Since companies have some concerns on the privacy, we could consider the predicted serving cells as a start point to discuss the predicted UE trajectory. |
| LGE | Yes | During handover, the target node that receives the UE trajectory prediction can use for further mobility optimization. |
| AT&T | Yes | Both cell-based, beam-based (e.g. for FR2), and UE geographic location should be further considered. The usefulness and feasibility of the different granularities of the information will depend on the use case, frequency layer, and time-scale involved, so tradeoffs between the options in terms of accuracy and simplicity need more discussion. |
| Nokia | Yes, with clarification | Cell-level UE Trajectory prediction can be forwarded to neighbouring gNBs. Since, all the actions that we have considered so far are on a cell level, we don’t think that we need to consider location information of lower granularity, e.g., based on GNSS. Also, in our view UE Trajectory prediction does not need to be calculated per UE. We think that it is enough if for a given cell it includes a probability with which a different sequence of cells is traversed starting at the given cell. |
| Lenovo | Yes | Agree that at least the predicted cell level mobility can be transferred to the target gNB. Open to discuss other options e.g., geographical location . |
| China Unicom | Yes | In general, UE trajectory prediction can be helpful for mobility optimization. Further discussion for detailed solution may needed. |
| Qualcomm | Yes | Transferring Predicted UE trajectory is a useful information for Mobility optimization. Procedures used for transferring UE Trajectory and the inputs and outputs for UE Trajectory Prediction can be FFS for now. |
| Intel | Need further discussion | We share the same view with Ericsson that it needs to be further clarified how UE trajectory can be transmitted over Xn interface, e.g. definition, format, etc. |

Moderator’s summary:

12 companies agree that transferring predicted AI/ML trajectory benefits AI/ML based Mobility Optimization, and the solution needs to be further discussed. And 2 companies think the predicted cell-level UE trajectory can be accepted, and other level (e.g. UE geographical location) needs to be further discussed.

Proposal 3: Cell-level predicted UE trajectory can be considered as external output information for AI/ML based mobility optimization. Other level of UE trajectory needs to be further discussed.

Procedures for AI/ML function

Following are two agreements during online session for a new procedure used for AI/ML related information:

Define a new procedure over Xn can be used for AI/ML related information, e.g., predicted information

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

The conflict can be found based on the contributions and the discussion in 1st round, whether the procedure for AI/ML related information should be non-UE associated procedure or UE associated procedure.

Q4: Companies are invited to provide their views on whether the procedure for AI/ML related information should be non-UE associated procedure or UE associated procedure?

| Company | Non-UE/UE associated | Comment |
| --- | --- | --- |
| Huawei | Non-UE associated | In general, we think it should be non-UE associated, since this new procedure should serve different purpose, e.g. data input, feedback, we think this should not be UE associated only. In addition, the input of AI/ML model anyway doesn’t need to identify a specific UE, and some cases, the feedback is based on cell level or node level. |
| Samsung | Case by case | It should be discussed case by case. For resource status and energy efficiency prediction, it can be the cell level. For UE traffic prediction and UE trajectory prediction to optimize mobility decision, it should be the UE-associated procedure. |
| China Telecom | case by case | Agree with Samsung. |
| Ericsson | Non-UE Associated | We have already adopted this principle in SON, where the Access And Mobility Indication is a non-UE associated procedure which includes UE related information together with UE AP IDs. Therefore there is no need to define UE associated procedures, we can reuse the design already adopted for SON. |
| CATT | Both | New non-UE-associated procedure carries e.g. EE (as DV and EC separately); New UE-associated procedure carries e.g. UE data volume (used for per-UE mobility and traffic steering). |
| InterDigital | Case by case | It is clear that there are cell level outputs, and UE feedback using SON principles would be non-UE associated, but it isn’t yet clear there isn’t data that is UE related while a UE context exists in the source gNB. |
| ZTE | Non-UE Associated | Share same view as E///. Predicted information involves UE related information and UE un-related information. But the procedure can be designed as non-UE associated procedure with UE IDs to carry UE related information (e.g., predicted UE traffic, predicted UE trajectory), as similar as the procedure for paging. |
| LGE | Non-UE associated | The input data for AI/ML training/inference and the feedback can be provided using the non-UE associated procedure. For the UE related information, we have a similar view with Ericsson. |
| Nokia | Non-UE Associated | Non UE-associated procedures are capable to communicate information even if this information is related to specific Ues. We agree with Ericsson. |
| Lenovo | Non-UE associated | Prefer a unified solution, since non UE associated procedure can also carry UE AP ID. |
| China Unicom | Non-UE associated | In our understanding, cell level information transferring would be enough. |
| Qualcomm | Non-UE associated | Agree with E///. Further details on cell level or node level or including UE AP ID can be FFS for now. |
| Intel | Non-UE associated | Same view with E/// that further details on cell level or other information can be FFS, since predicted information can be used for all UEs within a cell, etc. Also, non-UE associated procedure can also carry UE AP ID. |

Moderator’s summary:

9 companies prefer to design a new non-UE associated procedure for AI/ML related information. And 4 companies think it needs to be discussed case by case.

Proposal 4: The new procedure over Xn used for AI/ML related information should be non-UE associated.

Discussion - 1st

Standard impacts on procedures for AI/ML function

Based on the TR37.817, majority companies in their contributions mention that for some information  for three agreed AI/ML based use cases, (Network Energy Saving, Load Balancing, and Mobility Optimization) needs to be enhanced over Xn/F1/E1 interface.The information for AI/ML function is divided to input information, output information, and feedback information.

Since there is some overlapped information across these use cases (e.g., predicted resource status, current resource status, and UE performance, etc), there are two options as follows:

- Option 1: Enhance/reuse the existing procedure if the revelant procedure can be found.  [16][21][23][24][38][39][40][44][45][46] [25][26][27]

- Option 2: Define the new procedure for the information used for AI/ML.[8][32][35][36][37][38] [50]

Q1: Companies are invited to provide their views on which option above is preferred to support AI/ML function for the overlapped information (e.g. predicted resource status, current resource status, predicted UE trajectory and UE performance, etc )?

| Company | Which option? | Comment |
| --- | --- | --- |
| Lenovo |  | We are fine to go for either way, both have pros and cons as we discussed in our paper R3-224421. |
| Huawei | Option 1 preferred | Option 1 is simpler, technically we see pros and cons for both options, we are also open to further discuss new procedures, but in our understanding, we don’t see the need to introduce new procedure in E1/F1, maybe we could start from this point. |
| Qualcomm | Option 1 | Option 1 is preferred as mentioned in our paper. Resource Status Reporting procedure and Handover related procedures can be reused to transfer the AI/ML related data. If there a need to introduce new message, we are open to discuss them further. |
| Samsung | Option1 | According to the R17 discussion, most input/output/feedback information identified for AI/ML training or inference has a relevant existing procedure to collect. Hence, if applicable, taking existing procedure as baseline and enhancing it to support AI/ML functionality are convenient. |
| vivo | Option 1, FFS for Option 2 | It is straightforward to reuse the existing relevant procedures, e.g., reusing Resource Status request/response for the predicted resource status transfer. If there is no relevant procedure, e.g., UE associated feedback, new procedures shall be introduced. |
| Ericsson | Option 2 | RAN3 has so far followed a modular design. That is the reason why the Resource Status Reporting procedures over Xn have been defined for a specific function, namely Mobility Load Balancing. Mixing data used for AI/ML in a procedure that is today associated to a specific function (MLB) does not allow for a modular design. The same procedure needs to be used for different functions and subject to failures due to different reasons (nodes behaviours and processes). Troubleshooting problems becomes therefore more difficult. Dependencies between functions become more difficult. Also, Resource Status Reporting procedures follow a structure that may not be optimal for AI/ML. For example, the Resource Status Request may need to be enhanced with a new “Cell To Report” list, indicating the cells for which AI/ML information are required, which might not be the cells for which MLB data are required. This makes the procedure more complex. Also consider message sizes. Putting in the same message massive amount of information for MLB and AI/ML and on top signal these message frequently creates a big challenge at transport and implementation level. Smaller messages are easier to handle. Finally, even in terms of compliance, it is more appropriate to define dedicated procedures. Reusing existing procedures would bring ambiguity to AI/ML compliance as support of the legacy procedures reused for AI/ML may be interpreted as compliance to AI/ML support. We do not see the need for new procedures over F1 and E1. |
| CATT | Slightly prefer O2 | We have slight preference on option 2 while option 1 is also acceptable for us. |
| Intel | Option 1 for current status information, UE performance, feedback; Option 2 for predicted information | For current information, e.g. current resource status, UE performance, other feedback information, since existing procedure already support some information exchanging, option 1 is preferred for current information. However, for predicted information, since it highly depends on whether the requested NG-RAN node has such AI/ML capability to generate such information or not, a separate procedure for predicted information exchange is preferred. Additionally, the nature of predicted information is also different from the current information, as discussed in R3-224770. The predicted data is available only after the results being generated from Model Inference, which is different from current information which are always available. The legacy procedure (i.e. periodically reported) may not be suitable for predicted information. |
| Nokia | Option 2 | We support defining new procedures to exchange AI/ML information. This option will make AI/ML functionality across different use cases simpler and it would also avoid extending each legacy procedure with new IEs and extra complication. We think it is a more clean way of introducing AI/ML functionality in the RAN. We agree with Ericsson on their analysis. |
| LGE | Option 2 | We think the existing procedure, e.g., Resource Status Reporting procedure, may not be appropriate for AI/ML, as Ericsson’s example above. |
| ChinaTelecom | Slightly Option 2 | We are fine with both options, but we are inclined to Option 2. - Goals of design: The existing procedures specified in XnAP, e.g., Resource Status Reporting procedures are applicable for SON MLB operation. It aims to acquire the long-term measurement result from the neighbor nodes. However, AI/ML procedures aim to acquire long term or short-term data or feedback information. Clearly, the design objective of MLB and AI/ML operation are different. - Extensibility: To support AI/ML Operation in more use cases, the procedure for AI/ML should have good extensibility. The signaling procedure we talked here is mainly leveraged for transmitting predicted information or dedicated AI/ML data, which involves timing information as well. Therefore, the most of the existing IEs in Resource Status Reporting procedures could not be directly reused in AI/ML operation. Too much data with different usage, e.g., load balance, ES, and Mobility included in Resource Status Reporting procedures makes the excessive amount of signaling message. Compares says, a new procedure is easier to achieve good extensibility. Implementation &IoT issue: Here, we share the same view as Ericsson that troubleshooting problems becomes more difficult if MLB and AI/ML operation merged in a single signaling procedure. |
| Xiaomi | Option 2 or depends | Both are OK to us, considering AI/ML is new function in NG-RAN, option 2 may be more clear and future proof. But we also acknowledge that some can reuse the existing procedure and some may need new procedure e.g. prediction related. |
| InterDigital | Slightly Option 2 | We think that Option 2 is better but have assumed option 1 in our discussion papers so either will work |
| ZTE | Option2 | We prefer Option2. As explanied in our contribution [39], Option2 seems much clearer and simpler, and can be used for multiple AI/ML based use cases. For the future extension, we just needs to identify the common predicted information, and involve these prediction information into the unified AI/ML procedure, instead of finding a proper existing procedure to involve a new identified information for AI/ML based use cases. |
| CMCC | Option 1 | Prefer to reuse the existing procedure if the relevant procedure can be found, such as the current/predicted resource status, UE trajectory procedures. |

Moderator’s summary:

5/15 companies prefer to reuse the existing procedure for AI/ML related information if the relevant procedure can be found, while 9/15 companies prefer to define the new dedicated procedure for AI/ML considering extensibility, clear design, and message size. And some companies also think both ways are fine considering that current information can use the existing procedure, and new procedure can be defined for predicted information.

Proposal 1-1: Define the new dedicated new procedure for AI/ML predicted information (e.g., predicted resource status, etc).

FFS whether to use the existing procedure or dedicated new procedure for other input , output and feedback information.

Following are the new procedures proposed by companies:

- Option 1: Define the new unified procedures for AI/ML function for input information, output information, and feedback information informatio [39][40]:

  - AI/ML Data Collection Procedure (to collect historical information)

  - AI/ML Predicted Information Procedure (to transfer predicted information)

  - AI/ML Feedback Information Procedure (to retrieve feedback information)

- Option 2: Define two non-UE associated procedure for predicted information and UE performance [44][45][46]:

  - AI/ML Assistance Data Reporting Initiation (for handling the subscription mechanism)

  - AI/ML Assistance Data Reporting (for handling the collection of subscribed information)

- Option 3: Define two non-UE associated procedure, and two UE-associated procedure [16]:

  - Node Data Collection Initiation procedure and Node Data Collection Reporting procedure

  - UE Data Collection Initiation procedure and the UE Data Collection Reporting procedure

- Option 4: Define the new procedure only for predicted information [23][24]:

  - Predicted Information Request/Response/Update

Q2: If you agree to define new procedures, companies are invited to provide their views on which stage3 signalling design option over interfaces above is preferred?

| Company | Which option? | Comment |
| --- | --- | --- |
| Lenovo | Option 4 | If we go for new procedures, it seems a cleaner option to define new procedure only for predicted information to distinguish from legacy procedures for the same . A general question needs to be clarified if we call the new procedure “AI/ML xxx procedure” as in Option 1 and 2 is that, would that imply the carried information cannot be used for non-AI/ML purpose? E.g., the UE performance feedback after the HO. Similarly, would that imply other legacy procedures will not be used for AI/ML purpose, which doesn’t seem to be the intention. |
| Huawei | See comment | In our understanding, if we would like to introduce new procedure, this procedure should be common for all use cases (including possible new use cases in the future), and should be common for request/response for the data of different purpose, e.g. history info/predicted info etc. |
| Qualcomm |  | See our comment for Q1. Prefer Reuse. But if there is a strong preference to use new message, we prefer a single Request/Response procedure for all the AI/ML use cases. |
| Samsung |  | If no relevant existing procedure, the detailed new procedure can be discussed case by case. |
| vivo |  | Revisit this issue when the Xn impact is concluded, e.g., feedback. |
| Ericsson | Option 2 | We believe that the new procedure to be defined should be use case and information agnostic. Namely, there should be a single procedure to setup and report all types of AI/ML data (inputs, outputs, feedback) and for all use cases. The reason for this is that there is no gain at a functional level in creating different procedures for each use case and for each type of data. The only node that needs to know for which use case information is needed and the type of information that is needed is the requesting node. The reporting node only needs to know which data it should report. Therefore, there is no need to make a dedicated procedure per data type and per use case. The requesting and reporting nodes can communicate which data is needed via a single procedure. We see that the new procedure could resemble the Resource Status Reporting Initiation and Resource Status Reporting over Xn. However, as explained in the previous question, we would like to have a dedicated procedure to make sure that resource status reporting is not impacted, given that this procedure has been used for many releases for MLB. We also think that defining a procedure like in Option 2, which is based on the Xn: Resource Status Reporting is advantageous because it allows us to adapt to AI/ML, e.g. reporting periods values may be added depending on the use case, partial stops for specific cells may be adopted, etc. |
| CATT | O3 > O4 > O2 > O1 | For O1&2, We share the view of Lenovo that “AI/ML xxx procedure” is not a good name as they can be used for other purpose as well. That is the very reason why we call it “xxx Data Collection xxx procedure”. O4 is acceptable for us anyhow. |
| Intel | Option 4 | As explained in Q1, the current data can be well supported by existing messages and procedures. The new procedure can be introduced for predicted information which can be commonly used by all use cases. Regarding to Lenovo’s question, the data exchanged over new procedure does not prevent the received NG-RAN node to use it for other purpose, e.g. implementation, etc. |
| Nokia |  | In general, it seems premature to decide already at the very first meeting of this WI the needed procedures (whether we need 1 or 2 or 3 of those, whether they need to be UE associated or non-UE associated). In our view, collecting data for Model Training may use existing reporting procedures. A lot of the currently identified input is based on existing information that can be exchanged over Xn interface between neighbours. The node providing information for Training may in some cases be completely oblivious on the purpose it provides the data. When it comes to reporting of predicted information, we don’t think that existing resource status procedure can be re-used. For instance, the procedure should be able support longer reporting duration, possibly smaller granularity/period to make sure enough predictions are received. Also, it needs to be able to capture changes in inference output due to changes in the environment, currently not supported by existing resource status procedure. We agree that the “principle” of reporting predictions from neighbours should be based on a “subscription-based” solution where a node requests a certain prediction after which reporting is started, same for feedback information. There may be need to exchange UE-associated information but at this point we haven’t identified the need to define UE-associated procedures. For instance, regarding UE Trajectory prediction we don’t think that a gNB will possibly keep trajectories for each UE, but some average will be calculated based on a number of UEs traversing a sequence of locations/cells. Thus, it seems at this point that non-UE associated procedures are sufficient. |
| LGE |  | We think the following non-UE specific procedures for reporting of AI/ML data are needed: - The procedure that requests the AI/ML data to its neighbor nodes; - The procedure that reports the requested AI/ML data; We also are not clear why a UE-associated procedure is needed. It is because the feedback information will be provided after the action (e.g., handover) is completed like SON report is sent over Xn. |
| ChinaTelecom | Option 2 | We prefer to define a unified procedure for one NG-RAN node to retrieve AI/ML related input, output and feedback data from other nodes, what exact the information contains in the input or output or feedback message, depends on the use cases. For instance, in case of Network Energy Saving, predicted energy efficiency and predicted resource status from neighbouring nodes are required. Besides, we think the new signaling could resemble the existing Resource Status Reporting procedure. |
| Xiaomi | Option 2 or option 4 | If a whole new procedure is needed, we prefer option 2. If only define new procedure for predication, option 4 seems ok. |
| InterDigital |  | I think we might need to wait, for example how to handle feedback which would be post-handover and handling potential signalling as part of handover. I do agree that it should be at least a single procedure outside of current resource status procedure for feedback. |
| ZTE | Option 1 | We prefer Option1 that define the procedures for the input, output and feedback for the AI/ML based Use cases. And we think the non-UE associated message is suitable for AI/ML related cases. If we need to transfer UE related information (e.g., UE trajectory, UE performance), we could use the temporary RAN UE ID over interfaces as the current mechanisms. |
| CMCC | Prefer Option 4 | As mentioned in Q1, we prefer to reuse the current procedures. Only for the predicated information which need to be exchanged between gNBs, it is specific introduced by AI/ML, may need to define new procedures. Option 4 is preferred. |

Moderator’s summary:

6 companies prefer option4 to define the new dedicated procedure for AI/ML predcited information if the new procedure is needed. 1 company prefer option1 that define the procedures for the input, output and feedback for the AI/ML based use cases. 5 companies mentioned that the procedure for AI/ML funtion should be based on “subscription-based” solution (e.g., request/response). And 3 companies think it is premature to disuss the needed procedure. And some companies are not sure whether the procedure should be non-UE associated procedure or UE associated procedure.

Proposal 1-2: The new procedure for predicted information should be based on subscription-based solution.

FFS whether the procedure should be non-UE associated procedure or UE associated procedure.

Network Energy Saving

Since, in RAN1 there is one study item “Study on network energy savings for NR”, [12][25][40] [51] propose to focus on the cell-level energy saving strategy (e.g., cell activation/deactivation), and avoid the overlapped discussion compared to RAN1 led SI.

Prosposal: Regarding AI/ML based Energy Saving, cell-level energy saving strategy  as a start point in RAN3, avoiding overlapped discussion for network energy saving in RAN1 SI.

Q3: Companies are invited to provide their views on the proposal above?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Lenovo | Yes | It would be safer to keep the discussion on cell level to avoid overlapped discussion. |
| Huawei | Yes | We could start from cell level activation/deactivation; and of course, overlap with SI on ES should be avoided. |
| Qualcomm | Yes | We could start with cell level and avoid overlap with RAN1 |
| Samsung | Yes | The cell-level energy saving has been supported by the spec, and AI/ML can help to do the optimization. There is another SI network energy saving, and other granularities maybe will be studied in this SI. In current stage, it is better to take cell switch on/off as the starting point. Other granularities depends on the progress of network energy saving SI, whose corresponding AI for RAN impact can be delayed. |
| vivo | Yes | Follow RAN1’s conclusion. |
| Ericsson | See comment | First of all, cell level energy saving strategies are not limited to cell activation/deactivation. TR37.817 mentions “Efficient energy consumption can also be achieved by other means such as reduction of load, coverage modification, or other RAN configuration adjustments.”. for this reason we propose to remove (cell activation/deactivation) from the proposal. We are fine to focus our efforts on cell level energy efficiency solutions, but we should also consider that any cell level energy saving action cannot be detrimental to UE energy consumption. Or at least the choice of cell level energy saving action that is detrimental to the UE should be taken in an educated way. This is why we suggest to also look at UE feedback when deciding which cell level energy saving action to take. |
| CATT | Yes |  |
| Intel | Yes | Agree with comments from above companies, overlapping with RAN1 new SI NES should be avoided. |
| Nokia | Yes | It is true that we should avoid overlapping discussions between groups. We should focus on cell level actions during this work. |
| ChinaTelecom | Yes | overlapping with RAN1 SI should be avoided |
| Xiaomi | Yes |  |
| InterDigital | yes |  |
| ZTE | Yes | Follow RAN1’s agreement, and focus on the cell-level energy strategy in RAN3. |
| CMCC | Yes | The discussion should be limited in cell-level energy saving strategy, to avoid the overlap/conflict with RAN1. |

Moderator’s summary:

All companies agree that RAN3 focus on the cell-level energy saving strategy as a start point, to avoid overlapped discussion for network energy saving in RAN1 SI.

Proposal 3: RAN3 focus on the cell-level energy saving strategy as a start point, to avoid overlapped discussion for network energy saving in RAN1 SI.

Following information for AI/ML based network energy saving use case which has standard impacts are summarized based on contributions:

Input Information

- Predicted resource status information from neighbour NG-RAN nodes over Xn [1][10][25][37][40][44][48]

- Historical resource status [40]

- Current/Predicted Energy Efficiency from neighbour NG-RAN nodes [1][8][25]

- Current/Predicted energy state from neighbour NG-RAN nodes [1][8]

- Current/Predicted Energy efficiency score [44]

- Predicted overload status information over Xn/E1/F1 [25]

- Energy efficiency [23]: NG-RAN data energy efficiency, Network slice energy efficiency, including energy efficiency of eMBB, uRLLC and mIoT, PNF power consumption, including average power, minimum power, and maximum power, PNF energy consumption ,Energy state (high/low/active/inactive)

- Measurement frequency for reporting EE [37][51]

Output Information

- Predicted energy saving strategy (e.g., predicted cell switch-on/off decision) [25]

- Time for switch-off indication to show when the cell switch-off [25]

- Predicted time length indication that the cell will stay in the current activation/deactivation. [12]

- The predicted load transferring plan [25]

Feedback

- UE QoS parameters (Handover interruption time, UL Data Rate, UL Data Rate) [1][8]

- Per-cell total DL/UL UE throughput (i.e. “DRB.UEThpDl” and “DRB.UEThpUl” in TS 28.552)[16]

- Per-cell “Average delay DL air-interface” and the “Average delay UL on over-the-air interface” [16]

- RVQoE measurements [44]

- UE energy consumption [44]

- UE level performance (e.g., UL/DL throughput, packet delay, packet loss) [44]

Q4: Companies are invited to provide their views on which information listed above has the standard impact with priority order. And Elaborate more on the detailed standard impact.

| Company | Which? | Comment |
| --- | --- | --- |
| Lenovo | Input: 1, 3 Output: 9 or 11, Feedback: 13 or 18, | 1: the predicted resource status can be carried in enhanced RESOURCE STATUS REPORT or new procedure depending on Q1. 3: new procedure is needed to exchange energy efficiency. 9 or 11: depending on if new procedure is used for transmitting the predicted e.g. cell activation/deactivation. At least it can be done in a way that when indicating the cell activation/deactivation as legacy, the gNB also indicate for how long the cell will stay in the activation/deactivation state. 13 or 18: After handover a UE due to energy saving, the UE performance after HO can be feedback to the source gNB. 13 and 18 look similar. |
| Huawei | See comment | Input: predicted resource status info 5, current/predicted EE 1 Output: predicted resource status info 5 Feedback: EE (energy efficiency), resource status info |
| Qualcomm |  | Input – 1, 6 Output – predicted resource status info, predicted cells to be deactivated/switched off and the duration Feedback – 14 Energy Efficiency, Energy State and Energy Score are useful only when there is a standardized procedure on how to calculate them. Else the values provided cannot be interpreted correctly by the consumer. UE Energy consumption is not needed to calculate a network energy efficiency. |
| Samsung | Yes for 1, 3, 6, 9, 10, 11, 12. Yes but more description is needed: 13, 16, 18 | For 1, yes. It needs to collect predicted resource status information from neighbor node to generate ES decision to avoid local overload, switch on/off ping-pong. For 2, it can be collected by existing resource status reporting procedure. For 3, yes. Energy efficiency exchange can help to realize the global optimization. For 4, it is better to start with cell switch on/off as Q3. For 5, energy efficiency is more accurate. For 6, yes. When predicted load status is high, nodes can send the predicted overload indication to peer node. In such mechanism, no request is needed so that the overload information can be transferred timely. For split architecture, DU and CUUP can send the predicted overload indication to CU. For 7, the energy efficiency definition can refer to 28.310 as the Data Volume divided by the Energy Consumption. For 8, the energy efficiency is kinds of resource status, and the detailed procedure can take the resource status reporting as the baseline. For 9 and 10, yes. The energy saving strategy can be the action for a time point/period for future. For example, a node predicts it will be switched off in one minute. The node can exchange such as predicted cell switch-on/off decision with its neighbours to inform the action plan in advance, so the neighbour cells can take it as reference information to make proper decision (such as UE handover, load transferring, switch on/off and so on) to avoid the unnecessary handover, handover ping-pong, switch-off/on ping-pong, local overload etc. For 11, yes. The predicted time information for the current activation/deactivation help the neighbor node to set SON decision to select target node to offload load or to handover UE. For 12, yes. If deciding to switch off a cell, the existing load needs to be offloaded to neighbor node. AI/ML model generates the predicted traffic/load transferring action for a period for future. For example, a node predicts it will switch off in the future and generates the predicted handover strategy or load transferring plan in advance to avoid local overload and consecutive handover. If the target neighbor node can not accept the load, the node can make other proper candidate plans to guarantee the successful handover/transferring. Otherwise, the energy saving action may be delayed due to remaining load that has not transferred out successfully. Thus, it is beneficial for energy saving plan. For 13, partial yes. The UL/DL data rate is fine, but handover interruption time is a little bit unclear. For 14 and 15, the cell throughput is related to multiple factors, such as resource allocation policy, service type, UE mobility, etc. Thus per cell performance can not reflect the impact of energy saving decision directly. For 16, yes but RVQoE should be the parameters of handed-over UEs due to energy saving decision. It can help to see whether the energy saving decision and related offloading policy are good or not. For 17, UE energy consumption seems not in the scope of current energy saving. Now, it mainly focus on the base station energy saving. For 18, yes but the UE level performance should be the parameters of handed-over UEs due to energy saving decision. It can help to judge the impact of energy saving decision. |
| vivo |  | Input: 1, 3 Output: none as all are internal output without standard impact. Feedback: 14, cell level feedback is needed |
| Ericsson | Input: 1, 2, 3, 5 Output: 12 Feedback: 16, 17, 18 | With regards to Inputs it is worth mentioning that Input 5 Is simply an index expression of Input 3. We believe that expressing energy efficiency is sensitive over inter vendor interfaces, hence an indexed version of this metric would be easier to support as it does not reveal the exact energy efficiency of a node, but only how it improves or degrades. With respect to Outputs: we consider output 9 as an internal output to the node running inference. With respect to feedback: We see that feedback 13 is largely contained in feedback 18. We also see no technical reason to provide per cell level feedback (feedback 14, 15) because such feedback does not allow to check if the result of a per UE AI/ML action had a positive or negative effect. |
| CATT | Input:1 Output:9 Feedback:14,15,16 | 1: We think the predicted resource status from neighbor node could provide assistant information for the energy saving decision. 9: The main intention of AI for Energy saving is to make decision on cell on/off. 14, 15,16: This information could help the NG-RAN node understand whether the energy saving decision is good or not. |
| Intel | Yes for Input: 1, 3, 4, 7; Output: 10 Feedback: 14, 15, 18 | 1: the new procedure for exchanging predicted information can be used to carry 1 over Xn interface 3: current energy efficiency is carried over existing procedure, e.g. resource status report, while the predicted energy efficiency is carried over the new procedure. 7 is the same as 3, which gives the definition of how to calculate energy efficiency, as defined in TS28.554. Besides, power, energy consumption defined in TS28.552 (e.g. PNF power consumption, etc) can also be included as part of energy efficiency. 4: the energy state of neighbouring NG-RAN node is essential to for the requesting NG-RAN node to decide whether it can be selected as the target cell for the impacted UEs. The state of NG-RAN node can be exchanged over Xn interface together with energy efficiency. 7: it’s detailed definition of 3, as defined in TS28.554 and TS28.552. 10: the time for switch-off indication can be treated as validity time of when the energy strategy predicted strategy becomes valid. Since the output of model inference is a predicted information in the future, the corresponding time of when the behavior should taken place should be known by the node itself, as well as by the actors. 14/15/18: we believe those information can reuse the definition in TS28.552 and TS38.314. The performance information can be exchanged over Xn interface. |
| Nokia | Input: 1 Output: 9, 10 (12) Feedback: 13,18 | 1.Predicted resource status information from neighbours can be sent over Xn. 2. We don’t support forcing a node to store historical data. In the availability of resource status reporting, historical load can be created at a node interested to “consume” this information. 4. Energy State seems redundant if the only options are that a cell is “on” or “off”. The way it is currently captured in the TR is confusing as it involves other states as well (active, high, low, inactive). 3, 7: We don’t support to exchange energy efficiency information between neighbours since in our view this is internal information of each gNB. Besides, the parameters required to calculate the Energy Efficiency, e.g. Data Volume (DV) and Energy Consumption (EC), other parameters may be needed to interpret variations in the Energy Efficiency KPI values from different network nodes. These can be classified into different classes dependent on demography, topography and climate and describe the network characteristics with regard to population density, geographical conditions and climate zones. 5: We agree with the intention of the energy efficiency score not to expose energy efficiency information but it is unclear it can be useful if a node cannot “exactly” interpret this information. An energy related score/cost could be useful, e.g., to capture what is the cost associated to adding traffic to a node or subtracting traffic from a node but could be difficult to be understood by the neighbours unless it is expressed in absolute values, e.g., additional energy consumed by adding X GBs of traffic to a cell. 6. Predicted overload could be determined at a node by using resource status from a neighbour and calculate a predicted load. No need to introduce a new predicted overload status information over Xn/F1/E1. 9,10: If a node predicts to switch off one of its cells it can indicate this information to its neighbours (using existing e.g., NG-RAN Node Configuration Update procedure), in which it can include a “Deactivation” indication and a time in the future or a time window when this will happen. 12: Seems to have the same effect as sending a predicted energy saving strategy (e.g., switch off a cell) in advance (9) with some timing indication (10). 13,18: Both seem related to feedback information related to UE performance for those UEs being handed-over from source to target. |
| ChinaTelecom | Input: 1, 3 Output: 9, 10 Feedback: 13,14 | 1: Predicted resource status information should be transmitted over Xn/F1 interface for ES decision making. 3: Current/Predicted Energy Efficiency is essential for network node to make analyze and optimization, and the information exchange need signaling enhancements. For 9 and 10, the purpose of the ES use case is using AI/ML tool for making optimized ES decisions, the corresponding ES strategy and execution time is essential. For 13 and 14, we think UE performance and cell throughput are important factors to be considered as feedback. |
| Xiaomi | Input: 1 Output: 9, 12 Feedback: 16, 18 | For input, we don’t see much value on the others except 1. For output, we think Predicted energy saving strategy is the baseline, and it would be good to have the predicted load transferring plan. For feedback, we think RVQoE can reflect the real performance of UE, however, RVQoE may not be used in some cases, where UL level performance metrics can be used. |
| InterDigital | Input 1, 3 or 5, 4 Output 12 Feedback 13, 16 | OK on use of 3 or 5 agree with Ericsson it is basically the same. Agree with Ericsson that 9 is internal output 12 would be the external one For feedback 13, but 18 is a similar set, probably should come to a conclusion on which parts of 13/18 are a good set – the agreed set would be feedback for the load balancing use case and probably the handover optimization case. Feedback 16 yes as it would help understanding if the ES decision was good. |
| ZTE | Input: 1, 2, 3 Output: 9, 10 Feedback: 18 | Input: Predicted resource status from neighbour NG-RAN nodes is needed for the optimization. And historical resource status is also needed. The current resource status reporting just supports one-shot and periodicity reporting, but cannot report the historical information in the past period. Hence, the historical information should be discussed. Regarding predicted energy saving state, predicted overload status information, this can be obtained by predicted energy efficiecny and predicted resource status. Regarding feedback, we think the UE level performance could be the start point to discuss in WI phase. |
| CMCC | Suggest mandatory for input 1 and 3; output 9 and 10; feedback 13-15. | For input 1 and 3: IE should be defined in XnAP to support the inter-gNB energy efficiency information exchange. For output: 9 and 10 should be mandatory;9 may introduce new procedure; 11, 12 can be optional and discussed later; For feedback: 13-15 should be mandatory, 16-17 can be optional, these measurements may need to be newly defined, and raise the requirements on UEs. 18 (UE performance) is similar with 13(UE QoS performance), should be merged. |

Moderator’s summary:

The following listed are in order of priority of over-half support:

1.Predicted resource status information from neighbour NG-RAN nodes over Xn (14 supporting)

3.Current/Predicted Energy Efficiency from neighbour NG-RAN nodes (10 supporting)

13/18. UE performance (UL/DL throughput, packet delay, packet loss) (10 supporting)

9. Predicted energy saving strategy (e.g., predicted cell switch-on/off decision) (9 supporting)

10. Time for switch-off indication to show when the cell switch-off (6 supporting)

16. RVQoE measurements (6 supporting)

Proposal 4: Regarding AI/ML based Energy Saving, the following information should be specified as a start point:

- Predcited resource status information from neighbour NG-RAN nodes over Xn

- Current/Predicted Energy Efficiency from neighbour NG-RAN nodes over Xn

- UE performance (UL/DL throughput, packet delay, packet loss)

Load Balancing

Following information as input which has standard impacts for AI/ML based load balancing use case are summarized based on contributions (except the overlapped information e.g., predicted own resource status and predicted resource status from neighbouring NG-RAN nodes):

Input Information

- Predicted UE performance received from a target NG-RAN node [52]

- Predicted or measured UE traffic over E1 [11]

- Current and Predicted resource status information of neighbouring NG-RAN node(s)

Output Information

- Predicted resource status information of neighbouring NG-RAN node(s) [6]

- Predicted load balancing strategy [26]

- Indentification of an incoming handover for the purpose of AI based load balancing [36]

Feedback

- UE level performance metrics (e.g., UL/DL throughput, packet delay, packet loss) [36][45][52]

- RVQoE measurements [45]

Q5: Companies are invited to provide their views on which information listed above has the standard impact with priority order. And Elaborate more on the detailed standard impact.

| Company | Which? | Comment |
| --- | --- | --- |
| Lenovo | Input: 2 Feedback: 6 | 2: In case of CU-CP CU-UP split architecture, some enhancements are needed for CU-CP to receive either actual UE traffic measurement, e.g., data volume from CU-UP to make a prediction, or CU-CP requests the CU-UP to provide a prediction result. |
| Huawei | See comments | Input: predicted resource status information (output from neighbor node), 3 Output: predicted resource status information, 3 Feedback: UE level performance metrics as feedback info, 6 |
| Qualcomm |  | Input – 2, 3 Output – 4, Mobility Actions Feedback – 6 |
| Samsung | Yes for 1, 2, 3, 5 Yes but more description is needed: 7, 8 | For 1, yes. Predicted UE performance from target node can help source node to choose proper target node to offload load to guarantee the UE performance during handover. For 2, yes. CUUP has the information of UE traffic information. E1 impact should be studied. For 3, yes. The current resource status of neighbour node can be collected by existing resource status reporting. The predicted resource status reporting can take the existing scheme as the baseline. For 4, the node can collect the resource status of neighbor nodes and then do the prediction to provide the reference information for SON decision. However, there is no need to transfer the predicted status back to the neighbor nodes. For 5, yes. Same as energy saving. AI/ML model generates the predicted traffic/load transferring action for a period for future. For example, a node predicts it will switch off in the future and generates the predicted handover strategy or load transferring plan in advance to avoid local overload and consecutive handover. If the target neighbor node can not accept the load, the node can make other proper candidate plans to guarantee the successful handover/transferring. Otherwise, the energy saving action may be delayed due to remaining load that has not transferred out successfully. For 6, it is a little bit unclear about the identification of an incoming handover. For 7, yes but the UE level performance should be the parameters of handed-over UEs due to load balancing decision. It can help to judge the impact of load balancing decision. For 8, yes but RVQoE should be the parameters of handed-over UEs due to load balancing decision. It can help to see whether the related offloading policy are good or not. |
| vivo |  | Input: 2, 3 Output: none as all are internal output without standard impact. Feedback: 7 UE level performance feedback. |
| Ericsson | Inputs: 3 Output: 4 Feedback: 7, 8 | Concerning Inputs: We believe it is not possible to express input 1 because the neighbor RAN is not able to derive the radio conditions and therefore the performance of the UE once handed over to the target cell Concerning Outputs: We consider the predicted load balancing strategy (output 5) as a node internal output. Output 6 is also in a way node internal as it may come as the inference output of the IA/ML function |
| CATT | Input:2 Output :4 Feedback:7 | 2: predicted and measured UE traffic is useful when deciding which UE should be offloaded 4:This is natural the output of load balance use case 7:RVQoE provide assistant information on whether load balancing decision is proper or not. |
| Intel | Yes for (after number revision) Input: 4 Output: 4 Feedback: 7 | Following what agreed in TR37.817. |
| Nokia | Input: 1, 3 Output: 4 Feedback: 7 (with a clarification) | 1: Predicted UE performance from the neighbour could be based on QoS performance prediction of the UE’s PDU sessions. For instance the target could predict the probability that a certain PDU session is dropped after a Handover. Or a probability with which the QoS requirements of a PDU session will be violated after the handover. In general this can be a prediction in a performance degradation that a UE may possibly experience after a handover to a target. 3: Current and Predicted Resource Status information is already in the agreed input from the neighbour in TR 37.817. 4: Predicted Resource status information of “neighbouring nodes” will be consumed internally so no standards impacts are foreseen. 7: UE level performance metrics (e.g., UL/DL throughput, packet delay, packet loss) “of handed over UEs” can be sent in the feedback as already agreed in the TR. |
| China Telecom | Input: 2, 3 Output: 4, 5 Feedback: 7 | 2: predicted and measured UE traffic is important and the E1 Interface case should not be excluded. 3: essential information to be considered for making LB strategies, and we suggest a new procedure to transmit the related information. For 4 and 5, they are basic output for Load Balancing use case. |
| Xiaomi | Input: 1, 2 or 3 Output: 5 Feedback: 7, 8 | For input, we think UE traffic can be reflected by resource status or vice visa. For output, we think Predicted load balancing strategy covers the other two. For feedback, same as the comments in 3.2 |
| InterDigital | Inputs 3 Output: 4 Feedback 7, 8 | As stated in previous point, feedback 7 should be aligned with the UE QoS feedback for energy savings. |
| ZTE | Input: 3 Feedback: 7 | Input: predicted resource status information is needed for optimization. Output: none as all are internal output without standard impact. |
| CMCC | Suggest mandatory for Input 3; output 4-5; Feedback 7. | For input 1, the predicted UE performance can help choose the proper node for load balance, but need to define the new procedure through Xn; for input 2, suggest to leave E1 impact for later discussion; for input 3, same as above procedure in Energy Saving; For output 4-5, they are needed but no impact on current standards, for internal use. For output 6: no need to mandatory. For Feedback 8: the RVQoE measurements may need to be defined and discussed later. |

Moderator’s summary:

The following listed are in order of priority of over-half support:

7.UE-level performance (e.g., UL/DL throughput, packet delay, packet loss) (14 supporting companies)

3.Current and Predicted resource status information of neighbouring NG-RAN node(s) (11 supporting companies)

4.Predicted resource status information of neighbouring NG-RAN node(s) (9 supporting comapnies)

2.Predicted or measured UE traffic over E1 (6 supporting companies)

Proposal 5: Regarding AI/ML based Load Balancing, the following information should be specified as a start point:

- Predicted resource status information of neighbouring NG-RAN node(s)

- UE performance (UL/DL throughput, packet delay, packet loss)

Proposal 6: Predicted resource status information of neighbouring NG-RAN node(s) as output is internally used, and no standard impacts.

Mobility Optimization

Following information as input which has standard impacts for AI/ML based Mobility Optimization use case are summarized based on contributions (except the overlapped information as predicted own resource status, and predicted resource status from neighbouring NG-RAN nodes):

Input Information

- Predcited UE traffic over Xn/E1 [5][8][13][16] [27]

- Predicted QoS performanceo of UE. [53]

- Prediction regading an expected (incident) load from target NG-RAN node.[53]

- RSRP prediction from UE [20]

Output Information

- UE trajectory prediction over Xn (e.g., predicted serving cell, …) [8][13][16][27][41]

- Handover execution timing [5][23][35]

- Estimated Arrival Probability in CHO [5][16][23]

- Predicted resource reseveration time window for CHO [5]

- Predicted priority of selecting predicted target cell [23]

- Confidence level [23]

Feedback

- Existing SON reports [35]

- QoS parameters of handed-over UE [27]

- UE level performance metrics (e.g., UL/DL throughput, packet delay, packet loss) [16] [46]

- RVQoE measurements [46]

- Actual UE trajectory [13] [20][41]

- Actual UE traffic [13] [20]

Q6: Companies are invited to provide their views on which information listed above has the standard impact with priority order. And Elaborate more on the detailed standard impact.

| Company | Which? | Comment |
| --- | --- | --- |
| Lenovo | Input: 1 with clarification, Output: 5, 10 Feedback: 12 or 13, 15, 16 | 1: measured or predicted UE traffic sent from CU-UP to CU-CP can be input for mobility optimization. In another scenario, the target gNB can receive predicted UE traffic over Xn interface from source gNB and use it for further mobility optimization. 5: the target gNB can receive predicted UE trajectory over Xn interface from source gNB and use it for further mobility optimization. 10: confidence level could be provided together with the prediction result. 15, 16: In one scenario, assuming the source gNB has made prediction on UE traffic and UE trajectory and then handover the UE to another gNB, the source gNB needs to understand if the previously made prediction is correct or not. Thus, some method is needed for the source gNB to get the actual UE traffic/trajectory after the handover. This can be considered as part of the AI/ML model performance monitoring. |
| Huawei | See comments | Input: predicted resource status information, predicted trajectory Output: the timestamp of the HO, 6 Feedback: UE level performance metrics as feedback info, 13 |
| Qualcomm |  | Input – 1, predicted resource status information, predicted trajectory Output – 1, predicted resource status information, predicted trajectory, Mobility Action and 6 Feedback – 13 and 14 RSRP prediction from UE, Actual UE trajectory, Actual UE traffic should are not needed for input and feedback. |
| Samsung | Yes for 1, 2, 5, 11, 12 Yes but more description is needed: 13, 14 | For 1, yes. UP has the knowledge of traffic, so that UP is a proper location for traffic prediction. UP can do the prediction to provide reference info to set resource allocation policy, and UP can transfer the traffic prediction results to CP to assist CP to set HO decision. For 2, yes. The predicted QoS information of UE can help source node to choose the proper target node to guarantee the UE performance during handover. For 3, it needs more clarification of expected (incident). For 4, at current stage, it is better to not involve UE inference. For 5, yes. The predicted trajectory helps target node for further mobility optimization. For high-mobility UEs, the collected data amount of a small-coverage cell (i.e. mmW cell) is small, which is not sufficient for AI/ML model to generate accurate predicted UE trajectory information. And the node capability to support AI/ML model is diverse, so there may exist some nodes with no ability for AI/ML model inference or the function for UE trajectory prediction. Thus, exchanging predicted UE information is beneficial for node to get the accurate predicted UE position information and set the proper further mobility optimization strategy to improve the handover robustness and efficiency. For 6 and 8, they can be used internally to choose the candidate cells for CHO. If transferring the handover execution timing and predicted resource reservation time window to the target node, the target node may release the reserved resource based on the handover timing or the received time window. As the model inference result can not achieve 100% accuracy, the UE finds the condition is met just after node releasing the resource, so the CHO is failure. For 7, it is already supported by current spec. For 9, it can be used internally to choose the candidate cells and set the execution policy. It seems no need to transfer such information. For 10, it needs to be discussed case by case. For 11, yes, the SON report can be enhanced to carry the feedback information. For 12, yes. QoS parameters of handed-over UE can help to judge whether mobility decision is good or not. For 13 and 14, yes but should be the handed-over UE. Same reason as 12. For 15 and 16, UE trajectory prediction and UE traffic prediction are help to set the handover decision. When the QoS/QoE performance of handed-over UE is good, it means the handover decision is good, so that the UE trajectory prediction and UE traffic prediction results are proper. Hence, to save the signaling, there is no need to transfer actual UE trajectory and actual UE traffic. |
| vivo |  | Input: 1 Predicted UE traffic over E1. 4, RSRP prediction from UE For the 1 over Xn, the predicted UE traffic shall be internal output at the source node. 4 is quite beneficial to address HO to wrong cell issues via initial simulation. Output: 7 Predicted Arrival Probability for CHO. Feedback: 15, 16. |
| Ericsson | Inputs: see comments Feedback: 11, 13, 14 | Concerning Inputs: The following are all inputs already captured in the TR which we consider valid: predicted and current resource status are inputs, UE information such as UE history information, UE measurements (not only RSRP), UE position. Concerning Outputs: Output 7 is already available in the standard. Output 6 does not make technical sense because an HO is triggered when an HO triggering event occurs. It is not possible to state that an HO shall be triggered in the future without validating that the HO triggering conditions are satisfied. Concerning feedback: it seems that feedback 12 and feedback 13 are the same. Concerning Feedback 15, this is a very sensitive piece of information which is even difficult to express. Should it be expressed in terms of cells the UE goes through? If yes, then we already have that information in the form of the UE history. The source knows the UE history already because it can add to the UE history it has the HO target cell. |
| CATT | Input:1 Output:5, 7 Feedback:12,13,14,15,16 | 1: During handover or SN addition, providing the predicted UE traffic may help the target or SN to configure the radio resources better. 5: handover target node can make future UE trajectory prediction based on the received previous UE trajectory prediction. 7: CHO target cell may decide not to accept the CHO handover request in case of low Arrival Probability if overloaded. 12,13: QoS parameter or UE performance, such as UL/DL throughput, packet delay, packet loss, shall be considered after handover. 14: RVQoE can be seen as another type of UE performance parameter, so it shall be sent back to source. 15,16: agree with Lenovo. |
| Intel | Yes for: Output: 6, 7, 8, 9, 10 Feedback: 11, 13, 16 | 6: handover execution timing is treated as validity time of the handover strategy 8: the time window for CHO could help to reduce the measurement complexity at UE side, as well as help to optimize resource reservation at the CHO candidate cell. |
| Nokia | Input: 2,3 Output: 5,7,8 Feedback: 11,12, 13 | 2: see, above. 3: Resource status procedure provides information on load at a gNB level. We think that having information on predicted load that a given UE is expected to cause (incident load) at a given node is useful to give a smaller granularity of control 4: We haven’t agreed that a UE can have AI/ML functionality. Outputs 5,7, and 8 have been agreed in the TR 37.817 and we agree to them. Regarding UE Trajectory prediction, it may be internally consumed by a node (though could be agreeable to send it to a neighbour node). In our view UE Trajectory prediction is an average over a large number of trajectories providing sequences of locations/cells and corresponding time spent in those cells and not a “per UE” information. 6: Handover execution timing for normal Handover is not very meaningful since the handover will be executed when the event is met and this cannot be predicted (typically the horizon is too short for an accurate prediction). 9: On predicted priority, we are not sure what a target cell can do by knowing this information. 11: We support sending existing SON Reports. For sending those, existing procedures can be used. 12, 13: It seems from the corresponding references that both these are related to the UEs that have been handed over to a target gNB |
| LGE | Input: 1 Output: 5, 10 Feedback: 11, 12, 13 | 1: The target node or the secondary node that receives the predicated UE traffic can allocate the proper resource. 5: The target node uses the received UE trajectory prediction for further mobility optimization. 11: The existing SON reports can be used to carry the feedback information. 12, 13: After HO, based on QoS parameters and UE performance for the handed-over UE, we can check whether the AI/ML model is working well, for example, the source node may know whether the prediction for UE traffic or UE trajectory is accurate. |
| China Telecom | Output: 5, 6,7 Feedback:12,13 | 5, 7, 12 and 13 have been agreed in TR 37.817 and this time we still support them. 6: handover execution timing seems like validity time and should be considered as output. |
| Xiaomi | Input: either 1+ 3 or 2 Output: 5 can be baseline, others seems enhancements Feedback: 12/13 or 14 | For the input, we think the predicted QoS performance may already consider the UE traffic and load of the NG-RAN node. For the output, we think UE trajectory prediction can be a baseline, for other enhancements, we think 6,8 would be useful. For the feedback, we don’t see much difference between 12 and 13, either 12 or 13 is fine, RVQoE is needed anyway. |
| InterDigital | Inputs 1 Output 5, 6, 7, 8 Feedback 11,13, 14, 15 | For outputs 6 is the estimated handover execution time, that is very useful for avoiding ping-pong and to optimize resource reservation (e.g. data forwarding). To address Samsung having the estimated handover execution time wouldn’t in a good implementation cause the release of resources but optimize data forwarding. For Feedback, not sure what the difference is between 12 and 13 since 13 is QoS feedback… In any case this UE QoS feedback should be aligned with the other use cases |
| ZTE | Output: 5,6 Feedback: 13, 15 | Regarding feedback of UE trajectory, if we need evaluate the model which performs UE trajectroy prediction, the source NG-RAN node needs to know the actual trajectory to know the model performance is good or not. |
| CMCC | Suggest mandatory for input 1; output 5,6;feedback 13. | For Input: 1: also suggest to exclude E1 discussion in this meeting; and the predicated resource status information, predicted UE trajectory should also be the input. As comments above, try to use the current IE but can also agree on defining new procedure for predication data. For output: 5,6 should be mandatory. Output 6 is internal, has no impact on current standards. For feedback: 13 should be mandatory, 12 can be merged into 13. 15 and 16 are better to have if predicted UE trajectory and traffic are input. And also, if in this scenario, the validity time should be included in the IE of these predications. |

Moderator’s summary:

The following listed are in order of priority of over-half support:

12/13. UE level performance metrics (e.g., UL/DL throughput, packet delay, packet loss) (14 companies supporting)

5. UE trajectory prediction over Xn (e.g., predicted serving cell, …) (11 companies supporting)

1. Predcited UE traffic over Xn/E1 (9 companies supporting)

6. Handover execution timing (7 companies supporting)

11.Existing SON reports (6 companies supporting)

5. Estimated Arrival Probability in CHO (5 companies supporting)

14. RVQoE measurements (5 companies supporting)

Proposal 7: Regarding AI/ML based mobility optimization, the following information should be specified as a start point:

- UE level performance metrics (e.g., UL/DL throughput, packet delay, packet loss)

- Predicted resource status information of neighbouring NG-RAN node(s)

- UE trajectory prediction over Xn (e.g., predicted serving cell, …)

Validity time

Validity time has been discussed several times during SI phase, and it is left to be discussed in the normative phase. [2] thinks that validity time is a local node model output with no standard impacts unless an output is sent to another node in assistance, or an action is triggered to be initiate in another gNB or the UE. [8] think that validity time is included for each predictions as optional IE. [10][41] propose to include the information related to validity time (e.g., requested time and limited time, start time and duration) in the request message. [20] thinks validity time should be needed for the external output. [47] thinks validity time of the predicted data (model inference output) should also be included in the messages of requesting/responding/updating predicted data between the current and neighbouring NG-RAN nodes. In [45] it is argued that when an NG-RAN node requests predictions to another NG-RAN node, it is understood that the request includes the time horizon or validity time of such prediction in the form of the Reporting Period, hence the validy time is expressed via the reporting period.

Q7: Companies are invited to provide their views on whether the validity time is needed for the predicted information or decision? If agree, should it be explicitly indicated in request/response/update message?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Lenovo | Yes | It would be very strange, if the prediction receiving node doesn’t know the prediction is about what time point in the future. It seems most reasonable for the requesting node to indicate some time information when requesting the prediction, e.g., the peer node should provide prediction result about what time point or time window. |
| Huawei | See comment | We are open to discuss the necessity of validity time, technically validity time could take effect when inference output is sent to neighbor node, however one could argue that new inference output could be generated before validity (i.e. validity time is not needed); while on the other hand, if further inference output could not be provided in time, some implementations have to be considered at target side. |
| Qualcomm |  | As mentioned in our paper, Validity time is needed. Any predicted information should have a validity associated to it. Response and Feedback messages where predicted data is sent, a validity time should also be sent along. |
| Samsung | Yes, maybe can be discussed case by case | For prediction information, it should show for which future time interval that the information is valid. If applying the predicted information in a wrong time period, the network performance may downgrades. Maybe it can be discussed case by case. Suggest to identify which inputs/outputs/feedback have the spec impact and then to study whether to need validity time for each of them. |
| vivo |  | The model output validity time is needed for the external output that does not associate with a timestamp. That is, if one output is for internal use or is already associated with a timestamp, the validity time is not needed. It should be explicitly indicated in the response/update message. |
| Ericsson | See comments | Validity time needs to be discussed use case by use case. We believe that the validity time can be deduced from the Reporting Period. Namely, if a node reports predictions with a given periodicity, the prediction should be valid for that period, after which the prediction is updated. |
| CATT | Somehow | There are two types of prediction. One is e.g. the next cell which the UE will move into. This type of prediction is used either at the node which makes the prediction itself (typical scenario) or instantly at the target cell. There is no need for any time delivered over RAN3 interfaces. (A validity time may anyhow be generated, but without RAN3 impact who cares?) The other is time series, e.g. a list that 10s later the UE will be at Place A and 20s later the UE will be at Place B. The information “10s later” “20s later” should be delivered of course, but we don’t know whether it should be called “validity time”. |
| Intel | Yes | As discussed in our paper R3-224770, the validity time can be defined as the time when model inference output becomes valid in the system. The validity time is also important for the requesting NG-RAN node to use such information to predict for the corresponding timing of its own prediction. |
| Nokia |  | We think that validity time is model-dependent information. For different input information provided to Model Inference, its validity time will be different. For instance, a predicted UE trajectory over a cell granularity (input is a cell id) will have a much larger validity time than a predicted UE Trajectory over detailed location information (input is GNSS coordinates). But once the input/output of the model is fixed, the validity time of a prediction remains fixed. If a gNB subscribes to receive information about a ML Model supported by a neighbour, then for a given input and requested output the validity time will be fixed. This can be done without revealing any proprietary information about the model. Whether to include or not the validity time depends on the reporting procedures. Therefore we think that this topic can be solved/ revisited later, when we discuss reporting in detail. |
| LGE |  | Depending on that which information is predicted by the AI/ML model inference, the validity time may or may not be necessary. So, according to the predicted information, it can be discussed case by case. |
| China Telecom | Yes | We think validity time is necessary and should be investigated case by case. As for the predicted information, the current NG-RAN node and neighbouring NG-RAN node should be aware of its corresponding validity time to make better use of related information, otherwise, due to delay caused by data request and response processes, nodes that receive prediction information won’t know how to make use of that information. The decision information might be considered as internal output sometimes and not needful to be outputted with validity time together. Therefore, further discussions for validity time is needed. |
| Xiaomi |  | We’re fine to further discuss validity time case by case. |
| InterDigital |  | Our paper 2 defines a proposal which I will modify based on the discussion so far: Validity time is a local node model output with no standards impact unless an output is sent to another node in assistance, or an action is triggered to be initiated in another gNB or the UE. The validity time may be able to be inferred from reporting periods or timestamps. The part in italic here isn’t absolutely needed since it is not applicable to any of the current use cases but might be applicable in future use cases. |
| ZTE | Yes for request message. | The output information for each use case could be divided into two types, e.g., decision type and prediction type. In our understanding, decision type output information will be taken at the moment when it needed, so decision type does not need the validity time. For the prediction infomration, we agree to introduce the start time or duration as validity time in the request message that request the related AI/ML information. In this way, the NG-RAN node will consider the configuration of start time or duration, and report the prediction information in the validity time. |
| CMCC | Yes, prefer to have | It is reasonable to include the validity time (start, end time) in each prediction IE, in response/update message, not very useful in request time. For the Energy Saving strategy, UE trajectory, validity time is an important information. |

Moderator’s summary:

8 companies state that validity time is needed for the predicted information while 5 compaines state that validity time should be discussed case by case. And some companies think whether validity time is neede or not can be discussed later. And one company propose that validity time is a local node model output with no standards impact unless an output is sent to another node in assistance, or an action is triggered to be initiated in another gNB or the UE. The validity time may be able to be inferred from reporting periods or timestamps.

Above all, moderator tries to conclude that validity time is a local node model output with no standards impact unless an output is sent to another node in assistance. And whether validity time is needed is based on the specific information output to other node.

Proposal 8: Validity time is a local node model output with no standards impact unless an output is identified to be sent to another node in assistance.

MDT enhancement

There is one Note in the objectives that specify MDT enhancement if needed.

[41] proposes to enhance the MDT procedure to support the consecutive AI/ML data collection for the certain time-series AI/ML model. When UE location information needs to be leveraged as input, model training or model inference in NG-RAN node shall collect the historical UE information including UE location information in the past period of time. And [47][54] thinks the existing MDT and RRM measurement procedures are re-used for data collection for AI/ML in NG-RAN without further enhancement.

[13] asks RAN3 to discuss mechanisms for the old NG-RAN node, that has made UE trajectory prediction before transferring UE to RRC Inactive/Idle state, to obtain logged UE trajectory information when UE enters RRC Connected state and reports to the new NG-RAN node. Such that the old NG-RAN node can understand how accurate the prediction was, and retrain the AI/ML model if necessary.

[49] finds that EM can directly send the m-based MDT activation to gNB-DU or gNB-CU-UP without gNB-CU-CP involvement, and the gNB-DU or gNB-CU-UP can send the MDT report to TCE without gNB-CU-CP involvement. And ask RAN3 to discuss how the gNB-CU-CP obtains the MDT measurement that is directly activated to gNB-DU or gNB-CU-UP by EM.

[56] argues to enhance the current MDT based activation to enable a more granular selection of UEs based on enhanced MDT Configuration information. It also proposes to discuss how to map AI/ML Feedback information to AI/ML Actions and report them over MDT

Moderator concludes the potential MDT enhancements proposed by companies below:

- Enhance the MDT procedure to solve the issue how to support the consecutive AI/ML data collection for the certain time-series AI/ML model.

- How the gNB-CU-CP obtains the MDT measurement that is directly activated to gNB-DU or gNB-CU-UP by EM

- How the source NG-RAN node obtains logged UE trajectory information when UE enters RRC Connected state and reports to the new NG-RAN node.

- How to enable a more granular selection of UEs based on enhanced MDT Configuration information in management based MDT

- How to map AI/ML Feedback information to AI/ML Actions and report them over MDT

Q8: Companies are invited to provide their views on whether the MDT enhancement proposed above is needed?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Lenovo | Yes | The raised MDT related issues seem reasonable and would be good to resolve from data collection and model performance monitoring point of view. The exact solution can be further discussed. |
| Huawei | Maybe not | From the discussions happened so far, no new measurement quantity is needed, which means that there is no need to enhance MDT. In addition, using MDT is for the purpose of training, the more important point for training is the amount of data collected, not the time spent for data collection. |
| Qualcomm | NO | For 1st bullet – As mentioned in our paper R3-224308, no new UE measurements are identified for AI/ML. Also MDT measurements from the UE can be reused for AI/ML purposes. We do not understand what is meant by consecutive AI/ML data collection. 2nd bullet – CU-CP can request for MDT information from DU or CU-UP as and when needed for AI/ML training. MDT information need not be sent to CU, whenever it is enabled in DU or CU-UP. 3rd bullet – UE Trajectory prediction can be done at NG-RAN based on UE history information and the HO related information. UE need not send the logged or predicted trajectory to the network. This simply complicates the procedure and needs RAN2 and SA3 interactions. |
| Samsung | Yes for 3 | For 1, yes, but it seems belong to RAN2 scope. For 2, the current MDT procedure to collect location info is CUCP to send configuration to UE via RRC signaling. So there is no need of additional activation. For 3, yes, the way for source node to get logged location info needs to be discussed. |
| vivo | Yes for 1 | The MDT enhancement shall focus on the new measurements or new formats. The consecutive AI/ML data is a new format of existing measurements. |
| Ericsson | Yes for 4 and 5 | 1. seems to be already possible by setting MDT traces for a given AI/ML process. 2. is based on a wrong assumption, which is that MDT is used to gather data at the RAN. This is not the case as MDT is used to gather data at the OAM, hence the issue is not valild. 3. is not justified by any use case. |
| CATT | 1. Slightly yes 2. Prefer no 3. Neutral | 1. In our understanding this bullet aims to collect more information than legacy MDT, e.g. periodical UE geographical coordination. Although its use may be limited to only Rel-18+ UEs, we may have to make such enhancement someday so why not earlier? 2. Well, we are RAN3, don’t bother the EM unless necessary…And another drawback is that the granularities in TS 28.552 are often too fine for RAN AI/ML use cases, increasing the signalling load. |
| Intel | Not at this stage | So far, we don’t see a strong motivation to enhance MDT procedure. Existing MDT procedure can be reused for AI/ML data collection. But we are open to study UE selection to reduce overhead of data collection. |
| Nokia | 1,2,3: Solutions are not necessary to enhance MDT | 1. UE can provide the RAN with UE location information according to agreements. UE location information can be provided by RRM measurements, that do not need to be retrieved by MDT. Obtaining detailed location information in OAM 2. The agreed UE measurements in TR 37.817 are location information and radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR. These are RRC measurements configured by CU-CP. Configuring MDT to DU or to CU-UP is not needed here. 3.At this stage at least we can acknowledge the underlying problem, i.e. that the source node would need the further trajectory information but can't currently get it. 5.The issue on how to map AI/ML feedback to actions is more general than just limit it to MDT, which assumes AI/ML Training in OAM. Mapping AI/ML feedback to actions is a valid problem also when AI/ML Training is in the RAN, so maybe a broader scope for this is needed. 4. We are not yet convinced about this problem. If we solve the problem of mapping AI/ML actions to feedback, doesn’t this also help to identify UEs in a more granular way? |
| China Telecom | Maybe not | Agree with Huawei. At this stage we should focus on the signaling design and the details of input/output/feedback information for each use cases. We don’t see any need to enhance the existing MDT. We perfer this WI has no RAN2 impact in Rel-18 |
| Xiaomi | Yes for 2 | Some clarification for 2. In our understanding, MDT measurements in DU or CU-UP can provide the throughput, packet delay, packet loss and UL measurements of the UEs, which can be used as Input data and feedback data as mentioned in the use cases in 3.2, 3.3 and 3.5, for these data, the more the better, it would be beneficial for AI training and inference. |
| InterDigital | Yes for 1 and 4, possibly 5 | 1.the issue of consecutive data does not relate to MDT only. It is intrinsic to ML that input data should follow a certain granularity (time gap between samples) and the data points should have continuity (consistency of the ) for proper training. The problem has therefore a broader scope. For connected UEs, the current measurement reporting framework is insufficient because it will either not collect data properly or it will increase signaling significantly. This is detailed in our paper [3] However, the discussion on UE measurements in general has not been broached in this discussion we are fine with discussing further at a later time. |
| ZTE | 1, 3 | For model training and model inference, technically speaking, input data should be consecutive and a time series of information per certain granularity. For the AI/ML based mobility optimization, even for other AI/ML based use cases, input information would be retrieved from UE side. However, current MDT mechanism could not support input information collection for AI/ML model training and inference. As we said, the time-series input data information should not be interrupted, when the model inference is triggered. If the input information is not obtained at one momonet, the model inference will be failed. Hence, the MDT enhancement should be discussed in WI phase. |
| CMCC | No, only prefer 4 and 5 | Don’t think 1-3 are necessary. For 4 and 5, also suggest to discuss later. |

Moderator’s summary:

6 companies think the MDT should be enhanced to support the consecutive AI/ML data collection, and it may be RAN2’s scope. One company think MDT measurements in DU or CU-UP can provide the throughput, packet delay, packet loss and UL measurements of the Ues, which can be provided to CU-CP. And 4 companies consider how source node gets logged location info needs to be discussed.

3 companies perfer to consider how to enable a more granular selection of UEs based on enhanced MDT Configuration information in management based MDT, and 4 companies prefer to discuss how to map AI/ML Feedback information to AI/ML Actions and report them over MDT

5 companies state that the MDT enhancement related issues could be discussed later or not at this stage.

Proposal 9: Potential MDT enhancement related issues as follows, need more time to discuss the details and potential standard impacts, coordination with RAN2/SA5 if needed:

- Enhance the MDT procedure to solve the issue how to support the consecutive AI/ML data collection for the certain time-series AI/ML model.

- How the source NG-RAN node obtains logged UE trajectory information when UE enters RRC Connected state and reports to the new NG-RAN node.

- How to enable a more granular selection of UEs based on enhanced MDT Configuration information in management based MDT

- How to map AI/ML Feedback information to AI/ML Actions and report them over MDT

Others

Q9: Any other essential issues are needed to be discussed in this meeting?

| Company | Comment |
| --- | --- |

Conclusion, Recommendations [if needed]

References

- R3-224231 AI/ML parameter Open Issue List Discussion (InterDigital )	discussion

- R3-224232 Validity Time Discussion (InterDigital )	discussion

- R3-224254 AI/ML Radio Measurement Discussion (InterDigital )	discussion

- R3-224255 AI/ML UE location Discussion (InterDigital)	discussion

- R3-224256 Discussion on Mobility Optimization Model Outputs (InterDigital )	discussion

- R3-224257 Discussion on Load Balancing Model Outputs  (InterDigital Finland Oy)	discussion

- R3-224258 QoS Feedback (InterDigital)	discussion

- R3-224306 XN enhancements for NG-RAN AI/ML (Qualcomm India Pvt Ltd)	discussion

- R3-224359 Discussion on data collection enhancements and signaling support (NTT DOCOMO, INC.)	discussion

- R3-224422 Discussion on resource status prediction (Lenovo)	discussion

- R3-224423 Discussion on UE traffic prediction (Lenovo)	discussion

- R3-224424 Discussion on AI assisted network energy saving (Lenovo)	discussion

- R3-224425 Discussion on prediction and feedback transfer in mobility scenario (Lenovo)	discussion

- R3-224426 (TP for TS37.483 TS37.480) Support UE traffic prediction over E1 interface (Lenovo)	other

- R3-224491 BL CR to TS 38.423: Support for AI/ML in NG-RAN (Ericsson)	CR0869r, TS 38.423 v17.1.0, Rel-18, Cat. B

- R3-224655 Discussion on AI/ML deployment and Stage-3 impacts (CATT)	discussion

- R3-224656 TP on TS 37.483 for AI/ML (CATT)	other

- R3-224657 TP on TS 38.423 for AI/ML (CATT)	other

- R3-224658 TP on TS 38.473 for AI/ML (CATT)	other

- R3-224675 Discussion on remaining issues of Mobility Optimization (VIVO TECH GmbH)	discussion

- R3-224716 Discussion on stage 3 related impacts of mobility optimization (LG Electronics)	discussion

- R3-224774 Discussion on Common Aspects in Stage 3 of AI/ML based Use cases (Intel Corporation)	discussion

- R3-224775 Discussion on Stage 3 of AI/ML based network energy saving and mobility optimization (Intel Corporation)	discussion

- R3-224776 (TP for NR_AIML_NGRAN BL CR for TS 38.423) (Intel Corporation)	other

- R3-224850 Discussion on AI/ML based Network Energy Saving (Stage 3) (Samsung)	discussion

- R3-224851 Discussion on AI/ML based Load Balancing (Stage 3) (Samsung)	discussion

- R3-224852 Discussion on AI/ML based Mobility Optimization (Stage 3) (Samsung)	discussion

- R3-224853 Correction of AI/ML for NG-RAN (Samsung)	CR0889r, TS 38.423 v17.1.0, Rel-18, Cat. F

- R3-224854 Correction of AI/ML for NG-RAN (Samsung)	CR1023r, TS 38.473 v17.1.0, Rel-18, Cat. F

- R3-224855 Correction of AI/ML for NG-RAN (Samsung)	CR0036r, TS 37.483 v17.1.0, Rel-18, Cat. F

- R3-224881 Discussion on AI/ML energy saving strategy (China Unicom)	discussion

- R3-224882 Discussion on AI/ML resource status (China Unicom)	discussion

- R3-224892 Introduction of RAN AI/ML (Huawei)	CR0894r, TS 38.423 v17.1.0, Rel-18, Cat. B

- R3-224893 Introduction of RAN AI/ML (Huawei)	CR1026r, TS 38.473 v17.1.0, Rel-18, Cat. B

- R3-224894 Discussion on the support of mobility enhancements using AI&ML (Huawei)	discussion

- R3-224895 Discussion on the support of load balancing using AI&ML (Huawei)	discussion

- R3-224896 Discussion on the support of energy saving using AI&ML (Huawei)	discussion

- R3-224909 Stage 3 issues on Rel-18 AI ML for NG-RAN (CMCC)	discussion

- R3-224960 Discussion on the standard impacts of AI-RAN (ZTE)	discussion

- R3-224961 Discussion on standards impacts of load prediction across multiple AI/ML based use cases (ZTE)	discussion

- R3-224962 Discussion on standards impacts of trajectory prediction across multiple AI/ML based use cases (ZTE)	discussion

- R3-224963 CR to TS38.423 for the unified AI/ML procedure (ZTE)	CR0896r, TS 38.423 v17.1.0, Rel-18, Cat. B

- R3-224964 CR to TS38.423 to enhance the existing procedure for AIML function (ZTE)	CR0897r, TS 38.423 v17.1.0, Rel-18, Cat. B

- R3-224488 AI/ML Network Energy Saving (Ericsson)	Discussion

- R3-224489 AI/ML Load Balancing (Ericsson)	Discussion

- R3-224490 AI/ML Mobility Optimization (Ericsson)	Discussion

- R3-224770 Discuss

- ion on Data Collection and Signaling Support of AI/ML for NG-RAN (Intel Corporation)	Discussion

- R3-224874 Discussion of potential impacts on signalling procedures  (China Unicom)	Discussion

- R3-224978 Data collection via MDT in split architecture (Beijing Xiaomi Mobile Software)	Discussion

- R3-224957 Signalling Procedure to support AI/ML in RAN (China Telecom)	Discussion

- R3-224560 AI/ML Network Energy Saving (Nokia, Nokia Shanghai Bell)	Discussion

- R3-224561 AI/ML Load Balancing (Nokia, Nokia Shanghai Bell)	Discussion

- R3-224562 AI/ML Mobility Optimization (Nokia, Nokia Shanghai Bell)	Discussion

- R3-224308 UE impacts in NG-RAN AI/ML (Qualcomm India Pvt Ltd) Discussion

- R3-224493, MDT enhancements to support AI/ML based processes (Ericsson)