3GPP TSG RAN WG3 Meeting #117bis-e	R3-225338

E-meeting, 10th – 18th Oct 2022

Title: 	Broadcast reception in a certain area

Source: 	TD Tech, Chengdu TD Tech

Agenda item:	15.2

Document Type:	Discussion

1. Introduction

NR MBS was introduced in 3GPP R17 by using multicast mode and broadcast mode to provide multicast sessions and broadcast sessions respectively. According to multicast mode, for a multicast session, all the UEs receiving the multicast session shall stay in RRC_CONNECTED state to receive the multicast session with PTM mode and/or PTP mode. In order to reduce the power consumption in a UE for receiving a multicast session and support the sharing processing for both unicast reception and broadcast reception, the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services” [1] was approved. In the contribution, broadcast reception in a certain area is discussed with the related proposals suggested.

2. Broadcast reception in a certain area

During email discussion after the last RAN2 e-meeting, the application of the PTM configuration information to a certain area is discussed. If there’s scenario that the PTM configuration information of a multicast session is applied to a certain area, it’s better to configure the certain area to RRC_IANCTIVE UEs. With the knowledge of the certain area, RRC_INACTIVE UE can move within the certain area without RRC state switching and service interruption.

For broadcast reception, the same scenario exists. If the PTM configuration information for broadcast reception is applied to a certain area and the certain area is indicated to RRC_INACTIVE/RRC_IDLE UE on MCCH, RRC_INACTIVE/RRC_IDLE UE can move within the certain area without RRC state switching and service interruption just as RRC_INACTIVE UE does for multicast reception.

Based on the discussion above, the following proposals are suggested for broadcast reception in a certain area.

Proposal 1: If the PTM configuration information of a broadcast session is applied to a certain area, indicate the certain area on MCCH.

Proposal 2: RAN3 supports the certain area for broadcast reception over F1/E1/Xn.

3. Conclusions

Based on the discussion in the above section, the following proposals are suggested.

Proposal 1: If the PTM configuration information of a broadcast session is applied to a certain area, indicate the certain area on MCCH.

Proposal 2: RAN3 supports the certain area for broadcast reception over F1/E1/Xn.

4. References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e