3GPP TSG-RAN WG3 #117bis-e	 				      R3-225374

E-meeting, 10-18 October 2022

Agenda Item:  19.2

Source:       NEC

Title:		UAV Support over NG-RAN interface

Document for:	decision

Introduction

This contribution discussion some aspects that related with RAN3 interfaces, for the Rel-18 NR support for UAV (Uncrewed Aerial Vehicles)

Discussion

It is shown in the WID [1] that:

.... 2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. ....

First checked the LTE UAV from 36.300:

- 23.17.2 Subscription based identification of Aerial UE function Support of Aerial UE function is stored in the user's subscription information in HSS. HSS transfers this information to the MME during Attach, Service Request and Tracking Area Update procedures. The subscription information can be provided from the MME to the eNB via the S1-AP Initial Context Setup Request during Attach, Tracking Area Update and Service Request procedures. The subscription information can also be updated via the S1-AP UE Context Modification Request message. In addition, for X2-based handover, the source eNodeB can include the subscription information in the X2-AP Handover Request message to the target eNodeB. For the intra and inter MME S1 based handover, the MME provides the subscription information to the target eNB after the handover procedure.

Then following this stage 2 description, RAN3 introduced the Areal UE subscription Information IE with ENUMERATED (allowed, not allowed,…) in the following RAN3 stage 3 specifications and signalling messages:

- S1AP: INITIAL CONTEXT SETUP, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE messages

- X2AP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE messages.

For the Rel-18 NR support for UAV, this will be straightforward if just to follow the LTE since the similar information can be introduced.

Proposal  : Follow the similar information in LTE, for Rel-18 NR UAV, provide Areal UE subscription Information with ENUMERATED (allowed, not allowed,…) from AMF to gNB during Initial Context Setup, UE Context Modification and inter-gNB handover to the target gNB. Also source gNB to target gNB during the Xn-HO and Retrieve UE Context Response.

Proposal

Proposal  : Follow the similar information in LTE, for Rel-18 NR UAV, provide Areal UE subscription Information with ENUMERATED (allowed, not allowed,…) from AMF to gNB during Initial Context Setup, UE Context Modification and inter-gNB handover to the target gNB. Also source gNB to target gNB during the Xn-HO and Retrieve UE Context Response.

References

[1] RP-213600: New WID on NR support for UAV (Uncrewed Aerial Vehicles)