3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225472

Online, 10 – 18 October 2022

Agenda item:	 10.2.1

Source:		Lenovo

Title:		SON enhancements for successful PSCell change report

Document for:		Discussion and Decision

1	Introduction

In RAN3#117-e meeting [1], it was agreed to to consider successful PSCell change report for NR-DC including:

•	SN- and MN-initiated classic PSCell change / CPC

•	intra-SN classic PSCell change / CPC

•	classic Addition / CPA

•	HO with SN change are not prohibited, but possibly addressed once the basic solution for SPCR is known.

In this paper, details of SON enhancements for successful PSCell addition/change report are further discussed.

2	Discussion

In R17, Successful Handover Report (i.e. SHR) is introduced to provide a more robust mobility via reporting failure events observed during successful handovers. The UE could be configured with triggering conditions to compile the SHR, the report would be triggered only when at least one condition is met. This limits UE reporting relevant cases, such as underlying issues detected by RLM detected upon a successful handover event.

Legacy PSCell addition or PSCell change is supported in R15. In R17, to further enhance robustness of PSCell change, CPAC in NR-NR DC or EN-DC scenario is completely frozen. If the UE performs a successful PSCell addition/change to a target PSCell, there is a possibility that this successful PSCell addition/change is on the verge of failure, e.g. the target PSCell is not stable/good enough, and PSCell addition/change is close to failure. A near-failure successful PSCell addition or PSCell change may happen in legacy PSCell addition or PSCell change, as well as CPA or CPC.. Similar as SHR in R17, for mobility in dual connectivity scenario, reporting failure events observed during successful successful PSCell addition/change is beneficial for robustness.

Similar as T310/T312/T304 related trigger condition for SHR, for reporting successful PSCell addition/change, configuration for generating successful PSCell addition/change report e.g. T310/T312/T304 trigger threshold associated with PSCell should be configured to the UE. For example, in case of MN initiated PSCell change or CPC, T310/T312 trigger threshold may be generated by MN, and T304 trigger threshold may be generated by taregt SN; in case of SN initiated PSCell change or CPC, T310/T312 trigger threshold may be generated by source SN, and T304 trigger threshold may be generated by target SN; in case of PSCell addition or CPA, T304 trigger threshold may be generated by taregt SN. It is MN provides the configuration for generating successful PSCell addition/change report to the UE.

Proposal 1: Configuration for generating successful PSCell addition/change report is configured to the UE by the network.

When at least one trigger condition for successful PSCell addition/change report is fulfilled, the successful PSCell addition/change report is stored by the UE including some necessary information for analysis of the near-failure PSCell addition/change, e.g. source PSCell ID, target PSCell ID, cause of successful PSCell addition/change report, or measurement results when PSCell addition/change is executed and completed.

Specially, in case of a successful PSCell addition/change in a CPA/CPC procedure, since it is the UE to decide when to perform CPAC execution after receiving CPAC configuration i.e. the UE performs PSCell addition/change towards a target PSCell when its corresponding CPAC execution condition is fulfilled, it is necessary for the UE to store and report the time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell, which is beneficail for near-failure analysis, for example, if the elapsed time duration is long, network may understand failure may be caused due to strict CPAC execution condition.

After the PSCell addition/change is successfully completed, an indication for availability of successful PSCell addition/change report may be transmitted to MN, MN may request the successful PSCell addition/change report from the UE, and then the UE may transmit the stored successful PSCell addition/change report to MN.

Proposal 2: MN may request and receive the successful PSCell addition/change report from the UE after PSCell addition/change is successfully completed.

For analysis of the successful PSCell addition/change report, the node which generates the trigger condition that acutually triggers reporting the successful PSCell addition/change report should be the proper node to perform final root cause analysis. For example,

- in case of MN initiated PSCell change or CPC, if cause of successful PSCell change report is T310 or T312 trigger threshold is fulfilled, it is MN that analyzes the received report from the UE, if successful PSCell change report is triggered due to T304 trigger threshold is fulfilled, MN may forward the successful PSCell change report to target SN, then target SN performs analysis.

- in case of SN initiated PSCell change or CPC, if cause of successful PSCell change report is T310 or T312 trigger threshold is fulfilled, MN may forward the successful PSCell change report to source SN, then source SN performs final root cause analysis, if it is T304 related condition triggers the report, MN may forward the successful PSCell change report to target SN, then target SN performs final root cause analysis.

- in case of PSCell addition or CPA, when successful PSCell addition report is triggered due to T304 trigger threshold is fulfilled, MN may forward the successful PSCell addition report to target SN, then target SN performs corresponding analysis.

Proposal 3: If successful PSCell addition/change report is triggered due to T304 trigger threshold is fulfilled, MN may forward the successful PSCell addition/change report to target SN for root cause analysis.

Proposal 4: In case of MN initiated PSCell change or CPC, if successful PSCell change report is triggered due to T310 or T312 trigger threshold is fulfilled, MN performs root cause analysis.

Proposal 5: In case of SN initiated PSCell change or CPC, if successful PSCell change report is triggered due to T310 or T312 trigger threshold is fulfilled, MN may forward the successful PSCell change report to source SN for root cause analysis.

After the source SN receives the successful PSCell change report, there may be a possibility that UE context is released in the source SN, therefore the source SN can not identify the association between the UE and the successful PSCell change report. How to solve the issue needs to be discussed, e.g. SN Mobility Information may also be transferred to the source SN together with the successful PSCell change report via Xn interface.

Proposal 6: RAN3 discuss how to identify the successful PSCell change report when UE context is released in source SN.

3	Conclusion

In this contribution, the details of SON enhancements for successful PSCell addition/change report are discussed. We have the following proposals:

Proposal 1: Configuration for generating successful PSCell addition/change report is configured to the UE by the network.

Proposal 2: MN may request and receive the successful PSCell addition/change report from the UE after PSCell addition/change is successfully completed.

Proposal 3: If successful PSCell addition/change report is triggered due to T304 trigger threshold is fulfilled, MN may forward the successful PSCell addition/change report to target SN for root cause analysis.

Proposal 4: In case of MN initiated PSCell change or CPC, if successful PSCell change report is triggered due to T310 or T312 trigger threshold is fulfilled, MN performs root cause analysis.

Proposal 5: In case of SN initiated PSCell change or CPC, if successful PSCell change report is triggered due to T310 or T312 trigger threshold is fulfilled, MN may forward the successful PSCell change report to source SN for root cause analysis.

Proposal 6: RAN3 discuss how to identify the successful PSCell change report when UE context is released in source SN.

Reference

[1] Chairman notes for RAN3#117-e meeting