3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225497

E-meeting, 10th – 18th October 2022

Agenda item:	 15.3

Source:		Lenovo

Title:		PTM configuration for multicast reception in RRC_INACTIVE

Document for:		Discussion and Decision

- Introduction

This contribution discusses PTM configuration provision and update for the following objective [1]:

- PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2].

- Discussion

- PTM configuration Provision

- In RRC_CONNECTED state, the PTM configuration of an multicast MRB is provided by RRC dedicated signalling, i.e., the RRCReconfiguration message. Usually, when UE enters RRC_INACTIVE state, the RRC dedicated configuration is suspended.  One possible way is that the UE can continue using the dedicated PTM configuration for multicast reception in RRC_INACTIVE state.

If the RRCRelease message includes multicast reception in RRC_INACTIVE indication, the UE shall perform multicast reception in RRC_INACTIVE operation. The behaviours of UE may include:

-	the UE suspends or releases the PTP leg if any.

-	the UE disables or suspends the dedicated PTM configuration for RRC_CONNECTED state, e.g. the UE disables HARQ feedback, ROHC related functions.

-	the UE continues the multicast reception of PTM leg in RRC_INACTIVE with continuing using the PTM configuration.

-	the UE suspends the PDCP entities of all DRBs expect the MRB. i.e. the UE does not suspend the PDCP entity of the MRB:

-	the UE does not stop and reset the t-reordering if running;

-	the UE continues to use the existing value of RX_NEXT and RX_DELIV, i.e. the UE does not set RX_NEXT and RX_DELIV to the initial value.

-	the UE performs partial MAC reset, e.g. the UE does not flush the soft buffers for DL HARQ processes related to the PTM transmission, i.e. the UE only flushes the soft buffers for all DL HARQ process related to unicast SRBs and DRBs.

-	during state transition from RRC_CONNECTED to RRC_INACTIVE, if the RRCRelease message includes multicast reception in RRC_INACTIVE indication, the UE shall attempt to select the cell which provides the multicast service (either indicated by SIB, or by MCCH or by the RRC dedicated signalling), e.g. the UE may attempt to select the serving cell.

The UE continues using the dedicated PTM configuration for multicast reception in RRC_INACTIVE state.

- In order to support UE mobility in multiple cells without entering RRC_CONECTED state, PTM configuration of a list of cells can be provided to the UE by RRC dedicated signaling. When the UE moves among the list of cells, the UE does not need enter to RRC_CONNECTED state and use the PTM configuration of the corresponding cell for multicast reception in RRC_INACTIVE.

-	Before sending the UE to RRC_INACTIVE, the gNB decides the list of cells for multicast reception in RRC_INACTIVE and coordinates with negibour cell about the PTM configuration.

-	And the gNB may send multicast MRB configuration for RRC_INACTIVE request to the neighbour gNB that the cell(s) belongs to, and the gNB may also provide a list of cell IDs to the neighbour gNB, for request the multicast MRB configuration for RRC_INACTIVE of the cells belonging to the neighbour gNB. The neighbour gNB provides the multicast MRB configuration for RRC_INACTIVE of the cells to the gNB. The exchange between the gNB and the neighbour gNB may be carried by Xn-AP signalling.

-	The gNB provides PTM configuration for RRC_INACTIVE of a list of cells to the UE.

-	During RRC_INACTIVE state, the UE should prioritize to reselect a cell that is provided PTM configuration for RRC_INACTIVE and/or the UE should treat the frequency that the cell belongs to as the highest priority for cell reselection. In order to prioritize to the cell and/or frequency, an cell or frequency based offset of the threshold for cell reselection can be configured and used during cell reselection evaluation.

-	when the UE reselects to a cell with PTM configuration for RRC_INACTIVE, the UE should apply the corresponding configuration of the cell for multicast reception.

-	when the UE reselects to a cell without PTM configuration for RRC_INACTIVE, the UE should initialize RRC Resume procedure to enter RRC_CONNECTED or enter to RRC_IDLE directly.

Figure 1. PTM configuration of a list of cells based solution

In order to support UE mobility in multiple cells without entering RRC_CONECTED state, PTM configuration of a list of cells can be provided to the UE by RRC dedicated signalling.

The gNB coordinates with the neighbour gNB about the PTM configuration of the neighbour cells.

- PTM configuration Provision

- In the RRC_CONNECTED, the PTM configuration of a multicast MRB is provided by RRC dedicated signalling, i.e., the RRCReconfiguration message. One possible way is that the UE can continue using the dedicated PTM configuration for multicast reception in RRC_INACTIVE. In this case, when PTM configuration update,

-	the gNB sends paging message to the UE for notifying the UE enters into RRC_CONNECTED state;

-	when receiving the paging message, the UE performs RRC state transition from RRC_INACTIVE to RRC_CONNECTED state;

-	after UE entering the RRC_CONNECTED state, the gNB sends the updated PTM configuration by the RRCReconfiguration message;

-	after the PTM configuration update, the gNB may send the UE into RRC_INACTIVE by RRCRelease message.

- This procedure of PTM configuration update by RRC dedicated signalling causes large latency, signalling overhead and large service interruption.

Figure 2. An example of fast PTM configuration update

- A fast PTM configuration update is proposed:

-	when the network determines the update of the PTM configuration, the network sends a paging message to the UE for indicating the PTM configuration update.

-	In case of gNB-DU determines to update part of parameters of the multicast PTM configuration, the gNB-DU sends an PTM Configuration Update Request message to the gNB-CU. the PTM Configuration

-	According to the information e.g. the MBS session ID in the PTM Configuration Update message, the gNB-CU determines the UEs that needs to be paged and sending corresponding paging message to the UE

-	in response to the paging message, the UE indicates PTM configuration update indication to the network in the RRCResumeRequest message. The PTM configuration update indication is used to indicate the RRCResumeRequest is only for acquiring the updated PTM configuration which implies there is no need to resume the UE into RRC_CONNECTED state.

-	in response to the PTM configuration update indication in the RRCResumeRequest message, the network can directly provide the updated PTM configuration in the RRCRelease message without entering RRC_CONECTED state.

A fast PTM configuration update without entering RRC_CONNECTED state should be supported.

3	Conclusion

This contribution discusses PTM configuration provision and update for multicast reception in RRC_INACTIVE.

- The UE continues using the dedicated PTM configuration for multicast reception in RRC_INACTIVE state.

In order to support UE mobility in multiple cells without entering RRC_CONECTED state, PTM configuration of a list of cells can be provided to the UE by RRC dedicated signalling.

The gNB coordinates with the neighbour gNB about the PTM configuration of the neighbour cells.

A fast PTM configuration update without entering RRC_CONNECTED state should be supported.

Reference

[1] RP-221458. Revised WID: Enhancements of NR Multicast and Broadcast Services. CATT, CBN.