3GPP TSG-RAN WG3 Meeting #117-bis-e	R3-225507

E-meeting, 10 October – 18 October 2022

Title: 	Analysis of RAN dependency of FS_eNS_Ph3

Source: 	Ericsson

Agenda item:	8.1

Document Type:	Discussion and Decision

# Introduction

RAN3 has received from SA2 the LS in [1]. This LS presents questions to RAN3 concerning “Key Issue #3:  Network Slice Area of Service for services not mapping to existing TAs boundaries, and Temporary network slices”.

In this contribution an analysis of the questions asked by SA2 is provided and feedback on the LS from SA2 is given.

# Discussion

KI#3 is defined in TR23.700-41 v1.0.0 as follows:

Network Slices are deployed for services over an Area of Service which may match the existing TAs or for which the Area of Service can be different. Currently, the network slice availability (i.e. where the network slices are defined to be supported) is designed to match deployed TA boundaries. In addition, the UEs and network configuration can be impacted when network slices are deployed and decommissioned over certain time interval (e.g. the Configured NSSAI can change when a network slice is no longer available or becomes available, this can affect the Allowed NSSAI and other parameters and in turn the RA may need to change, etc.).

This Key Issue will study how to address the issues described above and whether system level improvements are needed to mitigate e.g. the deployment and control plane issues that arise due to the currently defined system behaviour:

-	The support of services over network slices when the services have Area of Service not matching the existing deployed TA boundaries.

-	The support of network slices which have a limited lifetime (including how to gracefully terminate a network slice which can apply also to network slices which have a longer lifespan in order to avoid abrupt PDU Session release).

From the above we understand that the target of KI#3 is that of enabling policies that allow certain services to be available in certain areas without being subject to the constraint of being available in the whole coverage of the TAs forming the RA.

We note that there is no requirement on the availability of a service outside its area of service. Namely, if a service has to be available in e.g. Cell x, Cell y and Cell z, there is no requirement on whether such service may or may not be available outside these cells coverage. The service may well be available outside its area of service but subject to specific operator´s policies.

Conclusion 1: KI#3 focuses on enabling policies to allow a service to be available in areas that do not have to match with a TA coverage. The service may be available outside its area of service, provided that operator´s policies rule what such availability entails.

Also, we note that there is no requirement on modifying mobility mechanisms, such as network registration. We understand that such mechanisms shall be left untouched if possible.

In general, and as a common principle followed by 3GPP, the solution to be sought should be one fulfilling the objective of the key issue while minimising the impacts on current specifications.

Conclusion 2: The solution to be sought for KI#3 should minimise the impacts on the network while fulfilling the main objectives of the issue

## 2.1 Feedback on Q1

With the above in mind, we analyse the three questions asked in the LS in [1].

Q1.Whether NG-RAN can broadcast one or more Secondary TAIs (up to a number RAN2 agrees, we note that for NTN is already possible to broadcast TWO TACs) via an updated SIB or new SIB, and report them to the CN and between gNBs as per existing Tracking Area related information exchange procedures but with indication they are secondary. The additional TAIs are associated with specific S-NSSAI(s) like the existing TAs and will be treated by UEs supporting secondary TAs as a normal Tracking area from RM standpoint (as described in solution#9)

The solution in Q1 proposes additional secondary TAIs, which are  for the sole purpose of enabling the use of slices that don’t fully align with an existing TA. The secondary TAIs are to be broadcast over an existing and updated SIB or via a new SIB and, for that, they can be used by new UEs only. Namely, this solution does not work for legacy UEs. The latter means that if any slice is supported only in the secondary TAIs, such slice will not be accessible to a legacy UE, despite the user for the legacy UE might have subscribed to such slice. The latter is non-trivial given that a user may, for example, contract access to a slice deployed on a secondary TAI and move its SIM card to a legacy device not supporting secondary TAIs. Hence the user will be denied access to the contracted slice services.

We note that this solution has impacts on the whole network, namely:

- The solution impacts UEs, which need to support decoding the secondary TAIs in new SIB information and behaving accordingly

- The solution impacts the RAN, which has to broadcast the secondary TAIs via modified or new SIBs, signal them over common interfaces to CN and other NG-RAN nodes and adopt specific behaviours associated with them.

- The solution impacts the CN, which needs to consider such secondary TAIs for processes such as network registration, paging, user location information etc.

- The solution impacts the OAM, which needs to configure the secondary TAIs on a per cell basis and the mapping between them and the supported slices.

Observation 1: The solution presented in Q1 has an impact on the whole 5G system and it  has a serious impact on legacy UEs in that slices deployed in the secondary TAs are not available to the UEs even if they have a subscription for those slices.

As hinted above, it is not clear what the UE should do when entering such secondary TAIs. If such TAIs are equivalent in functionalities to legacy TAIs, then the solution modifies mobility and paging procedures by introducing, for example, new, extra registration updates at secondary TAI change. This would incur in an increase of signalling over the air and over the RAN-CN interface, as well as changing well established mobility and paging functionalities.

Observation 2: The solution in Q1 causes an increase of over the air and network signalling and changes well established mobility and paging functionalities

Whether the solution solves the issue raised by KI#3 is also questionable. If KI#3 is motivated by the fact that existing tracking areas are difficult to adapt to the area of service for a given slice, then why would the introduction of yet another type of TAI resolve the problem? Wouldn´t it be that secondary TAIs would also be difficult to modify in coverage once they are established? In this case, the area of service for new slices introduced after the secondary TAIs have been setup would probably not match the coverage of the secondary TAI. Would then the solution be to introduce yet another type of TAI? 
A clear conclusion is that this solution is not scalable or flexible to adapt to the area of service that slices deployed at different points in time would demand.

Observation 3: The solution in Q1 does not fulfil the objective of KI#3 due to lack of flexibility and scalability. This is due to the fixed coverage of secondary TAIs, which, like with normal TAIs, would not be easy to adapt to areas of service once it is configured.

One further aspect of the solution in Q1 is that some RAN behaviours would become ambiguous. For example, how should the RAN decide on mobility targets on the basis of the neighbour cells´ TAI Support List and Secondary TAI Support List? If a UE is allowed to access some slices in normal TAIs and some others in secondary TAIs, should the RAN steer mobility towards cells supporting “normal” TAIs, or should it choose cells supporting “secondary TAIs”?

Another example of how this solution distorts legacy behaviours concerns connected mode mobility, which is currently controlled by the Mobility Restriction List. Namely, a cell would support multiple TAs, legacy ones and secondary ones. What would happen if, e.g. a secondary TA is allowed and the legacy one is forbidden (or vice versa)? It is not clear if the UE is allowed to be handed over to the cell or not.

Observation 4: The solution in Q1 creates ambiguity in certain RAN behaviours such as connected mode mobility

In light of the observations above the following can be concluded

Conclusion 3: it can be concluded that the solution in Q1 is not feasible to solve KI#3 because:

- It has a high impact on the whole 5G system

- It does not work for legacy UEs and it prevents legacy UEs from using the slices available in Secondary TAIs

- It causes an increase of over the air and network interface signalling, while impacting some well established functions such as mobility and paging

- It does not fulfil the objective of KI#3 due to lack of flexibility and scalability

- It does create ambiguity in nodes behaviours, for example in case of connected mode mobility

## 2.2 Feedback on Q2

In [1] Q2 is captured as follows:

Q2. Whether the NG-RAN can be configured with a slice availability on a per-cell basis and

  - inform AMF and other gNBs in NGAP messages (as described in solution#11 and others)

  - Whether in Constrained Service Area the network slice is still supported but since no dedicated resources are allocated for the network slice the SLA of the network slice is not guaranteed.(as described in solution#45).

It is understood from Q2 that even though a slice is supported (e.g., over one or more TAs), it may be subject to different degrees of availability, namely it will be served with different policies, depending on the cell.

The solution in Q2 is based on the configuration at the RAN of a policy by which the RAN makes some network slices fully available (i.e. served according to fulfil their SLA) in some cells while leaving the availability of such slices in other cells up to an operator´s configured policy.

This approach allows fulfilment of SLAs within those cells configured with slice availability. For cells where the slice availability is not configured, the service may be served as best effort (reduced resources allocated to the slice), or it may not be served at all (no resources allocated to the slice).

Concerning question 2.a: we do not see the reason to communicate a list of cells with configured availability over NGAP and XnAP. Reasons for this are:

- The slice availability policy is configured and executed at the RAN for a subset of cells in the TA. Nevertheless, the concept of supported slices per TA remains unchanged, hence there seem to be no reason to signal a list of cells with slice availability to the AMF, given that the AMF is already aware of all supported slices per TA and this is all the AMF needs to know to run its processes

- It is already possible for RAN nodes to understand the availability of resources for a slice in a neighbour cell (via the Xn: Resource Status Reporting procedure), which is all it is needed to carry out connected mode moiblity. This is done via the Resource Status Reporting procedure over Xn. Hence, cells where slice availability is configured will show higher available resources, while cells where no slice availability has been configured will show low or no available resources.

Concerning question 2.b: the same observations as for question 2.a are valid. Additionally, we believe it is plausible that the policy on how to serve slices outside the slice availability area can be configured and it depends on operators´ choices. In one example, slices outside the slice availability area may be served as best effort. In another example, the operator may choose to configure the RAN not to allocate any resources to these slices in cells outside the slice availability area.

Moreover, we note that the solution in Q2.b can be achieved by means of existing tools. Namely, the RRM policies specified in TS28.541 allow already to configure the RAN with per slice resources, hence enabling the concept of “Constrained Service Area”

In light of the above, the following can be concluded:

Conclusion 4: It is feasible to configure the NG-RAN with an additional per-cell availability for a supported slice, however

- There is no need to signal a list of cells configured with slice availability over RAN interfaces

- It is feasible to leave up to operators´ configuration and based on existing RRM policy configuration tools what resources a slice may access outside its slice availability area

## 2.3 Feedback on Q3

In [1] Q3 is captured as follows:

Q3.	The NG-RAN receives in solution 29 (but conceivably this would be needed for similar solutions) the partially allowed S-NSSAIs in addition to the Allowed NSSAI. Can the NG-RAN in principle trigger handover procedure to a supporting TAI of the partially allowed S-NSSAIs when it is possible to do so? this can happen while in connected mode or when the UE is engaged in transition from Idle to connected mode. The reason is to enable the support of the maximum number of S-NSSAIs in the Allowed and partly allowed S-NSSAIs lists.

The solution in Q3 is based on the introduction of a partially allowed NSSAI, in addition to the Allowed NSSAI. The question asks if the NG/RAN can “in principle trigger handover procedure to a supporting TAI of the partially allowed S-NSSAIs when it is possible to do so”. Bringing this question in the context of SA2´s discussion, the use case from which the question is derived is one where a UE is handed over to a given cell on the basis of its partially allowed S-NSSAIs, without necessarily having any active user plane resources for the partially allowed S-NSSAIs.

Of course, if a UE has PDU sessions with an active user plane, which are associated to slices supported by the target cell of a TAI supporting the partially allowed S-NSSAIs, the RAN can trigger mobility towards that target cell, provided that the target cell fulfils the established radio requirements. However, this consists of legacy functionalities, and it is unclear why the introduction of partially allowed S-NSSAIs would be needed for that.

If the use case is to trigger an handover towards a target cell, only because it supports the partially allowed S-NSSAIs and without the UE having any active UP connection for those S-NSSAIs, the situation is different.

Handovers are normally performed to maximize the coverage and performance of existing active data bearers or to acquire needed service support once a UE requests a certain service. Triggering handovers for a UE to another frequency or cell, purely on the basis of partially supported S-NSSAIs at the target cell is, may have harmful effects on network performance and it should not be taken as a reference use case in 3GPP.

One of the major concerns would be whether radio conditions at the target cell/frequency will degrade the coverage performance of the active data bearers. In addition, if handovers are triggered purely on the basis of whether the target supports partially available S-NSSAIs, it is likely to happen that, during mobility to different cells, the partially allowed S-NSSAIs becomes unavailable. In this case, the UE may need to be moved back to the original frequency/coverage layer. These back-and-forth handovers which are not motivated by actual active UP connections will lead to unnecessary increases in drop-call probabilities, signaling and processing load in the network.

Moreover, this solution is not compatible with legacy UEs, hence similar issues as described in the reply to Q1 would arise, i.e. the eventual impact on the RAN needed to change the RAN behaviour and to make the solution work would have a limited benefit because a very large portion of UEs would not be able to make use of this feature.

On the basis of the above, the following can be concluded:

Conclusion 5 Legacy mobility functions already allow the RAN to carry out handovers of UEs with active UP connections associated to specific slices to those cells that support some or all of such slices. 
Handing over a UE to a target cell/frequency, only because that mobility target supports the partially allowed S-NSSAIs (i.e. without any active UP connection for the partially allowed S-NSSAIs) may be harmful and should not be considered as a reference use case in 3GPP. 
Hence, it is not clear why partially allowed S-NSSAIs would be needed. 
The adoption of partially allowed S-NSSAIs triggers a considerable impact on functions like mobility, admission control and resource management, without full benefit due to the non backwards compatibility of the solution towards legacy UEs

# 3. Conclusion

In this paper, the LS from SA2 in [1] has been analysed and the following observations and conclusions have been derived:

Conclusion 1: KI#3 focuses on enabling policies to allow a service to be available in areas that do not have to match with a TA coverage. The service may be available outside its area of service, provided that operator´s policies rule what such availability entails.

Conclusion 2: The solution to be sought for KI#3 should minimise the impacts on the network while fulfilling the main objectives of the issue

Observation 1: The solution presented in Q1 has an impact on the whole 5G system and it does not work for legacy UEs

Observation 2: The solution in Q1 causes an increase of over the air and network signalling and changes well established mobility and paging functionalities

Observation 3: The solution in Q1 does not fulfil the objective of KI#3 due to lack of flexibility and scalability. This is due to the fixed coverage of secondary TAIs, which, like with normal TAIs, would not be easy to adapt to areas of service once it is configured.

bservation 4: The solution in Q1 creates ambiguity in certain RAN behaviours such as connected mode mobility

Conclusion 3: it can be concluded that the solution in Q1 is not feasible to solve KI#3 because:

- It has a high impact on the whole 5G system

- It does not work for legacy UEs, hence it does not guarantee correct access for the slices available in Secondary TAIs

- It causes an increase of over the air and network interface signalling, while impacting some well established functions such as mobility and paging

- It does not fulfil the objective of KI#3 due to lack of flexibility and scalability

- It does create ambiguity in nodes behaviours, for example in case of connected mode mobility

Conclusion 4: It is feasible to configure the NG-RAN with an additional per-cell availability for a supported slice, however

- There is no need to signal a list of cells configured with slice availability over RAN interfaces

- It is feasible to leave up to operators´ configuration and based on existing RRM policy configuration tools what resources a slice may access outside its slice availability area

Conclusion 5 Legacy mobility functions already allow the RAN to carry out handovers of UEs with active UP connections associated to specific slices to those cells that support some or all of such slices. 
Handing over a UE to a target cell/frequency, only because that mobility target supports the partially allowed S-NSSAIs (i.e. without any active UP connection for the partially allowed S-NSSAIs) may be harmful and should not be considered as a reference use case in 3GPP. 
Hence, it is not clear why partially allowed S-NSSAIs would be needed. 
The adoption of partially allowed S-NSSAIs triggers a considerable impact on functions like mobility, admission control and resource management, without full benefit due to the non backwards compatibility of the solution towards legacy UEs

In line with the conclusions above, a reply LS has been made available in [2].

# 4. References

[1]	R3-225320, LS Out on RAN dependency of FS_eNS_Ph3, SA2

[2]	R3-225507, Reply LS Out on RAN dependency of FS_eNS_Ph3, Ericsson