3GPP TSG-RAN WG3 Meeting #117-bis-e	R3-225560

# Online, 10 - 18 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0086 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of timing error margin for UE/TRP TEGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Timing Error Group (TEG) was introduced in NRPPa/F1AP at RAN3#115 based on RAN1 agreements, and some corrections agreed at RAN3#116 (CR0063r4) to further align with RAN1/RAN2 decisions. At RAN3#117e, an LS was received from RAN4 (R3-224216) listing agreements on timing error margins for UE/TRP Tx/Rx/RxTx TEGs. Based on the RAN4 agreements, RAN3 added timing error margin associated to the UE/TRP Tx/Rx/RxTx TEGs. Because the timing error margins were introduced as a late correction to complete the TEG framework, the IEs were made optional in order to be backwards compatible but an FFS captured on how to interpret the absence of margin values. In fact, the sender (UE/gNB) must always know the timing error margins if it is supporting TEG as a feature, so there is no reason why the sender should not always include the margins. The only reason the IEs are optional is that they were introduced as a backwards compatible correction to the incomplete TEG framework, which was unfortunately introduced piecemeal in NRPPa/F1AP due to agreements being reached in RAN1 and RAN4 at different meetings. For the TEG framework to work without undue complexity, the timing error margins should be mandatory to avoid abnormal conditions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The timing error margins are made mandatory if the associated TEG ID is included, as follows: - 9.2.39: TRP Rx Timing Error Margin IE is made mandatory if TRP Rx TEG ID IE is included - 9.2.78: UE Tx Timing Error Margin IE is made mandatory - 9.2.79: TRP Tx Timing Error Margin IE is made mandatory - 9.2.80: For RxTxTEG, the TRP RxTx Timing Error Margin IE is made mandatory, and the TRP Tx Timing Error Margin IE is made conditional on inclusion of the TRP Tx TEG ID IE. - 9.2.80: For Rx TEG, both the TRP Rx Timing Error Margin IE and the TRP Tx Timing Error Margin IE are made mandatory. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol point of view. The impact can be considered isolated because the change affects only timing error margin functionality. This CR is non-backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Timing error margin functionality is unnecessarily complex due to potential abnormal scenarios if timing error margin is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.39, 9.2.40, 9.2.78, 9.2.79, 9.2.80, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 1046 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modification

### 9.2.39	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP Rx TEG Information |  | 0..1 |  |  | YES | ignore |
| >TRP Rx TEG ID | M |  | INTEGER (0..31) |  | - |  |
| >TRP Rx Timing Error Margin | M |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Rx TEG ID. | - |  |

### 9.2.40	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP TEG Information | O |  | 9.2.80 |  | YES | ignore |

Next Modification

### 9.2.78	UE Tx TEG Association List

This information element contains the list of UE Tx TEG associations.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE Tx TEG Association item |  | 1..<maxnoUETEGs> |  |  |  |  |
| >UE Tx TEG ID | M |  | INTEGER (0..7) |  |  |  |
| >Positioning SRS Resource ID List |  | 1 |  |  |  |  |
| >>Positioning SRS Resource ID Item |  | 1..<maxnoSRS-PosResources> |  |  |  |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |  |  |
| >Time Stamp | M |  | 9.2.42 |  |  |  |
| >Carrier Frequency |  | 0..1 |  | Indicates the frequency of the positioning SRS resources |  |  |
| >>Point A | M |  | INTEGER (0..3279165) | NR ARFCN |  |  |
| >>Offset to Carrier | M |  | INTEGER (0.2199, …) |  |  |  |
| >UE Tx Timing Error Margin | M |  | Timing Error Margin 9.2.84 | Timing error margin associated to the UE Tx TEG ID. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoUETEGs | Maximum no of reported UE Tx TEG associations. Value is 256. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources. Value is 64. |

### 9.2.79	TRP Tx TEG Association

This information element contains the TRP Tx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TRP TEG item |  | 1..<maxnoTRPTEGs> |  |  |  |  |
| >TRP Tx TEG ID | M |  | INTEGER (0..7) |  |  |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |  |  |
| >DL-PRS Resource ID List |  | 0..1 |  |  |  |  |
| >>DL-PRS Resource ID Item |  | 1..<maxPRS-ResourcesPerSet> |  |  |  |  |
| >>>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >TRP Tx Timing Error Margin | M |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Tx TEG ID. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPTEGs | Maximum no of reported TRP Tx TEG association. Value is 8. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

Next Modification

### 9.2.80	TRP TEG Information

This information element contains the TRP TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE TRP TEG | M |  |  |  |  |  |
| >RxTx TEG |  |  |  |  |  |  |
| >>TRP RxTx TEG ID | M |  | INTEGER (0..255) |  |  |  |
| >>TRP Tx TEG ID | O |  | INTEGER (0..7) |  |  |  |
| >>TRP RxTx Timing Error Margin | M |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP RxTx TEG ID. |  |  |
| >>TRP Tx Timing Error Margin | C-ifTRP-TxTEG-ID |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Tx TEG ID. |  |  |
| >Rx TEG |  |  |  |  |  |  |
| >>TRP Rx TEG ID | M |  | INTEGER (0..31) |  |  |  |
| >>TRP Tx TEG ID | M |  | INTEGER (0..7) |  |  |  |
| >>TRP Rx Timing Error Margin | M |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Rx TEG ID. |  |  |
| >>TRP Tx Timing Error Margin | M |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Tx TEG ID. |  |  |

| Condition | Explanation |
| --- | --- |
| ifTRP-TxTEG-ID | This IE shall be present if the TRP Tx TEG ID IE is present. |

Next Modification

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

** Unchanged text skipped **

id-TRPTEGInformation,

id-TRPRxTEGInformation,

id-TRPBeamAntennaInformation,

id-NR-TADV,

id-pathPower,

id-SRSPortIndex,

** Unchanged text skipped **

GNB-RxTxTimeDiff ::= SEQUENCE {

rxTxTimeDiff		GNBRxTxTimeDiffMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { GNB-RxTxTimeDiff-ExtIEs} }	OPTIONAL,

...

}

GNB-RxTxTimeDiff-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore EXTENSION ExtendedAdditionalPathList PRESENCE optional}|

{ ID id-TRPTEGInformation		CRITICALITY ignore EXTENSION TRPTEGInformation	PRESENCE optional },

...

}

** Unchanged text skipped **

TRPTEGInformation ::= CHOICE {

rxTx-TEG			RxTxTEG,

rx-TEG				RxTEG,

choice-extension		ProtocolIE-Single-Container { { TRPTEGInformation-ExtIEs} }

}

TRPTEGInformation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

RxTxTEG ::= SEQUENCE {

tRP-RxTx-TEGID					INTEGER (0..255),

tRP-Tx-TEGID					INTEGER (0..7)	OPTIONAL,

tRPRxTxTimingErrorMargin		TimingErrorMargin,

tRPTxTimingErrorMargin			TimingErrorMargin		OPTIONAL,

-- The IE shall be present if the TRP Tx TEG ID IE is present

iE-extensions		ProtocolExtensionContainer { { RxTxTEG-ExtIEs } }	OPTIONAL,

...

}

RxTxTEG-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RxTEG ::= SEQUENCE {

tRP-Rx-TEGID				INTEGER (0..31),

tRP-Tx-TEGID				INTEGER (0..7),

tRPRxTimingErrorMargin		TimingErrorMargin,

tRPTxTimingErrorMargin		TimingErrorMargin,

iE-extensions      ProtocolExtensionContainer { { RxTEG-ExtIEs } }	OPTIONAL,

...

}

RxTEG-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

** Unchanged text skipped **

TRPRxTEGInformation ::= SEQUENCE {

tRP-Rx-TEG-ID				INTEGER (0..31),

tRPRxTimingErrorMargin		TimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRPRxTEGInformation-ExtIEs } } OPTIONAL,

...

}

TRPRxTEGInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPTxTEGAssociation ::= SEQUENCE (SIZE(1.. maxnoTRPTEGs)) OF TRPTEGItem

TRPTEGItem ::= SEQUENCE {

tRP-Tx-TEG-ID			INTEGER (0..7),

dl-PRSResourceSetID		PRS-Resource-Set-ID,

dl-PRSResourceID-List	SEQUENCE (SIZE(1.. maxPRS-ResourcesPerSet)) OF DLPRSResourceID-Item	OPTIONAL,

tRPTxTimingErrorMargin	TimingErrorMargin,

iE-Extensions					ProtocolExtensionContainer { { TRPTEGItem-ExtIEs } } OPTIONAL,

...

}

TRPTEGItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

** Unchanged text skipped **

UETxTEGAssociationList ::= SEQUENCE (SIZE(1.. maxnoUETEGs)) OF UETxTEGAssociationItem

UETxTEGAssociationItem ::= SEQUENCE {

uE-Tx-TEG-ID				INTEGER (0..7),

posSRSResourceID-List		PosSRSResourceID-List,

timeStamp					TimeStamp,

carrierFreq					CarrierFreq				OPTIONAL,

uETxTimingErrorMargin		TimingErrorMargin,

iE-Extensions					ProtocolExtensionContainer { { UETxTEGAssociationItem-ExtIEs } } OPTIONAL,

...

}

UETxTEGAssociationItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

** Unchanged text skipped **

UL-RTOAMeasurement ::= SEQUENCE {

uLRTOAmeas			ULRTOAMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-RTOAMeasurement-ExtIEs } }	OPTIONAL,	...

}

UL-RTOAMeasurement-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore EXTENSION ExtendedAdditionalPathList	PRESENCE optional}|

{ ID id-TRPRxTEGInformation			CRITICALITY ignore EXTENSION TRPRxTEGInformation		PRESENCE optional},

...

}

Last Modification

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

** Unchanged text skipped **

id-TRPTEGInformation											ProtocolIE-ID ::= 85

id-TRPRxTEGInformation													ProtocolIE-ID ::= 86

id-TRP-PRS-Information-List										ProtocolIE-ID ::= 87

id-PRS-Measurements-Info-List									ProtocolIE-ID ::= 88

id-PRSConfigRequestType											ProtocolIE-ID ::= 89

id-UE-TEG-Info-Request											ProtocolIE-ID ::= 90

id-MeasurementTimeOccasion										ProtocolIE-ID ::= 91

id-MeasurementCharacteristicsRequestIndicator					ProtocolIE-ID ::= 92

id-TRPBeamAntennaInformation									ProtocolIE-ID ::= 93

id-NR-TADV														ProtocolIE-ID ::= 94

id-MeasurementAmount											ProtocolIE-ID ::= 95

id-pathPower													ProtocolIE-ID ::= 96

id-PreconfigurationResult										ProtocolIE-ID ::= 97

id-RequestType													ProtocolIE-ID ::= 98

id-UE-TEG-ReportingPeriodicity									ProtocolIE-ID ::= 99

id-SRSPortIndex													ProtocolIE-ID ::= 100

id-procedure-code-101-not-to-be-used							ProtocolIE-ID ::= 101

id-procedure-code-102-not-to-be-used							ProtocolIE-ID ::= 102

id-procedure-code-103-not-to-be-used							ProtocolIE-ID ::= 103

id-procedure-code-104-not-to-be-used							ProtocolIE-ID ::= 104

id-MeasurementPeriodicityNR-AoA									ProtocolIE-ID ::= 105