3GPP TSG-RAN WG3 Meeting #117bis-e 	R3-225588

E-meeting, 10 – 18 October 2022

Agenda item:	12.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.423) AI/ML Related Information and Procedures in Xn

Document for:	Text Proposal

# 1 Introduction

In this contribution, we provide TP related to our contributions R3-225584 and R3-225587. We also address the agreement from previous meeting:

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information.

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

# Annex – TP for BL CR for TS 38.423

<<< start of changes >>>

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| Prediction Data Reporting Initiation | PREDICTION DATA REQUEST | PREDICTION DATA RESPONSE | PREDICTION DATA FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| Prediction Data Reporting | PREDICTION DATA UPDATE |

<<< next change >>>

### 8.4.x	Prediction Data Reporting Initiation

#### 8.4.x.1	General

This procedure is used by an NG-RAN node to request the reporting of predictions to another NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: Prediction Data Reporting Initiation, successful operation

NG-RAN node1 initiates the reporting procedure by sending the PREDICTION DATA REQUEST message to NG-RAN node2 to start a prediction, stop a prediction or add cells to report for a prediction. Upon receipt, NG-RAN node2:

-	shall initiate the requested predictions reporting according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells predictions and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the reporting of prediction data initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If reporting of prediction data is already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the PREDICTION DATA REQUEST message and the Report Characteristics IE indicates cell specific predictions, the Cell To Report List IE shall be included.

If the Registration Request IE is set to "add" in the PREDICTION DATA REQUEST message, the Cell To Report List IE shall be included.

If the Reporting Type IE is set to periodic then NG-RAN node2 shall report the predicted information in a periodic manner, with periodicity indicated in the Reporting Periodicity IE. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the Reporting Type IE is set to event-based then NG-RAN node2 shall report the predicted information in an event-based manner if the corresponding event indicated in the Event List IE is satisfied.

If the Reporting Type IE is set to both, then NG-RAN node2 may report the predicted information in a periodic way, with periodicity indicated in the Reporting Periodicity IE, as well as in an event-based manner, if an event indicated in the Event List IE is satisfied.

If NG-RAN node2 is capable to provide all requested predicted data information, it shall initiate the prediction as requested by NG-RAN node1 and respond with the PREDICTION DATA RESPONSE message.

Interaction with other procedures

When starting the reporting of predictions, the Report Characteristics IE in the PREDICTION DATA REQUEST indicates the type of objects NG-RAN node2 shall perform predictions on. For each cell, NG-RAN node2 shall include in the PREDICTION DATA UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, “Predicted Load” of the Report Characteristics IE included in the PREDICTION DATA REQUEST message is set to "1". If NG-RAN node2 is a gNB and if the cell for which Predicted Radio Resource Status IE is requested to be reported supports more than one SSB, the Predicted Radio Resource Status IE for such cell shall include the Predicted SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall include the requested Predicted SSB Area Radio Resource Status List IE; If the cell for which Predicted Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the requested Predicted Slice Radio Resource Status Item IE;

- 	the Predicted EE metric IE, if the second bit “Predicted EE metric” of the Report Characteristics IE included in the PREDICTION DATA REQUEST message is set to "1".

#### 8.4.x.3	Unsuccessful Operation

Figure 8.4.x.3-1: Prediction Data Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, NG-RAN node2 shall send the PREDICTION DATA FAILURE message with an appropriate cause value.

#### 8.4.x.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the PREDICTION DATA REQUEST message or the PREDICTION DATA FAILURE message, the NG-RAN node1 may reinitiate the Prediction Data Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new PREDICTION DATA REQUEST message is identical to the content of the previously unacknowledged PREDICTION DATA REQUEST message.

If the NG-RAN node2 receives a PREDICTION DATA REQUEST message which includes the Registration Request IE set to "add" or "stop" and if the NG-RAN node2 Measurement ID value received in the PREDICTION DATA REQUEST message is not used, the NG-RAN node2 shall initiate PREDICTION DATA FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the PREDICTION DATA REQUEST message then NG-RAN node2 shall initiate a PREDICTION DATA FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives a PREDICTION DATA REQUEST message which includes the Registration Request IE set to "start" and the NG-RAN node1Measurement ID IE corresponding to an existing on-going load prediction reporting, then NG-RAN node2 shall initiate a PREDICTION DATA FAILURE message with an appropriate cause value.

### 8.4.y	Prediction Data Reporting

#### 8.4.y.1	General

This procedure is initiated by an NG-RAN node to report the result of predictions admitted by the NG-RAN node following a successful Prediction Data Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.4.y.2	Successful Operation

Figure 8.4.y.2-1: Prediction Data Reporting, successful operation

NG-RAN node2 shall report the results of the admitted predictions in PREDICTION DATA UPDATE message. The admitted predictions are the predictions that were successfully initiated during the preceding Prediction Data Reporting initiation procedure.

#### 8.4.y.3	Unsuccessful Operation

Not applicable.

#### 8.4.y.4	Abnormal Conditions

Void

<<< next change >>>

#### 9.1.3.w	PREDICTION DATA REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested prediction according to the parameters given in the message.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | reject |
| Reporting Type | M |  | ENUMERATED(periodic, event-based, both) |  | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = Predicted Load, Second Bit = Predicted EE metric. Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | – |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  | – |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | S-NSSAI 9.2.3.21 |  | – |  |
| Predicted Load Input | C-ifReportCharacteristicsFirstBitSet |  | BITSTRING (SIZE(32)) | First Bit = Predicted PRB, Second Bit = Predicted TNL Capacity Ind, Third Bit = Predicted Composite Available Capacity, Fourth Bit = Predicted Number of Active UEs, Fifth Bit = Predicted RRC connections Other bits shall be ignored by the NG-RAN node2. |  |  |
| >Predicted PRB Load | C-ifPredictedLoadFirstBitSet |  |  |  |  |  |
| >> Info about requested prediction |  |  |  |  |  |  |
| >>> Time of prediction |  |  |  |  |  |  |
| >Predicted TNL Capacity Ind | C-ifPredictedLoadSecondBitSet |  |  |  |  |  |
| >> Info about requested prediction |  |  |  |  |  |  |
| >>> Time of prediction |  |  |  |  |  |  |
| >Predicted Composite Available Capacity | C-ifPredictedLoadThirdBitSet |  |  |  |  |  |
| >> Info about requested prediction |  |  |  |  |  |  |
| >>> Time of prediction |  |  |  |  |  |  |
| >Predicted Number of Active UEs | C-ifPredictedLoadFourthBitSet |  |  |  |  |  |
| >> Info about requested prediction |  |  |  |  |  |  |
| >>> Time of prediction |  |  |  |  |  |  |
| >Predicted RRC connections | C-ifPredictedLoadFifthBitSet |  |  |  |  |  |
| >> Info about requested prediction |  |  |  |  |  |  |
| >>> Time of prediction |  |  |  |  |  |  |
| Predicted Energy Efficiency | C-ifReportCharacteristicsSecondBitSet |  |  |  |  |  |
| >> Info about requested prediction |  |  |  |  |  |  |
| >>>PRB Load |  |  | 9.2.2.50 |  |  |  |
| Reporting Periodicity | C-ifPeriodicTypeSetTrue |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. | YES | ignore |
| Event List | C-ifEventTypeSetTrue |  | ENUMERATED (inference updated, …) | List of events that can be used for reporting of indicated measurements. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |
| ifPeriodicTypeSetTrue | This IE shall be present if the Reporting Type IE is set to “periodic”. |
| ifEventBasedTypeSetTrue | This IE shall be present if the Reporting Type IE is set to “event-based”. |
| ifReportCharacteristicsFirstBitSet | This IE shall be present if the first bit in Report Characteristics IE is set to 1. |
| ifReportCharacteristicsSecondBitSet | This IE shall be present if the second bit in Report Characteristics IE is set to 1. |
| ifPredictedLoadFirstBitSet | This IE shall be present if the first bit of Predicted Load Input IE is set to 1. |
| ifPredictedLoadSecondBitSet | This IE shall be present if the second bit of Predicted Load Input IE is set to 1. |
| ifPredictedLoadThirdBitSet | This IE shall be present if the third bit of Predicted Load Input IE is set to 1. |
| ifPredictedLoadFourthBitSet | This IE shall be present if the fourth bit of Predicted Load Input IE is set to 1. |
| IfPredictedLoadFifthBitSet | This IE shall be present if the fifth bit of Predicted Load Input IE is set to 1. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.1.3.x	PREDICTION DATA RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested prediction data, for all of the measurement objects included in the prediction data is successfully initiated.

Direction: NG-RAN node2 ® NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.y	PREDICTION DATA FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested prediction data measurements the measurement cannot be initiated.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.z	PREDICTION DATA UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested prediction data.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.x |  | – |  |
| >>Predicted EE Metric | O |  | INTEGER (0..100) | Value 100 corresponds to maximum predicted EE metric. Value 0 corresponds to minimum predicted EE metric. Values are calculated in linear scale. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<< next change >>>

#### 9.2.2.x	Predicted Radio Resource Status

This message IE provides load predictions:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted PRB Load | O |  | 9.2.2.50 | Predicted value of Radio Resource Status IE |
| Predicted TNL Capacity Ind | O |  | 9.2.2.49 | Predicted value of TNL Capacity IE |
| Predicted Composite Available Capacity | O |  | 9.2.2.51 | Predicted value of Composite Available Capacity IE |
| Predicted Number of Active UEs | O |  | 9.2.2.62 | Predicted value of the number of Active UEs IE |
| Predicted RRC connections | O |  | 9.2.2.56 | Predicted value of number of RRC Connections IE |

<<< end of changes >>>