3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225590

E-meeting, 10 – 18 October, 2022

Agenda item:	11.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Handling of QMC configuration for NR-DC

Document for:	Discussion and Decision

# 1	Introduction

Discussion at RAN3#117 allowed to identify several open points concerning handling of MR-DC in QMC activation phase, and we here provide further analysis taking into account principles used for m-based MDT.

# 2	Discussion

The following three scenarios for m-based QoE activation were discussed at RAN#117 (see SoD in R3-225235):

- M-based QoE configuration is only received by MN;

- M-based QoE configuration is only received by SN;

- M-based QoE configuration is received by both MN and SN.

## 2.1	Background from m-based MDT

Although, as per current standard a gNB supports both MN and SN roles, the scenario description above has a parallel in the description of immediate MDT for MR-DC case in TS 37.320: "In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently." However, the technical solution in the RAN relies on independent MDT operation within the MN and the SN, as described further in the same specification: "For immediate MDT configuration, MN and SN can independently configure and receive measurement from the UE."

Observation 1: The m-based MDT solution in the RAN relies on independent MDT operation within the MN and the SN.

A consequence is that the area scope provided together with the m-based MDT configuration relates to Pcells when the configured gNB acts as MN, and relates to PScells when the configured gNB acts as SN. In case of carrier aggregation (CA), we also assume that the gNB additionally considers SCells (within the same node) when selecting UEs for m-based MDT.

A further illustration of the same principle is that TS 37.320 also indicates that it is the OAM of the node in charge of the PDCP termination of a given bearer that configures per DRB measurements (e.g. average UE throughput or packet delay), irrespective of which node is responsible for the actual transmission of the bearer: "For MN terminated SCG bearer and SN terminated MCG bearer, the terminated node, e.g., MN in case of MN terminated SCG bearer, configures the configuration to UE." This means that in case where the operator needs to monitor e.g. MN-terminated SCG bearers, the configured area scope will correspond to the Pcell even though the data is actually transferred over e.g. a PScell. However, we believe that the operator today has sufficient information to correctly configure m-based MDT also for this case, in particular by taking into account that the OAM is informed about neighbour relations (the PScell in MR-DC is necessarily a neighbour of the Pcell), cf. TS 38.300: "The OAM system is informed about changes in the NCRT.".

This also means that for MDT there is no need for the RAN to coordinate m-based configurations provided by the OAM independently to gNBs acting as MN and SN, respectively.

Observation 2: For MDT, there is no need for the RAN to coordinate m-based configurations provided by the OAM independently to gNBs acting as MN and SN.

## 2.2	M-based QMC configuration principles

From the discussion above, we can see that m-based MDT configuration relies on the principle of autonomy of each node, i.e. MDT measurements are always configured towards the node that is in charge of the corresponding measurements. And in case of MR-DC, some measurements are performed by the MN while other measurements are performed by the SN (hence TS 37.320: "In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently.").

In case of QMC the RAN does not itself perform any QoE measurements but forwards the QMC configuration to the application layer in the UE. A mechanism based on the OAM system providing the QMC configuration to both MN and SN independently can therefore not be motivated by autonomous QMC handling in the nodes. A potential motivation could be related to area scope definition, i.e. how the operator can configure m-based QMC for cells deployed for Scell operation in MR-DC. But as discussed above, a similar scenario for MDT would be per-DRB measurements of MN-terminated SCG bearers, and in this case the m-based MDT configuration is sent to the MN with area scope corresponding to MCG cells. In our understanding, this scenario works today well in terms of configuration.

Furthermore, we don't believe that there is any QMC analogy of the scenario where m-based MDT is configured for SN-terminated MCG bearers.

Based on this analysis, we would therefore like make the following comments and proposals based on agreements and open points identified at RAN3#117 for m-based QMC configuration:

For M-based QoE configuration in NR-DC, coordination between MN and SN is needed.

The corresponding coordination can be made by OAM. The required coordination can be enabled by clarifying that the area scope for QMC corresponds to MCG cells.

Proposal 1: For M-based QoE configuration in NR-DC, coordination between MN and SN is performed by the OAM system providing the required configuration to the MN, using area scope for QMC corresponding to MCG cells.

If the M-based QoE configuration is received by the MN, the MN should make the decision on the UE selection and on which node sends the QoE configuration to the UE.

We confirm that m-based QMC configuration can be received by the MN.

If the M-based QoE configuration is received only by the SN, whether the MN or the SN performs UE selection and sends the QoE configuration to the UE needs to be further discussed.

Following our proposal 1 there is no need to send m-based QMC configuration to the SN.

Proposal 2: There is no need to support m-based QMC configuration to the SN.

## 2.3	QMC configuration principles for RVQoE

The following working assumption and agreement were taken at RAN3#117:

WA: MN and SN can generate RVQoE configurations.

MN and SN should coordinate about configuring a dual-connected UE with RVQoE measurements. The details of the coordination are FFS.

The rationale is that MN and SN independently consume RVQoE measurements, e.g. for their respective schedulers, and configurations may therefore be different. Still, we believe RAN3 should take into account the impact of potentially very frequent reconfigurations of RVQoE in the UE application layer in terms of:

- RRC signalling load for reconfigurations

- load on the application layer to process reconfigurations

- impact on measurement validity, e.g. the observation period required by some measurements may not allow potentially frequent stop/start of these measurements

In Rel-17 QMC, the QoE configuration, including RVQoE, will typically remain stable while the UE is served by the same Pcell. Furthermore, current SA4 specification doesn't allow reconfiguration of ongoing QMC sessions. Taking into account these issues, we believe the same configuration stability should be ensured also in Rel-18. In that respect, the above WA could have been confirmed if the coordination between MN and SN (FFS above) resulted in the superset of MN and SN metrics being configured towards the UE. However in a typical scenario the SN may change while the MN remains the same, and it therefore doesn't seem possible to ensure this stability of the RVQoE configuration. A better solution would therefore be that the MN has configured information reflecting the needs for RVQoE metrics for SNs under the coverage of the Pcell, so that the MN can provide an RVQoE configuration corresponding to a superset of the required metrics and hence remaining valid for the full time the UE stays under this cell or in a wider area, e.g. under the gNB serving as MN. In this way frequent reconfigurations can be avoided.

Proposal 3: Avoid frequent QoE reconfigurations by using configured information reflecting the needs for RVQoE metrics for SNs under the coverage of the Pcell, and configure the superset of required metrics towards the UE.

# 3	Conclusion

We have made the following observations and proposals:

Observation 1: The m-based MDT solution in the RAN relies on independent MDT operation within the MN and the SN.

Observation 2: For MDT, there is no need for the RAN to coordinate m-based configurations provided by the OAM independently to gNBs acting as MN and SN.

Proposal 1: For M-based QoE configuration in NR-DC, coordination between MN and SN is performed by the OAM system providing the required configuration to the MN, using area scope for QMC corresponding to MCG cells.

Proposal 2: There is no need to support m-based QMC configuration to the SN.

Proposal 3: Avoid frequent QoE reconfigurations by using configured information reflecting the needs for RVQoE metrics for SNs under the coverage of the Pcell, and configure the superset of required metrics towards the UE.