3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225639

E-meeting, 10 – 18 October 2022

Title: 	Discussion on positioning SI delivery over F1AP

Source: 	Huawei, China Unicom, CMCC

Agenda item:	9.2.4

Document Type:	Discussion

1. Introduction

In this document, we discuss the support of PosSIB delivery over F1AP.

2. Discussion

According to TS38.331, as cited and highlighted below, the RRCSystemInfoRequest IE can be used by the UE to request SI messages configured by SI-SchedulingInfo or posSI-SchedulingInfo in SIB1. In other words, the UE can request the gNB to broadcast either the requested SI or requested posSI.

RRCSystemInfoRequest message -- ASN1START -- TAG-RRCSYSTEMINFOREQUEST-START RRCSystemInfoRequest ::= SEQUENCE { criticalExtensions CHOICE { rrcSystemInfoRequest RRCSystemInfoRequest-IEs, criticalExtensionsFuture-r16 CHOICE { rrcPosSystemInfoRequest-r16 RRC-PosSystemInfoRequest-r16-IEs, criticalExtensionsFuture SEQUENCE {} } } } RRCSystemInfoRequest-IEs ::= SEQUENCE { requested-SI-List BIT STRING (SIZE (maxSI-Message)), --32bits spare BIT STRING (SIZE (12)) } RRC-PosSystemInfoRequest-r16-IEs ::= SEQUENCE { requestedPosSI-List BIT STRING (SIZE (maxSI-Message)), --32bits spare BIT STRING (SIZE (11)) } -- TAG-RRCSYSTEMINFOREQUEST-STOP -- ASN1STOP RRCSystemInfoRequest-IEs field descriptions. requested-SI-List Contains a list of requested SI messages. According to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on.. requestedPosSI-List Contains a list of requested SI messages. According to the order of entry in the list of SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo in SIB1, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on..

In CU DU split case, upon receiving the request, the gNB-CU shall send the System Information Delivery message to indicate the gNB-DU to broadcast the requested system information. In this message, the SIType List IE (clause 9.3.1.62, TS38.473 [1]) can be used by the gNB-CU to indicate the list of other SI. However, there is currently no method to enable the gNB-CU to provide the requested posSI configured in posSI-SchedulingInfo IE to the gNB-DU.

- Observation1: the UE can request to broadcast posSI configured in posSI-SchedulingInfo IE by sending RRCSystemInfoRequest message. however, the gNB-CU cannot indicate the requested posSI to the gNB-DU on F1AP.

- Since both the SIType List IE in System Information Delivery Command message and the SI Type IE in SIType List IE are mandatory, there are two options to correct this issue:

- Option 1: Define a new F1AP message (e.g., Positioning System Information Delivery Command) to enable the gNB-CU to request the gNB-DU to broadcast the requested posSI.

- Option 2: Add a new indicator (e.g. PosSIRequested) in the System Information Delivery Command message, to indicate to the gNB-DU that the SIType List IE is the list of requested posSI configured in posSI-SchedulingInfo IE for broadcast. This is because that the SIType List in the message is mandatory.

- It should be noticed that both option 1 and option 2 are BC changes. Hence, to minimize specification impacts, it is proposed to adopt option 2 as the solution.

- An example of option 2 is given as follows:

#### 9.2.5.1 SYSTEM INFORMATION DELIVERY COMMAND This message is sent by the gNB-CU and is used to enable the gNB-DU to broadcast the requested other SI or posSI. Direction: gNB-CU gNB-DU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. Transaction ID; M; 9.3.1.23; YES; reject. NR CGI; M; 9.3.1.12; NR cell identifier; YES; reject. SIType List; M; 9.3.1.62; Indicates the positioning SI if PosSIRequested is present.; YES; reject. Confirmed UE ID; M; gNB-DU UE F1AP ID 9.3.1.5; YES; reject. PosSIRequested; O; ENUMERATED(true,…); YES; reject. #### *** Unrelated Text Skipped *** #### 9.3.1.62 SIType List This IE is used by gNB-CU to provide SI list of other SI or posSI for gNB-DU. IE/Group Name; Presence; Range; IE type and reference; Semantics description. SI type item IEs; 1.. <maxnoofSITypes>. >SI Type; M; INTEGER (1..32, ...); Indicates a certain SI type required to be broadcasted by the gNB-DU. The SI Type value of other SI starts from 2. Range bound; Explanation. maxnoofSITypes; Maximum no. of SI types, the maximum value is 32..

- Although we usually introduce new F1AP procedures for positioning, option 2 seems simpler and sufficient to correct this issue.

Proposal 1: Add a new indicator (e.g. PosSIRequested) in the System Information Delivery Command message, to indicate to the gNB-DU that the SIType List IE is the list of requested posSI configured in posSI-SchedulingInfo IE for broadcast.

3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation1: the UE can request to broadcast posSI configured in posSI-SchedulingInfo IE by sending RRCSystemInfoRequest message. however, the gNB-CU cannot indicate the requested posSI to the gNB-DU on F1AP.

Proposal 1: Add a new indicator (e.g. PosSIRequested) in the System Information Delivery Command message, to indicate to the gNB-DU that the SIType List IE is the list of requested posSI configured in posSI-SchedulingInfo IE for broadcast.

The corresponding CRs are provided in [2] and [3].

4. Reference

- 3GPP TS 38.473 V17.2.0

- R3-225640	Correction on positioning SI delivery over F1AP

- R3-225641	Correction on positioning SI delivery over F1AP