3GPP TSG-RAN WG3 #117bis-e	R3-225729

E-meeting, 10th – 18th October 2022

Source:	CATT

Title:	TP on TS 38.420 for AI/ML

Agenda Item:	12.2.2.1

Document for:	Discussion and decision

- Introduction

This is a TP based on the discussion paper R3-225726 and R3-225727.

- TP for TS 38.420

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

- 6	Xn interface procedures

## 6.1	General

The Xn interface supports procedures over the control plane (Xn-C) and user plane (Xn-U).

## 6.2	Control plane protocol procedures

### 6.2.1	Mobility management procedures

The mobility management procedures are used to manage the UE mobility in Connected or RRC_Inactive modes:

-	Handover Preparation

-	Handover Cancel

-	SN Status Transfer

-	Retrieve UE Context

-	RAN Paging

-	Xn-U Address Indication

-	UE Context Release

-	Handover Success Indication

-	Conditional Handover Cancel

-	Retrieve UE Context Confirm

### 6.2.2	Dual Connectivity procedures

The dual connectivity procedures are used to add, modify and releases resources for the operation of Dual Connectivity:

-	S-NG-RAN-node Addition Preparation

-	S-NG-RAN-node Reconfiguration Completion

-	M-NG-RAN-node initiated S-NG-RAN-node Modification Preparation

-	S-NG-RAN-node initiated S-NG-RAN-node Modification

-	M-NG-RAN-node initiated S-NG-RAN-node Release

-	S-NG-RAN-node initiated S-NG-RAN-node Release

-	S-NG-RAN-node Counter Check

-	RRC Transfer

-	Notification Control Indication

-	Activity Notification

-	Secondary RAT Data Usage Report

-	Conditional PSCell Change Cancel

### 6.2.3	Global procedures

The global procedures are used to exchange configuration level data between two NG-RAN nodes, or to remove Xn connectivity between two NG-RAN nodes in a controlled manner:

-	Xn Setup

-	NG-RAN-node Configuration Update

-	Xn Removal

### 6.2.4	Interface Management procedures

The interface management procedures are used to align resources between two NG-RAN nodes in the event of failures, and to report detected protocol errors:

-	Reset

-	Error Indication

### 6.2.5	Energy saving procedures

-	Cell Activation procedure: enables an NG-RAN node to request the activation of a previously deactivated cell hosted in another NG-RAN node.

### 6.2.6	Resource coordination procedures

-	E-UTRA - NR Cell Resource Coordination procedure: enables an ng-eNB and a gNB to interact for resource coordination purposes.

### 6.2.7	UE Tracing procedures

The following procedures are used to trace the UE:

-	Trace Start procedure

-	Deactivate Trace procedure

-	Cell Traffic Trace

### 6.2.8	Load management procedures

The load management procedures are used by NG-RAN nodes to indicate resource status, overload and traffic load to each other.

-	Resource Status Reporting Initiation

-	Resource Status Reporting

### 6.2.9	Data exchange for self-optimisation procedures

The data exchange for self-optimisation procedures are used to transfer failure and mobility related information among NG-RAN nodes to enable self-optimisation

-	Failure Indication

-	Handover report

-	Mobility Settings Change

-	Access and Mobility Indication

-	SCG Failure Information Report

-	SCG Failure Transfer

### 6.2.10	IAB procedures

The IAB procedures are used to enable the transfer of F1/non-F1 traffic for IAB, to exchange information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, to enable the delivery of F1-C traffic between the M-NG-RAN node and the S-NG-RAN node serving a dual-connected non-boundary IAB-node, and to exchange resource multiplexing related information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node:

-	F1-C Traffic Transfer

-	IAB Transport Migration Management

-	IAB Transport Migration Modification

-	IAB Resource Coordination

### 6.2.11	MBS Management procedures

The MBS management procedures are used to manage the MBS Session:

-	RAN Multicast Group Paging procedure

### 6.2.12	Small data transmission procedures

-	Partial UE Context Transfer: enables exchange of information between NG-RAN nodes for SDT transmission without anchor relocation

Small data transmission is also supported by the following procedures:

-	RRC Transfer

-	Retrieve UE Context Confirm

### 6.2.13	QMC support procedures

The following procedures are used to transfer QMC configuration and session information to the target NG-RAN node during a UE’s intra-system intra-RAT mobility:

-	Handover Preparation

-	Retrieve UE Context

### 6.2.x	Data collection procedures

The following procedures are used to collect data across NG-RAN nodes, mainly for the purpose of Artificial Intelligence / Machine Learning:

-	Data Collection Initiation

-	Data Collection Reporting

## 6.3	User plane protocol procedures

The user plane protocol procedures are used to exchange user plane information between Xn-U protocol peers:

-	Transfer of Downlink User Data procedure: enables the node hosting the NR PDCP entity to provide user plane information to the corresponding node.

-	Downlink Data Delivery Status procedure: enables the corresponding node to provide feedback to the node hosting the NR PDCP entity.

-	Transfer of Assistance Information: enables the corresponding node to provide assistance information to the node hosting the NR PDCP entity.

-	Transfer of PDU Session Information procedure: enables an NG-RAN node to provide user plane information associated with the forwarding of data towards a peer NG-RAN node, when using PDU session tunnels.

///////////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////////