3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225738

# E-meeting, 10 – 18 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1065 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration Restriction Info for DAPS handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Qualcomm Incorporate |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Previous RAN3#117 meeting discussed how to support the configuration restriction information for DAPS handover in case of CU/DU split case (see the SoD in R3-225193). The key question is whether the source CU or the source DU can decide it, then the configRestrictInfoDAPS IE can be included in the Handover Preparation Information message. Below provide details in the TS 38.331. ConfigRestrictInfoDAPS-r16 ::= SEQUENCE { powerCoordination-r16 SEQUENCE { p-DAPS-Source-r16 P-Max, p-DAPS-Target-r16 P-Max, uplinkPowerSharingDAPS-Mode-r16 ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic } } OPTIONAL } ConfigRestrictInfoDAPS-v1640 ::= SEQUENCE { sourceFeatureSetPerDownlinkCC-r16 FeatureSetDownlinkPerCC-Id, sourceFeatureSetPerUplinkCC-r16 FeatureSetUplinkPerCC-Id } It is reasonable that the source DU can decide the restriction information including the power coordination and the feature set. Compared with the option that the source CU decides it, this option has the following characterises. - The power coordination defines the maximum total transmit power to be used by the UE in the source and target, which is much related to the physical /L1 parameters. Hence it should be decided for the MN DU discretion. - The feature set information indicates the index selected by the source for DL and UL, which includes a set of features that the UE supports on one band entry with detailed physical parameters. Hence it is up to the MN DU decision. - For MR-DC case at the SN side, the SN-DU will provide the Requested P-MaxFR1/ Requested P-MaxFR2/ FeatureSetEntryIndex etc to the SN CU, indicating the maximum value for FR1/FR2 and feature set entry index to be used at the SN side. This proves that the DU decide the DAPS lower layer parameters, for the DAPS handover. Hence to align with the handlings for MR-DC, this CR proposes that the source DU decides the configuration restriction information, and signal to the CU. In order for the DU to provide the accurate information, the target cell ID related information is also provided to the source DU so that the source DU can select the more proper restriction information. Note that this can be sent together with the GNB-DU Configuration Query IE in the UE CONTEXT MODIFICATION REQUEST message, when the gNB-CU requests the latest lower layer information before the handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the ConfigRestrictInfoDAPS1 IE and ConfigRestrictInfoDAPS2 IE in the DU to CU RRC Information. - Add the Target Cell Information DAPS IE in the UE CONTEXT MODIFICATION REQUEST message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the configuration restriction information for DAPS handover over F1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration restriction information is not supported in the CU-DU split architecture for DAPS handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.2, 9.2.2.7, 9.3.1.26, 9.3.1.aaa, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

<Unchanged Text Omitted>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

<Unchanged Text Omitted>

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the Target Cell Information DAPS IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the ConfigRestrictInfoDAPS1 IE and/or the ConfigRestrictInfoDAPS2 IE included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-CU shall use the received information for DAPS handover as described in TS 38.331 [8].

<Unchanged Text Omitted>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT SRB. | YES | reject |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | YES | reject |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 |  | YES | ignore |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >>RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| >>SDT Indicator Modify | O |  | ENUMTERATED (true, false, …) | Indicates SDT DRB or not. | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Released Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| IAB Conditional RRC Message Delivery Indication | O |  | ENUMERATED (true, …) | Indicates whether the RRC message within should be withheld. This IE is only applicable if the UE is an IAB-MT, and the gNB-DU is an IAB-DU. | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 | This IE is only applicable if the UE is an IAB-MT. | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Updated Remote UE Local ID | O |  | Remote UE Local ID 9.3.1.267 | This IE indicates the updated Remote UE Local ID for the U2N Remote UE associated with the F1AP-IDs | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | O |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to Be Released Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  |  |  |
| >>CHOICE PC5 RLC Channel QoS Information | O |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicate the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Released Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| Multicast MBS Session Remove List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has left. | YES | reject |
| UE Multicast MRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| UE Multicast MRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| SL DRX Cycle List |  | 0..1 |  |  | YES | ignore |
| >SL DRX Cycle Item IEs |  | 1 .. <maxnoofSLdestinations > |  |  | EACH | ignore |
| >>RX UE ID | M |  | BIT STRING (SIZE(24)) | Indicates the destination L2 ID of RX UE associated to this UE. | - |  |
| >>CHOICE SL DRX Information | M |  |  |  | - |  |
| >>>SL DRX Cycle |  |  |  |  |  |  |
| >>>>SL DRX Cycle Length | M |  | ENUMERATED (ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...) | Indicates the desired SL DRX cycle for RX UE associated to this UE. | - |  |
| >>>No SL DRX |  |  |  |  | - |  |
| >>>>SL DRX configuration indicator | M |  | ENUMERATED(release,...) |  | - |  |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.274 |  | YES | ignore |
| SDT Bearer Configuration Query Indication | O |  | ENUMERATED (true, ...) |  | YES | Ignore |
| Target Cell Information DAPS | O |  | 9.3.1.aaa |  | YES | ignore |

<Unchanged Text Omitted>

#### 9.3.1.26	DU to CU RRC Information

This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CellGroupConfig | M |  | OCTET STRING | CellGroupConfig, as defined in TS 38.331 [8]. |  |  |
| MeasGapConfig | O |  | OCTET STRING | MeasGapConfig as defined in TS 38.331 [8]. For EN-DC/NGEN-DC operation, includes the gap for FR2, as requested by the gNB-CU via MeasConfig IE. For NG-RAN,NE-DC and MN for NR-NR DC, includes the gap(s) for FR1 and/or FR2, as requested by the gNB-CU via MeasConfig IE and according to the requested gap type (per-UE or per-FR). |  |  |
| Requested P-MaxFR1 | O |  | OCTET STRING | requestedP-MaxFR1, as defined in TS 38.331 [8]. For EN-DC, NGEN-DC and NR-DC operation, this IE should be included. |  |  |
| DRX Long Cycle Start Offset | O |  | INTEGER (0..10239) | Identical to the value of the drx-LongCycleStartOffset IE within the DRX-Config as defined in TS 38.331 [8]. This field is not used in NR-DC. |  |  |
| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination; if this IE is included, the gNB-CU uses this information to deduce the selected band. | YES | ignore |
| Selected FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected FeatureSet. | YES | ignore |
| Ph-InfoSCG | O |  | OCTET STRING | PH-TypeListSCG, as defined in TS 38.331 [8].For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in SN. | Yes | ignore |
| Requested BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Band Combination. | YES | ignore |
| Requested FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Feature Set. | YES | ignore |
| DRX Config | O |  | OCTET STRING | DRX-Config, as defined in TS 38.331 [8]. This field is only used in NR-DC. | YES | ignore |
| PDCCH BlindDetectionSCG | O |  | OCTET STRING | pdcch-BlindDetectionSCG, as defined in TS 38.331 [8]. This IE is used between the MgNB-DU and the MgNB-CU. | YES | ignore |
| Requested PDCCH BlindDetectionSCG | O |  | OCTET STRING | requestedPDCCH-BlindDetectionSCG, as defined in TS 38.331 [8]. This IE is used between the SgNB-DU and the SgNB-CU. | YES | ignore |
| Ph-InfoMCG | O |  | OCTET STRING | PH-TypeListMCG, as defined in TS 38.331 [8]. For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in MCG. | YES | ignore |
| MeasGapSharingConfig | O |  | OCTET STRING | MeasGapSharingConfig as defined in TS 38.331 [8]. | YES | ignore |
| SL-PHY-MAC-RLC-Config | O |  | OCTET STRING | SL-PHY-MAC-RLC-Config as defined in TS 38.331 [8]. | YES | ignore |
| SL-ConfigDedicatedEUTRA-Info | O |  | OCTET STRING | SL-ConfigDedicatedEUTRA-Info as defined in TS 38.331 [8]. | YES | ignore |
| Requested P-MaxFR2 | O |  | OCTET STRING | RequestedP-MaxFR2, as defined in TS 38.331 [8]. For NR-DC operation, this IE should be included. | YES | ignore |
| SDT-MAC-PHY-CG-Config | O |  | OCTET STRING | SDT-MAC-PHY-CG-Config, as defined in TS 38.331 [8]. | YES | ignore |
| MUSIM-GapConfig | O |  | OCTET STRING | MUSIM-GapConfig as defined in TS 38.331 [8]. | YES | ignore |
| SL-RLC-ChannelToAddModList | O |  | OCTET STRING | sl-RLC-ChannelToAddModList-r17, as defined in TS 38.331 [8] | YES | ignore |
| InterFrequencyConfig-NoGap | O |  | ENUMERATED (true, …) | Identical to the value of the interFrequencyConfig-NoGap-r16 IE, as defined in TS 38.331 [8]. | YES | ignore |
| ul-GapFR2-Config | O |  | OCTET STRING | ul-GapFR2-Config-r17, as specifed in TS 38.331 [8]. | YES | Ignore |
| ConfigRestrictInfoDAPS1 | O |  | OCTET STRING | ConfigRestrictInfoDAPS-r16, as specifed in TS 38.331 [8]. | YES | ignore |
| ConfigRestrictInfoDAPS2 | O |  | OCTET STRING | ConfigRestrictInfoDAPS-v1640, as specifed in TS 38.331 [8]. | YES | ignore |

<Unchanged Text Omitted>

#### 9.3.1.aaa	Target Cell Information DAPS

This IE contains target cell information for DAPS handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID |
| CHOICE NR-Mode-Info | M |  |  |  |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. |
| >>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. |
| >>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  |
| >>>Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  |
| >>>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | The tdd-UL-DL-ConfigurationCommon as defined in TS 38.331 [8] |
| >>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. |

<Unchanged Text Omitted>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

<Unchanged Text Omitted>

UE-MulticastMRBs-ToBeSetup-Item,

PosMeasurementAmount,

BAP-Header-Rewriting-Removed-List-Item,

SLDRXCycleList,

MDTPLMNModificationList,

ActivationRequestType,

PosMeasGapPreConfigList,

PosMeasurementPeriodicityNR-AoA,

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

TargetCellInformationDAPS

<Unchanged Text Omitted>

id-BAP-Header-Rewriting-Removed-List-Item,

id-SLDRXCycleList,

id-ManagementBasedMDTPLMNModificationList,

id-ActivationRequestType,

id-PosMeasGapPreConfigList,

id-PosMeasurementPeriodicityNR-AoA,

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-TargetCellInformationDAPS,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

<Unchanged Text Omitted>

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator			PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 							PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-DRXConfigurationIndicator				CRITICALITY ignore	TYPE DRXConfigurationIndicator						PRESENCE optional	}|

{ ID id-RLCFailureIndication					CRITICALITY ignore	TYPE RLCFailureIndication								PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentListInformation	CRITICALITY ignore	TYPE UplinkTxDirectCurrentListInformation		PRESENCE optional	}|

{ ID id-GNB-DUConfigurationQuery				CRITICALITY reject	TYPE GNB-DUConfigurationQuery						PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE optional	}|

{ ID id-ExecuteDuplication						CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-NeedforGap								CRITICALITY ignore	TYPE NeedforGap											PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	TYPE LowerLayerPresenceStatusChange				PRESENCE optional	}|

{ ID id-BHChannels-ToBeSetupMod-List			CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeModified-List			CRITICALITY reject	TYPE BHChannels-ToBeModified-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeReleased-List			CRITICALITY reject	TYPE BHChannels-ToBeReleased-List					PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetupMod-List				CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeModified-List				CRITICALITY reject	TYPE SLDRBs-ToBeModified-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeReleased-List				CRITICALITY reject	TYPE SLDRBs-ToBeReleased-List						PRESENCE optional	}|

{ ID id-ConditionalIntraDUMobilityInformation	CRITICALITY reject	TYPE ConditionalIntraDUMobilityInformation		PRESENCE optional}|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore	TYPE SCGIndicator										PRESENCE optional	}|

{ ID id-IABConditionalRRCMessageDeliveryIndication				CRITICALITY reject	TYPE IABConditionalRRCMessageDeliveryIndication							PRESENCE optional	}|

{ ID id-F1CTransferPathNRDC						CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional }|

{ ID id-MDTPollutedMeasurementIndicator			CRITICALITY ignore	TYPE MDTPollutedMeasurementIndicator				PRESENCE optional }|

{ ID id-SCGActivationRequest					CRITICALITY ignore	TYPE SCGActivationRequest								PRESENCE optional }|

{ ID id-CG-SDTQueryIndication					CRITICALITY ignore	TYPE CG-SDTQueryIndication							PRESENCE optional	}|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional }|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitrate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-FiveG-ProSePC5LinkAMBR					CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-UpdatedRemoteUELocalID					CRITICALITY ignore	TYPE RemoteUELocalID				 					PRESENCE optional	}|

{ ID id-UuRLCChannelToBeSetupList				CRITICALITY reject	TYPE UuRLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelToBeModifiedList			CRITICALITY reject	TYPE UuRLCChannelToBeModifiedList					PRESENCE optional}|

{ ID id-UuRLCChannelToBeReleasedList			CRITICALITY reject	TYPE UuRLCChannelToBeReleasedList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeSetupList				CRITICALITY reject	TYPE PC5RLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-PC5RLCChannelToBeModifiedList			CRITICALITY reject	TYPE PC5RLCChannelToBeModifiedList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeReleasedList			CRITICALITY reject	TYPE PC5RLCChannelToBeReleasedList					PRESENCE optional}|

{ ID id-PathSwitchConfiguration					CRITICALITY ignore	TYPE PathSwitchConfiguration				 			PRESENCE optional	}|

{ ID id-GNBDUUESliceMaximumBitRateList			CRITICALITY ignore	TYPE GNBDUUESliceMaximumBitRateList				PRESENCE optional }|

{ ID id-MulticastMBSSessionSetupList			CRITICALITY reject	TYPE MulticastMBSSessionList							PRESENCE optional }|

{ ID id-MulticastMBSSessionRemoveList			CRITICALITY reject	TYPE MulticastMBSSessionList							PRESENCE optional }|

{ ID id-UE-MulticastMRBs-ToBeSetup-List			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-List				PRESENCE optional	}|

{ ID id-UE-MulticastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeReleased-List			PRESENCE optional	}|

{ ID id-SLDRXCycleList							CRITICALITY ignore  TYPE SLDRXCycleList 										PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNModificationList	CRITICALITY ignore	TYPE MDTPLMNModificationList							PRESENCE optional }|

{ ID id-SDTBearerConfigurationQueryIndication	CRITICALITY ignore	TYPE SDTBearerConfigurationQueryIndication		PRESENCE optional }|

{ ID id-TargetCellInformationDAPS				CRITICALITY ignore	TYPE TargetCellInformationDAPS						PRESENCE optional	},

...

}

<Unchanged Text Omitted>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<Unchanged Text Omitted>

id-NeedForGapNCSGInfoEUTRA,

id-TRPTxTimingErrorMargin,

id-TRPRxTimingErrorMargin,

id-TRPRxTxTimingErrorMargin,

id-Source-MRB-ID,

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS1,

id-ConfigRestrictInfoDAPS2,

maxNRARFCN,

maxnoofErrors,

<Unchanged Text Omitted>

DUtoCURRCInformation ::= SEQUENCE {

cellGroupConfig		CellGroupConfig,

measGapConfig			MeasGapConfig	OPTIONAL,

requestedP-MaxFR1				OCTET STRING				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { DUtoCURRCInformation-ExtIEs} } OPTIONAL,

...

}

DUtoCURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DRX-LongCycleStartOffset			CRITICALITY ignore	EXTENSION DRX-LongCycleStartOffset				PRESENCE optional }|

{ ID id-SelectedBandCombinationIndex		CRITICALITY ignore	EXTENSION SelectedBandCombinationIndex			PRESENCE optional }|

{ ID id-SelectedFeatureSetEntryIndex		CRITICALITY ignore	EXTENSION SelectedFeatureSetEntryIndex			PRESENCE optional }|

{ ID id-Ph-InfoSCG							CRITICALITY ignore	EXTENSION Ph-InfoSCG								PRESENCE optional }|

{ ID id-RequestedBandCombinationIndex		CRITICALITY ignore	EXTENSION RequestedBandCombinationIndex		PRESENCE optional }|

{ ID id-RequestedFeatureSetEntryIndex		CRITICALITY ignore	EXTENSION RequestedFeatureSetEntryIndex		PRESENCE optional }|

{ ID id-DRX-Config							CRITICALITY ignore	EXTENSION DRX-Config								PRESENCE optional }|

{ ID id-PDCCH-BlindDetectionSCG				CRITICALITY ignore	EXTENSION PDCCH-BlindDetectionSCG				PRESENCE optional }|

{ ID id-Requested-PDCCH-BlindDetectionSCG	CRITICALITY ignore	EXTENSION Requested-PDCCH-BlindDetectionSCG	PRESENCE optional }|

{ ID id-Ph-InfoMCG							CRITICALITY ignore	EXTENSION Ph-InfoMCG								PRESENCE optional }|

{ ID id-MeasGapSharingConfig				CRITICALITY ignore	EXTENSION MeasGapSharingConfig					PRESENCE optional }|

{ ID id-SL-PHY-MAC-RLC-Config				CRITICALITY ignore	EXTENSION SL-PHY-MAC-RLC-Config					PRESENCE optional }|

{ ID id-SL-ConfigDedicatedEUTRA-Info		CRITICALITY ignore	EXTENSION SL-ConfigDedicatedEUTRA-Info			PRESENCE optional }|

{ ID id-RequestedP-MaxFR2					CRITICALITY ignore	EXTENSION RequestedP-MaxFR2							PRESENCE optional }|

{ ID id-SDT-MAC-PHY-CG-Config				CRITICALITY ignore	EXTENSION SDT-MAC-PHY-CG-Config							PRESENCE optional }|

{ ID id-MUSIM-GapConfig						CRITICALITY ignore	EXTENSION MUSIM-GapConfig							PRESENCE optional }|

{ ID id-SL-RLC-ChannelToAddModList			CRITICALITY ignore	EXTENSION SL-RLC-ChannelToAddModList			PRESENCE optional }|

{ ID id-InterFrequencyConfig-NoGap			CRITICALITY ignore	EXTENSION InterFrequencyConfig-NoGap			PRESENCE optional }|

{ ID id-UL-GapFR2-Config					CRITICALITY ignore	EXTENSION UL-GapFR2-Config							PRESENCE optional }|

{ ID id-ConfigRestrictInfoDAPS1				CRITICALITY ignore	EXTENSION ConfigRestrictInfoDAPS					PRESENCE optional }|

{ ID id-ConfigRestrictInfoDAPS2				CRITICALITY ignore	EXTENSION ConfigRestrictInfoDAPS					PRESENCE optional },

...

}

<Unchanged Text Omitted>

ConditionalIntraDUMobilityInformation ::= SEQUENCE {

cho-trigger						CHOtrigger-IntraDU,

targetCellsTocancel				TargetCellList								OPTIONAL,

-- This IE may be present if the cho-trigger IE is present and set to "cho-cancel"

iE-Extensions					ProtocolExtensionContainer { { ConditionalIntraDUMobilityInformation-ExtIEs} }	OPTIONAL,

...

}

ConditionalIntraDUMobilityInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

{ ID id-EstimatedArrivalProbability		CRITICALITY ignore		EXTENSION CHO-Probability		PRESENCE optional	},

...

}

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

ConfigRestrictInfoDAPS ::= OCTET STRING

CoordinateID ::= INTEGER (0..511, ...)

<Unchanged Text Omitted>

#### -- T

FiveGS-TAC ::= OCTET STRING (SIZE(3))

Configured-EPS-TAC ::= OCTET STRING (SIZE(2))

TargetCellInformationDAPS ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

nR-Mode-Info					NR-Mode-Info,

iE-Extensions					ProtocolExtensionContainer { { TargetCellInformationDAPS-ExtIEs} } OPTIONAL,

...

}

TargetCellInformationDAPS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

TargetCellList-Item ::= SEQUENCE {

target-cell								NRCGI,

iE-Extensions							ProtocolExtensionContainer { { TargetCellList-Item-ExtIEs} } OPTIONAL

}

TargetCellList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TDD-Info ::= SEQUENCE {

nRFreqInfo							NRFreqInfo,

transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { {TDD-Info-ExtIEs} } OPTIONAL,

...

}

TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-IntendedTDD-DL-ULConfig	CRITICALITY ignore	EXTENSION	IntendedTDD-DL-ULConfig	PRESENCE optional}|

{ID id-TDD-UL-DLConfigCommonNR	CRITICALITY ignore	EXTENSION TDD-UL-DLConfigCommonNR	PRESENCE optional }|

{ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList				PRESENCE optional },

...

}

<Unchanged Text Omitted>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-PosMeasurementPeriodicityNR-AoA					ProtocolIE-ID ::= 672

id-RedCapIndication									ProtocolIE-ID ::= 673

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS1							ProtocolIE-ID ::= aaa

id-ConfigRestrictInfoDAPS2							ProtocolIE-ID ::= bbb

id-TargetCellInformationDAPS						ProtocolIE-ID ::= ccc

<Unchanged Text Omitted>

Change Ends