3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225772

Electronic Meeting, October 10th – 18th, 2022

Agenda item: 		10.2.1

Source:	Intel Corporation

Title:	SHR for NR to LTE HO

Document for:		Discussion and Decision

# Introduction

In RAN3#117-e meeting, the following agreement is achieved for SHR for intra-system inter-RAT handover [1].

SHR for intra-system inter-RAT, HO from NR to LTE will be treated first

This contribution makes an analysis on the SHR issue for NR to LTE HO case using intra-NR HO solution as baseline.

# Discussion

Successful handover report (SHR) is generated in case of failure events during HO (normal HO, CHO, DAPS). For intra-NR HO, the following conditions are used for a UE to trigger generating SHR, which can be reused for NR to LTE HO case.

- Upon exceeding thresholds on T310 (configured by the source cell)

- Upon exceeding thresholds on T312 (configured by the source cell)

- Upon exceeding thresholds on T304 (configured by the target cell)

Proposal 1: The UE logs the SHR if, during NR to LTE HO, T310 or T312 or T304 value exceeds a threshold.

For intra-NR HO, as shown below [2], the source cell and target cell related identifiers and measurements are included in the SHR.

SuccessHO-Report-r17 ::=                 SEQUENCE {

sourceCellInfo-r17                       SEQUENCE {

sourcePCellId-r17                        CGI-Info-Logging-r16,

sourceCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL,

rlf-InSourceDAPS-r17                     ENUMERATED {true}                               OPTIONAL

},

targetCellInfo-r17                       SEQUENCE {

targetPCellId-r17                        CGI-Info-Logging-r16,

targetCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r17                 SEQUENCE {

measResultListNR-r17                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r17                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

locationInfo-r17                         LocationInfo-r16                                    OPTIONAL,

timeSinceCHO-Reconfig-r17                TimeSinceCHO-Reconfig-r17                           OPTIONAL,

shr-Cause-r17                            SHR-Cause-r17                                       OPTIONAL,

ra-InformationCommon-r17                 RA-InformationCommon-r16                            OPTIONAL,

upInterruptionTimeAtHO-r17               UPInterruptionTimeAtHO-r17                          OPTIONAL,

c-RNTI-r17                               RNTI-Value                                          OPTIONAL,

...

}

Since, for NR to LTE HO case, the existing parameters in SuccessHO-Report IE are sufficient to support basic SHR functionality, the existing SuccessHO-Report IE can be reused. But this proposal still needs further RAN2 decision.

Proposal 2: The existing SuccessHO-Report IE can be reused for NR to LTE HO case.

Similar as RLF report, the target LTE node receiving the SHR should transfer it to the source NG-RAN node by triggering the DOWNLINK RAN CONFIGURATION TRANSFER procedure over NG or ACCESS AND MOBILITY INDICATION procedure over Xn.

Proposal 3: The target LTE node transfers the receiving SHR to the source NG-RAN node by triggering the DOWNLINK RAN CONFIGURATION TRANSFER procedure over NG or ACCESS AND MOBILITY INDICATION procedure over Xn.

# Conclusion

In this contribution we analyzed the SHR for NR to LTE HO case, and the following proposals were drawn from the analysis.

Proposal 1: The UE logs the SHR if, during NR to LTE HO, T310 or T312 or T304 value exceeds a threshold.

Proposal 2: The existing SuccessHO-Report IE can be reused for NR to LTE HO case.

Proposal 3: The target LTE node transfers the receiving SHR to the source NG-RAN node by triggering the DOWNLINK RAN CONFIGURATION TRANSFER procedure over NG or ACCESS AND MOBILITY INDICATION procedure over Xn.

# References

- RAN3_117-e_agenda_20220825_EOM1

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification