3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225780

Electronic Meeting, October 10th – 18th, 2022

Agenda item:		12.2.2.1

Source:	Intel Corporation

Title:	Discussion on UE Performance and System KPI (including TP for TS 38.423)

Document for:		Discussion and Decision

# Introduction

During Rel-17, AI/ML based network energy saving, load balancing and mobility optimization were studied in [1], in which several potential solutions and input/output/feedback information are summarized. In this contribution, we mainly focus on resource status request/response/update enhancement for UE performance of handed-over UE(s) and system KPI.

# Discussion

### UE performance

Similar as other measurement objects, e.g. PRB periodic, TNL capability and Periodic, etc. in Resource Status Request message, the UE performance can be treated as a new measurement object. A new bit can be introduced as one of "Report Characteristics" to request the performance of the handed-over UE from the neighbouring NG-RAN node.

Proposal 1: Introduce a new bit within the Report Characteristics of Resource Status Request message to request the performance of requested UE(s).

It is also noted that AI/ML based load balancing and mobility optimization use cases further require the UE performance measurement and/or QoS parameters of the handed-over UE from the offloaded neighbouring cell, as "input" and "feedback" for Model Training and Model Inference.

TR 37.817 [1] Section 5.2.2.4 Input of AI/ML based Load Balancing From neighbouring NG-RAN Nodes: - Current and predicted resource status - UE performance measurement at traffic offloaded neighbouring cell TR 37.817 [1] Section 5.2.2.6 Feedback of AI/ML-based Load Balancing To optimize the performance of AI/ML-based load balancing model, following feedback can be considered to be collected from NG-RAN nodes: - UE performance information from target NG-RAN (for those UEs handed over from the source NG-RAN node) - Resource status information updates from target NG-RAN - System KPIs (e.g., throughput, delay, RLF of current and neighbours) TR 37.817 [1] Section 5.3.2.4 Input of AI/ML based Mobility Optimization From the neighbouring RAN nodes: - UE’s history information from neighbour - Position, QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.) - Current/predicted resource status - UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism. TR 37.817 [1] Section 5.3.2.6 Feedback of AI/ML-based Mobility Optimization The following data is required as feedback data for mobility optimization. - QoS parameters such as throughput, packet delay of the handed-over UE, etc. - Resource status information updates from target NG-RAN. - Performance information from target NG-RAN. The details of performance information are to be discussed during normative work phase.

Note that feedback information is basically for "performance feedback" of the past inferencing and thus one may think this should not be used as an input for the next inferencing. However, given we already agreed that they can be provided as feedback and the same info was agreed to be used as an input, there is no reason not to use them for the next inferencing, and we think feedback is indeed useful especially for model tuning purpose.

Proposal 2: For AI/ML based load balancing and mobility optimization, AI/ML Model Training and Model Inference re-use UE performance feedback information from the previous iteration as input for Model Inference in the next iteration.

Additionally, as discussed in the companion contribution [2], the XnAP ID of the UE(s) impacted by handover strategy should be provided by the source NG-RAN node to the target NG-RAN node, so that the corresponding performance can be collected and reported. Therefore, the Resource Status Request message should also introduce a new field “UE to Report List” under “Cell To Report Item”, so that the XnAP ID(s) at source NG-RAN node of the handed-over UE(s) can be included in this “UE to Report List” in the Resource Status Request message.

Proposal 3: Add a new IE “UE to Report List” under the “Cell To Report Item” in the Resource Status Request message to include the XnAP ID(s) of the impacted UE(s) by the AI/ML based network energy saving strategy.

As for the UE performance at the target NG-RAN node, some metrics such as bitrate, packet loss and latency have been captured as examples in TR 37.817 [1]. First of all, based on our understanding, similar as other L2 measurements, the UE performance should be measured and collected at the network side, rather than being collected from the UE. There’s no need to impact air interface to let UE report new information as UE performance indicator.

Observation 1: UE performance is collected at the network side without impacting air interface.

Latency and packet loss defined in [3] as L2 measurement are collected by NG-RAN node, indicating per UE performance. Therefore, such information can be further propagated to the neighbouring NG-RAN node over Xn interface as a feedback of UE performance. The Resource Status Request message can be used to carry the related information, e.g. delay, RLC packet delay, PDCP packet delay, etc. Moreover, based on the UE throughput calculation defined in [4] and section 4.2.1.6 in [3], an individual UE throughput can also be collected by NG-RAN node, where the UE data volume refers to the total volume scheduled for the UE.

The granularity for PDCP SDU Data Volume measurement defined in TS 28.552 [4] is per DRB per UE. The granularity for Average UE throughput measurement defined in TS 28.552 [4] is per UE and per DRB per UE.

Proposal 4: The Resource Status Update message includes the following information when “UE performance” is being requested in the Resource Status Request message:

- XnAP ID of the handed-over UE at source/target NG-RAN node

- Average over-the-air interface packet delay in the UL per DRB

- Average RLC packet delay in the UL per DRB

- Average PDCP packet re-ordering delay in the UL per DRB

- Packet Uu loss rate in the DL per DRB

- DL PDCP SDU Data Volume per DRB

- UL PDCP SDU Data Volume per DRB

- DL UE throughput in gNB

- UL UE throughput in gNB

Additionally, Average over-the air interface packet delay in the UL per DRB/Average RLC packet delay in UL per DRB/Packet Uu loss rate in the DL per DRB are collected at MAC/RLC, respectively [3]. The F1 interface should also be enhanced to report such information to CU.

Proposal 5: Enhance F1AP to support the following information collection from DU to CU:

- Average over-the-air interface packet delay in the UL per DRB

- Average RLC packet delay in the UL per DRB

- Packet Uu loss rate in the DL per DRB

### System KPI

As captured in [1], system KPI may include throughput, delay, RLF of the current and neighbouring NG-RAN nodes. Performance measurements for gNB are defined in TS 28.552 Section 5.1 [4], where NG-RAN can measure and collect its performance and report to upper layer. To receive system KPIs from the neighbouring NG-RAN nodes, those information (e.g. packet delay, radio resource utilization, UE throughput, RRC connection number, PDU session management, Mobility management, etc.) needs to be exchanged over Xn interface when system KPI is required by the Resource Status Request message.

Similarly for the UE performance discussed above, system KPI can also be treated as a new measurement object in resource status request/response/update procedure.

Proposal 6: Introduce a new bit within of the Report Characteristics of Resource Status Request message to request the system KPI from the target NG-RAN node, where the following performance measurements for gNB defined in TS 28.552 [4] Section 5.1 are included in the corresponding Resource Status Update message as system KPI measurement report for throughput, delay and RLF:

- Packet delay in TS 28.552 [4] Section 5.1.1.1

- UE throughput in TS 28.552 [4] Section 5.1.1.3

- Mobility Management in TS 28.552 [4] Section 5.1.1.6

- Measurements related to MRO in TS 28.552 [4] Section 5.1.1.25

- PDCP Data Volume in TS 28.552 [4] Section 5.1.2.1

- Packet loss rate, packet drop rate, packet delay in TS 28.552 [4] Section 5.1.3

# Conclusion

In this contribution, we discussed resource status request/response/update enhancement for UE performance of handed-over UE(s) and system KPI.

We propose the following observations and proposals:

Observation 1: UE performance is collected at the network side without impacting air interface.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Proposal 1: Introduce a new bit within the Report Characteristics of Resource Status Request message to request the performance of requested UE(s).

Proposal 2: For AI/ML based load balancing and mobility optimization, AI/ML Model Training and Model Inference re-use UE performance feedback information from the previous iteration as input for Model Inference in the next iteration.

Proposal 3: Add a new IE “UE to Report List” under the “Cell To Report Item” in the Resource Status Request message to include the XnAP ID(s) of the impacted UE(s) by the AI/ML based network energy saving strategy.

Proposal 4: The Resource Status Update message includes the following information when “UE performance” is being requested in the Resource Status Request message:

- XnAP ID of the handed-over UE at source/target NG-RAN node

- Average over-the-air interface packet delay in the UL per DRB

- Average RLC packet delay in the UL per DRB

- Average PDCP packet re-ordering delay in the UL per DRB

- Packet Uu loss rate in the DL per DRB

- DL PDCP SDU Data Volume per DRB

- UL PDCP SDU Data Volume per DRB

- DL UE throughput in gNB

- UL UE throughput in gNB

Proposal 5: Enhance F1AP to support the following information collection from DU to CU:

- Average over-the-air interface packet delay in the UL per DRB

- Average RLC packet delay in the UL per DRB

- Packet Uu loss rate in the DL per DRB

Proposal 6: Introduce a new bit within of the Report Characteristics of Resource Status Request message to request the system KPI from the target NG-RAN node, where the following performance measurements for gNB defined in TS 28.552 [4] Section 5.1 are included in the corresponding Resource Status Update message as system KPI measurement report for throughput, delay and RLF:

- Packet delay in TS 28.552 [4] Section 5.1.1.1

- UE throughput in TS 28.552 [4] Section 5.1.1.3

- Mobility Management in TS 28.552 [4] Section 5.1.1.6

- Measurements related to MRO in TS 28.552 [4] Section 5.1.1.25

- PDCP Data Volume in TS 28.552 [4] Section 5.1.2.1

- Packet loss rate, packet drop rate, packet delay in TS 28.552 [4] Section 5.1.3

The corresponding text proposal for TS 38.423 is provided in Annex.

# References

- TR 37.817, Study on enhancement for Data Collection for NR and EN-DC

- R3-225776, "Discussion on Stage 2 of AI/ML based Use Cases", Intel Corporation

- TS 38.314, Layer 2 measurements

- TS 28.552, 5G performance measurements

# Annex – Text Proposal for TS 38.423

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	NG-RAN Node and Cell Configuration related IE definitions

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.aa	UE Performance

The UE Performance IE indicates the packet delay, packet loss and throughput of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Performance List |  | 1 |  |  | YES | Ignore |
| >UE Performance Item |  | 1..<maxnoofUEContext> |  |  | – |  |
| >>Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| >>Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| >>Average Over-The-Air Interface Packet Delay | O |  | TBD | Per DRB UL average over-the-air interface packet delay as defined in TS 38.314 [42]. | – |  |
| >>Average RLC Packet Delay | O |  | TBD | Per DRB UL average RLC packet delay as defined in TS 38.314 [42]. | – |  |
| >>Average PDCP Packet Re-ordering Delay | O |  | TBD | Per DRB UL PDCP packet re-ordering delay as defined in TS 38.314 [42]. | – |  |
| >>Packet Uu Loss Rate | O |  | TBD | Per DRB DL packet Uu loss rate as defined in TS 38.314 [42]. | – |  |
| >>DL PDCP SDU Data Volume | O |  | TBD | Per DRB DL PDCP SDU Data volume as defined in TS 38.314 [42]. | – |  |
| >>UL PDCP SDU Data Volume | O |  | TBD | Per DRB UL PDCP SDU Data volume as defined in TS 38.314 [42]. | – |  |
| >>DL UE Throughput | O |  | TBD | DL UE throughput as defined in TS 38.314 [42]. | – |  |
| >>UL UE Throughput | O |  | TBD | UL UE throughput as defined in TS 38.314 [42]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEContexts | Maximum no. of UE Contexts. Value is 8192. |

#### 9.2.2.bb	System KPI

The System KPI IE indicates the throughput, delay, RLF event status of the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE System KPI Type | M |  |  |  |  |  |
| >Packet delay |  |  |  |  |  |  |
| >>Average Delay DL Air-Interface | O |  | TBD | This measurement provides the average (arithmetic mean) time that takes for packet transmission over air interface in the downlink direction. |  |  |
| >>Distribution of Delay DL Air Interface | O |  | TBD | This measurement provides the distribution of the time that takes for packet transmission over air-interface in the downlink direction. |  |  |
| >>Average Delay UL Over-The-Air-Interface | O |  | TBD | This measurement provides the average (arithmetic mean) over-the-air packet delay on the uplink. |  |  |
| >>Average RLC Packet Delay in the UL | O |  | TBD | This measurement provides the average (arithmetic mean) RLC packet delay on the uplink, i.e. the delay within the gNB-DU. |  |  |
| >>Average PDCP Re-ordering Delay in the UL | O |  | TBD | This measurement provides the average (arithmetic mean) PDCP re-ordering delay on the uplink, i.e. the delay within the gNB-CU-UP. |  |  |
| >>Distribution of DL Delay between NG-RAN and UE | O |  | TBD | This measurement provides the distribution of DL packet delay between NG-RAN and UE, which is the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface. |  |  |
| >>Distribution of UL Delay between NG-RAN and UE | O |  | TBD | This measurement provides the distribution of UL packet delay between NG-RAN and UE, which is the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface. |  |  |
| >UE throughput |  |  |  |  |  |  |
| >>Average DL UE Throughput | O |  | TBD | This measurement provides the average UE throughput in downlink. |  |  |
| >>Distribution of DL UE Throughput | O |  | TBD | This measurement provides the distribution of the UE throughput in downlink. |  |  |
| >>Average UL UE Throughput | O |  | TBD | This measurement provides the average UE throughput in uplink. |  |  |
| >>Distribution of UL UE Throughput | O |  | TBD | This measurement provides the distribution of the UE throughput in uplink. |  |  |
| >>Percentage of Unrestricted DL UE Data Volume | O |  | TBD | This measurement provides the percentage of DL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. |  |  |
| >>Percentage of Unrestricted UL UE Data Volume | O |  | TBD | This measurement provides the percentage of UL data volume for UEs in the cell that is classified as unrestricted, i.e., when the volume is so low that all data can be transferred in one slot and no UE throughput sample could be calculated. |  |  |
| >Measurement related to MRO |  |  |  |  |  |  |
| >>Handover Failures related to MRO for Intra-System Mobility | O |  | TBD | This measurement provides the number of handover failure events related to MRO detected during the intra-system mobility within 5GS, see TS 38.300 [49] clause 15.5.2. |  |  |
| >>Handover Failures related to MRO for Inter-System Mobility | O |  | TBD | This measurement provides the number of handover failure events delated to MRO detected during the inter-system mobility from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. |  |  |
| >>Unnecessary Handovers for Inter-System Mobility | O |  | TBD | This measurement provides the number of unnecessary handover events detected during the inter-system mobility from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. |  |  |
| >>Handover Ping-Pong for Inter-System Mobility | O |  | TBD | This measurement provides the number of handover ping-pong events detected during the inter-system mobility from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. |  |  |
| >>Handover Failures for Beam-Cell Pair related to MRO for Intra-System Mobility | O |  | TBD | This measurement provides the number of handover failure events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) related to MRO detected during the intra-system mobility within 5GS. |  |  |
| >PDCP Data Volume for non-split gNB |  |  |  |  |  |  |
| >>DL PDCP SDU Data Volume Measurements | O |  | TBD |  |  |  |
| >>UL PDCP SDU Data Volume Measurements | O |  | TBD |  |  |  |
| >Performance measurement for split gNB |  |  |  |  |  |  |
| >>UL PDCP SDU Loss Rate | O |  | TBD |  |  |  |
| >>UL F1-U Packet Loss Rate | O |  | TBD |  |  |  |
| >>DL F1-U Packet Loss Rate | O |  | TBD |  |  |  |
| >>DL PDCP SDU Drop Rate in gNB-CU-UP | O |  | TBD |  |  |  |
| >>DL Packet Drop Rate in gNB-DU | O |  | TBD |  |  |  |
| >>Average Delay DL in CU-UP | O |  | TBD |  |  |  |
| >>Average Delay DL on F1-U | O |  | TBD |  |  |  |
| >>Average Delay DL in gNB-DU | O |  | TBD |  |  |  |
| >>Distribution of Delay DL in CU-UP | O |  | TBD |  |  |  |
| >>Distribution of Delay DL on F1-U | O |  | TBD |  |  |  |
| >>Distribution of Delay DL in gNB-DU | O |  | TBD |  |  |  |