3GPP TSG-RAN WG3 Meeting #117-bis-e	R3-225829

# E-meeting, 10 Oct - 18 Oct 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0924 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for TS 38.423 on UHI in MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Samsung, Lenovo, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-9-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A correlation error will be caused if the PSCell dwelling time exceeds the upper bound value. This issue can be solved by allowing the SN to add new cell entries with the same PSCell ID when the PSCell dwelling time exceeds the upper bound value. However, there are two main drawback for this solution. The first one is that the correlation error still exists if the actual PSCell dwelling time exceeds 32768s (8×4096s). Secondly, if the UE is staying in the same PSCell for a long time, repeating PSCell entries would lead to reduced unique cell entries which further leads to potential loss of information. Considering the above two drawbacks, it is necessary to extend the PSCell numbers from 8 to 16 to support this solution. Thus, longer PSCell dwelling time can be covered and more unique PSCell entries are kept to help the target NG-RAN node to make the right decision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extend the value of maxnoofPSCellsPerSN from 8 to 16. Impact assessment towards the previous version of the specification (same release): This CR has limited impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The MN cannot accurately correlate MN and SN UHI if the actual PSCell dwelling time exceeds 32768s (8×4096s). Furthermore, reduced unique cell entries can lead to potential loss of information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.151, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.423 CR 1721 TS 38.413 CR 0897 TS 36.413 CR 1902 TS 37.340 CR draft |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------Start of the change---------------------------------------------------

#### 9.2.3.151	SCG UE History Information

This IE contains information about the PSCells served by the secondary node in an active state.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerSN> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. |
| >Last Visited PSCell Information | M |  | OCTET STRING | Defined in TS 38.413 [5] |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerSN | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 16. |

*** Start of ASN.1 change ***

-----------------------------------------------------------Next change-----------------------------------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-------- skip unchanged part ----------

maxnoofWLANName									INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofMBSFSAs										INTEGER ::= 256

maxnoofUEIDIndicesforMBSPaging				INTEGER ::= 4096

maxnoofMBSQoSFlows								INTEGER ::= 64

maxnoofMRBs											INTEGER ::= 32

maxnoofCellsforMBS								INTEGER ::= 8192

maxnoofMBSServiceAreaInformation				INTEGER ::= 256

maxnoofTAIforMBS 									INTEGER ::= 1024

maxnoofAssociatedMBSSessions					INTEGER ::= 32

maxnoofMBSSessions								INTEGER ::= 256

maxnoofSuccessfulHOReports						INTEGER ::= 64

maxnoofPSCellsPerSN 								INTEGER ::= 16

maxnoofNR-UChannelIDs							INTEGER ::= 16

maxnoofCellsinCHO									INTEGER ::= 8

maxnoofCHOexecutioncond							INTEGER ::= 2

maxnoofServedCellsIAB							INTEGER ::= 512

maxnoofServingCells								INTEGER ::= 32

maxnoofBHInfo										INTEGER ::= 1024

maxnoofTrafficIndexEntries						INTEGER	::=	1024

maxnoofTLAsIAB										INTEGER	::=	1024

maxnoofBAPControlPDURLCCHs						INTEGER ::= 2

maxnoofIABSTCInfo									INTEGER ::= 45

-------- skip unchanged part ----------

-----------------------------------------------------End of the change-------------------------------------------------------