3GPP TSG-RAN WG3#117bis-e                                             	                     R3-225944

E-Meeting, 10– 18 October 2022

Agenda item:	9.2.5

Source:	Nokia, Nokia Shanghai Bell

Title: 	Response to R3-225444

Document for:	Approval

# Introduction

Tdoc R3-225444 is proposing new call flows for TS 38.401 which makes challenging assumptions on the way resources are allocated and UE-associated/non-UE associated procedures are used. Also, we think that TS 38.401 should:

- Either contain a generic call flow with many optinality as in R3-225652 (a)

- Or present the nominal call flows i.e. the ones which are typically used (b).

Our view is that the call flows presented in R3-225444 are not representative of nominal call flows and therefore this paper presents an alternative proposal of what are the nominal call flows for multicast context establishment.

It is proposed to agree (a) or the below proposal if (b) is chosen.

# Discussion

3GPP TSG-RAN WG3 #117bis-e	R3-22xxxx

# Online, 10th - 18th October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR |  | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the example message flow for multicast MBS Context establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The example message flow for multicast MBS Context establishment is not precise and not representative of the nominal call flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The example message flow for multicast MBS Context establishment was devided into three scenarios: a) a first UE joining an inactive multicast MBS session as the first UE in a gNB b) multicast session activation c) MRB type configuration change for a UE and establishment of an F1-U ptp retransmission tunnel with the possibility for the gNB-CU-CP to establish and leave established gNB-DU multicast MBS session contexts during deactivated multicast MBS session context and F1-U resources.. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it restructures stage 2 specification of the multicast MBS function in NG-RAN. The impact can be considered isolated because the change affects only the stage 2 specification of the multicast MBS function in NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 specification of the multicast MBS function in NG-RAN remains not precise and not representative of the nominal call flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.15.1.2	Multicast MBS Session Context Establishment

Figures 8.15.1.2-1, 8.15.1.2-2 and 8.15.1.2-3 illustrate an examplified interaction of NGAP, E1AP, F1AP and RRC protocol functions for Multicast MBS Session Context Establishment.

A UE joins an inactive multicast MBS Session as first UE in a gNB:

Figure 8.15.1.2-1: Multicast MBS Session Context establishment - 
a UE joins an inactive multicast MBS Session as first UE in a gNB

0.	For a certain multicast MBS session (TMGI), which is currently inactive, no multicast MBS session context is yet established in the gNB.

1.	The gNB(-CU-CP) receives for a UE within the NGAP PDU Session Modification Request message the information that the UE has joined a multicast MBS session. A multicast session context is established by the 5GC in the gNB.

4/5. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL/UL GTP TEIDs are exchanged, for multicast transport, multicast address information is fetched from the 5GC.

6/7. The NG-U UP entity is established by means of the E1AP MC Bearer Context Modification procedure, providing the 5GC NG-U TNL address to the gNB-CU-UP,.

8.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

9.	The gNB successfully terminates the NGAP procedure for establishing the multicast session context.

On NG-U, in case of location dependent multicast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU- CP may request the gNB-CU-UP to either apply the available MRB configuration of the shared NG-U termination, or to apply the MRB configuration requested by the gNB-CU-CP. The gNB-CU-UP provides the MRB configuration to the gNB-CU-CP if the MRB configuration requested by the gNB-CU-CP and the available MRB configuration of the shared NG-U termination are different.

The multicast MBS session is activated:

Figure 8.15.1.2-2: Multicast MBS Session Context establishment - 
Activation of the multicast MBS session

1.	The multicast MBS session is activated by the 5GC.

2./7. Depending on the number of UEs having joined, the gNB-CU-CP decides to configure the PTM resources in the cell and initiates the F1AP Multicast Context Setup Request procedure. The gNB-DU triggers the the establishment of F1-U tunnel resources in steps 4-7 by means of the F1AP Multicast Distribution Setup procedure.

8.-9. The gNB-CU CP initates for each UE the F1AP UE Context Modidication Request message informing that the UE joined and the gNB-DU selects the MRB bearer type and includes the corresponding lower layer configuration (DU to CU RRC Information) in the F1AP UE Context Modification Response.

10.	The gNB-CU-CP configures the UE with MRB resources.

11.	The multicast MBS media stream is provided to the UEs.

Change of the MRB type configuration with F1-U ptp retransmission tunnel establishment:

Figure 8.15.1.2-3: Multicast MBS Session Context establishment - 
change of the MRB type configuration with F1-U ptp retransmission tunnel establishment

1.	The gNB-DU decides to change the MRB type configuration for a UE and provides the new lower layer configuration to the gNB-CU-CP along with a per-MRB MRB type reconfiguration indication and the MRB Reconfigured RLC mode.

2.	The gNB-CU-CP confirms the modification request and indicates this to the gNB-DU. The gNB-CU-CP may request the gNB-DU to establish F1-U ptp retransmission resources in order for the UE to perform PDCP Status Report and receive retransmitted PDCP PDUs for the reconfigured MRBs.

4. - 7.	If necessary, the gNB-DU requests the establishment of F1-U resources for ptp retransmission by means of triggering the F1AP Multicast Distribution Setup procedure, which in turn triggers the E1AP MC Bearer Context Modification procedure to exchange F1-U tunnel end-point addresses.

8./9. The RRC Reconfiguration procedure is performed.

10.	PDCP Status Report and PDCP PDU retransmission can take place via the established F1-U ptp retransmission resources.