3GPP TSG-RAN WG3 Meeting #117bis-e	R3-226019

# E-meeting, 10 – 18 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1059 | rev | 1 | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on positioning SI delivery over F1AP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, CMCC, Ericsson, Nokia, Nokia Shanghai Bell, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2022-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.331, RRCSystemInfoRequest message can be used by UE to request SI messages configured by SI-SchedulingInfo IE or posSI-SchedulingInfo IE in SIB1. Upon receiving the request, the gNB-CU shall send the System Information Delivery message over F1AP to command the gNB-DU to broadcast the requested system information. For example, in this message, the SIType List IE can be used by the gNB-CU to provide the SI list of other SI to be broadcasted. The issue is that requesting the gNB-DU to broadcast the requested posSI configured in posSI-SchedulingInfo is not supported yet on F1AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new message Positioning System Information Delivery Command to support the broadcast of the requested posSI. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has a limited impact on the protocol as it adds an optional IE. This CR has functionality impact. This CR can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reguesting the gNB-DU to broadcast the required posSI configured in posSI-SchedulingInfo IE is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.13, 9.2.12, 9.3.1, 9.4.3, 9.4.4, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.470 CR 0106 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1, update based on comments in #117bis-e meeting. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

8.1	List of F1AP Elementary procedures

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Positioning System Information Delivery |  | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |  |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

8.13.x	Positioning System Information Delivery

8.13.x.1	General

The purpose of the Positioning System Information Delivery procedure is to command the gNB-DU to broadcast the requested positioning SI. The procedure uses non-UE associated signalling.

8.13.x.2	Successful Operation

POSITIONING SYSTEM INFORMATION

COMMAND

gNB

-

DU

gNB

-

CU

Figure 8.6.x.2-1: Positioning System Information Delivery procedure. Successful operation.

The gNB-CU initiates the procedure by sending a POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message to the gNB-DU.

Upon reception of the POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU shall broadcast the requested positioning SI, and delete the UE context corresponding to the Confirmed UE ID IE, if any.

Interactions with gNB-DU Configuration Update procedure:

Upon reception of POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU Configuration Update procedure may be performed, and as part of such procedure the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in GNB-DU CONFIGURATION UPDATE message for UEs that are unable to receive system information from broadcast.

8.13.x.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.12.x	POSITIONING SYSTEM INFORMATION DELIVERY COMMAND

This message is sent by the gNB-CU and is used to enable the gNB-DU to broadcast the requested positioning SI.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 | NR cell identifier | YES | reject |
| PosSIType List | M |  | 9.3.1.y |  | YES | reject |
| Confirmed UE ID | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.1.y	PosSIType List

This IE is used by gNB-CU to provide SI list of positioning SI for gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SI type item IEs |  | 1.. <maxnoofSITypes> |  |  |
| >SI Type | M |  | INTEGER (1..32, ...) | Indicates a certain SI type required to be broadcasted by the gNB-DU. The SI Type value of positioning SI starts from 1. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSITypes | Maximum no. of SI types, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

PosSystemInformationDeliveryCommand

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-PosSystemInformationDeliveryCommand

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

posSystemInformationDelivery,

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

posSystemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PosSystemInformationDeliveryCommand

PROCEDURE CODE			id-PosSystemInformationDeliveryCommand

CRITICALITY				ignore

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- System Information ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- System information Delivery Command

--

-- **************************************************************

SystemInformationDeliveryCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ SystemInformationDeliveryCommandIEs}},

...

}

SystemInformationDeliveryCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-SItype-List				CRITICALITY reject	TYPE SItype-List					PRESENCE mandatory	}|

{ ID id-ConfirmedUEID 			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- Positioning System information Delivery Command

--

-- **************************************************************

PosSystemInformationDeliveryCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PosSystemInformationDeliveryCommandIEs}},

...

}

PosSystemInformationDeliveryCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-PosSItypeList			CRITICALITY reject	TYPE SItype-List					PRESENCE mandatory	}|

{ ID id-ConfirmedUEID 			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	},

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-PosMeasurementPeriodicityExtended,

id-PosMeasurementPeriodicityNR-AoA,

id-PosSItypeList,

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

id-PositioningInformationUpdate				ProcedureCode ::= 56

id-ReferenceTimeInformationReport			ProcedureCode ::= 57

id-ReferenceTimeInformationReportingControl	ProcedureCode ::= 58

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= aa

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

id-LocationMeasurementInformation					ProtocolIE-ID ::= 440

id-InterFrequencyConfig-NoGap						ProtocolIE-ID ::= 651

id-NeedForGapsInfoNR								ProtocolIE-ID ::= 665

id-PosMeasurementPeriodicityNR-AoA					ProtocolIE-ID ::= 672

id-PosSItypeList									ProtocolIE-ID ::= bbb

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>